| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A021 |
| --- | --- |

| Title: | Coding tools using parametric representations to improve coding efficiency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park, Seungwook Park, and Byeongmoon Jeon |  |  |
|  | 221, Yangjae-Dong, Seocho-Gu, Seoul, Korea | Tel: Email: | jy.park@lge.com seungwook.park@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents two coding tools using parametric representations. The first tool is an adaptive warped reference (AWR) method to handle complex motions of the video sequence. The AWR method generates warped reference pictures that compensate complex motions between a recently decoded reference picture and a current picture to encode. The complex motions are modeled as a parametric image transformation function such as a homography transformation function. Then, the parameters of the transformation function are quantized and encoded in bit streams so that a decoder can warp the recently decoded reference picture in the same way as the encoder did. The second tool is a parametric adaptive interpolation filter (PAIF) method that is one of advanced AIFs. The PAIF represents an interpolation filter using a few parameters instead of many individual filter coefficients by approximating the interpolation filter with a parametric function. It is known that PAIF needs less bits than conventional AIFs to represent a filter and that its representation is closer to the optimal filter than those of conventional AIFs. In the proposed PAIF method, we used our parametric function and an offset value, which is computed for each reference frame

These tools were used to furher improve LG’s response model to the CfP. However, the evaluation of the LG’s response model was done without these tools because they were under development during the evaluation period.

# Introduction

Approximating a complex function with a parametric function provides a compact representation. In video coding problems, the compact representation is an important factor because it reduces the amount of data to be encoded. A good example is the adaptive interpolation filter (AIF) technique. Conventional AIF techniques represent a interpolation filter using a large number of indiviual coefficients whereas it can be represented by a few parameters if the interpolation filter is approximated with a parametric function.

In this document, we introduce two coding tools using paramteric representations: adaptive warped reference (AWR) and parametric adaptive interpolation filter (PAIF), which have been developed to further improve LG’s reponse model to CfP. Details of the two tools are explained in the following sections.

# Tool I (AWR)

Motion estimation and compensation are efficient tools to exploit and reduce the correlation between consecutive frames, so that it has been incorporated in the most video coding standards like MPEG-1, MPEG-2, MPEG-4 and H.264/AVC. However, these coding tools consider simple translational motion, which is not sufficient to express complex motions such as zooming, rotation, shearing and so on.

Recently, BTZMCP (block-matching translation and zoom motion compensated prediction) method was proposed [1], which covers zooming motion as well as translational motion within the conventional framework. Although this model showed good coding efficiency, it may increase the complexity of motion estimation step to find the best combination of motion vectors and zooming scale. In addition, it is difficult to describe complexmotions such as rotation and perspective distortion.

The proposed AWR method also takes the benefits of parametric representation of the complex motions as affine and polynomial motion models have taken. The parametric motion models are good for describing the global motions in efficient way. However, one obstable of the warping techniques is the increasing computational complexity needed to find the optimal motion paramters and the increasing memory complexity to save the warped reference picture. The proposed AWR method used feature point-based motion search algorithm to alleviate the computational complexity problem.

## Proposed Method

The proposed AWR algorithm generates a warped reference picture that compensates complex motions between a reference image and a current image to encode, where the complex motions are modeled as a parametric image transformation function. Previous studies about warping-based coding algorithm suffered from the increasing complexity required to find optimal warping parameters. The proposed algorithm uses an efficient method to find the warping parameters.

Computation of warping parameters

To find and compute a warping function that is appropriate for the pair of images, we used KLT feature tracker [2] in order to detect feature points in the current picture and track them in the reference picture resulting a lot of feature pairs. Because a pair of pictures can be composed of different kind of motions, the feature pairs are segmented into several groups so that the feature pairs in a group to be explained by one parametric function. We used the homography tranformation function, which is a non-linear mapping function that maps a point (x,y) to (x’,y’) as

x'y'1=Hxy1=h11h12h13h21h22h23h31h321xy1,                    (1)

where H is a 3x3 matrix that contains 8 parameters of the homography transformation matrix H.

The feature pairs are grouped by a segmentation algorithm based on a region growing approach [3]. Then, for each group of feature pairs, we computed the 8 parameters of the homography transformation function using normalized direct linear transform algorithm [4]. Fig. 1 shows an example of grouped feature pairs.

Fig. 1 Example of grouped feature pairs; five pairs are grouped into one group.

Next, we warped the reference picture according to the computed warping parameters for each group. For example, Fig. 2 shows (a) a reference picture, (b) a current picture, (c) a wapred reference picture corresponding to 0th group, and (d) another warped reference picture corresponding to 1st group. In this example, the first and second warped reference pictures explains two motions of the static background region, and the rotating trump card, respectively.

| - a reference picture | - a current picture |
| --- | --- |
| - the first warped reference picture (by 0th group) | - the second warped reference picture (by 1st group) |

Selection of best warped reference

Although we can generate multiple warped references to expain the complex motions in the video sequence, not all the warped reference pictures are useful nor the significant level of them are known yet. So, we made a criterion to estimate how much gains can be obtained from each warped refernece picture when compared to the first reference picture. Using the criterion, we choose the best warped reference picture and used the selected warped reference picture. The other warped reference pictures are ignored.

Encoding of the motion parameters

When the homography tranformation function is used as the parameteric motion model, the 8 floating point numbers should be encoded. However, the motion parameters are too sensitive to be quantized; small quantization errors make large differences to the warping results. In order to minimize the effect of the quantization errors and the size of side information (parameters) to encode, we took another representation of the homography transform function. We applied the homography tranformation function to 4 corner points of the current picture to obtain transformed coordinates of them. Then, we encoded the 4 motion vectors of the 4 corner points instead of the 8 parameters. This representation makes the warping results less sensitive to the quantization errors and has smaller range of values than using the 8 parameters of the homography tranformation function.

Reordering of the reference picture list

The warped reference pictures should be inserted to the reference picture list to be used in the motion estimation and motion compensation processes. To increase the selection ratio of warped reference pictures, we reorder the reference picture as follows. If n warped reference pictures are generated from i’th reference picture, then the n warped references are inserted right after the i’th reference picture. Fig. 3 shows an example of reordered reference picture list when the first (0th) referenence picture is used to generate one warped reference picture.

Fig. 3 Reordering of the reference picture list.

## Experimental Results

We implemented the AWR method on the LG’s response model and measured the performance of AWR under the same set of conditions used for the evaluation of LG’s response model (IbBbBbBbP).

Table 1 show the experimental results, where we uses only one warped reference picture in addition to original reference pictures. To calculate average gain, we used BDrate computation tool [5]. Maximum gain among all test sequences was 7.08% at the Cactus sequence, where the RD curve of Cactus is shown in Fig. 4. The average gains for the three classes B, C, and D were 1.31%, -0.12%, -0.14%. The reason of high gain at the Cactus sequence was that the sequence contains in-plane rotating motion which could be efficiently compensated by the proposed AWR. On the other hand, there was no evident loss at any sequence when the proposed method was applied.

Another good test sequence for AWR was Jets (1280x720p, 60Hz) test sequence, which is composed of zooming out and panning camera motion. For the Jets test sequence, AWR showed -8.22% BD bitrate reduction, where the RD curve is shown in Fig. 5.

Table 1. BD rate (%) reduction of AWR (anchor is LG’s response model).

| Class | seq. | BD-rate(%) |
| --- | --- | --- |
| B | BasketballDrive | -0.09 |
|  | BQTerrace | 0.53 |
|  | Cactus | -7.08 |
|  | Kimono | 0.05 |
|  | ParkScene | 0.05 |
| Class B Avg. |  | -1.31 |
| C | BasketballDrill | -0.10 |
|  | BQMall | 0.06 |
|  | PartyScene | -0.14 |
|  | RaceHorses | -0.29 |
| Class C Avg. |  | -0.12 |
| D | BasketballPass | -0.15 |
|  | BlowingBubbles | -0.09 |
|  | BQSquare | -0.17 |
|  | RaceHorses | -0.14 |
| Class D Avg. |  | -0.14 |

Fig. 4 R-D curve of Cactus test sequence

Fig. 5 R-D curve of Jets test sequence.

# Tool II (PAIF)

In the KTA model, adaptive interpolation filter (AIF) scheme is included to reduce the energy of prediction error [6]. Due to the characteristic of the AIF, this scheme adds a significant amount of additional complexity and side-information. In order to resolve this, many advanced AIF methods are introduced in [7] and [8]. But the conventional AIF still has the trade-off between the accuracy of coefficients and the size of side information.

Another kind of adaptive filter has been developed for solving this trade-off in the conventional AIF. The PAIF represents an interpolation filter using five parameters instead of many individual filter coefficients by approximating the interpolation filter with a parametric function [9]. The proposed PAIF scheme enhances the existing AIF scheme by reducing quantization error of AIF coefficients and also reducing the side-information for coding AIF coefficients. We designed a parametrized impulse response function of a filter on an experimental basis, and float-valued offset is additionally introduced for delivering supplementary coding gain.

## Proposed method

A parameterized function, representing all the coefficients for each sub-pel, has to be designed for changing the conventional AIF into a parameterized filter. This function can be regarded as the impulse response of PAIF. We modeled the function on an experimental basis as follows.

h(m,n)=N(m,n)sinc(α1m)sinc(α2n)[γ+sinc(β1m)sinc(β2n)]		(2)

where α1,α2,β1,β2, and γ are the five paramters to be transmitted. The values of N(m,n) are constants. We do not guarantee the gain of filter to be 1 for each sub-pel. As a result, the filter coefficients may have more flexible values reflecting the characteristic of the interpolated reference frame. This modeling fuction is motivated from Lanczos function. It basically follows the shape of the standard 6-tap AVC/H.264 filter as shown in Fig. 6.

Fig. 6 impulse response of PAIF with initial parameters

The parameter set has to be determined at every frame and coded for each frame using PAIF. Experimentally, the range of parameter set and initial values are given as follows.

α1 and α2	: [0.5, 1.0], initially π/4

β1 and β2	: [0, π/11], initially π/12

γ		: [-0.1, 0.1], initially 0

We need to find the optimal parameters that minimize the mean squared prediction error. In order to find them, we used the numerical optimization method. Initial values are determined to be close to the standard AVC/H.264 filter. And also, Broyden–Fletcher–Goldfarb–Shanno (BFGS) Quasi-Newton method with a golden section search is applied to find the optimal parameter set.

Before coded, the 5 parameters are quantized by fixed length coding. The magnitude of β1 ,β2, and γis uniformly quantized to 8196 steps and coded as 13-bit. And the magnitude of α1 and α2 is also uniformly quantized to 2048 steps, coded as 11-bit. For γ, one additional bit is required to represent the sign value.

In addition, we use the offset value in the motion compensation process. In the recently proposed filtering technique such as SIFO and enhanced AIF, they usually include the offset in addition to newly designed filters. We also added offset values with a PAIF filter. Similar with the offset usage in case of the weighted prediction in AVC/H.264, we added one offset value for each reference frame. And also the precision of the offset value is set to be not integer but 0.1 in order to secure more accuracy.

## Experimental results

The P-AIF scheme was implemented based on LG CfP model. To test the performance of the P-AIF scheme, the same testing conditions in LG CfP model (IbBbBbBbP) are used in our simulation. Table 2 shows the performance of the P-AIF scheme in terms of BD rate reduction [5]. We compare the performance of P-AIF with SIFO(singlepass) and 2D-AIF for class C and D. High precision h.264 filter is used at the anchor. Maximum gain among all test sequences was 8.89% at BQSquare sequence, where the RD curve of BQSquare is shown in Fig. 7. The average bit rate reduction for class C is 0.76% (up to 1.79%) and for class D is 2.67% (up to 8.89%). PAIF shows the better performance than singlepass SIFO and 2D-AIF at the most sequences.

Table 2: BD rate (%) reduction relative to anchor (high precision h.264 filter)

| Class | seq. | singlepass SIFO (LG CfP model) | 2D-AIF | P-AIF |
| --- | --- | --- | --- | --- |
| C | BasketballDrill | -0.46 | -0.31 | -0.56 |
|  | BQMall | -0.25 | -0.40 | -0.68 |
|  | PartyScene | -0.71 | -0.90 | -1.79 |
|  | RaceHorses | -0.37 | -0.48 | -0.02 |
| Class C Avg. |  | -0.45 | -0.52 | -0.76 |
| D | BasketballPass | 0.42 | -0.08 | -0.13 |
|  | BlowingBubbles | -0.32 | 0.28 | -1.83 |
|  | BQSquare | -2.72 | -1.19 | -8.89 |
|  | RaceHorses | 0.64 | 0.04 | 0.15 |
| Class D Avg. |  | -0.49 | -0.24 | -2.67 |

Fig. 7 R-D curve of BQSquare test sequence

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] K. Wong, L. Po, K. Cheung, and K. Ng, “Block-Matching Translation and Zoom Motion-Compensated Prediction by Subsampling,” IEEE International Conference on Image Processing, pp. 1597-1600, 2009.

[2]  C. Tomasi, T. Kanade, “Detection and Tracking of Point Features”, Technical Report CUM-CS-91-132, 1991.

[3] S. J. Pundlik, S. T. Birchfield, “Motion Segmentation at Any Speed”, British Machine Vision Conference, Sept. 2006.

[4] H. Richard. Z. Andrew, Multiple view geometry in computer vision, Cambridge University Press, UK., 2000.

[5] G. Bjontegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SC16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[6] Yuri Vatis, Bernd Edler, Diew Thanh Nguyen, and Jörn Ostermann, “Two-dimensional non-separable Adaptive Wiener Interpolation Filter for H.264/AVC”, ITU-T SC16/Q6, 26th VCEG Meeting, Busan, Korea, April 2005, Doc. VCEG-Z17.

[7] Matsushita, "Separable Adaptive Interpolation Filter", COM16-C219 July 2007.

[8] D. Rusanovskyy, K. Ugur, J. Lainema, "Adaptive Interpolation with Directional Filters", ITU-T Q.6/SG16 VCEG, VCEG-AG21 , October 2007

[9] Jie Dong, King Ngi Ngan, “Parametric interpolation filter for motion compensated prediction”, IEEE International Conference on Image Processing, Nov., 2009