| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A022 |
| --- | --- |

| Title: | Multiple Predictor Sets for Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Lingzhi Liu 2330 Cental Expy. Santa Clara, CA 95050, USA | Tel: Email: | +1-408-3304996 lzliu@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

To achieve better coding gain of intra (I) frames in H.264/MPEG AVC, multiple predictor sets for intra coding (MPSI) are proposed to improve the accuracy of intra prediction. A new predictor set is introduced with similar subsets as the original one in H.264/AVC, in which 8 are directional and 1 is non-directional. The same method in H.264/AVC is used to predict the intra prediction mode of current block. Experimental results show that the proposed method achieves 3.14 % average bit reduction for I frames at constraint set 1 and constraint set 2 with CABAC, and 3.89 % average bit reduction at constraint set 2 of CAVLC. The technique is compatible with other KTA tools like Adaptive Loop Filter, and achieves more bit reduction when used together.

# Introduction

H.264/AVC uses intra prediction to reduce the spatial redundancy between spatially adjacent blocks, in which pixels from the already coded neighboring blocks are used to predict the current block. For luminance block, H.264 has a predictor set of 9 predictors for 4x4 and 8x8 blocks. The predictors can be divided into two subsets according to the texture feature of the predictors. One is directional subset with 8 directional predictors; another is non-directional with 1 DC predictor as shown in Fig. 1.

8 Directional Prediction Mode

1 no-directionnel Mode (DC Mode)

Fig 1 Prediction Modes in H.264/MPEG AVC

It has been proved that the above method works well for general sequences. However the weakness of the prediction method is that it is not able to represent sample prediction blocks with complex textures.

In this contribution, a framework to improve the intra coding efficiency is proposed. A new predictor set is introduced with similar subsets as the original one in H.264/AVC, in which there are also 8 directional and 1 non-directional predictors. The method has two advantages: (1) The same method in H.264/AVC can be used to predict the intra prediction mode of current block; (2) The technique is compatible with other KTA tools like Adaptive Loop Filter, and shows more coding gain.

# Multiple predictor sets for intra coding

As described in Section 1, the new predictor set for intra prediction also consists of one directional subset and one non-directional subset.

## Directional subset

The directional subset has 8 directional predictors which are uniform rotated from original predictors of H.264/AVC. Keeping the prediction modes and rotating the directional subset in H.264/AVC of 12.5o counterclockwise, the new 8 directional predictors and their corresponding prediction modes are shown in Fig.2 (b).

Fig 2 Directional Subsets for Intra Coding (a) Directional Subset in H.264/AVC; (b) New Directional Subset after rotating counterclockwise (2) with arctan(4); (c) All Directions

The pixel values in current block are also predicted by the pixels from the already decoded neighboring blocks. The prediction value is calculated through linear interpolation of its neighboring decoded pixels. The calculations also only need simple additions and right shifts as H.264/AVC. Thus the computational complexity on the decoder side is minimum.

## Non-directional subset

Template matching [3] is used for the non-directional subset in the new predictor set. To reduce the computational complexity, the searching range is limited to the decoded pixels of current Macroblock and an “L” shape with the width of 48 and height of 32 pixels as shown in Fig 3. The template used for searching is the 9 adjacent pixels of a 4x4 block or 17 pixels of an 8x8 one.

Fig 3 Template Matching for non-directional prediction subset

## Encoding and decoding processes

In H.264/AVC, the predicted value of current intra prediction mode is set to the smaller intra prediction mode of upper block and left block. Because the predictors with same intra prediction mode in different predictor set have similar texture features, the predicted value of intra prediction mode can also use the smaller intra prediction mode of its upper left block in spite of which predictor set it belongs to.

For example, if the upper block has a prediction mode 4 of set 0, and the left block has a prediction mode 1 of set 1, the predicted mode for the current mode is set to 1.

The rate-distortion optimization technique is used at the Macroblock level to decide which predictor set will be adapted for the Macroblock. A bit of predictor_set_flag is introduced to indicate the set. The syntax in the bit-stream is as follows:

if( transform_8x8_mode_flag && mb_type = = I_NxN )

{

transform_size_8x8_flag

predictor_set_flag

}

At the decoder side, the decoder first decodes the predictor_set_flag, then follows the same procedure as in H.264/AVC.

…

# Experimental results

The proposed method was implemented into jm11.0kta2.6r1 software, and experiments have been performed on I frames according to JCT CFP satisfies constraint set 1 and 2.

Experimental conditions are as follows:

Constraint set 1 and 2 (All I-frame, CABAC ), Constraint set 2( All I-frame, CAVLC )

- MPSI is applied to Intra 4x4 and Intra 8x8 prediction for all constrain sets

- RDO enabled

- RDOQ enabled

- Full length CFP test sequences (WQVGA, WVGA, 720P and HD)

- Use QP defined in Alpha anchor.

Tab 1 shows the results in the Constraint set 1 and 2 (CABAC)[5], and Tab 2 shows the experimental results in the case of Constraint set 2 (CAVLC ). Average PSNR gain and bit reduction are calculated based on the Bjontegarrd method [1][2].

In the case of Constraint set 1 and 2 (CABAC), the average bit reduction is about 3.14% for all sequences, 1.84% for WQVGA, 3.61% for WVGA, 4.18% for 720p and 3.18% for 1080p. Picture size larger than WQVGA has apparently better gain.

In the case of Constraint set 2 (CAVLC), the average bit reduction is 3.89% for all sequences, 2.20% for WQVGA, about 4.1% for WVGA, and about 5.32 % for 720p, and 4.22% for 1080p. Picture size larger than WQVGA appears to have better gain.

Tab 1 Coding efficiency of Constraint set 1 and 2 (CABAC, MPSI only)

| Resolution | Sequence | Bitrate | ΔBitrate (%) | ΔPSNR (dB) |
| --- | --- | --- | --- | --- |
| Class D WQVGA 416x240 | BasketballPass | High | -2.13 | 0.13 |
|  |  | Low | -2.39 | 0.13 |
|  | BQSqure | High | -2.01 | 0.17 |
|  |  | Low | -2.31 | 0.17 |
|  | BlowingBubbles | High | -1.23 | 0.09 |
|  |  | Low | -1.52 | 0.09 |
|  | RaceHorses | High | -1.37 | 0.10 |
|  |  | Low | -1.75 | 0.11 |
| Average coding efficiency for WQVAG |  |  | -1.84 | 0.13 |
| Class C WVGA 832x480 | PartyScene | High | -1.56 | 0.09 |
|  |  | Low | -1.79 | 0.08 |
|  | BQMall | High | -3.10 | 0.17 |
|  |  | Low | -3.41 | 0.18 |
|  | BasketballDrill | High | -7.33 | 0.34 |
|  |  | Low | -7.22 | 0.33 |
|  | RaceHorses | High | -2.12 | 0.11 |
|  |  | Low | -2.33 | 0.11 |
| Average coding efficiency for WVAG |  |  | -3.61 | 0.18 |
| Class E 720P 1280x720 | Vidyo1 | High | -4.81 | 0.26 |
|  |  | Low | -4.96 | 0.28 |
|  | Vidyo3 | High | -3.91 | 0.24 |
|  |  | Low | -4.59 | 0.28 |
|  | Vidyo4 | High | -3.25 | 0.17 |
|  |  | Low | -3.57 | 0.18 |
| Average coding efficiency for 720P |  |  | -4.18 | 0.23 |
| Class B 1080P 832x480 | Kimono | High | -1.60 | 0.05 |
|  |  | Low | -1.87 | 0.07 |
|  | Cactus | High | -3.64 | 0.14 |
|  |  | Low | -4.37 | 0.17 |
|  | BasketballDrive | High | -3.50 | 0.11 |
|  |  | Low | -4.36 | 0.14 |
|  | BQTerrace | High | -4.77 | 0.21 |
|  |  | Low | -5.55 | 0.24 |
|  | Parkscene | High | -0.97 | 0.05 |
|  |  | Low | -1.16 | 0.05 |
| Average coding efficiency for 1080P |  |  | -3.18 | 0.12 |
| Average Coding efficiency for all sequences |  |  | -3.14 | 0.16 |

Tab 2 Coding efficiency of Constraint set 2 (CAVLC, MPSI only)

| Resolution | Sequence | Bitrate | ΔBitrate (%) | ΔPSNR (dB) |
| --- | --- | --- | --- | --- |
| Class D WQVGA 416x240 | BasketballPass | High | -2.67 | 0.18 |
|  |  | Low | -3.05 | 0.18 |
|  | BQSqure | High | -2.36 | 0.21 |
|  |  | Low | -2.66 | 0.21 |
|  | BlowingBubbles | High | -1.39 | 0.11 |
|  |  | Low | -1.68 | 0.11 |
|  | RaceHorses | High | -1.69 | 0.13 |
|  |  | Low | -2.12 | 0.14 |
| Average coding efficiency for WQVAG |  |  | -2.20 | 0.16 |
| Class C WVGA 832x480 | PartyScene | High | -1.75 | 0.10 |
|  |  | Low | -1.93 | 0.10 |
|  | BQMall | High | -3.84 | 0.24 |
|  |  | Low | -4.18 | 0.25 |
|  | BasketballDrill | High | -7.91 | 0.41 |
|  |  | Low | -7.54 | 0.38 |
|  | RaceHorses | High | -2.64 | 0.15 |
|  |  | Low | -2.99 | 0.15 |
| Average coding efficiency for WVAG |  |  | -4.10 | 0.22 |
| Class E 720P 1280x720 | Vidyo1 | High | -5.66 | 0.35 |
|  |  | Low | -5.74 | 0.37 |
|  | Vidyo3 | High | -5.55 | 0.38 |
|  |  | Low | -6.34 | 0.44 |
|  | Vidyo4 | High | -4.18 | 0.24 |
|  |  | Low | -4.42 | 0.26 |
| Average coding efficiency for 720P |  |  | -5.32 | 0.34 |
| Class B 1080P 832x480 | Kimono | High | -2.65 | 0.10 |
|  |  | Low | -3.13 | 0.14 |
|  | Cactus | High | -4.47 | 0.19 |
|  |  | Low | -5.26 | 0.23 |
|  | BasketballDrive | High | -4.69 | 0.17 |
|  |  | Low | -5.63 | 0.22 |
|  | BQTerrace | High | -6.05 | 0.29 |
|  |  | Low | -6.97 | 0.34 |
|  | Parkscene | High | -1.49 | 0.08 |
|  |  | Low | -1.80 | 0.09 |
| Average coding efficiency for 1080P |  |  | -4.22 | 0.19 |
| Average Coding efficiency for all sequences |  |  | -3.89 | 0.22 |

Tab 3. shows the results when MPSI works together with Adaptive Loop Filter (ALF). The anchor is only turning on ALF, and the proposed is turning on both MPSI and ALF. The case of Constraint set 1 and 2 (CABAC) is tested. The average bit reduction is 3.36% for all sequences, 1.89% for WQVGA, 3.65% for WVGA, 4.74% for 720p and 3.48% for 1080p.

Tab 3 Coding efficiency of Constraint set 1 and 2 (CABAC, MPSI + ALF)

Anchor: Only ALF is turned on; Proposed: Both MPSI and ALF are turned on

| Resolution | Sequence | Bitrate | ΔBitrate (%) | ΔPSNR (dB) |
| --- | --- | --- | --- | --- |
| Class D WQVGA 416x240 | BasketballPass | High | -2.33 | 0.15 |
|  |  | Low | -2.53 | 0.14 |
|  | BQSqure | High | -1.99 | 0.17 |
|  |  | Low | -2.29 | 0.17 |
|  | BlowingBubbles | High | -1.22 | 0.09 |
|  |  | Low | -1.53 | 0.09 |
|  | RaceHorses | High | -1.46 | 0.11 |
|  |  | Low | -1.78 | 0.11 |
| Average coding efficiency for WQVAG |  |  | -1.89 | 0.13 |
| Class C WVGA 832x480 | PartyScene | High | -1.56 | 0.09 |
|  |  | Low | -1.74 | 0.08 |
|  | BQMall | High | -3.26 | 0.18 |
|  |  | Low | -3.51 | 0.19 |
|  | BasketballDrill | High | -7.21 | 0.34 |
|  |  | Low | -7.14 | 0.33 |
|  | RaceHorses | High | -2.20 | 0.12 |
|  |  | Low | -2.55 | 0.12 |
| Average coding efficiency for WVAG |  |  | -3.65 | 0.18 |
| Class E 720P 1280x720 | Vidyo1 | High | -5.20 | 0.28 |
|  |  | Low | -5.31 | 0.30 |
|  | Vidyo3 | High | -4.83 | 0.30 |
|  |  | Low | -5.59 | 0.35 |
|  | Vidyo4 | High | -3.61 | 0.19 |
|  |  | Low | -3.93 | 0.20 |
| Average coding efficiency for 720P |  |  | -4.74 | 0.27 |
| Class B 1080P 832x480 | Kimono | High | -1.99 | 0.06 |
|  |  | Low | -2.23 | 0.08 |
|  | Cactus | High | -4.00 | 0.15 |
|  |  | Low | -4.80 | 0.19 |
|  | BasketballDrive | High | -3.98 | 0.12 |
|  |  | Low | -4.89 | 0.16 |
|  | BQTerrace | High | -4.82 | 0.20 |
|  |  | Low | -5.53 | 0.23 |
|  | Parkscene | High | -1.18 | 0.06 |
|  |  | Low | -1.35 | 0.06 |
| Average coding efficiency for 1080P |  |  | -3.48 | 0.13 |
| Average Coding efficiency for all sequences |  |  | -3.36 | 0.17 |

# Complexity analysis

## Encoding time and measurement methodology

The encoding time is measured on a DellT7500 workstation. The computer has 8 2.4G Xeon cores with 12GB installed memory. The operation system is 64 bit Windows 7 professional. The anchor and proposed are running simultaneously to test the encoding time. Tab 4 and Tab 5 gives the comparison results of encoding time. The average encoding time increasing is 102.24% for all sequences of Constraint set 1 and 2 (CABAC, MPSI) and 103.99% for all sequences of Constraint set 2 (CAVLC, MPSI). Because the number of candidates for deciding the best predictor set is double, the encoding time is also doubled.

Tab 4 Encoding time comparison of Constraint set 1 and 2 (CABAC, MPSI only)

| Resolution | Sequence | Average ΔTime (%) |
| --- | --- | --- |
| Class D WQVGA 416x240 | BasketballPass | 102.4457547 |
|  | BQSqure | 98.79243861 |
|  | BlowingBubbles | 95.27065458 |
|  | RaceHorses | 99.66727287 |
| WQVGA Average |  | 99.04403018 |
| Class C WVGA 832x480 | PartyScene | 102.0096679 |
|  | BQMall | 105.091504 |
|  | BasketballDrill | 104.4768222 |
|  | RaceHorses | 104.3397826 |
| WVGA Average |  | 103.9794442 |
| Class E 720P 1280x720 | Vidyo1 | 105.6655913 |
|  | Vidyo3 | 108.5042937 |
|  | Vidyo4 | 107.5252656 |
| 720P Average |  | 107.2317169 |
| Class B 1080P 832x480 | Kimono | 100.2774867 |
|  | Cactus | 100.9915225 |
|  | BasketballDrive | 103.4756683 |
|  | BQTerrace | 100.0307788 |
|  | Parkscene | 97.33944029 |
| 1080P Average |  | 100.4229793 |
| All Average |  | 102.2439965 |

Tab 5 Encoding time comparison of Constraint set 1 and 2 (CAVLC, MPSI only)

| Resolution | Sequence | Average ΔTime (%) |
| --- | --- | --- |
| Class D WQVGA 416x240 | BasketballPass | 102.4477706 |
|  | BQSqure | 98.4877664 |
|  | BlowingBubbles | 98.05588703 |
|  | RaceHorses | 99.12481752 |
| WQVGA Average |  | 99.52906038 |
| Class C WVGA 832x480 | PartyScene | 102.1773402 |
|  | BQMall | 105.9983546 |
|  | BasketballDrill | 104.5288646 |
|  | RaceHorses | 104.8334235 |
| WVGA Average |  | 104.3844957 |
| Class E 720P 1280x720 | Vidyo1 | 110.5624321 |
|  | Vidyo3 | 112.7659786 |
|  | Vidyo4 | 111.6514325 |
| 720P Average |  | 111.6599477 |
| Class B 1080P 832x480 | Kimono | 103.3904962 |
|  | Cactus | 102.0298208 |
|  | BasketballDrive | 105.5615468 |
|  | BQTerrace | 101.0145559 |
|  | Parkscene | 101.2611463 |
| 1080P Average |  | 102.6515132 |
| All Average |  | 103.9932271 |

## Decoding time and measurement methodology

The decoding time is measured by the same methodology as encoding time measurement. Tab 6 and Tab 7 gives the comparison results of decoding time. The average decoding time increasing is 27.43% for all sequences of Constraint set 1 and 2 (CABAC, MPSI) and 56.95% for all sequences of Constraint set 2 (CAVLC, MPSI). The decoding time increasing is mainly caused by template matching, because the best matching block is also to be derived at the decoder side.

Tab 6 Decoding time comparison of Constraint set 1 and 2 (CABAC, MPSI only)

| Resolution | Sequence | Average ΔTime (%) |
| --- | --- | --- |
| Class D WQVGA 416x240 | BasketballPass | 23.47309712 |
|  | BQSqure | 33.46427433 |
|  | BlowingBubbles | 15.89815009 |
|  | RaceHorses | 15.48483755 |
| WQVGA Average |  | 22.08008977 |
| Class C WVGA 832x480 | PartyScene | 21.05240191 |
|  | BQMall | 28.798849 |
|  | BasketballDrill | 48.77927175 |
|  | RaceHorses | 22.12729643 |
| WVGA Average |  | 30.18945477 |
| Class E 720P 1280x720 | Vidyo1 | 33.76548223 |
|  | Vidyo3 | 35.36019598 |
|  | Vidyo4 | 26.42970573 |
| 720P Average |  | 31.85179465 |
| Class B 1080P 832x480 | Kimono | 15.79685619 |
|  | Cactus | 29.38907485 |
|  | BasketballDrive | 32.09230938 |
|  | BQTerrace | 42.74053321 |
|  | Parkscene | 14.1785734 |
| 1080P Average |  | 26.8394694 |
| All Average |  | 27.42693182 |

Tab 7 Decoding time comparison of Constraint set 2 (CAVLC, MPSI only)

| Resolution | Sequence | Average ΔTime (%) |
| --- | --- | --- |
| Class D WQVGA 416x240 | BasketballPass | 55.28289284 |
|  | BQSqure | 75.37015466 |
|  | BlowingBubbles | 34.19103086 |
|  | RaceHorses | 30.50940605 |
| WQVGA Average |  | 48.8383711 |
| Class C WVGA 832x480 | PartyScene | 50.33924228 |
|  | BQMall | 64.09627475 |
|  | BasketballDrill | 90.0725856 |
|  | RaceHorses | 48.77977019 |
| WVGA Average |  | 63.32196821 |
| Class E 720P 1280x720 | Vidyo1 | 60.10529794 |
|  | Vidyo3 | 64.88537557 |
|  | Vidyo4 | 51.62854064 |
| 720P Average |  | 58.87307138 |
| Class B 1080P 832x480 | Kimono | 45.21076348 |
|  | Cactus | 59.31883964 |
|  | BasketballDrive | 57.08535512 |
|  | BQTerrace | 81.52498469 |
|  | Parkscene | 42.76055492 |
| 1080P Average |  | 57.18009957 |
| All Average |  | 56.94756683 |

# Analysis

For MPSI, the analysis of the selection repartition between the original or new predictor sets is depicted in Tab 8. Set0 is the original predictor set in H.264/AVC, and Set1 is the new predictor set in this proposal. 4x4, 8x8 and 16x16 represents the block size. It shows that on average on the test set, the new predictor is selected 12.8% for 4x4 blocks and 24.29% for 8x8 blocks. Given that the selection results from a RD choice, this result confirms that the MPSI is useful. Note that these values exclude the cases where both predictors provide the same value, under which the original predictor has the priority to be selected.

Tab 8 Average percentage of the appearance of different predictor sets of

Constraint set 1 and 2 (CABAC, MPSI only)

| Resolution | Sequence | Average Percentage of Appearance (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 0 4x4 | Set1 4x4 | Set0 8x8 | Set1 8x8 | 16*16 |
| Class D WQVGA 416x240 | BasketballPass | 18.37 | 12.39 | 30.63 | 22.57 | 16.04 |
|  | BQSqure | 32.19 | 31.95 | 12.25 | 11.33 | 12.28 |
|  | BlowingBubbles | 41.14 | 26.35 | 17.94 | 11.99 | 2.58 |
|  | RaceHorses | 20.01 | 18.12 | 32.59 | 24.78 | 4.51 |
| WQVGA Average |  | 27.92 | 22.20 | 23.35 | 17.67 | 8.85 |
| Class C WVGA 832x480 | PartyScene | 30.09 | 16.13 | 23.75 | 15.35 | 14.68 |
|  | BQMall | 17.48 | 9.62 | 26.47 | 21.61 | 24.82 |
|  | BasketballDrill | 6.61 | 16.10 | 22.03 | 39.84 | 15.41 |
|  | RaceHorses | 8.96 | 10.21 | 27.77 | 28.29 | 24.76 |
| WVGA Average |  | 15.79 | 13.02 | 25.00 | 26.27 | 19.92 |
| Class E 720P 1280x720 | Vidyo1 | 5.16 | 5.57 | 22.41 | 28.64 | 38.22 |
|  | Vidyo3 | 9.37 | 6.49 | 19.05 | 19.51 | 45.57 |
|  | Vidyo4 | 7.04 | 4.32 | 25.67 | 24.98 | 37.98 |
| 720P Average |  | 7.19 | 5.46 | 22.38 | 24.38 | 40.59 |
| Class D WQVGA 416x240 | Kimono | 2.50 | 2.72 | 38.58 | 36.66 | 19.54 |
|  | Cactus | 10.82 | 11.35 | 29.75 | 29.15 | 18.93 |
|  | BasketballDrive | 10.94 | 5.68 | 29.69 | 23.46 | 30.22 |
|  | BQTerrace | 13.35 | 18.74 | 22.14 | 25.03 | 20.74 |
|  | Parkscene | 12.15 | 9.06 | 35.22 | 25.51 | 18.06 |
| 1080P Average |  | 9.95 | 9.51 | 31.07 | 27.96 | 21.50 |
| All Average |  | 15.39 | 12.80 | 26.00 | 24.29 | 21.52 |

# Conclusion

A new method is proposed to improve the coding efficiency of Intra frames by using multiple predictor sets. A new predictor set with similar subsets as in H.264/AVC is introduced to get better prediction of local images with complex textures. The same method in H.264/AVC can be used to predict the intra prediction mode. The selected predictor set is indicated on a macroblock level. MPSI achieves 3.14 % bit reduction at Constraint set 1 and 2 (CABAC), and 3.89% bit reduction at Constraint set 2 (CAVLC) on average. Moreover, MPSI is is compatible with other KTA tools, and is able to achieve more coding gain when used together.

This technique can also be integrated into block size larger than 8x8 with more predictor sets, which will be our next work.

# References

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-Curves,” ITU-T SG16 Q.6 Document, VCEG-M33, Austin, US, April 2001

- G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document, VCEG-AI11, Berlin, Germany, July 2008

- J. Yang, B. Yin, Y Sun and N. Zhang, “A Block-Matching Based Intra Frame Prediction for H.264/AVC,” IEEE. International Conference on Multimedia and Expo., Jan. 2000

- T. Tsukuba, T. Yamamoto, Y Tokumo and T. Aono, “Adaptive Multidirectional Intra Prediction,” ITU-T Q.6/SG16 VCEG, VCEG-AG05, Shenzhen China, Oct. 2007.

- JCT-VC, “Joint call for proposals on video compression technology”, ISO/IEC JTC1/SC29/WG11 document N11113, Jan. 2010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).