| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A025 |
| --- | --- |

| Title: | New intra prediction using the correlation between pixels and lines |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changcai Lai, Yongbing Lin HiSilicon, Huawei Technologies Co., Ltd Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86-755-28422002 laichangcai@huawei.com |
| Source: | HiSilicon, Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents a new spatial prediction scheme for intra encoding by taking full use of the correlation between lines and pixels instead of blocks. Unlike to traditional block-based intra prediction, both line-based intra prediction and resample-based intra prediction are proposed to improve compression performance of intra encoding. Experimental results show that an average about 4% bit reduction is achieved compared with H.264/AVC High Profile under CfP common condition except that GOP structure is all intra pictures.

# Introduction

Intra prediction in H.264/AVC performs zero-order extrapolation using the adjacent boundary pixels of reconstructed neighboring blocks to get the prediction of the entire coding block. One problem of such a block-based intra prediction design is that the prediction error increases as the distance from the reference pixel to the current prediction pixel increases. To solve the problem, a new intra prediction scheme using the correlation between lines and pixels are proposed.

# Algorithm description

## Line-based intra prediction

Fig. 1. Directional prediction of 1x16 and 2x8 mode

Firstly, we introduce the line-based intra prediction (LIP) to use the correlation between lines and pixels. Currently, the line-based intra prediction consists of 4 modes, i.e., 1x16, 16x1, 2x8 and 8x2 modes. For each intra MB, the lip_flag is used to indicate whether LIP is used. And a lip_type symbol is used to specify the selected LIP mode. Mode selection is based on RD evaluation. The LIP mode 16x1 and 8x2 are illustrated in Fig. 1. For each LIP mode, nine directional predictions are evaluated, the one with minimum RD cost is selected as the prediction direction. For horizontal and vertical prediction, the prediction is set the value of the reference pixel. For other directions, the reference pixel that lies on the prediction direction and its two neighboring reference pixels are low-pass filtered to get the prediction value. According to the LIP mode used, DCT of size 1x16, 16x1, 2x8 or 8x2 is applied. The zigzag scan order is changed accordingly for coefficient blocks of size 8x2 and 2x8.

## Resample-based intra prediction

Secondly, a resample-based intra prediction (RIP) is proposed for intra prediction, which aims to improve coding efficiency by taking full advantage of the correlation of pixels instead of blocks.

Fig. 2. The proposed RIP partitions and signal methods

As illustrated in Fig. 2, three new MB partitions are proposed, of which one is one-level decomposition and the others are two-level decomposition. In the first-level decomposition, a MB is decomposed into 4 blocks by down-sampling. In the second-level decomposition, one of the down-sampled blocks can be further decomposed into 4 sub-blocks by using down-sampling or not. These three new MB partitions are used for intra encoding as well as traditional intra MB partitions. The final selected intra MB partition is determined by using RDO at MB level. Two flags, use_RIP_flag and hierarchical_structure_flag, are combined with the existing transform_size_8x8_flag to signal the proposed RIP partitions.

,

Fig. 3. Intra prediction and interpolation in RIP

Let’s take the proposed partition of Intra_RIP_8x8 as an example. As shown in Fig. 3, a MB is decomposed into 4 blocks (i.e. I0, P1, P2 and P3) by down-sampling. I0 block is encoded with traditional block-based intra prediction, where all of the reference pixels are from the encoded neighboring MB, and 9 intra prediction modes are used. P1 and P2 blocks are predicted pixel-by-pixel with the reference of the encoded I0 block by interpolation with 4-tap filter [-1, 5, 5, -1] horizontally and vertically. P3 block is predicted pixel-by-pixel with the reference of the encoded P1 and P2 blocks by averaging the four neighboring pixels. It is noticed that unavailable reference pixels are replaced by the nearest pixels when performing the prediction on the boundaries of blocks. It should be noticed that the proposed RIP technique takes full advantage of the correlation of pixels instead of blocks and has higher prediction accuracy in comparison to the traditional block-based intra prediction.

The prediction residuals of all blocks are processed with transform, quantization and entropy coding. In quantization process, block-based QP shift scheme is employed to achieve higher coding performance, where smaller QP is assigned to the reference block(s). Change to entropy coding of CBP (Coded Block Pattern) syntax element is necessary since all the decomposed blocks don’t have similar distribution of residual coefficients. The in-loop filter process inside a MB is disabled if the MB is encoded with the proposed partitions.

# Compression performance discussion

Table I.  Comparison of coding efficiency for CfP sequences

|  | low-bitrate range |  | high-bitrate range |  |
| --- | --- | --- | --- | --- |
| sequence | BD-bitrate | BD-PSNR | BD-bitrate | BD-PSNR |
| RaceHorses_416x240_30 | -2.07% | 0.13 | -1.90% | 0.14 |
| BasketballPass_416x240_50 | -3.35% | 0.18 | -2.77% | 0.17 |
| BlowingBubbles_416x240_50 | -2.00% | 0.12 | -1.82% | 0.14 |
| BQSquare_416x240_60 | -1.98% | 0.15 | -1.60% | 0.13 |
| Average QWVGA | -2.35% | 0.15 | -2.02% | 0.15 |
| RaceHorses_832x480_30 | -3.14% | 0.14 | -2.84% | 0.15 |
| BasketballDrill_832x480_50 | -6.51% | 0.29 | -6.27% | 0.28 |
| PartyScene_832x480_50 | -3.36% | 0.16 | -2.88% | 0.16 |
| BQMall_832x480_60 | -3.94% | 0.21 | -3.73% | 0.21 |
| Average WVGA | -4.24% | 0.20 | -3.93% | 0.20 |
| vidyo1_720p_60 | -6.30% | 0.35 | -6.48% | 0.36 |
| vidyo3_720p_60 | -4.79% | 0.29 | -4.34% | 0.27 |
| vidyo4_720p_60 | -5.18% | 0.27 | -4.93% | 0.25 |
| Average 720P | -5.42% | 0.30 | -5.25% | 0.29 |
| Kimono1_1920x1080_24 | -8.68% | 0.32 | -5.60% | 0.17 |
| ParkScene_1920x1080_24 | -1.57% | 0.07 | -0.74% | 0.03 |
| BasketballDrive_1920x1080_50 | -4.66% | 0.16 | -3.95% | 0.12 |
| Cactus_1920x1080_50 | -3.65% | 0.14 | -2.76% | 0.10 |
| BQTerrace_1920x1080_60 | -5.14% | 0.22 | -4.27% | 0.18 |
| Average 1080P | -4.74% | 0.18 | -3.46% | 0.12 |
| PeopleOnStreet_2560x1600_30_crop | -6.28% | 0.35 | -5.43% | 0.30 |
| Traffic_2560x1600_30_crop | -3.89% | 0.21 | -3.10% | 0.17 |
| Average 1600P | -5.09% | 0.28 | -4.27% | 0.24 |
| Average | -4.25% | 0.21 | -3.63% | 0.19 |

The proposed technique has been integrated into KTA2.6r1 reference software. It is noted that the proposed technique is only applied to luminance component. Encoding parameters are configured according to CfP common test condition except that GOP structure is all intra pictures. Anchor is H.264/AVC High Profile, all CfP sequences with full length are tested, CABAC entropy coding mode is enabled, RDO and RDOQ (Rate Distortion Optimized Quantization) are enabled. 5 QP points used in the experiment are the same as CfP Alpha anchors Error: Reference source not found.

Table I shows the experimental results at low-bitrate and high-bitrate ranges. Average PSNR gain and bitrate reduction are calculated based on VCEG-M33 Error: Reference source not found. It shows that average 4.25% bitrate reduction for low-bitrate range is achieved and average 3.63% bitrate reduction for high-bitrate range is achieved.

It is observed that performance gain depends on sequence’s resolution. Generally speaking, the proposed technique provides more performance gain for HD (High Definition) sequences (720p, 1080p and 1600p) than those sequences with low resolution (WVGA and WQVGA). The average bit reduction for HD sequences reaches up to 5%. This is because that more correlation between pixels and lines in HD sequences can be employed by the proposed technique.

# Reference

- ITU-T Q6/16 and ISO/IEC JTC1/SC29/WG11, “Joint Call for Proposals on Video Compression Technology,” MPEG document w11113, 91st MPEG meeting, Kyoto, Japan, Jan. 2010

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” VCEG document VCEG-M33, ITU-T Q.6/SG16, 13th VCEG Meeting, Austin, Texas, USA, Apr. 2001

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).