| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A027 |
| --- | --- |

| Title: | Arithmetic coding based on probability-aggregation and delayed-subdivision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Tel: Email: | hurricanezhb@hotmail.com |
| Source: | Hongbo Zhu |  |  |

_____________________________

# Abstract

A parallel-friendly arithmetic coder is proposed in this proposal. One shortage of the current design, m-coder is the dependency on range when more than two symbols are coded/decoded. After one symbol is coded, the range must be renormalized to range0x100,0x1ff. As the combination of range and probability are very big, it is difficult to determine the range before a symbol is decoded. In other word, it is necessary to decode all symbols one by one serially.

# Algorithm Description

The new arithmetic coding uses log-probability (logProb) to denote the interval. As illuminated in figure 1, in which Rprev denotes the length of the current interval, Range=0x10000, base=0.51128. The 32×logbasep1, 32×logbasepLPS and 32×logbasepMPS are logProb. Ix is a map from logProb to interval length. Because of Iy+128×32×n=Iy>>n, so only a 128-entry table is needed to calculate the interval length from logProb. SoIx=Tabtrvlx>>5∧127>>x>>12, in which Tabtrvl is a 128-entry table that is listed in figure 3.

It can be gotten from figure 1 that two logProb are needed for the LPS and MPS of each probability state in H.264, they are named as LPSlogprob and MPSlogprob. There are all logProb for 64 probability states in figure 2, in which lgprobaddvl[state][0] is LPSlogprob, while lgprobaddvl[state][1] is MPSlogprob. The design of LPSlogprob and MPSlogprob must meet the following two conditions:

- minp×log2p+1−p×log21−p−p×log2baseLPSlogprob32−1−p×log2baseMPSlogprob32,in which p is the probability of LPS

- Ix≥Ix+MPSlogProb+Ix+LPSlogProb,x∈[0,0x8000)

The basic operation of the encoder is to select the interval of LPS or MPS according to the value of input symbol. As illustrated in figure 4, when the encoder is run firstly, initialize the logProb, low and code to 0. When the symbol to be coded is MPS, increment the logProb by MPSlogprob and the low remains unchanged (corresponding to select the MPS interval). When the symbol to be coded is LPS, increment the logProb by LPSlogprob and add the low by I(logProb+MPSlogprob) (corresponding to select the LPS interval).

When the logProb is more than or equal to 0x8000 (corresponding to the current interval is less than 0x100), enlarge the current interval by one byte (illustrated in figure 5): logProb=logProb−0x8000;code=code<<iobits+low;low=0; output the never-changed bytes in code to bitstream (there must be some complex operations to cope with outstanding). As I(logProb-0x8000)>= I(logProb)<<8, so increase the logProb-0x8000 to logProbnew using an algorithm to meet I(logProbnew)<=(I(logProb)<<8).

The decoder is the reverse of the encoder. As illustrated in figure 6, initialize the value using the first two bytes in bitstream and set logProb to 0. When decoding a symbol, compare the value with I(logProb+MPSlogprob), if greater than or equal to, output LPS and select the LPS interval and subtract the value with I(logProb+MPSlogprob), else output MPS and select the MPS interval. Selecting the LPS/MPS interval is similar to that in encoder. If the logProb is greater than or equal to 0x8000, renormalization the logProb and range as that in encoder and enlarge the value by one byte and read a byte from bitstream to fill the lower byte of value.

# Fast decoding algorithms and development in future

Figure 7 illustrates the decoding of 3 symbols in parallel, in which lP is logProb and the MiLi denotes the MPSlogprob/LPSlogprob of i-th symbol to be decoded. The r0-r6 is all possible division of 3 symbols, they can be computed and compared with value in parallel, and find the first 0 in the result bit-string of the comparison, the lower bound of the resulting interval is found. Increasing the comparison devices can give more than 3 symbols decoding in parallel.

Figure 8 illustrates the decoding of multiple MPS in parallel. Compute the r0-r4 and compare them with value in parallel. The resulting interval can be gotten from the result of the comparison. If the interval of significance=0 in 4x4 or 8x8 block is always allocated to the lower part without considering if the significance=0 is MPS, the fast algorithm in figure 8 can decode one position of non-zero coefficient, so the decoding complexity is decreased from bin-bounded to symbol-bounded.

Developments in future are listed as follows:

- convert the byte-based input/output to word-based input/output

- Re-design the MPSlogprob/LPSlogprob to combine the MPSlogprob with the probability state, so decrease one table lookup.

- Re-design the subinterval_division to simplify the renormalization of logProb

- Design a table to map the value to logProbOfvalue approximately, and convert the comparison between value and I(logProb+MPSlogprob) to the comparison between logProbOfvalue and logProb+MPSlogprob.

# Primary experiment results

The new arithmetic coding is implemented in jm17.0 to get the results. Some basic configurations are as follows:

IntraPeriod = 32

FrameSkip = 0

SearchRange = 128

NumberReferenceFrames = 4

EnableIPCM = 0

NumberBFrames = 7

BList0References = 4

BList1References = 4

HierarchicalCoding = 2

HierarchyLevelQPEnable  =  1

BiPredMotionEstimation = 1

BiPredMERefinements    = 3

BiPredMESearchRange    = 16.

BiPredMESubPel         = 2

WeightedPrediction       =  0

WeightedBiprediction     =  2

RDOptimization =  1

SearchMode = 1

QP of I Slice = 22,27,32,37

QP of P Slice = 23,28,33,38

QP of B Slice = 24,29,34,39

Foreman_cif (300 frames), mobile_cif (300 frames) and carphone_cif (382 frames) are used to generated the results illustrated in table 1. More results are available in the next meeting because I finish the algorithm recently and have no time to get all results.

|  | BDPSNR | BDbitrate |
| --- | --- | --- |
| Mobile_cif | -0.01796 | 0.380485 |
| Foreman_cif | -0.02202 | 0.518041 |
| Carphone_cif | -0.00988 | 0.256317 |
| Average: | -0.01662 | 0.3849 |

Allrough new arithmetic coding is only above the entropy by about 0.4% (gotten by random generated symbols), it is inferior to the m-coder by 0.01662dB. This is perhaps due to the actual distribution deviates the 0.5×0.018750.5i63 for the LPS of i-th probability state (the current MPSlogprob/LPSlogprob is designed assuming that probility of the LPS of i-th probability state is equal to 0.5×0.018750.5i63). It is expected that training the MPSlogprob/LPSlogprob using the actual distribution gives better results.

# Patent rights declaration(s)

The author may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).