| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A028 |
| --- | --- |

| Title: | Adaptive frequency weighting quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianhua Zheng Hisilicon, Huawei Technologies Co., Ltd. Section B2, Huawei Industrial Base, Bantian Longgang District, Shenzhen 518129, China | Tel: Email: | Tel: +86-10-82836652 Email: zhengjianhua@huawei.com |
| Source: | Hisilicon, Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

In H.264/AVC standard, non-uniform quantization is available in picture level via loading 8x8 and 4x4 quantization scaling matrices in picture parameter set. However, different regions in picture have different sorts of texture, non-uniform quantization should be considered in macroblock level in High performance Video Coding (HVC). This proposal considers a macroblock level quantization tool to allow parameterized frequency weighting in picture level and non–uniform quantization in macroblock level meanwhile no bits used in macroblock.

# Contents

Abstract	1

Contents	2

1	Introduction	3

2	Algorithm description	3

2.1	Parameterized frequency weighting	3

2.2	Adaptive quantization mode selection	4

2.3	Syntax	7

3	Compression performance discussion	7

3.1	Constraint set 1 configuration relative to Alpha anchor	7

3.2	Constraint set 2 configuration relative to Beta anchors	8

4	Complexity analysis	9

4.1	Encoding time and measurement methodology and comparison	9

4.2	Decoding time and measurement methodology and comparison	10

4.3	Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2	10

4.4	Complexity characteristics of encoder quantization and quantization type selection	10

4.5	Complexity characteristics of decoder inverse quantization	10

5	Closing remarks	11

6	Patent rights declaration(s)	11

# Introduction

In H.264/AVC standard, 8x8 and 4x4 quantization weighting matrices are defined in picture parameter set, so as to select appropriate quantization matrices at the picture level for different video pictures since picture contents in a sequence varies. This allows non-uniform quantization in picture level. However, different regions in picture have different sorts of texture, non-uniform quantization should be considered in macroblock level to meet the picture content in High performance Video Coding (HVC). This proposal considers a macroblock level quantization tool for the following reasons,

1)	Quantization matrices loaded in picture level result in bits overhead and hard to be used in small picture size.

2)	Quantization weighting matrices are flexible but hard for end-user to control the coding picture quality via each value in the quantization weighting matrices.

3)	Non-uniform quantization is not available for macroblock level for considering the picture content such as textures, details and undetails.

Adaptive frequency weighting quantization (AFWQ) is proposed in this document using parameterized frequency weighting models in picture level and implement non–uniform quantization in macroblock level for considering the property of local textures. Fewer bits used in picture level and no extra bits needed in macroblock.

# Algorithm description

## Parameterized frequency weighting

According to the characteristics of Human Vision System (HVS), different frequency position in transformed coefficient block will lead to different perceptual effect for human eyes. One frequency point has similar perceptual property with its nearby position. Hence, the transformed block could be divided into different frequency bands according to HVS. Frequency band in one transformed block stands for different perceptual property. Transformed coefficients in same frequency band have similar perceptual features. According to the importance of the frequency bands, the weighting values can be allocated to the frequency bands, with the purpose of controlling the quality and rate-distortion capability of a block. Hence, each frequency band will be allocated one parameter. The whole matrix of a transformed block can be modeled as several frequency band weighting factors and their distribution.

Figure 1 illustrates three typical frequency bands distribution models for 8x8 block. In figure 1, every frequency band is responding to a weighting factor, wq_param[i] (i=0…6), and every frequency band distribution is modeled and indicated by weighting_quant_model (weighting_quant_model =0,1,2). Figure 1 are three typical distributions, in which pD, pa,pb,pc,pd,ph are band weighting parameters and indexed by i. Bands (pa-pd) have high sensitivity frequencies to human eyes. To combine the frequency bands distribution model and the frequency bands weighting parameters, an 8x8 frequency weighting matrix can be constructed.

Based on frequency bands distribution model (indicated by weighting_quant_model), the adjustment of the frequency bands’ weighting parameter can efficiently and easily control the block’s subjective quality. weighting_quant_model and wquant_param[i] (i=0…6) should be coded in bitstream to let them be changed adaptively for each picture.

a)

weighting_quant_model=0   weighting_quant_model=1   weighting_quant_model=2

b)

Figure 1. The frequency band distribution models

In H.264/AVC, the quantization processing is formulated as follows:

(1)

The dequantization is defined by

(2)

where  is the quantized transformed coefficient, is the DCT transform coefficients and QP is the quantization parameters ranging from 0 to 51. and  are the scaling factor of quantization, andis the rounding of quantization. The scaling factor can be calculated as follows.

A(QP,i,i)=WS(i,j)×Norm(QP,i,j)                                         (3)

B(QP,i,i)=WS(i,j)×Norm(QP,i,j)                                         (4)

where  is the scale adjustment parameter and  is the quantization matrix. The frequency weighting can be formulated at formula (5) ~ (8) as follow:

(5)

Xrij=(XqijB(QP,i,j)>>7)<<QP6                                          (6)

A(QP,i,i)=Norm(QP,i,j)<<7                                              (7)

(8)

Where  is the weighting parameters for the position (i,j) in blocks, and k indicats a frequency band.  is the integration operation.

Applying the frequency bands distribution model and the frequency bands weighting parameters, frequency weighting matrix can be derived and applied to luma and chroma coefficient blocks. For 4x4 luma and chorma blocks, the 6-parameter distribution models are used. The band weighting parameters of 6-parameter distribution models of 4x4 blocks are mapped from the 7-parameter distribution models of 8x8 blocks, so as with the chroma blocks, see figure 1b. Hence only 2 bit index indicated the distribution model and band weighting parameters of 8x8 luma need be coded in picture level. The weighting parameters predicted error of chroma and 4x4 luma from those parameters of 8x8 luma need to be coded. In current implementation, encoder can obtain a set of user defined weighting parameters in the configuration file.

## Adaptive quantization mode selection

For frequency weighting adapted to the local texture in the quantization of each macroblock, quantization mode is proposed to indicate the different quantization quality. For different quantization qualities, quantization modes may be classified into the following type, each one corresponding to certain sort of quantization qualities.

(1) Quantization mode 0:

Default quantization mode, that is, the frequency weighting matrix used for the current macroblock is same as the current default quantization mode. Usually, it is a flat quantization matrix.

(2) Quantization mode 1:

Details-preserving mode, that is, when the frequency weighting matrix used for the current macroblock , the quantized/dequantized block should be preserves image details as more as enough. Hence, this quantization mode belongs to the high quality quantization mode.

(3) Quantization mode 2:

Undetailed quantization mode, that is, when the frequency weighting matrix used for the current macroblock , the quantized/dequantized block should not preserve the details more. This quantization mode belongs to the low quality quantization mode.

In the above three cases, quantization modes with different quality may be used for characteristics of different image textures, wherein the high quality quantization modes and low quality quantization modes may be specified by a set of frequency band weighting parameters above.

Figure 2  Block partition

From Fig.2, it can be observed that the small partition size is always selected for the areas with more details; the larger partition size is usually used for the background with less details. For the areas with details, the detail reserved frequency weighting strategy should be used, and for the areas not belonging to image details, the undetail frequency weighting strategy should be used.

The characteristics of current macroblock local textures can be identified by its neighboring blocks with the mb-type for inter coding and intra_predict_mode for intra coding. Small motion partitions or small intra block as well as the intra prediction directions indicate the image details, then quantization mode 1 is used. Larger motion partitions or larger intra block as well as the intra prediction directions indicate it does not belong to the image details, then quantization mode 0 is used.

Figure 3  Neighboring MB

The corresponding rules are used to select the quantization mode.

(1) For regions belonging to details of an image should be maintained as possible as enough, and the corresponding quantization mode to be used should maintain more high frequency information, that is, high quality quantization modes should be used.

(2) For regions belonging to edges of a moving part of an image, high frequency information should be maintained, and high quality quantization modes also need to be used.

(3) For regions with rapid motion of an image, a certain decrease of objective quality would not impose great influence on subjective quality, and thus slightly rough quantization modes, i.e., low quality quantization modes may be used.

In case of an intra-coding picture, intra-prediction mode and block type of neighbor blocks may be used in the quantization mode section. In case of inter-coding picture, inter-prediction mode (including block size and block type) of neighbor blocks may be used in the quantization section conditions. An example is illustrated in figure4.

Fig.4a illustrates the video coding structure integrated with Adaptive Frequency Weighting qutization(denoted as AFWQ). AFWQ perform the quantization mode selection based on the coding side information of neighboring block. By the determined quantization mode, frequency weighting on current block is performed. Fig.4b illustrated that several quantization mode are predefined both in encoding side and decoding side.

Entropy

Coding

Scaling & Inv. Transform

Motion-

Compensation

Control

Data

Quant.

Transf. coeffs

Motion

Data

Intra/Inter

Coder

Control

Decoder

Motion

Estimation

Scal./Quant.

-

Input

Video

MB

Intra-frame

Prediction

De-blocking

Filter

Output

Video

Signal

Trans

form

AFWQ

Fig.4a

Entropy

Coding

Scaling & Inv. Transform

Motion-

Compensation

Control

Data

Quant.

Transf. coeffs

Motion

Data

Intra/Inter

Coder

Control

Decoder

Motion

Estimation

Scal./Quant.

-

Input

Video

Signal

Intra-frame

Prediction

De-blocking

Filter

Output

Video

Signal

Trans

form

AFWQ

T

Scal/Quant.

Inv.Scal/Quant.

Inv.T

Encoder

MBAFW

Frequency Weighting

W0

W1

W2

MBAWD

FW Mode

MBAFW

T

Scal/Quant.

Inv.Scal/Quant.

Inv.T

Encoder

Frequency Weighting

W0

W1

W2

Frequency Weighting

W0

W1

W2

MBAWD

Quantization Mode

qMode_1

qMode_2

Fig. 4b

## Syntax

Syntax for adaptive frequency weighting quantization is proposed as below.

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| … … … … |  |  |
| adaptive_frequency_weighting_flag | 1 | u(1) |
| if(adaptive_frequency_weighting_flag){ |  |  |
| mb_adaptive_weighting_quant_enable | 1 | u(1) |
| weighting quant model | 2 | u(2) |
| for(i=0; i<7; i++) |  |  |
| weighting_quant_param_detailed |  | se(v) |
| for(i=0; i<7; i++) |  |  |
| weighting_quant_param_undetailed |  | se(v) |
| } |  |  |
| } |  |  |

adaptive_frequency_weighting_flag is a switch flag coded to turning on/off this tool. Weighting quant model is the frequency band parameters distribution. Weighting_quant_param_detailed (i=0…6)and weighting_quant_param_undetailed(i=0…6) are frequency band weighting parameters for quantization mode 1 and quantization mode 2 respectively.

# Compression performance discussion

The proposed adaptive frequency weighting quantization has been implemented into JM11.0_KTA2.6 r1 reference software. Test condition in simulation is same as the Alpha anchor (constraint set 1) and Beta anchor (constraint set 2) described in N11113[1]. The anchor is JM11.0_KTA2.6 r1 reference software with KTA new tools turning off and setting QP number of RDOQ to 1. The QP points setting are fixed QP as same as the configuration in the Alpha anchor and Beta anchor bitstreams. Three frequency weighting quantization modes(0～2) are used in all simulation. Coding performance under alpha and beta common test condition showed in Table 1 and Table 2 respectively. The BD PSNR and rate computed via [3][4] criteria for the first 4 QP points(Rate1~Rate4,[1]) is labeled as low bitrate and the BD PSNR and rate for the last 4 QP points(Rate2~Rate5,[1]) is labeled s high bit rate.

## Constraint set 1 configuration relative to Alpha anchor

Table 1  Simulation results (alpha condition)

| alpha | High bitrate |  | Low bitrate |  |
| --- | --- | --- | --- | --- |
|  | Bitrate Saving | PSNR Gain | Bitrate Saving | PSNR Gain |
| Class D_WQVGA |  |  |  |  |
| BQSquare_416x240_60 | 1.97 | -0.07 | 0.78 | -0.03 |
| BasketballPass_416x240_50 | -1.68 | 0.08 | -2.24 | 0.10 |
| BlowingBubbles_416x240_50 | -0.74 | 0.03 | -2.68 | 0.10 |
| RaceHorses_416x240_30 | -0.86 | 0.04 | -1.67 | 0.08 |
| average | -0.33 | 0.02 | -1.45 | 0.06 |
| Class C_WVGA |  |  |  |  |
| PartyScene_832x480_50 | -3.90 | 0.15 | -4.83 | 0.18 |
| BQMall_832x480_60 | -2.31 | 0.11 | -2.61 | 0.14 |
| BasketballDrill_832x480_50 | -4.65 | 0.19 | -5.18 | 0.23 |
| RaceHorses_832x480_30 | -1.71 | 0.07 | -2.33 | 0.10 |
| average | -3.14 | 0.13 | -3.74 | 0.16 |
| Class B_1080P |  |  |  |  |
| Kimono1_1920x1080_24 | -2.88 | 0.09 | -3.35 | 0.13 |
| Cactus_1920x1080_50 | -3.32 | 0.09 | -3.82 | 0.12 |
| BasketballDrive_1920x1080_50 | -2.14 | 0.06 | -2.64 | 0.09 |
| ParkScene_1920x1080_24 | -3.31 | 0.12 | -3.18 | 0.12 |
| BQTerrace_1920x1080_60 | -2.61 | 0.04 | -2.90 | 0.06 |
| ChristmasTree_1920x1080_50 | -2.36 | 0.07 | -3.00 | 0.10 |
| Wisley2_1920x1080_50 | -6.31 | 0.23 | -6.19 | 0.24 |
| average | -3.28 | 0.10 | -3.58 | 0.12 |
| Class A_2K |  |  |  |  |
| Traffic_2560x1600_30 | -4.77 | 0.17 | -4.27 | 0.18 |
| PeopleOnStreet_2560x1600_30 | -1.18 | 0.06 | -1.54 | 0.08 |
| average | -2.97 | 0.12 | -2.90 | 0.13 |
| average(High rate/Low rate) | -2.51 | 0.09 | -3.04 | 0.12 |

| alpha | Bitrate Saving | PSNR Gain |
| --- | --- | --- |
| High bitrate | -2.51 | 0.09 |
| Low bitrate | -3.04 | 0.12 |
| average(all) | -2.78 | 0.11 |

## Constraint set 2 configuration relative to Beta anchors

Table 2  Simulation results (Beta condition)

| Beta | High bitrate |  | Low bitrate |  |
| --- | --- | --- | --- | --- |
|  | Bitrate Saving | PSNR Gain | Bitrate Saving | PSNR Gain |
| Class D_WQVGA |  |  |  |  |
| BQSquare | 0.41 | -0.01 | 1.33 | -0.04 |
| BasketballPass | -3.79 | 0.18 | -4.04 | 0.18 |
| BlowingBubbles_416x240_50 | -0.49 | 0.02 | -1.14 | 0.04 |
| RaceHorses_416x240_30 | -1.21 | 0.06 | -2.47 | 0.11 |
| average | -1.27 | 0.06 | -1.58 | 0.07 |
| Class C_WVGA |  |  |  |  |
| PartyScene | -2.01 | 0.07 | -3.43 | 0.11 |
| BQMall 2 | -1.72 | 0.08 | -1.84 | 0.09 |
| BasketballDrill | -3.70 | 0.15 | -4.33 | 0.18 |
| RaceHorses | -3.33 | 0.13 | -4.19 | 0.16 |
| average | -2.69 | 0.11 | -3.45 | 0.14 |
| Class B_1080P |  |  |  |  |
| Kimono | -1.39 | 0.05 | -1.78 | 0.08 |
| Cactus | -3.75 | 0.12 | -3.83 | 0.13 |
| BasketballDrive | -3.25 | 0.11 | -3.47 | 0.14 |
| Parkscene | -2.72 | 0.10 | -3.02 | 0.11 |
| BQTerrace_1920x1080_60 | -2.06 | 0.05 | -1.67 | 0.05 |
| ChristmasTree_1920x1080_50 | -2.37 | 0.08 | -2.86 | 0.10 |
| Wisley2_1920x1080_50 | -7.41 | 0.24 | -7.88 | 0.27 |
| average | -3.28 | 0.11 | -3.50 | 0.13 |
| Class A_720P |  |  |  |  |
| vidyo1_720p_60 | -3.46 | 0.15 | -2.42 | 0.12 |
| vidyo3_720p_60 | -1.11 | 0.05 | -1.00 | 0.05 |
| vidyo4_720p_60 | -1.75 | 0.07 | -1.52 | 0.07 |
| average | -2.11 | 0.09 | -1.65 | 0.08 |
| average(High rate/Low rate) | -2.51 | 0.09 | -2.75 | 0.11 |

| Beta | Bitrate Saving | PSNR Gain |
| --- | --- | --- |
| High bitrate | -2.51 | 0.09 |
| Low bitrate | -2.75 | 0.11 |
| average(all) | -2.63 | 0.10 |

# Complexity analysis

For comparing the computing complexity, JM(KTA), AFWQ and AQMS runs respectively on JM11.0_KTA2.6r1 reference software for encoding and decoding a 1080p test sequence, BasketballDrive_1920x1080_50 with length of 100 frames on alpha and Beta testing conditions[1]. The binary exe is compiled as 64 bit and run in Xeon X5440 Windows XP workstation.

## Encoding time and measurement methodology and comparison

Table 1  Encoding time comparison

|  |  | EncodingTime(second) 1 |  |  | EncodingTime(times) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | JM | AFWQ | AQMS | AFWQ_vs_JM | AQMS_vs_JM | AQMS_vs_AFWQ |
| alpha | 25 | 13890.899 | 13273.781 | 23041.899 | 0.956 | 1.659 | 1.736 |
|  | 27 | 12997.832 | 12878.395 | 22536.822 | 0.991 | 1.734 | 1.750 |
|  | 30 | 12991.921 | 12708.52 | 22159.925 | 0.978 | 1.706 | 1.744 |
|  | 33 | 12600.891 | 12315.385 | 20889.486 | 0.977 | 1.658 | 1.696 |
|  | 38 | 11835.699 | 11127.715 | 18743.736 | 0.940 | 1.584 | 1.684 |
| beta | 27 | 4791.643 | 4270.097 | 10635.058 | 0.891 | 2.220 | 2.491 |
|  | 30 | 5034.199 | 4345.924 | 11390.935 | 0.863 | 2.263 | 2.621 |
|  | 33 | 5606.589 | 5114.711 | 11959.278 | 0.912 | 2.133 | 2.338 |
|  | 37 | 6474.76 | 5649.957 | 12634.541 | 0.873 | 1.951 | 2.236 |
|  | 40 | 6577.924 | 6026.426 | 12640.646 | 0.916 | 1.922 | 2.098 |

1 Xeon X5440 2.93GHz , 16GB RAM, Windows XP SP2（x64）, BasketballDrive_1920x1080_50

Table1 makes it clear that the encoding time of AFWQ is observed no increasing comparing to the KTA JM anchor. In addition, it is observed that AFWQ saving encoding time a little than the anchor. That may because after frequency weighting in AFWQ, transformed coefficients becomes easily compressed comparing to the anchor, which may result in saving time in other codec module, entropy coding module etc. With comparison to AQMS, the encoding time of AQMS is about 1.7-2.6 times to AFWQ.

## Decoding time and measurement methodology and comparison

Table 2  Decoding time comparison

|  |  | DecodingTime(second) |  |  | DecodingTime(times) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | JM | AFWQ | AQMS | AFWQ_vs_JM | AQMS_vs_JM | AQMS_vs_AFWQ |
| alpha | 25 | 64.071 | 63.808 | 64.981 | 0.996 | 1.014 | 1.018 |
|  | 27 | 62.863 | 64.189 | 64.732 | 1.021 | 1.030 | 1.008 |
|  | 30 | 63.407 | 63.721 | 64.421 | 1.005 | 1.016 | 1.011 |
|  | 33 | 61.595 | 61.988 | 63.233 | 1.006 | 1.027 | 1.020 |
|  | 38 | 60.034 | 60.306 | 60.229 | 1.005 | 1.003 | 0.999 |
| beta | 27 | 54.241 | 55.925 | 55.169 | 1.031 | 1.017 | 0.986 |
|  | 30 | 60.488 | 54.394 | 54.775 | 0.899 | 0.906 | 1.007 |
|  | 33 | 53.647 | 54.098 | 54.003 | 1.008 | 1.007 | 0.998 |
|  | 37 | 52.039 | 51.879 | 54.315 | 0.997 | 1.044 | 1.047 |
|  | 40 | 50.898 | 50.676 | 50.868 | 0.996 | 0.999 | 1.004 |

1 Xeon X5460 3.16GHz , 16GB RAM, Windows XP professional x64 edition. SP2（x64）, BasketballDrive_1920x1080_50

It is observed from table 2，no decoding time increasing for AQMS and AFWQ.

## Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2

(64b versus 32b executables, clock speeds, use of multicore parallel processing, memory, hard drive characteristics, etc.)

Simulation platform in section 4.1 and 4.2 is as follows,

64bit executables,  Xeon X5460 3.16GHz , 16GB RAM, Windows XP professional x64 edition. SP2（x64）

## Complexity characteristics of encoder quantization and quantization type selection

Frequency band weighting parameters and weighting model index is loaded in slice level. Hence frequency weighting matrices is computed in slice level. Each frame needs such computation only one times. The complexity for frequency weighting can be ignored. Quantization mode selection is determined in macroblock level. For selection of quantization mode, the weighting matrices that are corresponding to the quantization mode should be stored in memory. However, the memory usage is very small. Therefore, the complexity of AFWQ can be ignored. It can also be observed from Table1.

## Complexity characteristics of decoder inverse quantization

Quantization mode selection is performed for each macroblock, and then frequency weighting need to performed on the macroblock. Hence, each block need an additional weighting operation for scaling coefficients or transformed coefficients.

# Closing remarks

AFWQ is currently implemented as one pass quantization tool. It implements non–uniform quantization in macroblock level for considering the property of local textures. Fewer bits used in picture level and no extra bits needed in macroblock.

References

[1]  MPEG output document, N11113, “Joint Call for Proposals on Video Compression Technology”

[2]  Gisle Bjontegaard, "Calculation of Average PSNR Differences between RD curves", ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[3]  Gisle Bjontegaard, "Improvements of the BD-PSNR model", ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, 16th - 18th July, 2008, Doc.VCEG-AI11.

# Patent rights declaration(s)

Huawei Technologies may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).