| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A030 |
| --- | --- |

| Title: | AVC based Intra Prediction for Improved Visual Quality |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Contact(s): | Ali Tabatabai 1730 N. First Street San Jose, CA, USA Teruhiko Suzuki Gate City Osaki West 12F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, JAPAN | Tel: Email: Tel: Email: | +1-408-352-4715 Ali.Tabatabai@am.sony.com +81-3-5435-3308 teruhikos@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In the current AVC intra coding framework, intra prediction is performed to remove the correlation between the pixels in the current block and reconstructed neighbors. There are DC prediction plus 8 directional prediction modes defined. The prediction mode number is signaled to the decoder using predictive coding. The current intra prediction has two major disadvantages: 1) the 8 directional modes does not provide sufficient precision to cover arbitrary directional patterns; and 2) the mode number prediction from neighbors is not accurate enough to exploit the geometric dependency between blocks. In this submission, we propose to extend AVC based intra prediction to improve the visual quality as well the coding performance for next generation video codec. Our proposed method leverages the inter-block geometric dependency at the decoder side. More specifically, prediction direction detection is performed at the decoder-side, in the reconstructed neighborhood of the current block to get the a priori prediction direction. Then the encoder sends side-information to signal the decoder whether to use this new prediction direction or AVC based prediction direction. Since the prediction direction detection is conducted at the decoder side, a high direction resolution can be obtained without requiring additional overhead bits in signaling. Simulation results shows that the new intra prediction method can provide as much as 10% bit-rate reduction compared to AVC intra prediction.

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).