| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A203 |
| --- | --- |

| Title: | Table of Proposal Design Elements for High Efficiency Video Coding (HEVC) |
| --- | --- |
| Status: | Approved output document |
| Source: | JCT-VC |

_____________________________

# Abstract

This document provides a table that identifies the algorithmic elements of the proposals submitted to the Joint Call for Proposals (CfP) on Video Compression Technology and their association to the proposal contributions.

| Proposal | Picture partitioning | Motion prediction, compensation & encoding | Intra prediction | Transform | Quantization | Loop filter | Entropy coding | Frame Buffer | Software |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JCTVC-A101 (TI & MIT) |  |  |  | OMDDT (orthogon. mode dep. directional transform) |  |  | Massively parallel CABAC | Compressed frame buffer (lossy) | KTA2.6r1 |
| JCTVC-A102 (Hitachi) | Extended block size up to 64x64 MB | MV competition E-AIF |  |  |  |  |  |  | Only 3 tools added to AVC |
| JCTVC-A103 (Sony) |  | Separable fixed interpolation filter Separable adaptive interpolation filter Recursive adaptive interpolation filter Adaptive MV predictor selection |  | Directional transforms (DCT, Haar Wavelet) |  | Separable adaptive loop filter |  |  | KTA |
| JCTVC-A104 (NEC) | Extended MB sizes up to 32x32 | MV competition |  | 16x16 integer DCT |  | Joint de-blocking/de-banding filter/ comfort noise Wiener filter (adaptive symmetric 5x5) |  |  | KTA |
| JCTVC-A105 (Sharp) | Superblock consisting of 2x2 Macroblocks | MV competition Multiple E-AIF (two parameter sets) | Parallel Intra with Adaptive multi-directional intra pred | 16x16 DCT |  | Loop filtering (QALF) with adaptive clipping | Parallel entropy coding by “entropy slices” |  | JM15.1 |
| JCTVC-A106 (Intel) |  | Self derivation of motion estimation (at decoder) |  |  |  | Adaptive Wiener filter (modified BALF and QALF) |  |  | Basis KTA2.6r1 |
| JCTVC-A107 (Mitsubishi) | MB Size extension (up to 64x64) | Hierarchical motion partitions Non-rectangular motion partition shapes Adaptive motion prediction (extension of MV comp.) Improved direct mode (selection of spatial/temporal candidate without signaling, SAD competition at decoder and encoder) MB-adaptive WP | Block-based pyramid prediction | 16x16 DCT Directional transforms |  | Enhanced BALF (two Wiener filters adaptively switched) Modified de-blocking filter |  |  | C Software written from scratch, multiple OS |
| JCTVC-A108 (JVC) |  | Geometry transform prediction of MVs Decoder-side block boundary decision Refinement of MC by decoder-side estimation | AC ramp prediction using DC and boundary |  |  | QALF from KTA |  |  | JM16.2 |
| JCTVC-A109 (MediaTek) | MB Size extension (up to 64x64) | Advanced adaptive IF Decoder-side MV derivation with various adaptations Enhanced spatio-temporal direct mode Scaled motion vector prediction | Enhanced intra prediction: Overlapped block, 9 modes for 16x16, context-dependent mode | DCT up to 32x32 Multiple model KLT for Intra |  | Adaptive LF: QALF with multiple filters and more granular switching | Adaptive scan of transform coefficients for intra |  | KTA2.6 |
| JCTVC-A110 (LG) | MB Size extension (up to 64x64) Inter-Intra mixed mode, partial skip | Scaled MV predictor Template-based illumination compensation Modified motion vector competition | Intra prediction for 32x32 Chroma-from-luma intra prediction | New MDDT kernels |  | Adaptive de-blocking filter based on Wiener filter, QALF |  |  | JM 11 |
| JCTVC-A111 (Huawei & Hisilicon) |  | Flexible macroblock partitions: H/V/D at arbitrary positions Template-based motion derivation Template-based inter-frame DC offset Second order prediction for residual signal with directional characteristics | Resample-based intra prediction Line-based intra prediction | RD optimized transform for intra prediction: Separable KLT Directional transform (non-separable KLT) to the residual signal of MC prediction | Adaptive frequency-weighted quantization |  |  |  | KTA 2.6 |
| JCTVC-A112 (RWTH Aachen) |  | Decoder-side motion vector derivation, P and B pictures, extended to larger MB sizes, candidate-based fast search |  |  |  |  |  |  | KTA2.6 |
| JCTVC-A113 (SK telecom, Sejong Univ. & Sungkyunkwan Univ.) | 32x32 MB and new partitions Tree coding for partition type | Adaptive motion vector precision (down to 1/8 pel) | Intra prediction of larger blocks, special coding of modes | Up to 16x16 DCT MDDT |  | QALF+HPF |  |  | Modified JM 15.2 |
| JCTVC-A114 (France Telecom, NTT, NTT DOCOMO, Panasonic, & Technicolor) | Basic structure: superblocks 8x8 | 1/8 pel precision for motion Basic motion unit 8x8, boundary can be diplaced 2 or 4 pixels MV comp. in P 2 interpolation modes: fixed and Wiener Zero-tree coding of (differential) motion information, displacement offset etc. | Intrapred. 8x8, 4x4, 8x2, 2x8 Directional pred. generalized for 16x16, special edge prediction mode intra template matching averaging Chroma intra prediction with adaptive filtering | 16x16 and 8x2, 2x8 transform for intra, switch to fixed KLT 16x16 for intra | Internal bit-depth increase to 14 bits Finer QP resolution (16 levels instead of 6), quant matrices | Non-linear denoising: Thresholding in over-complete transform domain (one 8x8 DCT per pixel) and frame-adaptive de-noising by Wiener filter (using prediction and residual additional to reconstruction) De-blocking similar to AVC can be signaled to be placed before or after de-noising | Zero-tree coding (binary splits) for sub-partitioning (grouping) of coefficient plane |  | Software written from scratch C++, multi-platform |
| JCTVC-A115 (Fujitsu) |  |  |  |  |  |  | Encoding of signs of DCT coefficients in CABAC based on sign estimation |  | Based on JM 16.1 |
| JCTVC-A116 (Fraunhofer HHI) | Flexible quadtree-based picture plane grouping, nested prediction and transform sub-trees | Generalized multi-hypothesis (more than 2) “Interleaved” motion prediction and coding for H/V Motion partition merging (replacing skip and direct mode) New interpolation filter “MOMS” (IIR/FIR) |  | Integer DCT up to NxN (up to 64x64 used) | Flexible internal bit-depth handling up to 14 bits | Wiener filter 1D sequential, quad-tree adaptation | Novel context-adaptive entropy coding: 12 interleaved threads with fixed Huffman tables |  | Software from scratch, C++ |
| JCTVC-A117 (Toshiba) | MB Size extension (up to 64x64) | New high accuracy interpolation filter Spatio-Temporal Direct Selection | Bidirectional weighted intra pred. | 16x16 DCT 16x8 DCT, 8x16 DCT DUT (Directional Unified Transform, 4x4, 8x8, 16x16) for intra | Subjectively Adaptive Quantization Matrix Selection | QALF with circular shape filters | Mode dependent fixed alternative scan for intra transform coefficients | IBDI(Internal Bit Depth Increase to 12 bits) with Decoded Picture Compression (to 8 bits) | C++ Software written from scratch |
| JCTVC-A118 (Microsoft) |  |  | Line-based/pixel based coding modes for intra; prediction uses switchable, Wiener, predefined directional filters or template matching |  |  | Content (edge) adaptive de-blocking |  |  | KTA 2.4 |
| JCTVC-A119 (Tandberg, Ericsson, Nokia) | Motion partitions up to 64x64 and down to 8x8 (no 32x16, 64x32, but 16x8) | Improved skip mode 2 candidates Low complexity direct mode Interpolation: Directional and separable filters switched | Intra prediction: 16x16 DC, h/v & planar; 8x8 DC and 32 angular predictions; 4x4 DC, H/V | DCT up to 64x64, 16 bit integer Spatial varying transform in inter, sizes 8x8, 16x4, 4x16 |  |  | VLC with improved context adaptivity |  | C Software written from scratch |
| JCTVC-A120 (RIM) |  |  |  |  | Soft-decision quantization (encoder optimization, adaptive) trellis) |  | Parallel entropy coding framework where context modeling and entropy coding engine are decoupled. Load balancing v2v codes as entropy coding engine. v2v codes in serial modes using primary and secondary Huffman codes |  | KTA 2.6r1 |
| JCTVC-A121 (Qualcomm) | Extended MB | Geometric partitioning Adaptive motion vector resolution (1/4 and 1/8) Switched interpolation filters with offset: 6-tap, 8-tap and 4x4 non-separable P direct mode Scaled MV prediction | MDDT 16x16 transform (LLM factorization) |  |  | In-loop filter (modified, merge of QALF and post filter, up to 16 filters, diamond-shaped filters to reduce overhead and complexity) |  |  | KTA optimized (faster) |
| JCTVC-A122 (NHK & Mitsubishi) | MB Size extension (up to 64x64) | Hierarchical motion partitions Adaptive direct vector derivation Spatio-temporal adaptive PMV decision Non-rectangular motion partition shapes | New 16x16 intra prediction modes: Global plane (surface) prediction, iterative adjustment prediction | 16x16 DCT Discrete Sine Transform (DST) |  | Enhanced BALF (two Wiener filters adaptively switched) Modified de-blocking filter |  |  | C Software from scratch, multiple OS (same platform as A107) |
| JCTVC-A123 (NCTU) |  | Parametric OBMC |  |  |  |  |  |  | KTA 2.4r1 |
| JCTVC-A124 Samsung & BBC | Flexible block structure (coding unit, prediction unit, transform unit), quad-tree based | Asymmetric partitions such as 1/4:3/4 Signaling of motion pred. choice DCT based interpolation filter High accuracy motion (1/12 bit refinement) | New intra pred methods: Arbitrary direction intra prediction, color comp. corr. pred. for intra, pixel-based template matching, combined intra prediction (ADI with local mean) | Integer DCT NxN (up to 64x64 used) Rotational DCT “Logical transform” with a wavelet-like tree for larger blocks | IBDI | Loop filtering: de-blocking, extreme correction, band correction, content adaptive dynamic range, ALF re-using CU partitions | Syntax-based CABAC, coding engine similar to JPEG annex D Adaptive coefficient scanning (hor/vert/diag) |  | C/C++ built from scratch (not object oriented) |
| JCTVC-A125 (BBC & Samsung; A124 without some or slightly modified tools) |  | No asymm. Partitions | No pixel-based template matching |  | IBDI | De-blocking Coding Unit - Based ALF |  |  | Same as A124 |
| JCTVC-A126 (Renesas) |  |  | Intra vector prediction by block matching Intra repetitive pixel replenishment based on template matching |  |  |  |  |  | KTA |
| JCTVC-A127 (ETRI) | Extended MB size 32x32 |  | Intra prediction mode 32x32, intra prediction 16x16 with directions Recursive intra prediction, pre-filter at boundaries before prediction, post filter after prediction | Up to 32x32 MDDT |  | Simplified deblocking filter, only boundary pixels Enhanced QALF |  |  | KTA2.3 |