| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A205 |
| --- | --- |

| Title: | Test Model under Consideration |
| --- | --- |
| Status: | Output Document (draft) |
| Source: | JCT-VC |

_____________________________

# Abstract

This document provides the description for a Test Model under consideration. This suggested Test Model is intended to provide a coding efficiency close to the best performing proposals in the subjective test of the CfP submissions and also to provide a complexity point that is close to that of the lowest complexity submissions shown to provide substantial improvement of coding efficiency. The inclusion of a technology in this document does not indicate a final adoption of the technology as an element of an approved test model of the committee. Rather, it indicates a preliminary selection which may require further evaluation and justification to achieve that status.

Abstract	1

1	Overview	2

2	Unit definition	2

2.1 Coding Tree Block (CTB)	26

2.1	Prediction unit (PU)	28

2.2	Transform unit (TU)	30

3	Motion representation	31

3.1	Motion vector prediction for rectangular partitions	31

3.2	Motion vector prediction for geometric block partitions	33

3.3	Interpolation Methods	34

3.3.1	Single pass switched Interpolation Filters with offsets (single pass SIFO)	34

3.3.2	Choice of filter set and offsets	35

3.3.3	Partition based Illumination Compensation	36

3.4	Adaptive Motion Vector Resolution	36

4	Intra-frame prediction	36

4.1	Adaptive reference sample smoothing	36

4.2	Planar prediction	36

4.3	Angular prediction	37

4.4	Arbitrary Directional Intra (ADI)	38

4.5	Edge based prediction	39

4.6	Combined Intra Prediction (CIP)	39

5	Spatial transforms	40

5.1	Large transform (16x16, 32x32, 64x64)	40

5.2	Rotational transform (ROT)	41

5.3	Mode dependent directional transforms for intra-prediction residuals	42

5.4	Transform Selection	43

5.5	Quantization	43

6	Deblocking filter	44

Luma filtering	44

Chroma filtering	45

Planar mode filtering	45

7	In-loop filtering	46

8	Entropy Coding	46

8.1	Low-Complexity Entropy Coding	46

8.2	High Coding Efficiency Entropy Coding	49

8.2.1	Novel entropy coding concept	49

8.2.2	V2V codes	50

8.2.3	Load Balancing	51

8.2.4	Binarization and context modeling	53

8.2.5	Improved context modeling scheme for coding of transform coefficients	53

# Overview

This suggested Test Model is intended to provide a coding efficiency close to the best performing proposals in the subjective test of the CfP submissions and also to provide a complexity point that is close to that of the lowest complexity submissions shown to provide substantial improvement of coding efficiency. The inclusion of a technology in this document does not indicate a final adoption of the technology as an element of an approved test model of the committee. Rather, it indicates a preliminary selection which may require further evaluation and justification to achieve that status.

Note that the bit depth for signal processing flow elements in this design is increased relative to that for conventional 8 bit video coding design elements, although the picture buffer storage bit depth may be 8 bits when coding 8 bit source video.

# Syntax and Semantics

## Syntax

### NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| NumBytesInRBSP = 0 |  |  |
| nalUnitHeaderBytes = 1 |  |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| i += 2 |  |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | All | f(8) |
| } else |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| } |  |  |
| } |  |  |

### Sequence parameter set RBSP syntax

[Ed.: Parameters should be removed and/or added as appropriate.]

| seq_parameter_set_rbsp( ) { | C |  | Descriptor |  |
| --- | --- | --- | --- | --- |
| profile_idc | 0 |  | u(8) |  |
| reserved_zero_8bits /* equal to 0 */ | 0 |  | u(8) |  |
| level_idc | 0 |  | u(8) |  |
| seq_parameter_set_id | 0 |  | ue(v) |  |
| chroma_format_idc | 0 |  | ue(v) |  |
| if( chroma_format_idc = = 3 ) |  |  |  |  |
| separate_colour_plane_flag | 0 |  | u(1) |  |
| bit_depth_luma_minus8 | 0 |  | ue(v) |  |
| bit_depth_chroma_minus8 | 0 |  | ue(v) |  |
| qpprime_y_zero_transform_bypass_flag | 0 |  | u(1) |  |
| seq_scaling_matrix_present_flag | 0 |  | u(1) |  |
| if( seq_scaling_matrix_present_flag ) |  |  |  |  |
| ... to be done ... |  |  |  |  |
| log2_max_frame_num_minus4 | 0 |  | ue(v) |  |
| pic_order_cnt_type | 0 |  | ue(v) |  |
| if( pic_order_cnt_type = = 0 ) |  |  |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 |  | ue(v) |  |
| else if( pic_order_cnt_type = = 1 ) { |  |  |  |  |
| delta_pic_order_always_zero_flag | 0 |  | u(1) |  |
| offset_for_non_ref_pic | 0 |  | se(v) |  |
| offset_for_top_to_bottom_field | 0 |  | se(v) |  |
| num_ref_frames_in_pic_order_cnt_cycle | 0 |  | ue(v) |  |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |  |  |
| offset_for_ref_frame[ i ] | 0 |  | se(v) |  |
| } |  |  |  |  |
| max_num_ref_frames | 0 |  | ue(v) |  |
| gaps_in_frame_num_value_allowed_flag | 0 |  | u(1) |  |
| log2_min_coding_unit_size_minus3 |  | 0 |  | ue(v) |
| max_coding_unit_hierarchy_depth |  | 0 |  | ue(v) |
| log2_min_transform_unit_size_minus2 |  | 0 |  | ue(v) |
| max_transform_unit_hierarchy_depth |  | 0 |  | ue(v) |
| pic_width_in_mbs_minus1 | 0 |  | ue(v) |  |
| pic_height_in_map_units_minus1 | 0 |  | ue(v) |  |
| frame_mbs_only_flag | 0 |  | u(1) |  |
| if( !frame_mbs_only_flag ) |  |  |  |  |
| mb_adaptive_frame_field_flag | 0 |  | u(1) |  |
| direct_8x8_inference_flag | 0 |  | u(1) |  |
| frame_cropping_flag | 0 |  | u(1) |  |
| if( frame_cropping_flag ) { |  |  |  |  |
| frame_crop_left_offset | 0 |  | ue(v) |  |
| frame_crop_right_offset | 0 |  | ue(v) |  |
| frame_crop_top_offset | 0 |  | ue(v) |  |
| frame_crop_bottom_offset | 0 |  | ue(v) |  |
| } |  |  |  |  |
| vui_parameters_present_flag | 0 |  | u(1) |  |
| if( vui_parameters_present_flag ) |  |  |  |  |
| vui_parameters( ) | 0 |  |  |  |
| rbsp_trailing_bits( ) | 0 |  |  |  |
| } |  |  |  |  |

### Picture parameter set RBSP syntax

[Ed.: Parameters should be removed and/or added as appropriate.]

| pic_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 1 | ue(v) |
| seq_parameter_set_id | 1 | ue(v) |
| entropy_coding_mode_flag | 1 | u(1) |
| bottom_field_pic_order_in_frame_present_flag | 1 | u(1) |
| num_slice_groups_minus1 | 1 | ue(v) |
| if( num_slice_groups_minus1 > 0 ) { |  |  |
| slice_group_map_type | 1 | ue(v) |
| if( slice_group_map_type = = 0 ) |  |  |
| for( iGroup = 0; iGroup <= num_slice_groups_minus1; iGroup++ ) |  |  |
| run_length_minus1[ iGroup ] | 1 | ue(v) |
| else if( slice_group_map_type = = 2 ) |  |  |
| for( iGroup = 0; iGroup < num_slice_groups_minus1; iGroup++ ) { |  |  |
| top_left[ iGroup ] | 1 | ue(v) |
| bottom_right[ iGroup ] | 1 | ue(v) |
| } |  |  |
| else if( slice_group_map_type = = 3 ¦ ¦ slice_group_map_type = = 4 ¦ ¦ slice_group_map_type = = 5 ) { |  |  |
| slice_group_change_direction_flag | 1 | u(1) |
| slice_group_change_rate_minus1 | 1 | ue(v) |
| } else if( slice_group_map_type = = 6 ) { |  |  |
| pic_size_in_map_units_minus1 | 1 | ue(v) |
| for( i = 0; i <= pic_size_in_map_units_minus1; i++ ) |  |  |
| slice_group_id[ i ] | 1 | u(v) |
| } |  |  |
| } |  |  |
| num_ref_idx_l0_default_active_minus1 | 1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | 1 | ue(v) |
| weighted_pred_flag | 1 | u(1) |
| weighted_bipred_idc | 1 | u(2) |
| pic_init_qp_minus26 /* relative to 26 */ | 1 | se(v) |
| pic_init_qs_minus26 /* relative to 26 */ | 1 | se(v) |
| chroma_qp_index_offset | 1 | se(v) |
| deblocking_filter_control_present_flag | 1 | u(1) |
| constrained_intra_pred_flag | 1 | u(1) |
| redundant_pic_cnt_present_flag | 1 | u(1) |
| if( more_rbsp_data( ) ) { |  |  |
| transform_8x8_mode_flag | 1 | u(1) |
| pic_scaling_matrix_present_flag | 1 | u(1) |
| if( pic_scaling_matrix_present_flag ) |  |  |
| ... to be done ... |  |  |
| second_chroma_qp_index_offset | 1 | se(v) |
| } |  |  |
| rbsp_trailing_bits( ) | 1 |  |
| } |  |  |

### Slice layer RBSP syntax

| slice_layer_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| slice_header( ) | 2 |  |
| slice_data( ) /* all categories of slice_data( ) syntax */ | 2 ¦ 3 ¦ 4 |  |
| rbsp_slice_trailing_bits( ) | 2 |  |
| } |  |  |

### RBSP slice trailing bits syntax

| rbsp_slice_trailing_bits( ) { | C | Descriptor |
| --- | --- | --- |
| rbsp_trailing_bits( ) | All |  |
| if( entropy_coding_mode_flag ) |  |  |
| while( more_rbsp_trailing_data( ) ) |  |  |
| cabac_zero_word /* equal to 0x0000 */ | All | f(16) |
| } |  |  |

### RBSP trailing bits syntax

| rbsp_trailing_bits( ) { | C | Descriptor |
| --- | --- | --- |
| rbsp_stop_one_bit /* equal to 1 */ | All | f(1) |
| while( !byte_aligned( ) ) |  |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | All | f(1) |
| } |  |  |

### Slice header syntax

[Ed.: Parameters should be removed and/or added as appropriate.]

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id | 2 | u(2) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| ref_pic_list_modification( ) | 2 |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| mc_interpolation_idc | 2 | ue(v) |
| mv_competition_flag | 2 | u(1) |
| } |  |  |

#### Reference picture list modification syntax

| ref_pic_list_modification( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type % 5 != 2 && slice_type % 5 != 4 ) { |  |  |
| ref_pic_list_modification_flag_l0 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| if( slice_type % 5 = = 1 ) { |  |  |
| ref_pic_list_modification_flag_l1 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |

#### Prediction weight table syntax

| pred_weight_table( ) { | C | Descriptor |
| --- | --- | --- |
| luma_log2_weight_denom | 2 | ue(v) |
| if( ChromaArrayType != 0 ) |  |  |
| chroma_log2_weight_denom | 2 | ue(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |  |
| luma_weight_l0_flag | 2 | u(1) |
| if( luma_weight_l0_flag ) { |  |  |
| luma_weight_l0[ i ] | 2 | se(v) |
| luma_offset_l0[ i ] | 2 | se(v) |
| } |  |  |
| if ( ChromaArrayType != 0 ) { |  |  |
| chroma_weight_l0_flag | 2 | u(1) |
| if( chroma_weight_l0_flag ) |  |  |
| for( j =0; j < 2; j++ ) { |  |  |
| chroma_weight_l0[ i ][ j ] | 2 | se(v) |
| chroma_offset_l0[ i ][ j ] | 2 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if( slice_type % 5 = = 1 ) |  |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |  |
| luma_weight_l1_flag | 2 | u(1) |
| if( luma_weight_l1_flag ) { |  |  |
| luma_weight_l1[ i ] | 2 | se(v) |
| luma_offset_l1[ i ] | 2 | se(v) |
| } |  |  |
| if( ChromaArrayType != 0 ) { |  |  |
| chroma_weight_l1_flag | 2 | u(1) |
| if( chroma_weight_l1_flag ) |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| chroma_weight_l1[ i ][ j ] | 2 | se(v) |
| chroma_offset_l1[ i ][ j ] | 2 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Decoded reference picture marking syntax

| dec_ref_pic_marking( ) { | C | Descriptor |
| --- | --- | --- |
| if( IdrPicFlag ) { |  |  |
| no_output_of_prior_pics_flag | 2 ¦ 5 | u(1) |
| long_term_reference_flag | 2 ¦ 5 | u(1) |
| } else { |  |  |
| adaptive_ref_pic_marking_mode_flag | 2 ¦ 5 | u(1) |
| if( adaptive_ref_pic_marking_mode_flag ) |  |  |
| do { |  |  |
| memory_management_control_operation | 2 ¦ 5 | ue(v) |
| if( memory_management_control_operation = = 1 ¦ ¦ memory_management_control_operation = = 3 ) |  |  |
| difference_of_pic_nums_minus1 | 2 ¦ 5 | ue(v) |
| if(memory_management_control_operation = = 2 ) |  |  |
| long_term_pic_num | 2 ¦ 5 | ue(v) |
| if( memory_management_control_operation = = 3 ¦ ¦ memory_management_control_operation = = 6 ) |  |  |
| long_term_frame_idx | 2 ¦ 5 | ue(v) |
| if( memory_management_control_operation = = 4 ) |  |  |
| max_long_term_frame_idx_plus1 | 2 ¦ 5 | ue(v) |
| } while( memory_management_control_operation != 0 ) |  |  |
| } |  |  |
| } |  |  |

### Slice data syntax

[Ed. first draft]

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| do { |  |  |
| xCU = HorLumaLocation( CurrMbAddr ) |  |  |
| yCU = VerLumaLocation( CurrMbAddr ) |  |  |
| coding_unit( xCU, yCU, MaxCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

### Coding unit syntax

[Ed. first draft]

| coding_unit( x0, y0, currCodingUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( currCodingUnitSize > MinCodingUnitSize ) |  |  |
| split_coding_unit_flag | 2 | u(1) ¦ ae(v) |
| if( split_coding_unit_flag ) { |  |  |
| splitCodingUnitSize = currCodingUnitSize >> 1 |  |  |
| x1 = x0 + splitCodingUnitSize |  |  |
| y1 = y0 + splitCodingUnitSize |  |  |
| coding_unit( x0, y0, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL ) |  |  |
| coding_unit( x1, y0, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| if( y1 < PicHeightInSamplesL ) |  |  |
| coding_unit( x0, y1, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |  |
| coding_unit( x1, y1, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| } else { |  |  |
| prediction_unit( x0, y0, currCodingUnitSize ) | 2 |  |
| transform_unit( x0, y0, currCodingUnitSize ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |

### Prediction unit syntax

[Ed. first draft]

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| intra_prediction_flag | 2 | u(1) ¦ ae(v) |
| if( intra_prediction_flag ) { |  |  |
| intra_split_flag | 2 | u(1) ¦ ae(v) |
| for( i = 0; i < ( intra_split_flag ? 4 : 1 ); i++ ) |  |  |
| intra_prediction_mode | 2 | ue(v) ¦ ae(v) |
| } else { |  |  |
| inter_partitioning_idc | 2 | ue(v) ¦ ae(v) |
| for( i= 0; i < NumInterPartitions; i++ ) { |  |  |
| if( NumMergeCandidates > 0 ) { |  |  |
| merge_flag | 2 | ue(1) ¦ ae(v) |
| if( merge_flag && NumMergeCandidates > 1 ) |  |  |
| merge_left_flag | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| if( !merge_flag ) { |  |  |
| inter_pred_idc | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L1 ) { |  |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |  |
| ref_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| mvd_l0[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l1[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| if( inter_pred_idc != Pred_L0 ) { |  |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |  |
| ref_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| mvd_l1[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l1[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Transform unit syntax

[Ed. first draft]

| transform_unit( x0, y0, currTransformUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( currTransformUnitSize > MinTransformUnitSize && currTransformUnitSize <= MaxTransformUnitSize ) |  |  |
| split_transform_unit_flag | 3 ¦ 4 | u(1) ¦ ae(v) |
| if( split_transform_unit_flag ) { |  |  |
| splitTransformUnitSize = currTransformUnitSize >> 1 |  |  |
| x1 = x0 + splitTransformUnitSize |  |  |
| y1 = y0 + splitTransformUnitSize |  |  |
| transform_unit( x0, y0, splitTransformUnitSize) | 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL ) |  |  |
| transform_unit( x1, y0, splitTransformUnitSize) | 3 ¦ 4 |  |
| if( y1 < PicHeightInSamplesL ) |  |  |
| transform_unit( x0, y1, splitTransformUnitSize) | 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |  |
| transform_unit( x1, y1, splitTransformUnitSize) | 3 ¦ 4 |  |
| } else { |  |  |
| coded_block_flag | 3 ¦ 4 | u(1) ¦ ae(v) |
| { transform coefficient values } // to be done |  |  |
| } |  |  |
| } |  |  |

## Semantics

### NAL unit semantics

NumBytesInNALunit specifies the size of the NAL unit in bytes. This value is required for decoding of the NAL unit. Some form of demarcation of NAL unit boundaries is necessary to enable inference of NumBytesInNALunit. One such demarcation method is specified in Annex B of ITU-T Rec. H.264 | ISO/IEC 14496-10 for the byte stream format.

forbidden_zero_bit shall be equal to 0.

nal_ref_idc not equal to 0 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, or a slice of a reference picture. nal_ref_idc equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture.

nal_ref_idc shall not be equal to 0 for sequence parameter set or picture parameter set NAL units. When nal_ref_idc is equal to 0 for one slice data NAL unit of a particular picture, it shall be equal to 0 for all slice data NAL units of the picture. nal_ref_idc shall not be equal to 0 for NAL units with nal_unit_type equal to 5.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 1.

The column marked "C" in Table 1 lists the categories of the syntax elements that may be present in the NAL unit. In addition, syntax elements with syntax category "All" may be present, as determined by the syntax and semantics of the RBSP data structure. The presence or absence of any syntax elements of a particular listed category is determined from the syntax and semantics of the associated RBSP data structure. VCL and non-VCL NAL units are specified in Table 1 in the column labelled " NAL unit type class".

When the value of nal_unit_type is equal to 5 for a slice data NAL unit of a particular picture, all slice data NAL units of the picture shall have nal_unit_type equal to 5.

Table 1 – NAL unit type codes, syntax element categories, and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | NAL unit type class |
| --- | --- | --- | --- |
| 0 | Unspecified |  | non-VCL |
| 1 | Coded slice of a non-IDR picture slice_layer_rbsp( ) | 2, 3, 4 | VCL |
| 2..4 | Reserved |  | non-VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | 2, 3 | VCL |
| 6 | Reserved |  | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | 0 | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | 1 | non-VCL |
| 9..23 | Reserved |  | non-VCL |
| 21..23 | Reserved |  | non-VCL |
| 24..31 | Unspecified |  | non-VCL |

In the text, coded slice NAL unit collectively refers to a coded slice of a non-IDR picture NAL unit or to a coded slice of an IDR picture NAL unit. The variable IdrPicFlag is specified as

IdrPicFlag = ( ( nal_unit_type  = =  5 )  ?  1  :  0 )		(1)

When the value of nal_unit_type is equal to 5 for a NAL unit containing a slice of a particular picture, the picture shall not contain NAL units with nal_unit_type equal to 1. Such a picture is referred to as an IDR picture.

rbsp_byte[ i ] is the i-th byte of an RBSP. An RBSP is specified as an ordered sequence of bytes as follows.

The RBSP contains an SODB as follows.

–	If the SODB is empty (i.e., zero bits in length), the RBSP is also empty.

–	Otherwise, the RBSP contains the SODB as follows:

1)	The first byte of the RBSP contains the (most significant, left-most) eight bits of the SODB; the next byte of the RBSP shall contain the next eight bits of the SODB, etc., until fewer than eight bits of the SODB remain.

2)	rbsp_trailing_bits( ) are present after the SODB as follows:

i)	The first (most significant, left-most) bits of the final RBSP byte contains the remaining bits of the SODB (if any).

ii)	The next bit consists of a single rbsp_stop_one_bit equal to 1.

iii)	When the rbsp_stop_one_bit is not the last bit of a byte-aligned byte, one or more rbsp_alignment_zero_bit is present to result in byte alignment.

3)	One or more cabac_zero_word 16-bit syntax elements equal to 0x0000 may be present in some RBSPs after the rbsp_trailing_bits( ) at the end of the RBSP.

Syntax structures having these RBSP properties are denoted in the syntax tables using an "_rbsp" suffix. These structures shall be carried within NAL units as the content of the rbsp_byte[ i ] data bytes. The association of the RBSP syntax structures to the NAL units shall be as specified in Table 1.

emulation_prevention_three_byte is a byte equal to 0x03. When an emulation_prevention_three_byte is present in the NAL unit, it shall be discarded by the decoding process.

The last byte of the NAL unit shall not be equal to 0x00.

Within the NAL unit, the following three-byte sequences shall not occur at any byte-aligned position:

–	0x000000

–	0x000001

–	0x000002

Within the NAL unit, any four-byte sequence that starts with 0x000003 other than the following sequences shall not occur at any byte-aligned position:

–	0x00000300

–	0x00000301

–	0x00000302

–	0x00000303

### Sequence parameter set RBSP semantics

[Ed.: Parameters should be removed and/or added as appropriate.]

profile_idc and level_idc indicate the profile and level to which the coded video sequence conforms.

reserved_zero_8bits shall be equal to 0. Decoders shall ignore the value of reserved_zero_8bits.

seq_parameter_set_id identifies the sequence parameter set that is referred to by the picture parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

chroma_format_idc specifies the chroma sampling relative to the luma sampling. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive.

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it shall be inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the primary coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb or Cr) that each use the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value.

Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows.

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

bit_depth_luma_minus8 specifies the bit depth of the samples of the luma array and the value of the luma quantisation parameter range offset QpBdOffsetY, as specified by

BitDepthY      = 8 + bit_depth_luma_minus8		(2)
QpBdOffsetY = 6 * bit_depth_luma_minus8		(3)

bit_depth_luma_minus8 shall be in the range of 0 to 6, inclusive.

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays and the value of the chroma quantisation parameter range offset QpBdOffsetC, as specified by

BitDepthC      = 8 + bit_depth_chroma_minus8		(4)
QpBdOffsetC = 6 * bit_depth_chroma_minus8		(5)

bit_depth_chroma_minus8 shall be in the range of 0 to 6, inclusive.

qpprime_y_zero_transform_bypass_flag equal to 1 specifies that, when QP′Y is equal to 0, a transform bypass operation for the transform coefficient decoding process and picture construction process prior to deblocking filter process shall be applied. qpprime_y_zero_transform_bypass_flag equal to 0 specifies that the transform coefficient decoding process and picture construction process prior to deblocking filter process shall not use the transform bypass operation.

seq_scaling_matrix_present_flag equal to 1 specifies that scaling lists are specified in the sequence parameter set RBSP. seq_scaling_matrix_present_flag equal to 0 specifies that scaling lists are not specified in the sequence parameter set RBSP.

log2_max_frame_num_minus4 specifies the value of the variable MaxFrameNum that is used in frame_num related derivations as follows:

MaxFrameNum = 2( log2_max_frame_num_minus4 + 4 )		(6)

The value of log2_max_frame_num_minus4 shall be in the range of 0 to 12, inclusive.

pic_order_cnt_type specifies the method to decode picture order count. The value of pic_order_cnt_type shall be in the range of 0 to 2, inclusive.

pic_order_cnt_type shall not be equal to 2 in a coded video sequence that contains any of the following:

–	an access unit containing a non-reference frame followed immediately by an access unit containing a non-reference picture,

–	two access units each containing a field with the two fields together forming a complementary non-reference field pair followed immediately by an access unit containing a non-reference picture,

–	an access unit containing a non-reference field followed immediately by an access unit containing another non-reference picture that does not form a complementary non-reference field pair with the first of the two access units.

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )	(7)

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

delta_pic_order_always_zero_flag equal to 1 specifies that delta_pic_order_cnt[ 0 ] and delta_pic_order_cnt[ 1 ] are not present in the slice headers of the sequence and shall be inferred to be equal to 0. delta_pic_order_always_zero_flag equal to 0 specifies that delta_pic_order_cnt[ 0 ] is present in the slice headers of the sequence and delta_pic_order_cnt[ 1 ] may be present in the slice headers of the sequence.

offset_for_non_ref_pic is used to calculate the picture order count of a non-reference picture. The value of offset_for_non_ref_pic shall be in the range of −231 + 1 to 231 − 1, inclusive.

offset_for_top_to_bottom_field is used to calculate the picture order count of a bottom field. The value of offset_for_top_to_bottom_field shall be in the range of −231 + 1 to 231 − 1, inclusive.

num_ref_frames_in_pic_order_cnt_cycle is used in the decoding process for picture order count. The value of num_ref_frames_in_pic_order_cnt_cycle shall be in the range of 0 to 255, inclusive.

offset_for_ref_frame[ i ] is an element of a list of num_ref_frames_in_pic_order_cnt_cycle values used in the decoding process for picture order count. The value of offset_for_ref_frame[ i ] shall be in the range of −231 + 1 to 231 − 1, inclusive.

When pic_order_cnt_type is equal to 1, the variable ExpectedDeltaPerPicOrderCntCycle is derived by

ExpectedDeltaPerPicOrderCntCycle = 0
for( i = 0;  i < num_ref_frames_in_pic_order_cnt_cycle;  i++ )
	ExpectedDeltaPerPicOrderCntCycle += offset_for_ref_frame[ i ]	(8)

max_num_ref_frames specifies the maximum number of short-term and long-term reference frames, complementary reference field pairs, and non-paired reference fields that may be used by the decoding process for inter prediction of any picture in the sequence. max_num_ref_frames also determines the size of the sliding window operation. The value of max_num_ref_frames shall be in the range of 0 to MaxDpbFrames, inclusive.

gaps_in_frame_num_value_allowed_flag specifies the allowed values of frame_num and the decoding process in case of an inferred gap between values of frame_num.

log2_min_coding_unit_size_minus3 specifies the minimum size of a coding unit.

The variable MinCodingUnitSize is derived as

MinCodingUnitSize = 1 << ( log2_min_coding_unit_size_minus3 + 3 )	(9)

max_coding_unit_hierarchy_depth specifies the maximum hierarchy depth for coding units.

The variable MaxCodingUnitSize is derived as

MaxCodingUnitSize = 1 << ( log2_min_coding_unit_size_minus3 + 3 +
                                                max_coding_unit_hierarchy_depth )	(10)

The bitstream shall not contain data that result in a value of MaxCodingUnitSize that is greater than 128.

log2_min_transform_unit_size_minus2 specifies the minimum size of a transform unit.

The variable MinTransformUnitSize is derived as

MinTransformUnitSize = 1 << ( log2_min_transform_unit_size_minus2 + 2 )	(11)

max_transform_unit_hierarchy_depth specifies the maximum hierarchy depth for transform units.

The variable MaxTransformUnitSize is derived as

MaxTransformUnitSize = 1 << ( log2_min_transform_unit_size_minus3 + 3 +
                                                     max_transform_unit_hierarchy_depth )	(12)

The bitstream shall not contain data that result in a value of MaxTransformUnitSize that is greater than MaxCodingUnitSize.

pic_width_in_mbs_minus1 plus 1 specifies the width of each decoded picture in units of macroblocks.

The variable for the picture width in units of macroblocks is derived as

PicWidthInMbs = pic_width_in_mbs_minus1 + 1		(13)

The variable for picture width for the luma component is derived as

PicWidthInSamplesL = PicWidthInMbs * 16		(14)

The variable for picture width for the chroma components is derived as

PicWidthInSamplesC = PicWidthInMbs * MbWidthC	(15)

pic_height_in_map_units_minus1 plus 1 specifies the height in slice group map units of a decoded frame or field.

The variables PicHeightInMapUnits and PicSizeInMapUnits are derived as

PicHeightInMapUnits = pic_height_in_map_units_minus1 + 1	(16)
PicSizeInMapUnits = PicWidthInMbs * PicHeightInMapUnits	(17)

frame_mbs_only_flag equal to 0 specifies that coded pictures of the coded video sequence may either be coded fields or coded frames. frame_mbs_only_flag equal to 1 specifies that every coded picture of the coded video sequence is a coded frame containing only frame macroblocks.

Depending on frame_mbs_only_flag, semantics are assigned to pic_height_in_map_units_minus1 as follows.

–	If frame_mbs_only_flag is equal to 0, pic_height_in_map_units_minus1 plus 1 is the height of a field in units of macroblocks.

–	Otherwise (frame_mbs_only_flag is equal to 1), pic_height_in_map_units_minus1 plus 1 is the height of a frame in units of macroblocks.

The variable FrameHeightInMbs is derived as

FrameHeightInMbs = ( 2 − frame_mbs_only_flag ) * PicHeightInMapUnits	(18)

mb_adaptive_frame_field_flag equal to 0 specifies no switching between frame and field macroblocks within a picture. mb_adaptive_frame_field_flag equal to 1 specifies the possible use of switching between frame and field macroblocks within frames. When mb_adaptive_frame_field_flag is not present, it shall be inferred to be equal to 0.

direct_8x8_inference_flag specifies the method used in the derivation process for luma motion vectors for B_Skip, B_Direct_16x16 and B_Direct_8x8. When frame_mbs_only_flag is equal to 0, direct_8x8_inference_flag shall be equal to 1.

frame_cropping_flag equal to 1 specifies that the frame cropping offset parameters follow next in the sequence parameter set. frame_cropping_flag equal to 0 specifies that the frame cropping offset parameters are not present.

frame_crop_left_offset, frame_crop_right_offset, frame_crop_top_offset, frame_crop_bottom_offset specify the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in frame coordinates for output.

The variables CropUnitX and CropUnitY are derived as follows.

–	If ChromaArrayType is equal to 0, CropUnitX and CropUnitY are derived as

CropUnitX = 1		(19)
CropUnitY = 2 − frame_mbs_only_flag		(20)

–	Otherwise (ChromaArrayType is equal to 1, 2, or 3), CropUnitX and CropUnitY are derived as

CropUnitX = SubWidthC		(21)
CropUnitY = SubHeightC * ( 2 − frame_mbs_only_flag )	(22)

The frame cropping rectangle contains luma samples with horizontal frame coordinates from CropUnitX * frame_crop_left_offset to PicWidthInSamplesL − ( CropUnitX * frame_crop_right_offset + 1 ) and vertical frame coordinates from CropUnitY * frame_crop_top_offset to ( 16 * FrameHeightInMbs ) − ( CropUnitY * frame_crop_bottom_offset + 1 ), inclusive. The value of frame_crop_left_offset shall be in the range of 0 to ( PicWidthInSamplesL / CropUnitX ) − ( frame_crop_right_offset + 1 ), inclusive; and the value of frame_crop_top_offset shall be in the range of 0 to ( 16 * FrameHeightInMbs / CropUnitY ) − ( frame_crop_bottom_offset + 1 ), inclusive.

When frame_cropping_flag is equal to 0, the values of frame_crop_left_offset, frame_crop_right_offset, frame_crop_top_offset, and frame_crop_bottom_offset shall be inferred to be equal to 0.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having frame coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the frame coordinates of the specified luma samples.

For decoded fields, the specified samples of the decoded field are the samples that fall within the rectangle specified in frame coordinates.

vui_parameters_present_flag equal to 1 specifies that the vui_parameters( ) syntax structure is present. vui_parameters_present_flag equal to 0 specifies that the vui_parameters( ) syntax structure is not present.

### Picture parameter set RBSP semantics

[Ed.: Parameters should be removed and/or added as appropriate.]

pic_parameter_set_id identifies the picture parameter set that is referred to in the slice header. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

seq_parameter_set_id refers to the active sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

entropy_coding_mode_flag selects the entropy decoding method to be applied for the syntax elements for which two descriptors appear in the syntax tables as follows.

–	If entropy_coding_mode_flag is equal to 0, the method specified by the left descriptor in the syntax table is applied.

–	Otherwise (entropy_coding_mode_flag is equal to 1), the method specified by the right descriptor in the syntax table is applied.

bottom_field_pic_order_in_frame_present_flag equal to 1 specifies that the syntax elements delta_pic_order_cnt_bottom (when pic_order_cnt_type is equal to 0) or delta_pic_order_cnt[ 1 ] (when pic_order_cnt_type is equal to 1), which are related to picture order counts for the bottom field of a coded frame, are present in the slice headers for coded frames. bottom_field_pic_order_in_frame_present_flag equal to 0 specifies that the syntax elements delta_pic_order_cnt_bottom and delta_pic_order_cnt[ 1 ] are not present in the slice headers.

num_slice_groups_minus1 plus 1 specifies the number of slice groups for a picture. When num_slice_groups_minus1 is equal to 0, all slices of the picture belong to the same slice group.

slice_group_map_type specifies how the mapping of slice group map units to slice groups is coded. The value of slice_group_map_type shall be in the range of 0 to 6, inclusive.

slice_group_map_type equal to 0 specifies interleaved slice groups.

slice_group_map_type equal to 1 specifies a dispersed slice group mapping.

slice_group_map_type equal to 2 specifies one or more "foreground" slice groups and a "leftover" slice group.

slice_group_map_type values equal to 3, 4, and 5 specify changing slice groups. When num_slice_groups_minus1 is not equal to 1, slice_group_map_type shall not be equal to 3, 4, or 5.

slice_group_map_type equal to 6 specifies an explicit assignment of a slice group to each slice group map unit.

Slice group map units are specified as follows.

–	If frame_mbs_only_flag is equal to 0 and mb_adaptive_frame_field_flag is equal to 1 and the coded picture is a frame, the slice group map units are macroblock pair units.

–	Otherwise, if frame_mbs_only_flag is equal to 1 or the coded picture is a field, the slice group map units are units of macroblocks.

–	Otherwise (frame_mbs_only_flag is equal to 0 and mb_adaptive_frame_field_flag is equal to 0 and the coded picture is a frame), the slice group map units are units of two macroblocks that are vertically contiguous as in a frame macroblock pair of an MBAFF frame.

run_length_minus1[ i ] is used to specify the number of consecutive slice group map units to be assigned to the i-th slice group in raster scan order of slice group map units. The value of run_length_minus1[ i ] shall be in the range of 0 to PicSizeInMapUnits − 1, inclusive.

top_left[ i ] and bottom_right[ i ] specify the top-left and bottom-right corners of a rectangle, respectively. top_left[ i ] and bottom_right[ i ] are slice group map unit positions in a raster scan of the picture for the slice group map units. For each rectangle i, all of the following constraints shall be obeyed by the values of the syntax elements top_left[ i ] and bottom_right[ i ]:

–	top_left[ i ] shall be less than or equal to bottom_right[ i ] and bottom_right[ i ] shall be less than PicSizeInMapUnits.

–	( top_left[ i ] % PicWidthInMbs ) shall be less than or equal to the value of ( bottom_right[ i ] % PicWidthInMbs ).

slice_group_change_direction_flag is used with slice_group_map_type to specify the refined map type when slice_group_map_type is 3, 4, or 5.

slice_group_change_rate_minus1 is used to specify the variable SliceGroupChangeRate. SliceGroupChangeRate specifies the multiple in number of slice group map units by which the size of a slice group can change from one picture to the next. The value of slice_group_change_rate_minus1 shall be in the range of 0 to PicSizeInMapUnits − 1, inclusive. The SliceGroupChangeRate variable is specified as follows:

SliceGroupChangeRate = slice_group_change_rate_minus1 + 1	(23)

pic_size_in_map_units_minus1 is used to specify the number of slice group map units in the picture. pic_size_in_map_units_minus1 shall be equal to PicSizeInMapUnits − 1.

slice_group_id[ i ] identifies a slice group of the i-th slice group map unit in raster scan order. The length of the slice_group_id[ i ] syntax element is Ceil( Log2( num_slice_groups_minus1 + 1 ) ) bits. The value of slice_group_id[ i ] shall be in the range of 0 to num_slice_groups_minus1, inclusive.

num_ref_idx_l0_default_active_minus1 specifies how num_ref_idx_l0_active_minus1 is inferred for P and B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l0_default_active_minus1 shall be in the range of 0 to 31, inclusive.

num_ref_idx_l1_default_active_minus1 specifies how num_ref_idx_l1_active_minus1 is inferred for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l1_default_active_minus1 shall be in the range of 0 to 31, inclusive.

weighted_pred_flag equal to 0 specifies that the default weighted prediction shall be applied to P slices. weighted_pred_flag equal to 1 specifies that explicit weighted prediction shall be applied to P slices.

weighted_bipred_idc equal to 0 specifies that the default weighted prediction shall be applied to B slices. weighted_bipred_idc equal to 1 specifies that explicit weighted prediction shall be applied to B slices. weighted_bipred_idc equal to 2 specifies that implicit weighted prediction shall be applied to B slices. The value of weighted_bipred_idc shall be in the range of 0 to 2, inclusive.

pic_init_qp_minus26 specifies the initial value minus 26 of SliceQPY for each slice. The initial value is modified at the slice layer when a non-zero value of slice_qp_delta is decoded, and is modified further when a non-zero value of mb_qp_delta is decoded at the macroblock layer. The value of pic_init_qp_minus26 shall be in the range of −(26 + QpBdOffsetY ) to +25, inclusive.

pic_init_qs_minus26 specifies the initial value minus 26 of SliceQSY for all macroblocks in SP or SI slices. The initial value is modified at the slice layer when a non-zero value of slice_qs_delta is decoded. The value of pic_init_qs_minus26 shall be in the range of −26 to +25, inclusive.

chroma_qp_index_offset specifies the offset that shall be added to QPY and QSY for addressing the table of QPC values for the Cb chroma component. The value of chroma_qp_index_offset shall be in the range of −12 to +12, inclusive.

deblocking_filter_control_present_flag equal to 1 specifies that a set of syntax elements controlling the characteristics of the deblocking filter is present in the slice header. deblocking_filter_control_present_flag equal to 0 specifies that the set of syntax elements controlling the characteristics of the deblocking filter is not present in the slice headers and their inferred values are in effect.

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring macroblocks coded using Inter macroblock prediction modes for the prediction of macroblocks coded using Intra macroblock prediction modes. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case prediction of macroblocks coded using Intra macroblock prediction modes only uses residual data and decoded samples from I macroblock types.

redundant_pic_cnt_present_flag equal to 0 specifies that the redundant_pic_cnt syntax element is not present in slice headers that refer to the picture parameter set. redundant_pic_cnt_present_flag equal to 1 specifies that the redundant_pic_cnt syntax element is present in all slice headers that refer to the picture parameter set.

transform_8x8_mode_flag equal to 1 specifies that the 8x8 transform decoding process may be in use. transform_8x8_mode_flag equal to 0 specifies that the 8x8 transform decoding process is not in use. When transform_8x8_mode_flag is not present, it shall be inferred to be 0.

pic_scaling_matrix_present_flag equal to 1 specifies that parameters are present to modify the scaling lists specified in the sequence parameter set. pic_scaling_matrix_present_flag equal to 0 specifies that the scaling lists used for the picture shall be inferred to be equal to those specified by the sequence parameter set. When pic_scaling_matrix_present_flag is not present, it shall be inferred to be equal to 0.

second_chroma_qp_index_offset specifies the offset that shall be added to QPY and QSY for addressing the table of QPC values for the Cr chroma component. The value of second_chroma_qp_index_offset shall be in the range of −12 to +12, inclusive.

When second_chroma_qp_index_offset is not present, it shall be inferred to be equal to chroma_qp_index_offset.

### Slice layer RBSP semantics

The slice layer RBSP consists of a slice header and slice data.

### RBSP slice trailing bits semantics

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

### RBSP trailing bits semantics

rbsp_stop_one_bit shall be equal to 1.

rbsp_alignment_zero_bit shall be equal to 0.

### Slice header semantics

[Ed.: Parameters should be removed and/or added as appropriate.]

When present, the value of the slice header syntax elements pic_parameter_set_id, frame_num, field_pic_flag, bottom_field_flag, idr_pic_id, pic_order_cnt_lsb, delta_pic_order_cnt_bottom, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], and slice_group_change_cycle shall be the same in all slice headers of a coded picture.

first_mb_in_slice specifies the address of the first macroblock in the slice.

The first macroblock address of the slice is derived as follows.

–	If MbaffFrameFlag is equal to 0, first_mb_in_slice is the macroblock address of the first macroblock in the slice, and first_mb_in_slice shall be in the range of 0 to PicSizeInMbs − 1, inclusive.

–	Otherwise (MbaffFrameFlag is equal to 1), first_mb_in_slice * 2 is the macroblock address of the first macroblock in the slice, which is the top macroblock of the first macroblock pair in the slice, and first_mb_in_slice shall be in the range of 0 to PicSizeInMbs / 2 − 1, inclusive.

slice_type specifies the coding type of the slice according to Table 2.

Table 2 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - P (P slice) |
| - 1 | - B (B slice) |
| - 2 | - I (I slice) |
| - 5 | - P (P slice) |
| - 6 | - B (B slice) |
| - 7 | - I (I slice) |

When slice_type has a value in the range 5..7, it is a requirement of bitstream conformance that all other slices of the current coded picture shall have a value of slice_type equal to the current value of slice_type or equal to the current value of slice_type minus 5.

When nal_unit_type is equal to 5 (IDR picture), slice_type shall be equal to 2 or 7.

When max_num_ref_frames is equal to 0, slice_type shall be equal to 2 or 7.

pic_parameter_set_id specifies the picture parameter set in use. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

colour_plane_id specifies the colour plane associated with the current slice RBSP when separate_colour_plane_flag is equal to 1. The value of colour_plane_id shall be in the range of 0 to 2, inclusive. colour_plane_id equal to 0, 1, and 2 correspond to the Y, Cb, and Cr planes, respectively.

frame_num is used as an identifier for pictures and shall be represented by log2_max_frame_num_minus4 + 4 bits in the bitstream.

field_pic_flag equal to 1 specifies that the slice is a slice of a coded field. field_pic_flag equal to 0 specifies that the slice is a slice of a coded frame. When field_pic_flag is not present it shall be inferred to be equal to 0.

The variable MbaffFrameFlag is derived as

MbaffFrameFlag = ( mb_adaptive_frame_field_flag  &&  !field_pic_flag )	(24)

The variable for the picture height in units of macroblocks is derived as

PicHeightInMbs = FrameHeightInMbs / ( 1 + field_pic_flag )	(25)

The variable for picture height for the luma component is derived as

PicHeightInSamplesL = PicHeightInMbs * 16		(26)

The variable for picture height for the chroma component is derived as

PicHeightInSamplesC = PicHeightInMbs * MbHeightC	(27)

The variable PicSizeInMbs for the current picture is derived as

PicSizeInMbs = PicWidthInMbs * PicHeightInMbs	(28)

The variable MaxPicNum is derived as follows.

–	If field_pic_flag is equal to 0, MaxPicNum is set equal to MaxFrameNum.

–	Otherwise (field_pic_flag is equal to 1), MaxPicNum is set equal to 2*MaxFrameNum.

The variable CurrPicNum is derived as follows.

–	If field_pic_flag is equal to 0, CurrPicNum is set equal to frame_num.

–	Otherwise (field_pic_flag is equal to 1), CurrPicNum is set equal to 2 * frame_num + 1.

bottom_field_flag equal to 1 specifies that the slice is part of a coded bottom field. bottom_field_flag equal to 0 specifies that the picture is a coded top field. When this syntax element is not present for the current slice, it shall be inferred to be equal to 0.

idr_pic_id identifies an IDR picture. The values of idr_pic_id in all the slices of an IDR picture shall remain unchanged. When two consecutive access units in decoding order are both IDR access units, the value of idr_pic_id in the slices of the first such IDR access unit shall differ from the idr_pic_id in the second such IDR access unit. The value of idr_pic_id shall be in the range of 0 to 65535, inclusive.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the top field of a coded frame or for a coded field. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

delta_pic_order_cnt_bottom specifies the picture order count difference between the bottom field and the top field of a coded frame as follows.

–	If the current picture includes a memory_management_control_operation equal to 5, the value of delta_pic_order_cnt_bottom shall be in the range of ( 1 − MaxPicOrderCntLsb ) to 231 − 1, inclusive.

–	Otherwise (the current picture does not include a memory_management_control_operation equal to 5), the value of delta_pic_order_cnt_bottom shall be in the range of −231 + 1 to 231 − 1, inclusive.

When this syntax element is not present in the bitstream for the current slice, it shall be inferred to be equal to 0.

delta_pic_order_cnt[ 0 ] specifies the picture order count difference from the expected picture order count for the top field of a coded frame or for a coded field. The value of delta_pic_order_cnt[ 0 ] shall be in the range of −231 + 1 to 231 − 1, inclusive. When this syntax element is not present in the bitstream for the current slice, it shall be inferred to be equal to 0.

delta_pic_order_cnt[ 1 ] specifies the picture order count difference from the expected picture order count for the bottom field of a coded frame. The value of delta_pic_order_cnt[ 1 ] shall be in the range of −231 + 1 to 231 − 1, inclusive. When this syntax element is not present in the bitstream for the current slice, it shall be inferred to be equal to 0.

redundant_pic_cnt shall be equal to 0 for slices belonging to the primary coded picture. The value of redundant_pic_cnt shall be greater than 0 for coded slices of a redundant coded picture. When redundant_pic_cnt is not present in the bitstream, its value shall be inferred to be equal to 0. The value of redundant_pic_cnt shall be in the range of 0 to 127, inclusive.

direct_spatial_mv_pred_flag specifies the method used in the decoding process to derive motion vectors and reference indices for inter prediction as follows.

–	If direct_spatial_mv_pred_flag is equal to 1, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 shall use spatial direct mode prediction.

–	Otherwise (direct_spatial_mv_pred_flag is equal to 0), the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 shall use temporal direct mode prediction.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_l0_active_minus1 is present for P and B slices and that the syntax element num_ref_idx_l1_active_minus1 is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are not present.

When the current slice is a P or B slice and field_pic_flag is equal to 0 and the value of num_ref_idx_l0_default_active_minus1 in the picture parameter set exceeds 15, num_ref_idx_active_override_flag shall be equal to 1.

When the current slice is a B slice and field_pic_flag is equal to 0 and the value of num_ref_idx_l1_default_active_minus1 in the picture parameter set exceeds 15, num_ref_idx_active_override_flag shall be equal to 1.

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that shall be used to decode the slice.

When the current slice is a P or B slice and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 shall be inferred to be equal to num_ref_idx_l0_default_active_minus1.

The range of num_ref_idx_l0_active_minus1 is specified as follows.

–	If field_pic_flag is equal to 0, num_ref_idx_l0_active_minus1 shall be in the range of 0 to 15, inclusive. When MbaffFrameFlag is equal to 1, num_ref_idx_l0_active_minus1 is the maximum index value for the decoding of frame macroblocks and 2 * num_ref_idx_l0_active_minus1 + 1 is the maximum index value for the decoding of field macroblocks.

–	Otherwise (field_pic_flag is equal to 1), num_ref_idx_l0_active_minus1 shall be in the range of 0 to 31, inclusive.

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice.

When the current slice is a B slice and num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 shall be inferred to be equal to num_ref_idx_l1_default_active_minus1.

The range of num_ref_idx_l1_active_minus1 is constrained as specified in the semantics for num_ref_idx_l0_active_minus1 with l0 and list 0 replaced by l1 and list 1, respectively.

cabac_init_idc specifies the index for determining the initialisation table used in the initialisation process for context variables. The value of cabac_init_idc shall be in the range of 0 to 2, inclusive.

slice_qp_delta specifies the initial value of QPY to be used for all the macroblocks in the slice until modified by the value of mb_qp_delta in the macroblock layer. The initial QPY quantisation parameter for the slice is computed as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(29)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

disable_deblocking_filter_idc specifies whether the operation of the deblocking filter shall be disabled across some block edges of the slice and specifies for which edges the filtering is disabled. When disable_deblocking_filter_idc is not present in the slice header, the value of disable_deblocking_filter_idc shall be inferred to be equal to 0.

The value of disable_deblocking_filter_idc shall be in the range of 0 to 2, inclusive.

slice_alpha_c0_offset_div2 specifies the offset used in accessing the α and tC0 deblocking filter tables for filtering operations controlled by the macroblocks within the slice. From this value, the offset that shall be applied when addressing these tables shall be computed as

FilterOffsetA = slice_alpha_c0_offset_div2 << 1		(30)

The value of slice_alpha_c0_offset_div2 shall be in the range of −6 to +6, inclusive. When slice_alpha_c0_offset_div2 is not present in the slice header, the value of slice_alpha_c0_offset_div2 shall be inferred to be equal to 0.

slice_beta_offset_div2 specifies the offset used in accessing the β deblocking filter table for filtering operations controlled by the macroblocks within the slice. From this value, the offset that is applied when addressing the β table of the deblocking filter shall be computed as

FilterOffsetB = slice_beta_offset_div2 << 1		(31)

The value of slice_beta_offset_div2 shall be in the range of −6 to +6, inclusive. When slice_beta_offset_div2 is not present in the slice header the value of slice_beta_offset_div2 shall be inferred to be equal to 0.

slice_group_change_cycle is used to derive the number of slice group map units in slice group 0 when slice_group_map_type is equal to 3, 4, or 5, as specified by

MapUnitsInSliceGroup0 = Min( slice_group_change_cycle * SliceGroupChangeRate,
                                                     PicSizeInMapUnits ) 	(32)

The value of slice_group_change_cycle is represented in the bitstream by the following number of bits

Ceil( Log2( PicSizeInMapUnits  SliceGroupChangeRate + 1 ) )	(33)

The value of slice_group_change_cycle shall be in the range of 0 to Ceil( PicSizeInMapUnitsSliceGroupChangeRate ), inclusive.

mc_interpolation_idc specifies the method that is used for deriving Inter prediction samples.

mv_competition_flag specifies whether motion vector competition is used.

#### Reference picture list modification semantics

The syntax elements modification_of_pic_nums_idc, abs_diff_pic_num_minus1, and long_term_pic_num specify the change from the initial reference picture lists to the reference picture lists to be used for decoding the slice.

ref_pic_list_modification_flag_l0 equal to 1 specifies that the syntax element modification_of_pic_nums_idc is present for specifying reference picture list 0. ref_pic_list_modification_flag_l0 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l0 is equal to 1, the number of times that modification_of_pic_nums_idc is not equal to 3 following ref_pic_list_modification_flag_l0 shall not exceed num_ref_idx_l0_active_minus1 + 1.

When RefPicList0[ num_ref_idx_l0_active_minus1 ] in the initial reference picture list is equal to "no reference picture", ref_pic_list_modification_flag_l0 shall be equal to 1 and modification_of_pic_nums_idc shall not be equal to 3 until RefPicList0[ num_ref_idx_l0_active_minus1 ] in the modified list is not equal to "no reference picture".

ref_pic_list_modification_flag_l1 equal to 1 specifies that the syntax element modification_of_pic_nums_idc is present for specifying reference picture list 1. ref_pic_list_modification_flag_l1 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l1 is equal to 1, the number of times that modification_of_pic_nums_idc is not equal to 3 following ref_pic_list_modification_flag_l1 shall not exceed num_ref_idx_l1_active_minus1 + 1.

When decoding a slice with slice_type equal to 1 or 6 and RefPicList1[ num_ref_idx_l1_active_minus1 ] in the initial reference picture list is equal to "no reference picture", ref_pic_list_modification_flag_l1 shall be equal to 1 and modification_of_pic_nums_idc shall not be equal to 3 until RefPicList1[ num_ref_idx_l1_active_minus1 ] in the modified list is not equal to "no reference picture".

modification_of_pic_nums_idc together with abs_diff_pic_num_minus1 or long_term_pic_num specifies which of the reference pictures are re-mapped. The values of modification_of_pic_nums_idc are specified in Table 3. The value of the first modification_of_pic_nums_idc that follows immediately after ref_pic_list_modification_flag_l0 or ref_pic_list_modification_flag_l1 shall not be equal to 3.

Table 3 – modification_of_pic_nums_idc operations for modification of reference picture lists

| modification_of_pic_nums_idc | modification specified |
| --- | --- |
| 0 | abs_diff_pic_num_minus1 is present and corresponds to a difference to subtract from a picture number prediction value |
| 1 | abs_diff_pic_num_minus1 is present and corresponds to a difference to add to a picture number prediction value |
| 2 | long_term_pic_num is present and specifies the long-term picture number for a reference picture |
| 3 | End loop for modification of the initial reference picture list |

abs_diff_pic_num_minus1 plus 1 specifies the absolute difference between the picture number of the picture being moved to the current index in the list and the picture number prediction value. abs_diff_pic_num_minus1 shall be in the range of 0 to MaxPicNum − 1.

long_term_pic_num specifies the long-term picture number of the picture being moved to the current index in the list. When decoding a coded frame, long_term_pic_num shall be equal to a LongTermPicNum assigned to one of the reference frames or complementary reference field pairs marked as "used for long-term reference". When decoding a coded field, long_term_pic_num shall be equal to a LongTermPicNum assigned to one of the reference fields marked as "used for long-term reference".

#### Prediction weight table semantics

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

chroma_log2_weight_denom is the base 2 logarithm of the denominator for all chroma weighting factors. The value of chroma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag equal to 1 specifies that weighting factors for the luma component of list 0 prediction are present. luma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

luma_weight_l0[ i ] is the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. When luma_weight_l0_flag is equal to 1, the value of luma_weight_l0[ i ] shall be in the range of −128 to 127, inclusive. When luma_weight_l0_flag is equal to 0, luma_weight_l0[ i ] shall be inferred to be equal to 2luma_log2_weight_denom for RefPicList0[ i ].

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive. When luma_weight_l0_flag is equal to 0, luma_offset_l0[ i ] shall be inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction are present. chroma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

chroma_weight_l0[ i ][ j ] is the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr. When chroma_weight_l0_flag is equal to 1, the value of chroma_weight_l0[ i ][ j ] shall be in the range of −128 to 127, inclusive. When chroma_weight_l0_flag is equal to 0, chroma_weight_l0[ i ][ j ] shall be inferred to be equal to 2chroma_log2_weight_denom for RefPicList0[ i ].

chroma_offset_l0[ i ][ j ] is the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr. The value of chroma_offset_l0[ i ][ j ] shall be in the range of −128 to 127, inclusive. When chroma_weight_l0_flag is equal to 0, chroma_offset_l0[ i ][ j ] shall be inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_l1_flag, luma_weight_l1, luma_offset_l1, chroma_weight_l1_flag, chroma_weight_l1, chroma_offset_l1 have the same semantics as luma_weight_l0_flag, luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, chroma_weight_l0, chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

#### Decoded reference picture marking semantics

The syntax elements no_output_of_prior_pics_flag, long_term_reference_flag, adaptive_ref_pic_marking_mode_flag, memory_management_control_operation, difference_of_pic_nums_minus1, long_term_frame_idx, long_term_pic_num, and max_long_term_frame_idx_plus1 specify marking of the reference pictures.

The marking of a reference picture can be "unused for reference", "used for short-term reference", or "used for long-term reference", but only one among these three. When a reference picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both). A reference picture that is marked as "used for short-term reference" is referred to as a short-term reference picture. A reference picture that is marked as "used for long-term reference" is referred to as a long-term reference picture.

The content of the decoded reference picture marking syntax structure shall be the same in all slice headers of the primary coded picture. When one or more redundant coded pictures are present, the content of the decoded reference picture marking syntax structure shall be the same in all slice headers of a redundant coded picture with a particular value of redundant_pic_cnt.

The syntax category of the decoded reference picture marking syntax structure shall be inferred as follows.

–	If the decoded reference picture marking syntax structure is in a slice header, the syntax category of the decoded reference picture marking syntax structure is inferred to be equal to 2.

–	Otherwise (the decoded reference picture marking syntax structure is in a decoded reference picture marking repetition SEI message), the syntax category of the decoded reference picture marking syntax structure is inferred to be equal to 5.

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR picture. When the IDR picture is the first IDR picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR picture is not the first IDR picture in the bitstream and the value of PicWidthInMbs, FrameHeightInMbs, or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of PicWidthInMbs, FrameHeightInMbs, or max_dec_frame_buffering derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.

long_term_reference_flag equal to 0 specifies that the MaxLongTermFrameIdx variable is set equal to "no long-term frame indices" and that the IDR picture is marked as "used for short-term reference". long_term_reference_flag equal to 1 specifies that the MaxLongTermFrameIdx variable is set equal to 0 and that the current IDR picture is marked "used for long-term reference" and is assigned LongTermFrameIdx equal to 0. When max_num_ref_frames is equal to 0, long_term_reference_flag shall be equal to 0.

adaptive_ref_pic_marking_mode_flag selects the reference picture marking mode of the currently decoded picture as specified in Table 4. adaptive_ref_pic_marking_mode_flag shall be equal to 1 when the number of frames, complementary field pairs, and non-paired fields that are currently marked as "used for long-term reference" is equal to Max( max_num_ref_frames, 1 ).

Table 4 – Interpretation of adaptive_ref_pic_marking_mode_flag

| adaptive_ref_pic_marking_mode_flag | Reference picture marking mode specified |
| --- | --- |
| 0 | Sliding window reference picture marking mode: A marking mode providing a first-in first-out mechanism for short-term reference pictures. |
| 1 | Adaptive reference picture marking mode: A reference picture marking mode providing syntax elements to specify marking of reference pictures as "unused for reference" and to assign long-term frame indices. |

memory_management_control_operation specifies a control operation to be applied to affect the reference picture marking. The memory_management_control_operation syntax element is followed by data necessary for the operation specified by the value of memory_management_control_operation. The values and control operations associated with memory_management_control_operation are specified in Table 5. The memory_management_control_operation syntax elements are processed by the decoding process in the order in which they appear in the slice header, and the semantics constraints expressed for each memory_management_control_operation apply at the specific position in that order at which that individual memory_management_control_operation is processed.

For interpretation of memory_management_control_operation, the term reference picture is interpreted as follows.

–	If the current picture is a frame, the term reference picture refers either to a reference frame or a complementary reference field pair.

–	Otherwise (the current picture is a field), the term reference picture refers either to a reference field or a field of a reference frame.

memory_management_control_operation shall not be equal to 1 in a slice header unless the specified reference picture is marked as "used for short-term reference" when the memory_management_control_operation is processed by the decoding process.

memory_management_control_operation shall not be equal to 2 in a slice header unless the specified long-term picture number refers to a reference picture that is marked as "used for long-term reference" when the memory_management_control_operation is processed by the decoding process.

memory_management_control_operation shall not be equal to 3 in a slice header unless the specified reference picture is marked as "used for short-term reference" when the memory_management_control_operation is processed by the decoding process.

memory_management_control_operation shall not be equal to 3 or 6 if the value of the variable MaxLongTermFrameIdx is equal to "no long-term frame indices" when the memory_management_control_operation is processed by the decoding process.

Not more than one memory_management_control_operation equal to 4 shall be present in a slice header.

Not more than one memory_management_control_operation equal to 5 shall be present in a slice header.

Not more than one memory_management_control_operation equal to 6 shall be present in a slice header.

memory_management_control_operation shall not be equal to 5 in a slice header unless no memory_management_control_operation in the range of 1 to 3 is present in the same decoded reference picture marking syntax structure.

A memory_management_control_operation equal to 5 shall not follow a memory_management_control_operation equal to 6 in the same slice header.

When a memory_management_control_operation equal to 6 is present, any memory_management_control_operation equal to 2, 3, or 4 that follows the memory_management_control_operation equal to 6 within the same slice header shall not specify the current picture to be marked as "unused for reference".

Table 5 – Memory management control operation (memory_management_control_operation) values

| memory_management_control_operation | Memory Management Control Operation |
| --- | --- |
| 0 | End memory_management_control_operation syntax element loop |
| 1 | Mark a short-term reference picture as "unused for reference" |
| 2 | Mark a long-term reference picture as "unused for reference" |
| 3 | Mark a short-term reference picture as "used for long-term reference" and assign a long-term frame index to it |
| 4 | Specify the maximum long-term frame index and mark all long-term reference pictures having long-term frame indices greater than the maximum value as "unused for reference" |
| 5 | Mark all reference pictures as "unused for reference" and set the MaxLongTermFrameIdx variable to "no long-term frame indices" |
| 6 | Mark the current picture as "used for long-term reference" and assign a long-term frame index to it |

When decoding a field and a memory_management_control_operation command equal to 3 is present that assigns a long-term frame index to a field that is part of a short-term reference frame or part of a complementary reference field pair, another memory_management_control_operation command (equal to 3 or 6) to assign the same long-term frame index to the other field of the same frame or complementary reference field pair shall be present in the same decoded reference picture marking syntax structure.

When the first field (in decoding order) of a complementary reference field pair included a long_term_reference_flag equal to 1 or a memory_management_control_operation command equal to 6, the decoded reference picture marking syntax structure for the second field of the complementary reference field pair shall contain a memory_management_control_operation command equal to 6 that assigns the same long-term frame index to the second field.

When the second field (in decoding order) of a complementary reference field pair includes a memory_management_control_operation command equal to 6 that assigns a long-term frame index to this field and the first field of the complementary reference field pair is marked as "used for short-term reference" when the memory_management_control_operation command equal to 6 is processed by the decoding process, the decoded reference picture marking syntax structure of that second field shall contain either a memory_management_control_operation command equal to 1 that marks the first field of the complementary field pair as "unused for reference" or a memory_management_control_operation command equal to 3 that marks the first field of the complementary field pair as "used for long-term reference" and assigns the same long-term frame index to the first field.

difference_of_pic_nums_minus1 is used (with memory_management_control_operation equal to 3 or 1) to assign a long-term frame index to a short-term reference picture or to mark a short-term reference picture as "unused for reference". When the associated memory_management_control_operation is processed by the decoding process, the resulting picture number derived from difference_of_pic_nums_minus1 shall be a picture number assigned to one of the reference pictures marked as "used for reference" and not previously assigned to a long-term frame index.

The resulting picture number is constrained as follows.

–	If field_pic_flag is equal to 0, the resulting picture number shall be one of the set of picture numbers assigned to reference frames or complementary reference field pairs.

–	Otherwise (field_pic_flag is equal to 1), the resulting picture number shall be one of the set of picture numbers assigned to reference fields.

long_term_pic_num is used (with memory_management_control_operation equal to 2) to mark a long-term reference picture as "unused for reference". When the associated memory_management_control_operation is processed by the decoding process, long_term_pic_num shall be equal to a long-term picture number assigned to one of the reference pictures that is currently marked as "used for long-term reference".

The resulting long-term picture number is constrained as follows.

–	If field_pic_flag is equal to 0, the resulting long-term picture number shall be one of the set of long-term picture numbers assigned to reference frames or complementary reference field pairs.

–	Otherwise (field_pic_flag is equal to 1), the resulting long-term picture number shall be one of the set of long-term picture numbers assigned to reference fields.

long_term_frame_idx is used (with memory_management_control_operation equal to 3 or 6) to assign a long-term frame index to a picture. When the associated memory_management_control_operation is processed by the decoding process, the value of long_term_frame_idx shall be in the range of 0 to MaxLongTermFrameIdx, inclusive.

max_long_term_frame_idx_plus1 minus 1 specifies the maximum value of long-term frame index allowed for long-term reference pictures (until receipt of another value of max_long_term_frame_idx_plus1). The value of max_long_term_frame_idx_plus1 shall be in the range of 0 to max_num_ref_frames, inclusive.

### Slice data semantics

end_of_slice_flag equal to 0 specifies that another macroblock is following in the slice. end_of_slice_flag equal to 1 specifies the end of the slice and that no further macroblock follows.

### Coding unit semantic

split_coding_unit_flag specifies whether a coding unit is split into coding units with half horizontal and vertical size. When split_coding_unit_flag is not present, it shall be inferred to be equal to 0.

### Prediction unit semantics

intra_prediction_flag specifies whether the current coding unit is intra are inter predicted.

intra_split_flag specifies whether the current coding unit is split for the purpose of intra prediction.

intra_prediction_mode[ i ] specifies the intra prediction mode for the current partition with partition index i.

inter_partitioning_idc specifies the partitioning of the coding unit for the purpose of inter prediction.

merge_flag[ i ] specifies whether the inter prediction parameters for the current partition with partition index i are inferred from a neighbouring inter-predicted partition.

merge_left_flag[ i ] specifies whether the inter prediction parameters for the current partition with partition index i are inferred from the left or the top neighbouring inter-predicted partition.

inter_pred_idc[ i ] specifies whether list 0, list 1, or bi-prediction is used for the current partition with partition index i.

ref_idx_l0[ i ], ref_idx_l1[ i ] specify the list 0 and list 1 reference picture indices, respectively, for the current partition with partition index i.

mvd_l0[ i ][ 0 ], mvd_l0[ i ][ 1 ] specify the horizontal and vertical component, respectively, of the list 0 motion vector difference for the current partition with partition index i.

mvd_l1[ i ][ 0 ], mvd_l1[ i ][ 1 ] specify the horizontal and vertical component, respectively, of the list 1 motion vector difference for the current partition with partition index i.

### Transform unit semantics

split_transform_unit_flag specifies whether a coding unit is split into coding units with half horizontal and vertical size. When split_transform_unit_flag is not present, it shall be inferred to be equal to ( currTransformUnitSize > MinTransformUnitSize ? 1 : 0 ).

coded_block_flag specifies whether the transform unit contains one or more non-zero transform coefficient levels.

[Ed. transform coefficients...]

# Unit definition

## Coding Tree Block (CTB)

The coding unit (CTB) is defined as a basic unit which has a square shape. Although it has a similar role to the macroblock and sub-macroblock in AVC, the main difference lies in the fact that CTB can have various sizes, with no distinction corresponding to its size. All processing except frame-based loop filtering is performed on a CTB basis, including intra/inter prediction, transform, quantization and entropy coding. Two special terms are defined: the largest coding unit (LCTB) and the smallest coding unit (SCTB). For convenient implementation, LCTB size and SCTB size are limited to values which are a power of 2 and which are greater than or equal to 8.

It is assumed that a picture consists of non-overlapped LCTBs. Since the CTB is restricted to be a square shape, the CTB structure within a LCTB can be expressed in a recursive tree representation adapted to the picture. That is, CTB is characterized by LCTB size and the hierarchical depth in the LCTB that the CTB belongs to.

Fig. 1: Illustration of recursive CTB structure (LCTB size = 128, maximum hierarchical depth = 5)

Figure 1 shows an example where LCTB size is 128 and the maximum hierarchical depth is 5. The recursive structure is represented by a series of split flags. For CTBd, which has depth d and size 2Nx2N, the coding of CTB is performed in the current depth when split flag is set to zero. When the split flag is set to 1, CTBd is split into 4 independent CTBd+1 which have depth (d+1) and size NxN. In this case, CTBd+1 is called a sub-CTB of CTBd similar to a sub-macroblock in AVC. Unless the depth of sub-CTB (d+1) is equal to the maximum allowed depth (4 in this case), each CTBd+1 is processed in a recursive manner. If the depth of sub-CTB (d+1) is equal to the maximum allowed depth, further splitting is not allowed. Note that a CTB can be further split into PUs.

The sizes of LCTB and SCTB are specified in the Sequence Parameter Set (SPS). The embedded information in the SPS is LCTB size (s) and the maximum hierarchical depth (h) in a LCTB. For example, if s = 128 and h = 5, then 5 kinds of CTB sizes are possible: 128x128 (LCTB), 64x64, 32x32, 16x16 and 8x8 (SCTB). If s = 16 and h = 2, then 16x16 (LCTB) and 8x8 (SCTB) are possible; this is a similar block structure to AVC. Therefore, if the LCTB size and maximum hierarchical depth are given, this defines the possible CTB sizes which are allowed.

This kind of arbitrary unit representation provides several major benefits. The first benefit comes from the support of CTB sizes greater than the conventional 16x16 macroblock. When the region of interest is homogeneous, a large CTB can represent the region in a smaller number of symbols than is possible in the case of several small blocks.

Furthermore, supporting arbitrary LCTB sizes enables the codec to be readily optimized for various content, applications and devices. Compared to the use of fixed size macroblock, support of various LCTB sizes is one of the major properties. It is especially useful for low resolution video services, which is still commonly used in the market. By choosing LCTB size and maximum hierarchical depth appropriately, the hierarchical block structure can be optimized in a better way for the targeted application. In the later stage of the standardization, the range of LCTB sizes could be specified in the Profiles and Levels section to match the requirements more specifically.

Finally, by eliminating the distinction between macroblock and sub-macroblock and using only one unit type, CTB, the multi-level hierarchical structure can be specified in a very simple and elegant way: LCTB size, maximum hierarchical depth and a series of split flags. Together with the proposed size-independent syntax representation, it’s sufficient to specify syntax items of one general size for the remaining coding tools. This kind of consistency can greatly simplify the specification effort as well as the actual parsing process.

The maximum depth for CTB hierarchy is arbitrary and potentially larger than the maximum depth allowed in the AVC block hierarchy. Therefore, the text includes a size-independent syntax representation which specifies all syntax elements in a consistent way independent of the CTB size. By contrast, in AVC block level syntax elements such as transform_8x8_mode_flag, coded_block_flag and intra_pred_modes are coded differently depending on whether the block size is 16, 8 or 4.  The splitting process for the CTB can be specified recursively and all other syntax elements for the leaf CTB are defined in the same way independent on the CTB size. This kind of representation is very useful in terms of reduced parsing complexity and improved clarity if a large hierarchical depth is allowed.

For the purpose of mode decision, transmission of the data associated with each block etc. all CTBs are traversed in raster scan order (left-to-right, top-down), and within each CTB, the subblocks are traversed in depth-first order.

## Prediction unit (PU)

Once the splitting process is done, prediction methods are specified for every CTB which is not further split i.e. the leaf nodes of the CTB hierarchical tree.

Coupled with CTB, a basic unit for the prediction mode is introduced: the prediction unit (PU). It should be noted that the PU is defined only for the last-depth CTB, i.e., the leaf nodes of the CTB-related quadtree and its size is limited to that of the CTB.

Similar to conventional standards, we define two different terms to specify the prediction method: the prediction type and the PU splitting. The prediction type is one of the values among intra or inter, which roughly describe the nature of the prediction method. After that, possible PU splittings are defined according to the prediction type.

The PU for intra has 2 different possible splittings: 2Nx2N (i.e. no split) and NxN (quarter split). The PU for inter has 8 different possible splittings: 4 symmetric splittings (2Nx2N, 2NxN, Nx2N, NxN) and 4 asymmetric splittings (2NxnU, 2NxnD, nLx2N and nRx2N). The number N is derived from the size of the CTB which the PU belongs to. For example, if the size of CTB is 128x128, then both 128x128 and 64x64 PUs for intra are possible. For inter prediction, 128x128, 128x64, 64x128, 64x64, 128x32 (for 2NxnU and 2NxnD), 128x96 (for 2NxnU and 2NxnD), 32x128 (for nRx2N and nLx2N) and 96x128 (for nRx2N and nLx2N) are possible.

Figure 1a shows an example where a 64x64 block is asymmetrically partitioned into 64x16, 64x48, 16x64, or 48x16 blocks, instead of two 64x32 or 32x64 blocks. Note that the motion partitions shown in the figure correspond to 2NxnU, 2NxnD, nLx2N, and nRx2N, respectively.

Figure 1a: Example of asymmetric motion partitions for 64x64 block

In addition, we have geometrical shapes for partitioning of inter PUs that are described as follows.

For each CTB, one type of PU splittings is defined by an arbitrary straight line across the CTB that divides the CTB into two regions. The boundary of the PU partition is defined by an angle subtended by a line perpendicular to the boundary with the X axis  and the distance of the partition line from the origin . The equation of the line defining the partition boundary is specified as

Two 32 bit lookup tables are used, one to store the slope, , and the other to store the Y-intercept, . The region to which each pixel belongs is calculated on the fly.

At each level CTB, 32 different values of  are defined (from 0 to  in steps of ). The number of values that   can take depends on the CTB size. For example, for CTB size of 16×16,  takes 8 possible values (from 0 to 7 in steps of 1). For block sizes of 32×32 and 64×64,  takes 16 and 32 possible values, respectively.

Figure 2: Parameters defining a geometry motion partition

For a CTB that is predicted in a manner described above, overlapped block motion compensation (OBMC) is applied. Let the two regions created by a geometry partition be denoted by region 1 and region 2. Let the corresponding motion vectors be denoted by  and , respectively. A pixel from region 1 (2) is defined to be a boundary pixel if any of its four connected neighbors (left, top, right, and bottom) belongs to region 2 (1). Fig. 2 shows an example where light blue pixels belong to the boundary of region 1 and white pixels belong to the boundary of region2. If a pixel is not a boundary pixel, normal motion compensation is performed using the appropriate motion vector. If a pixel is a boundary pixel, the motion compensation is performed using a weighted sum of the motion predictions from the two motion vectors,  and . The weights are   for the region containing the boundary pixel and  f for the other region.

Figure 3: Overlapped motion compensation for geometry partitions

All information related to prediction is signaled on a PU basis, for instance, the direction of intra prediction for intra or the motion vector difference and reference index for inter. Motion vector prediction and motion compensation are also performed on a PU basis.

For each inter predicted blocks, individual motion parameters are transmitted.  In order to achieve an improved coding efficiency, the block merging process enables to merge neighbouring blocks into so-called regions.  By doing so, the motion parameters do not need to be transmitted for each block of the region individually, but instead the parameters are transmitted only for once for the whole region.

For each inter prediction block, the set of all prediction blocks that are coded before that block in processing order is called the “set of causal blocks”.  The set of blocks that is admissible for merging with a particular block is called the “set of available blocks” and is always a subset of the set of causal blocks.  If a particular block is encoded and its set of available blocks is not empty, it is signaled whether this block is to be merged with one block out of this set and if so, with which of them.  Otherwise, merging cannot be used for this block.

The set of available blocks is formed as follows.  Starting from the top-left sample position of the current block, its left neighbouring sample position and its top neighbouring sample position is derived.  The set of available blocks can have only up to two elements, namely those blocks out of the set of causal blocks that contain one of the two sample positions.  Thus the set of available blocks can only have the two direct neighbouring blocks of the top-left sample position of the current block as its elements.  Note that only inter predicted blocks can be members of the set of available blocks.

If the set of available blocks is not empty, one flag called merge_flag is signaled, specifying whether the current block is merged with any of the available blocks.  Otherwise, or if the merge_flag is equal to 0 (for “false”), this block is not merged with one of its causal blocks and all parameters are transmitted ordinarily.  If the merge_flag is equal to 1 (for “true”), the following applies.  If the set of available blocks contains one and only one block, this block is used for merging.  Otherwise the set of available blocks contains exactly two blocks.  If the motion parameters of these two blocks are identical, these motion parameters are used for the current block.  Otherwise (the two blocks have different motion parameters), a flag called merge_left_flag is signaled.  If merge_left_flag is equal to 1 (for “true”), the block containing the left neighbouring sample position of the top-left sample position of the current block is selected out of the set of available blocks.  If merge_left_flag is equal to 0 (for “false”), the other (i.e., top neighbouring) block out of the set of available blocks is selected.  The motion parameters of the selected block are used for the current block.

Fig. 3: Two examples illustrating the set of causal blocks (grey) and the set of available blocks (A, B).

Fig. 3 shows two examples for a quadtree-based division of a picture into prediction blocks.  On the left side of Fig. 4, the top two blocks of the biggest size are so-called macroblocks, i.e., they are prediction blocks of the maximum possible size.  The other blocks in this left figure are obtained as a subdivision of their corresponding macroblock.  The current block is marked with an “X”.  All the grey-shaded blocks are encoded before the current block, so they form the set of causal blocks.  As explicated in the description of the derivation of the set of available blocks, only the blocks containing the direct (i.e. top or left) neighbouring samples of the top-left sample position of the current block can be members of the set of available blocks.  Thus the current block can be merged with either block “A” or block “B”.  If merge_flag is equal to 0 (for “false”), the current block “X” is not merged with any of the two blocks.  If blocks “A” and “B” have identical motion parameters, no distinction needs to be made, since merging with any of the two blocks will lead to the same result.  So, in this case, the merge_left_flag is not transmitted.  Otherwise, if blocks “A” and “B” have different prediction parameters, merge_left_flag equal to 1 (for “true”) will merge blocks “X” and “B”, whereas merge_left_flag equal to 0 (for “false”) will merge blocks “X” and “A”.

In Fig. 4 (right) another example is shown.  Here the current block “X” and the left neighbour block “B” are macroblocks, i.e., they have the maximum allowed block size.  The size of the top neighbour block “A” is one quarter of the macroblock size.  The blocks which are element of the set of causal blocks are grey-shaded.  Note that the current block “X” can only be merged with the two blocks “A” or “B”, not with any of the other top neighbouring blocks.

## Transform unit (TU)

In addition to the CTB and PU definitions, the transform unit (TU) for transform and quantization is defined separately. It should be noted that the size of the TU may be as large as the size of the CTB. Like the partitioning used for prediction, the partitioning of a single prediction error block into transform blocks is represented by quad trees. The greatest applicable transform size and the maximum depth of such quad trees (giving the minimum applicable size) are variable and are signaled in the bit stream. Transform blocks with an edge length of any positive power of two can be employed. Transform block sizes are constrained to the range 4x4 to 64x64 (for chroma, also 2x2 transforms may be used).

## Motion representation

## Motion vector prediction for rectangular partitions

A new method to form motion vector prediction is introduced. Unlike in AVC, this method takes the temporal distance of the reference frames into account for motion vector prediction. The neighboring blocks that are used for the prediction of the vertical components are illustrated in Figure 2.

Figure 2: Neighboring blocks used for motion vector prediction

But the motion vectors of neighboring blocks A, B, and C may point to different reference frames in the buffer. Therefore, the motion vectors are normalized based on their temporal distance from the current frame, before taking the median. This is illustrated in Figure 3 for an IPPP configuration. In the figure, blue colored frames are the reference frames stored in the buffer. Let  refer to the reference frames corresponding to blocks A, B, C, and X, respectively. Let their temporal distances from the current frame be denoted by , respectively. Then, each of the neighboring motion vectors,  is scaled as follows:

This scheme is applied to both P and B frames.

Figure 3: Reference frames and their temporal distances from the current frame

Furthermore, in order to reduce the bit rate required for transmitting the motion vectors, we employ a concept in which the prediction and coding of the components of a motion vector is interleaved. In a first step, the vertical motion vector component is predicted using motion vector competition and the difference between the actual vertical component and its prediction is coded. Then, only the motion vectors of the neighborhood for which the absolute difference between their vertical component and the coded vertical component for the current motion vector is minimized are used for the prediction of the horizontal component.

In the following, we consider the prediction and reconstruction of a motion vector currMv for a prediction block.

- Vertical component prediction. The prediction and coding of the components of a motion vector is interleaved. In a first step, the vertical motion vector component is predicted using a set of predictors (set_v).  The set of predictors contains the median motion vector computed from neighboring motion vectors as in AVC, and possibly the motion vector associated to the collocated block in the previously decoded frame. The difference between the actual vertical component and the selected predictor is coded. The selected predictor from set_v is signaled in the bitstream if necessary (i.e. if vertical components of the two predictors in the set are different).

- Then, only the motion vectors of the neighborhood for which the absolute difference between their vertical component and the coded vertical component for the current motion vector is minimized, are used for the prediction of the horizontal component.

At the slice level, 1 bit signals the configuration selected for set_v.

The location of the top-left luma sample of the prediction block, relative to the top-left luma sample of the picture, is denoted by ( x0, y0 ). The width and height of the prediction block are given by currW and currH, respectively. Given the transmitted motion vector difference mvDiff (with horizontal component mvDiff[ 0 ] and vertical component mvDiff[ 1 ]), the prediction and reconstruction of the motion vector currMv is done as described in the following. For the following description, only one reference list is considered. All motion vectors are motion vectors for this one reference list.

As first step, the vertical component mvPred[ 1 ] of the prediction vector mvPred is derived as specified by the following ordered steps:

- A list listMvCompY of vertical motion vector components is derived as follows.

    - Let blkA, blkB, and blkC be the prediction blocks that contain the luma samples at locations ( x0 − 1, y0 ), ( x0, y0 − 1 ), and ( x0 + currW, y0 − 1), respectively.

    - When blkC is not contained in the same slice as the current block, or it follows the current block in coding order, blkC is replaced with the prediction block that contains the luma sample at location ( x0 − 1, y0 − 1 ).

    - For each of the prediction blocks blkX (with X being replaced by A, B, and C), the following applies:

If the block blkX represents a motion-compensated block that is contained in the same slice as the current block and precedes the current block in coding order, the following applies with mvX being the motion vector that is associated with blkX:

The motion vector mvX is scaled as described above and inserted into the list listMvCompY.

- Let numCompY be the number of entries in the list listMvCompY and let listMvCompY[ k ], with k = 0..numCompY − 1, be the entries of the list listMvCompY. Depending on numCompY, the vertical component mvPred[ 1 ] of the motion vector predictor is derived as follows.

- If numCompY is equal to 0, mvPred[ 1 ] is set equal to 0.

- Otherwise, if numCompY is equal to 1, mvPred[ 1 ] is set equal to listMvCompY[ 0 ].

- Otherwise, if numCompY is equal to 2, mvPred[ 1 ] is set equal to ( listMvCompY[ 0 ] + listMvCompY[ 1 ] + 1 ) >> 1.

- Otherwise (numCompY is greater than 2), the entries in listMvCompY are sorted in increasing order and then mvPred[ 1 ] is set equal to listMvCompY[ numCompY >> 1 ].

Given the predictor mvPred[ 1 ] for the vertical motion vector component, the vertical motion vector component currMv[ 1 ] of the current motion vector is reconstructed by setting currMv[ 1 ] equal to ( mvPred[ 1 ] + mvDiff[ 1 ] ). Then, given the reconstructed vertical motion vector component currMv[ 1 ], the predictor mvPred[ 0 ] for the horizontal motion vector component is derived as follows:

- Let setOfBlks be the set of neighboring prediction blocks that are contained in the same slice as the current block, precede the current block in coding order, and contain a luma sample at any of the following locations:

- ( x0 − 1, y ) with y = 0..currH

- ( x, y0 − 1 ) with x = −1..currW + 1

- A list listMv of motion vectors is derived as follows:

For each block blkX of the set setOfBlks, the following applies with mvX being the motion vector that is associated with blkX:

The motion vector mvX is scaled as described above and inserted into the list listMv.

- Let numMv be the number of entries in the list listMv. If numMv is equal to 0, the predictor mvPred[ 0 ] for the horizontal motion vector component is set equal to 0; otherwise, mvPred[ 0 ] is derived as specified in the following with listMv[ k ] (k = 0..numMv − 1) representing the k-th entry in the list listMv:

  - The variable compY is initially set equal to listMv[ 0 ][ 1 ] and the variable minDelta is set equal to abs( listMv[ 0 ][ 1 ] − currMv[ 1 ] ), where abs( ) specifies the absolute value of the argument.

  - For i proceeding over the range of 1 to numMv − 1, inclusive, the following applies:

When abs( listMv[ i ][ 1 ] − currMv[ 1 ] ) is less than minDelta or (abs( listMv[ i ][ 1 ] − currMv[ 1 ] ) is equal to minDelta and listMv[ i ][ 1 ] is less than compY), compY is set equal to listMv[ i ][ 1 ].

  - The list listMvCompX is derived as follows.

For i proceeding over the range of 0 to numMv − 1, inclusive, if listMv[ i ][ 1 ] is equal to compY, the value listMv[ i ][ 0 ] is inserted into the list listMvCompX.

  - Let numCompX be the number of entries in the list listMvCompX and let listMvCompX[ k ], with k = 0..numCompX − 1, be the entries of the list listMvCompX. Depending on numCompX, the horizontal component mvPred[ 0 ] of the motion vector predictor is derived as follows.

    - If numCompX is equal to 1, mvPred[ 0 ] is set equal to listMvCompX[ 0 ].

    - Otherwise, if numCompX is equal to 2, mvPred[ 0 ] is set equal to ( listMvCompX[ 0 ] + listMvCompX[ 1 ] + 1 ) >> 1.

    - Otherwise (numCompX is greater than 2), the entries in listMvCompX are sorted in increasing order and then mvPred[ 0 ] is set equal to listMvCompX[ numCompX >> 1 ].

Given the predictor mvPred[ 0 ] for the horizontal motion vector component, the horizontal motion vector component currMv[ 0 ] of the current motion vector is reconstructed by setting currMv[ 0 ] equal to ( mvPred[ 0 ] + mvDiff[ 0 ] ).

## Motion vector prediction for geometric block partitions

The process of forming motion vector prediction for a geometric block partition is shown Figure 2. Motion vectors of neighboring blocks are used to form the motion vector prediction. Depending on the partition, a subset of blocks from  are used to form motion vector prediction. The choice of block for motion vector prediction depends upon whether they lie on the same side or opposite side of the geometry partition boundary.

Figure2: Motion vector prediction for a block having geometry partition

Let blkA, blkB, blkC, blkD, blkE and blkF be the prediction blocks that contain the luma samples at locations ( x0 − 1, y0 ), ( x0, y0 − 1 ), ( x0 + currW, y0 − 1), ( x0 − 1, y0 − 1 ), ( x0 + currW − 1, y0 − 1 ), and ( x0 − 1, y0 + currW − 1),   respectively. For each partition, if blkX (X being replaced A, B, C, D, E and F) is sharing a boundary with the partition, blkX is used to derive the prediction motion vector.

The prediction motion vector mvPred is derived as specified by the folowing ordered steps:

- If blkA, blkB and blkC are all connected to the current partition, median filter is used to derive  mvPred.

- Otherwise, if blkA is connected to the current partition, mvPred = mvA

- Otherwise, if blkB is connected to the current partition, mvPred = mvB

- Otherwise, if blkE is connected to the current partition, mvPred = mvE

- Otherwise, if blkF is connected to the current partition, mvPred = mvF

- Otherwise, if blkC is connected to the current partition, mvPred = mvC

- Otherwise mvPred = (0,0)

## Interpolation Methods

This text uses single pass switched interpolation filters with offsets (single pass SIFO). The 1/4th pixel motion positions are shown in the figure.

### Single pass switched Interpolation Filters with offsets (single pass SIFO)

In this text, 7 different filter sets are defined. Each filter set consists of 15 filters, one for each fractional pixel position.

- Filter set 0: This filter set uses high precision filtering with the filters shown in the following table with the exception of position ‘g’, where a non-separable filter is used.

Table 6: 6-tap separable filters

| 1/4 | { 8, -32, 224, 72, -24, 8 } (8 additions, 4 shifts) |
| --- | --- |
| 1/2 | { 8, -40, 160, 160, -40, 8 } (6 additions, 3 shifts) |
| 3/4 | { 8, -24, 72, 224, -32, 8 } (8 additions, 4 shifts) |

The filter coefficients are scaled by 256 and the intermediate data is kept in higher precision. For position ‘g’, the following filter is used (followed by right shift by 7 bits):

| 0 | 5 | 5 | 0 |
| --- | --- | --- | --- |
| 5 | 22 | 22 | 5 |
| 5 | 22 | 22 | 5 |
| 0 | 5 | 5 | 0 |

- Filter set 1: This filter set uses 12 tap filters for both horizontal and vertical filtering. The filter coefficients are scaled by 256. A higher bit depth of filter coefficients leads to more accurate interpolation; for comparison, AVC uses 5 bits for filter coefficients. Also, the intermediate values are maintained at higher bit precision.

Table 7: 12-tap separable filters

| ¼ | {-1, 5, -12, 20,-40, 229, 76, -32, 16, -8, 4,-1 } (18 additions, 6 shifts) |
| --- | --- |
| ½ | {-1, 8, -16, 24,-48, 161, 161, -48, 24, -16, 8,-1 } (15 additions, 4 shifts) |
| ¾ | {-1, 4, -8, 16, -32, 76, 229, -40, 20, -12, 5,-1 } (18 additions, 6 shifts) |

- Filter set 2: This set of interpolation filter is referred to as the Directional Interpolation Filter (DIF) and is used for all 15 quarter-pixel positions. For each of the three horizontal positions and the three vertical positions which are aligned with full pixel positions, a single 6-tap filter is used. For the 9 innermost quarter-pixel positions two 6-tap filters at +45 degree and -45 degree angles are used.

The filter coefficients for DIF are

[3 -15 111 37 -10 2]/128 for ¼ displacement (and mirrored for ¾ displacement)

[3 -17 78 78 -17 3]/128 for ½ displacement

- Filter set 3: The third set of interpolation filters consists of Separable Filters (SF) where interpolated samples are calculated by first applying a 6-tap filter horizontally and then vertically.

The filter coefficients for SF are

[3 -15 111 37 -10 2]/128 for ¼ displacement (and mirrored for ¾ displacement)

[3 -17 78 78 -17 3]/128 for ½ displacement

- Filter set 4:  Consists of a combination of a 2-tap pre-filter and a 4-tap interpolation filter.

  - The pre-filter is a 1D IIR filter in horizontal and vertical directions. The coefficients are given below:

    - Left-to-right filtering: [ 1 ] / [ 1, c ]

    - Right-to-left filtering: [ c ] / [ 1, c ]

where, the constant c = -11276. The pre-filter produces the same number of samples as the input to it. The filtering can be performed on the picture region to be interpolated.

  - The interpolation filter is implemented as a sequence of 1D horizontal and vertical filtering step. The FIR filters consist of 4-taps. The filter coefficients are the same for both horizontal and vertical directions. The filters used for different sub-pel positions are given below:

    - Full-pel: [6242,   20284,  6242,         0] / 32768

    - 1/4 pel : [2888,   19078,  10520,   280] / 32768

    - 1/2 pel : [1073,   15311,  15311, 1073] / 32768

    - 3/4 pel : [280,    10520,  19078,  2888] / 32768

- Filter set 5 and 6: For each set, positions a, b, and c use a six-tap horizontal filters. Positions d, h, and l use six-tap vertical filters. For the remaining fractional pixel positions, 4×4 non-separable filters are used. Each of the non-separable filters has horizontal, vertical or diagonal symmetry.

### Choice of filter set and offsets

The choice of the filter and offset is signalled to the decoder for each slice. For each of the 15 fractional pixel positions, the choice of the filter is signalled using 3 bits. Before encoding a frame, the encoder selects a filter for each fractional pixel position based on statistics gathered from previously encoded frames. In this text, the filter that minimizes prediction error for the previously encoded frames is selected. For each fractional pixel position, the minimization is performed only on blocks whose motion vector points to that fractional pixel location. The choice of filter remains the same irrespective of the reference frame in which the motion search is being performed.

For reference frame 0 from each list, offsets are sent to the decoder for each of the 15 fractional pixel positions as well as the full pixel position For other reference frames only one frame offset is sent. The offsets are encoded using exponential Golomb codes.

### Partition based Illumination Compensation

Intensity compensation parameters are signaled at each partition using scale and offset parameters.

In the single-list prediction case, one scale s and one offset o are transmitted. For bi-prediction, two transformed scales t0 and t1, and one offset o are transmitted. Scales s0 and s1 are derived from t0 and t1 as s0 = (64 + t0)/2+t1 and s1 = (64+t0) – s0.

The scales are applied to all components, whereas the offsets are applied only to the luma component (in the following the offset o is considered to be 0 for chroma components).

In the single-list prediction case, a predicted, illumination-compensated, sample w is set to ((64+s)*v)/64+o, where v is a motion-compensated sample. For bi-prediction, a predicted sample w is obtained by the weighted average (s0*v0+s1*v1)/64+o where v0 and v1 are the motion-compensated samples from each list.

The parameters s, t0, t1 and o are coded differentially in combination with motion vector differences.

This feature is enabled by a flag on slice level.

## Adaptive Motion Vector Resolution

For each region in a motion partition, the motion accuracy can be adaptively chosen to be 1/4th pixel or 1/8th pixel. We will refer to this as adaptive motion vector resolution. The choice of motion vector resolution is signaled to the decoder. For each motion vector, a motion vector resolution flag is encoded. If the flag is zero, the motion vector precision is 1/4th otherwise 1/8th. If the flag is 1 and motion vector is nonzero, refinement information for this motion vector is sent which specifies 1/8th pel precision.

The encoder always maintains the motion vector (MV) and MVD information at 1/8th pixel resolution. Then, the MV prediction for the current block is formed with 1/8th pixel accuracy. If the current block has only 1/4th pixel motion accuracy, the MV prediction is converted to 1/4th pixel accuracy. On the other hand if the current block has 1/8th pixel motion accuracy, the MVD is formed directly by subtracting the MV prediction from the motion vector for the current block. Once MVD is formed, if the current block has 1/4th pixel accuracy, for all the neighboring blocks used for determining the MVD contexts, the MVDs are converted to 1/4th pixel accuracy. Similar procedure is followed for 1/8th pixel accuracy.

# Intra-frame prediction

For blocks of size 64x64 : 33 Directions (ADI+Planar)

For blocks of size 32x32 : 33 Directions (ADI+Planar)

For blocks of size 16x16 : 33 Directions (ADI+Planar)

For blocks of size 8x8 : 33 directions (Angular+Planar)

For blocks of size 4x4: 9 directions (AVC)

## Adaptive reference sample smoothing

In addition, a smoothing operation can be applied to the reference samples as a pre-processing step before calculating the prediction.  This smoothing operation corresponds to applying an FIR-filter (1,2,1)>>2 with low-pass characteristics to the samples along the two reference rays.  Whether this smoothing operation is used is determined by the encoder and signaled in the bit stream.

## Planar prediction

The planar prediction is designed to be able to reconstruct smooth image segments in a visually pleasing way. It provides maximal continuity of the image plane at the macroblock borders and is able to follow gradual changes of the pixel values by signalling a planar gradient for each macroblock coded in this mode.  When a macroblock is coded in planar mode its bottom-right sample is signalled in the bitstream, the rightmost and bottom samples of the macroblock are linearly interpolated, and the middle samples are bilinearly interpolated from the border samples. When planar mode is signalled, the same algorithm is applied to luminance and both chrominance components separately with individual signalling of the bottom-right samples (16x16 based operation for luminance and 8x8 based for chrominance). The process is illustrated in the picture below.

Planar prediction of an 8x8 (chrominance) block. Bottom-right sample is signalled in the bitstream, rightmost and bottom samples are interpolated linearly, and the middle samples are interpolated bi-linearly.

## Angular prediction

In order to be able to accurately represent directional structures the Intra 8x8 coding mode provides a possibility to predict the blocks at any direction shown below.

Available prediction directions in the angular mode of Intra 8x8 macroblocks.

In the angular mode the prediction direction is given by the displacement of the bottom row of the block and the reference row above the block in the case of vertical prediction or displacement of the rightmost column of the block and reference column left from the block in the case of the horizontal prediction. The displacement is signalled at 1 pixel accuracy. When projection of the predicted pixel falls inbetween reference samples, the predicted value for the pixel is linearly interpolated from the reference samples (at 1/8th pel accuracy). The process is illustrated in the picture below for the sixth row of the block when building vertical prediction at +1 pixel displacement.

Figure 4 An example of angular prediction when operating the sixth row of the block with +1 pixel displacement. Projections of the pixels now fall at the 6/8 sub-pixel location (circles) in the reference row of pixels.

## Arbitrary Directional Intra (ADI)

Arbitrary Directional Intra (ADI) generates prediction pixels by directional extrapolation or calculation using the nearest boundary pixels of the already decoded area. But in ADI, even boundary pixels from the left down region may be used as context pixels for prediction as depicted in below

Example of context pixels for ADI

Whilst the 9 prediction modes are defined separately as Vertical, Horizontal, DC, Diagonal Down-Left, Diagonal Down-Right, Vertical-Right, Horizontal-Down, Vertical-Left, and Horizontal-Up in AVC, most of prediction modes in ADI are defined by integer pair information (dx, dy). The (dx, dy) pair represents the direction which each mode uses for context pixel extrapolation

prediction methods defined by (dx, dy)

## Edge based prediction

If the edge based prediction mode is switched on, the DC prediction mode can be replaced by an adaptive directional prediction mode depending on the presence of an edge in the neighboring reconstructed pixels.

The edge-based prediction mode consists of two steps:

- edge detection based on the Sobel operators to determine if an edge is present and if an edge is present, its direction

- extrapolation of the reference pixels along the direction of the detected edge

If an edge intersecting with the block to be predicted is detected, the reference pixels are extrapolated along the direction of the detected edge as showed in Figure 5.

wa=δx−floor(δx)wb=ceil(δx)−δxp=wa×a+wb×bFigure 5: Computation of the edge based prediction for intra coding

The prediction of the pixel p(x,y) is:

## Combined Intra Prediction (CIP)

The CIP is a low-complexity tool for providing improved prediction, especially in large blocks where a directional linear prediction may not work well, even with the large range of angles available to ADI. CIP predictions comprise a weighted combination of an ADI prediction together with a pixel-by-pixel mean prediction. It provides pixel-by-pixel adaptation but is a simpler tool than, for example, local template matching approaches.

Figure 4-12 Example of Combined Intra Prediction utilizing a local mean and ADI prediction

The local mean is constructed as the average of pixel values to the left, top-left and above the current picture. In the decoder and within the local decoder in the encoder, these values are the reconstructed values after inverse quantization and transform. In the encoder, the reconstructed values are not available because quantization and transform of the prediction residue have not yet been performed, so original values are used This local mean prediction is therefore different between forward prediction (encoder only) and reconstruction (encoder and decoder).

The combination factor is a 5-bit number between 0 and 31/32 which determines the proportion of local mean in the prediction. Since this number is less than 1, the prediction/reconstruction difference noise is damped, and pixel-by-pixel prediction adaptation is possible without noise blow-up. The ADI component provides an overall prediction direction and the local mean prediction provides local adaptation.

For each CTB, a single CIP flag is sent for all PUs. If set, the flag indicates that CIP prediction is used, otherwise ADI prediction is used. The use of CIP is signaled in the SPS.

# Spatial transforms

## Large transform (16x16, 32x32, 64x64)

For smooth data, a large transform has several advantages such as better energy compaction and reduced quantization error. In HD sequences, most image patterns in a macroblock represent a small part of objects or backgrounds which can be described as homogeneous texture patterns with little variation. Therefore, the coding efficiency of high resolution video can be improved by the use of large transforms as well as large block sizes. By contrast, AVC supports only 4x4 and 8x8 transform sizes. Larger transform sizes can also be chosen (or set) for each coding unit (blocks or partitions). Transform sizes larger than the prediction unit can also be supported.

3 additional sizes of transform are included: 16x16, 32x32 and 64x64. In a large transform design, it is very important to minimise complexity. The transforms in this text are based on Chen’s fast DCT algorithm [10]. Chen’s algorithm is not the fastest one, but it has reduced implementation complexity due to the regular butterfly structure. Moreover, it is readily extensible to larger transform sizes.

Figure 5-1 Signal flow graph of Chen’s fast 16-point DCT transform

Figure 5-1 shows the signal flow graph of Chen’s fast factorization of a 16 point DCT transform. In this figure, multiplication constants are represented by sinusoidal functions of specific angles, requiring floating point operations. To solve this problem, we scale and approximate the factors by fixed precision using pre-defined values, which can be calculated by cost effective shift operations. The approximated constants are shown in Table 0., with pre-defined precision value 64 in this case. Here, the ak’s are approximated values of cos(k*pi/32) for k =1,2,…,15.

Table 0 Approximated constants for 16 point transform

| a1 | a2 | a3 | a4 | a5 | a6 | a7 | a8 | a9 | a10 | a11 | a12 | a13 | a14 | a15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63/64 | 62/64 | 61/64 | 59/64 | 56/64 | 53/64 | 49/64 | 45/64 | 40/64 | 35/64 | 30/64 | 24/64 | 18/64 | 12/64 | 6/64 |

When the constant is approximated by the dyadic rational, the transform is no longer truly orthogonal. However, this minor non-orthogonality does not result in any perceptible negative effects on the compression performance, whilst the complexity can be significantly reduced.

## Rotational transform (ROT)

DCT is the most widely used transform in block based video and image codecs. The input to the transform is the residual, i.e. the difference between the prediction and the original signal. But it is well-known that the DCT basis functions are not optimal for some types of residual signal. For example, the residual signal having strong diagonal components cannot be represented efficiently with the DCT basis vectors.

Typically, directional transform schemes are exploited to solve this problem since spatial domain rotation makes it dificult to maintain the original square shape of the block and generally requires many floating point operations. However, it’s hard to develop directional transform cores, especially since many transforms of different sizes are needed. This text uses mode dependent directional transforms (MDDT) for block sizes of 4×4 and 8×8, and uses ROT for the higher block sizes. ROT can be applied as a 2nd transform after the DCT operation for block sizes of 16×16 or higher.

The main idea of ROT is to change the coordination system of the transform basis, instead of direct rotation of the input source. For this purpose, the following matrices are defined as

where Rvertical and Rhorizontal represent the rotation matrices for horizontal and vertical directions with the rotation angles of (a1, a2, a3) and (a4, a5, a6), respectively. In this text, only four out of all possible rotation angles are quantized and used, in order to minimize the encoder-side complexity. The chosen transform is signalled to the decoder using the chosen intra prediction direction as the context.

Furthermore, since ROT is a second transform, not all of the DCT coefficients are processed. For the TUs larger than 8×8, only 8x8 low-frequency areas are rotated by multiplying by the ROT matrix since most coefficients are already compacted into these low-frequency areas.

## Mode dependent directional transforms for intra-prediction residuals

In this text, nine directional intra prediction modes are used for block size of 4×4 and 33 directional intra prediction modes are used for block sizes of 8×8 (INTRA4x4 and INTRA8x8). For each directional prediction mode, the prediction residual exhibits a different directionality. In addition, the magnitude of the error increases for predicted pixels farther away from the boundary pixels used for prediction. To take advantage of this, mode dependent directional transforms (MDDT) are used to encode 4×4 and 8×8 intra prediction residuals. The type of MDDT is coupled with the selected intra prediction direction, so is not explicitly signaled. We briefly describe design and implementation of the MDDT.

For the 8×8 block size, the 33 intra prediction directions are clustered into nine separate directions. Thus, for both 4×4 and 8×8 blocks, nine separate MDDT transforms are designed. The Karhunen-Loève transform (KLT) derived from the statistics of the intra prediction residuals for that mode, would be the optimal transform choice from a rate-distortion perspective. However, KLT is a non-separable transform. For a residual block of size N×N, the size of the KLT matrix is N2×N2. Thus, KLT is prohibitively expensive in term of storage and computational requirement. A separable directional transform is used, which can be described as

Y=CiXRi,

where X, Y, C, and R are all of size  and i is the prediction mode. Singular Value Decomposition (SVD) is applied to the training set of residuals first in the row direction and then, in the column direction. A fixed-point approximation of the transform matrices to reduce computational cost is used.

Adaptive coefficient scanning

After applying separable directional transform, the 2-D transform coefficient matrix is converted to 1-D. In AVC, zigzag scanning order is used for this purpose. In the case of MDDT, even after separable directional transforms are applied, the resulting 2-D transform coefficient matrix still carries some directionality. For example, consider vertical prediction (mode 0). After prediction, transform and quantization, the nonzero coefficients tend to exist along the horizontal direction. By using coefficient scanning oriented in the horizontal direction instead of the zigzag scanning, the non-zero coefficients in the 2D matrix can be further compacted toward the beginning of the 1-D vector. This in turn improves entropy coding efficiency. Quantized transform coefficients of different prediction modes carry different statistics. Therefore, for each mode, adaptive coefficient scanning is used. This is accomplished as follows:

- At the beginning of each video slice, initialize the coefficient scanning order for each prediction mode;

- When a block is entropy-coded, for each non-zero coefficient coded, increment the count at the corresponding position by one;

- After each macroblock is coded, update the coefficient scanning order according to the statistics collected;

- Normalize the collected statistics if needed;

- Use the updated order for coding of future blocks. Go back to 2 until slice is finished.

## Transform Selection

For the luma component of the inter residual signal, two switchable transforms can be adaptively selected for each block size. Therefore, the selected transform can better fit the residue data. The selection is between the DCT and a KLT using a rate-distortion criterion. For transform sizes smaller than 16x16, no explicit flag is sent to indicate which transform is used for each block. Instead, the flag is incorporated within the transform coefficients themselves to reduce overhead. At the decoder, the transform coefficients are first recovered and the parity of their sum is computed. If the parity is even, the inverse DCT is applied. If the parity is odd, the inverse KLT is applied. When the transform size is 16x16 and above, an explicit flag is used to indicate which transform to apply.

For a residual block of size NxN, a separable KLT transform is used, which can be described as:

Y=CinterXCTinter,

where X, Y, Cinter, are of size NxN.

The KLT basis functions are derived from the statistics of the inter prediction residuals from a representative set of sequences. The offline training is done so that the KLT complements the DCT. The transform is implemented in integer arithmetic and the norms are forced to be the same as those of the DCT. In this way, the quantization tables and entropy coding of the DCT can be reused. Also, the first basis vector of the KLT is forced to be constant, so it is consistent with the first basis vector of the DCT, which improves subjective quality at low bit-rates.

The scanning order of the KLT coefficients is the same as for the DCT coefficients.

## Quantization

The basic principle for quantization and de-quantization of coefficients for large transforms is the same as that used in AVC, i.e. a scalar quantizer with dead-zone.

# Deblocking filter

Deblocking filter operates across 8x8 block edges for luma as well as for chroma blocks.

p30 p20 p10 p00

p31 p21 p11 p01

p32 p22 p12 p02

A p33 p23 p13 p03

P34 p24 p14 p04

P35 p25 p15 p05

P36 p26 p16 p06

P37 p27 p17 p07

q00 q10 q20 q30

q01 q11 q21 q31

q02 q12 q22 q32

q03 q13 q23 q33  B

q04 q14 q24 q34

q05 q15 q25 q35

q06 q16 q26 q36

q07 q17 q27 q37

Notation of Block Edge for Deblocking

### Luma filtering

The luma edge between blocks A and B can be filtered if one of the following conditions is true

- Block A or Block B has mbMode==INTRA

- or Block A or block B has nonzero transform coefficients

- or The absolute difference between the horizontal or vertical component of the motion vectors used for Block A and Block B is greater than or equal to 4 in units of quarter luma samples,

- and |p22 - 2*p12 + p02| + |q22 - 2*q12 + q02| + |p25 - 2*p15 + p05| + |q25 - 2*q15 + q05| < β

The luma edge between blocks A and B is not filtered if one of the following conditions is true.

- Block A and Block B belong to the same macroblock and mb_svt_flag==1

- or Block A and Block B belong to the same macroblock and (mbMode==INTER_16x16 or mbMode==INTRA) and transform 16x16 is used

- or the edge is an internal edge of a large macroblock LMB32

- or the edge is an internal edge of a large macroblock LMB64

- or the edge is a macroblock border and both Block A and Block B have planar_flag ==1.

If the edge between Block A and Block B is filtered, one of two types of filtering (weak or strong filtering) is performed. The choice between the strong and the weak filtering is done separately for each line depending on the following conditions. For each line i = 0,7, the strong filtering is performed if all the following conditions are true, otherwise, weak filtering is performed.

- d < (β>>2)

- and (|p3i - p0i| + |q0i – q3i|) < (β>>3)

- and |p0i – q0i| < ((5*tC + 1)>>1),

where tC and β depends on qp their relations are shown in Table 1. tC is increased by 4 when one of Block A or Block B has mbMode==INTRA.

Table 1 Corr_qpc, tc and β

| qp | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corr_qpc | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| tc | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Β | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| qp | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| Corr_qpc | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 6 |
| tc | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 |
| Β | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 |
| qp | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |  |  |  |  |  |  |  |  |
| Corr_qpc | 7 | 7 | 7 | 8 | 8 | 8 |  |  |  |  |  |  |  |  |  |  |  |  |
| tc | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |  |  |  |  |  |  |  |  |
| Β | 54 | 56 | 58 | 60 | 62 | 64 |  |  |  |  |  |  |  |  |  |  |  |  |

Weak filtering

Weak filtering mode filtering is performed based on the above conditions,

 = Clip(-tC,tC, (13*(q0i - p0i) + 4*( q1i - p1i) - 5*( q2i - p2i)+16)>>5))		i = 0,7

p0i = Clip0-255(p0i + )				i = 0,7

q0i = Clip0-255(q0i - ) 				i = 0,7

p1i = Clip0-255(p1i + /2) 				i = 0,7

q1i = Clip0-255(q1i - /2) 				i = 0,7

Strong filtering

Strong filtering mode is performed with the following set of operations.

p00=Clip0-255((p2i + 2*p1i + 2*p0i +2*q0i + q1i + 4)>>3);		i = 0,7

q00=Clip0-255((p1i + 2*p0i + 2*q0i + 2*q1i + q2i + 4)>>3);		i = 0,7

p10=Clip0-255((p2i + p1i + p0i + q0i +2)>>2);				i = 0,7

q10=Clip0-255((p0i + q0i + q1i + q2i +2)>>2);				i = 0,7

p20=Clip0-255((2*p3i + 3*p2i + p1i + p0i + q0i + 4)>>3);		i = 0,7

q20=Clip0-255((p0i + q0i + q1i + 3*q2i + 2*q3i + 4)>>3);			 i = 0,7

### Chroma filtering

The chroma edge between blocks A and B can be filtered if the following condition is true.

- Block A or Block B has mbMode==INTRA

The filtering is not applied if both following conditions are true.

- The edge is a macroblock edge

and both Block A and Block B have planar_flag==1.

If filtering is performed based on the above condition,

 = Clip(-tC,tC,((((q0i - p0i) << 2) + p1i - q1i + 4)  >> 3))		i = 0,7

p0i = Clip0-255(p0i + )						i = 0,7

q0i = Clip0-255(q0i - )						i = 0,7

tC depend on qp and their relations are shown in Table 1. For chroma filtering tC is increased by 4.

### Planar mode filtering

There is a special filtering mode which is applied to the macroblock edge when both macroblocks are encoded in intra planar mode. Luma and chroma edges between blocks A and B are filtered with planar filtering if all of the following conditions are true, otherwise normal filtering described in  and  are performed.

- Edge between Macroblock A and Macroblock B is a macroblock edge

- Both Macroblock A and Macroblock B have planar_flag==1

Planar luma filtering

The planar mode filtering for luma component is performed with the following set of operations.

p3i = (7*p3i + 3)/7			i = 0,15

p2i = (6*p3i + q3i	 + 3)/7			i = 0,15

p1i = (5*p3i + 2*q3i + 3)/7			i = 0,15

p0i = (4*p3i + 3*q3i + 3)/7			i = 0,15

q0i = (3*p3i + 4*q3i + 3)/7			i = 0,15

q1i = (2*p3i + 5*q3i + 3)/7			i = 0,15

q2i = (p3i + 6*q3i + 3)/7			i = 0,15

q3i = (7*q3i + 3)/7			i = 0,15

Planar chroma filtering

The planar mode filtering for chroma component is performed with the following set of operations.

p1i = (4*p1i + 2)/4			i = 0,7

p0i = (3*p1i + q1i	 + 2)/4			i = 0,7

q0i = (2*p1i + 2q1i + 2)/4			i = 0,7

q1i = (p1i + 3*q1i + 2)/4			i = 0,7

# In-loop filtering

The purpose of the adaptive loop filtering (ALF) process is to further reduce the distortion between the original picture and the reconstructed picture caused by complex lossy coding. Filters minimizing the distortion for both luma and chroma components are calculated using the Wiener filter approach. Based on the prediction signal s, the quantized prediction error e', and the reconstructed filter signal s'', the following adaptive loop filter operation is performed:

s'''=i=1Mwi⋅si''+i=1NwM+i⋅si+i=1OwM+N+i⋅ei'+c

After filters are applied in a frame or coding units, filter coefficients are explicitly sent in the bitstream. This adaptive loop filter increases reconstructed picture quality as well as reference picture quality for the next picture coding. The filtering control basis is usually obtained independently from the prediction structure, which results in a large amount of information being sent to the decoder and its inability to access prediction information in its process.

In this text, Coding Block Tree (CBT) synchronized ALF is presented. By utilizing the coding unit structure, the amount of control information to be sent is reduced. Due to the flexible coding unit structure from large to small block, coding unit boundary provides great flexibility to be used as control basis in the adaptive loop filter process. Since the optimal coding unit boundary obtained from previous coding stages reflects objects’ boundaries having similar properties, like prediction and quantization error, a simple control scheme which infers filter control structure from the coding unit structure and the optimal control depth reduces the complexity of the optimal control map while achieving similar performance.

The filter control information is signaled in the slice header in order to avoid re-multiplexing of the information at the CU level coded data.

Once the control units for filtering are determined, a control flag indicating whether filtering is applied is sent to maximize filter performance and reduce complexity of filter application. Depending on the largest block size, the maximum depth, sequence property and coding parameters, the number of coding units can be too many to be used as filter control units itself. Thus the optimal control depth is sent to indicate the maximum depth (i.e., the smallest size) of a coding unit boundary to be used as filter control map.

The text uses M set of separable filters. The M sets of separable filters is transmitted to the decoder for each frame. Whenever the ALF segmentation map indicates that a block should be filtered, for each pixel, a specific set of filters is chosen based on a measure of local characteristic of an image, called activity measure. The text uses the sum-modified Laplacian measure. The sum-modified Laplacian for pixel  is calculated as follows:

.

# Entropy Coding

## Low-Complexity Entropy Coding

General

All syntax elements are encoded using either FLC or “UVLC-like” VLC tables. Each syntax element use one out of 10 pre-determined VLC tables that are designed for different probability distributions. All VLC tables are organized in the conventional way with the shortest binary codeword at the beginning.

Encoding of a parameter/event using a VLC table is typically done in three steps:

- Convert the parameter/event value to a table index by using some enumeration scheme.

- Use the table index to generate a code number through lookup in a sorting table. The purpose of the sorting table is to assign code numbers according to increasing probability so that parameters/events with the high probability are assigned a code number with a low value.

- Use the code number to generate a binary codeword by lookup in the pre-determined VLC table.

The VLC encoding process is illustrated in Figure 6, while the VLC decoding process is illustrated in Error: Reference source not found.

event

table_index

code number

binary codeword

e.g. inter/

16x16/

ref=0

e.g. 12

e.g. 7

e.g. 010

enumeration

sorting table

VLC table

Figure 6 VLC encoding process

code number

table_index

event

e.g. 010

e.g. 7

e.g. 12

e.g. inter/

16x16/

ref=0

inverse VLC table

inverse sorting table

’inverse enumeration’

Figure 7 VLC decoding process

Adaptivity

Two different forms of adaptivity are applied to VLC encoding:

- For some parameters/events, the particular VLC table to be used depends on the value of previously encoded parameters/events. A context variable is maintained that indicates expected probability distribution of the events. This variable is updated as the events are encoded and it is used to look-up VLC table number.

- For some parameters/events, the sorting table is made adaptive. This process is most easily described in terms of decoder operations as follows: For each binary codeword that is decoded into a code number, determine the corresponding table index by lookup into an inverse sorting table. Next, swap that entry in the inverse sorting table with the entry immediately above. This mechanism ensures that a table index value that occurs frequently eventually propagates towards the top of the inverse sorting table, corresponding to the most likely value and the shortest binary codeword. On the encoder side, both a sorting table and the corresponding inverse sorting table need to be updated simultaneously.  Updating the inverse sorting table on the decoder side is illustrated in Figure 7.

code number = 7

before sorting:

after sorting:

12

3

9

3

12

9

0

6

7

0

6

7

table_index = 12

Figure 7 Update of inverse sorting table - example

Transform coefficients

Entropy coding of transform coefficients is performed by an improved version of CAVLC as follows:

  - Organize the quantized transform coefficients, each represented by a level and sign in a 1D array using the conventional zig-zag scan.

  - Coefficients are encoded backwards along the 1D array from the most high frequency coefficient towards the DC coefficients.

  - Each event (to be described below) is encoded using an “UVLC-like” table. Different VLC tables are used for different events.

  - First, the last significant coefficient (last_coeff) is coded in two steps:

    - Code the position (in backward scan order) of last_coeff (last_pos) and a flag (levelFlag = level>1) as a combined event.

    - If level>1 code the value of level as a single event

    - Code the sign as one bit

  - If level==1, continue encoding non-zero levels in run-mode.

  - If level>1, switch to level-mode if any of the following is true:

    - Cumulative sum of all level of coefficients coded so far >1 (excluding the level of last_coeff), is bigger than a threshold

    - The position of the current level is bigger than the threshold position

  - In run-mode, encode each non-zero coefficient using the preceding run and levelFlag as a combined event. Then encode the sign and level (if levelFlag==1).

  - In level-mode, encode each coefficient (including zero coefficients) as level and sign.

For the combined last_coeff/levelFlag event, an adaptive sorting table is used.

Side information

The following parameters are coded as a combined event for each macroblock:

- macroblock mode

- motion vector partition

- transform size

- macroblock level CBP information

- reference frame index for P frames

- dqp flag (indicating whether or not dqp is signalled for that MB)

For this combined event, an adaptive sorting table is used.

The following side information parameters are encoded separately as single events:

- block level CBP

- motion vector differences

- reference frame index for B frames

- intra prediction modes

Block level CBP is encoded using an adaptive sorting table.

## High Coding Efficiency Entropy Coding

For entropy coding, a variation of CABAC (as found in AVC) is employed. The binarization and context modeling are basically the same as in CABAC of AVC, except from a few modifications and additions as further explained below. However, the actual coding of the bins is based on a novel concept that has been introduced in order to support parallelized implementations of entropy encoding and decoding with load balancing.

Fig. 7  Illustration of the novel entropy coding concept

### Novel entropy coding concept

In Fig. 7, the basic entropy coding concept is illustrated. If a syntax element does not already represent a binary syntax element, it is first binarized, i.e., it is mapped onto a sequence of binary decisions (bins). For each bin a context is selected. A context represents a (binary) probability model for a class of bins; it is characterized by the probability and the value of the less probable bin (LPB). As in AVC, the LPB probability is represented by one out of 64 states. At the beginning of the encoding/decoding of a slice, the probability models are initialized using fixed values (as in CABAC of AVC). And after encoding/decoding a bin with a particular model, the model (i.e., the corresponding probability and value of the LPB) is updated. The probability model update, i.e., the probability estimation process is the same as in CABAC of AVC. The association of a bin with a context model is also similar as in CABAC of AVC. It depends on the syntax element, the bin number, and, for some bins, the values of neighboring syntax elements.

The above described binarization and association of the bins with context models is basically the same as in CABAC of AVC. The main difference is the next step. Instead of arithmetically coding the bins with the associated probability models using a single arithmetic coding engine as in CABAC of AVC, the estimated LPB probabilities are quantized, i.e., they are mapped onto a small number of LPB probability intervals and for each of these probability intervals a separate bin encoder/decoder is operated. In one implementation, we used 12 probability intervals and thus 12 different bin encoders/decoders. Each bin encoder/decoder operates at a fixed LPB probability, which can be considered as the representative probability for an LPB interval. The selection of a bin encoder/decoder is implemented via a look-up table that associates a bin encoder with each of the 64 state indices for the LPB probability. Hence, the 64 states that are used for estimating the LPB probability of a context model are mapped onto 12 probability classes, for each of which a separate bin encoder/decoder is operated. The partial bitstreams that are generated by the bin encoders can be written to different partitions of a NAL unit (with the partitioning of the NAL unit being indicated in the slice header) or they can be interleaved into a single bitstream.

The bin encoders and decoders represent simple variable length bin to variable length (V2V) codes, by which a variable number of bins is mapped onto variable length codewords and vice versa.

In the case where parallel processing at the decoder is desired, a load balancing scheme is provided to distribute the computational load onto multiple bin decoders evenly.

### V2V codes

For the bin encoders (cf. Fig. 7), we employ simple V2V codes. It should be again noted that each bin encoder operates at a fixed LPB probability. And for a fixed LPB probability, we can simply construct a code that maps a variable number of bins onto a variable length codeword and vice versa. As a particular example, the following table shows an example for such a mapping. In the example the bins "0" represent the more probable bin values and the bins "1" represent the less probable bin values. The example code has been designed for a LPB probability of 0.15 and it yields a redundancy of only 0.25 % relative to the entropy limit for this probability. Generally, it is possible to get closer to the entropy limit when the table size is increased.

Table 1: Example for a mapping between a variable number of bins and variable length codewords

| Sequence of bins (bin order is from left to right) | codewords (bits order is from left to right) |
| --- | --- |
| 0000 | 1 |
| 01 | 001 |
| 10 | 010 |
| 001 | 011 |
| 0001 00 | 0001 |
| 11 | 0000 1 |
| 0001 1 | 0000 00 |
| 0001 01 | 0000 01 |

At the end of a slice, it is possible that a bin sequence does not represent any bin sequence that is associated with a codeword. In order to terminate the codeword, the shortest codeword for a bin sequence that contains the current bin sequence as prefix is written. At the decoder side, the additionally decoded bins are discarded. As an example, we could use the mapping in the above table and have a bin sequence "00" at the end of a slice. Then we would write the codeword "1" (corresponding to the bin sequence "0000"). At the decoder side, the codeword "1" would be translated into the bin sequence "0000", but the decoder would discard (ignore) the last two decoded bins.

The partial bitstreams can be written to different partitions of the slice data NAL unit (with transmitting the partitioning information in the slice header). It is also possible to combine the partial bitstreams into one single bitstream. Let Lk be the encoded length of the kth partial bitstream, where 0 <= k < 64. We define a prefix code C the following way:

If n < 128, then C(n) = n << 1;
Else if n < 16512, then C(n) = ((n - 128) << 2) | 1;
Else if n < 2113664, then C(n) = ((n - 16512) << 3) | 3;
Else C(n) = ((n - 2113664) << 3) | 7;

where "<<" is a right shift, and "|" is a bit-wise OR.

The output of the parallel framework is the concatenation of the following:

1. [Header] C(Lk) for 0 <= k < 64,
2. [Payload] The encoded sequence for bitstream k (0 <= k < 64).

The decoder first decodes the Lk values from the header, and uses these values to find the starting point of the encoded sequence k within the payload. After this, the 64 bitstreams can be decoded independently of each other from the payload, hence parallel decoding is feasible.

It is also possible to interleave the codewords from different partial bitstreams.  This is in particular beneficial for small slice data NAL units, for which parallelized bin decoding is not required. In order to realize such an interleaving, a codeword buffer for variable-length codewords can be introduced at the encoder side. When a particular bin encoder starts the encoding of a new bin sequence (i.e., the bin encoder receives the first bin after having written the previous codeword), it reserves the next variable-length codeword entry in the codeword buffer. When the received bins represent a bin sequence that corresponds to a codeword, this codeword is written to the previously reserved codeword entry. Examples for the codeword buffer status are illustrated in Fig. 8. At the decoder side, the codewords can be directly read from the bitstream in decoding order.

Fig. 8  Examples for the codeword buffer status

It is also possible to combine the codeword interleaving with a low-delay buffer control. That means, we can suitably restrict the buffer delay that is introduced by the codeword interleaving. With such a low-delay control, a particular codeword is terminated when the maximum number of bits (this maximum number of bits represents the maximum codeword length for a particular bin encoder and is given by the corresponding mapping table) that are associated with the reserved or completed buffer entries following the buffer entry for the particular codeword is greater than a particular threshold. At the decoder side, the extra decoded bins must be discarded; this can be done by using the same measure of the maximum number of bits as at the encoder side.

### Load Balancing

In this section we describe an extension to the parallel framework described in Figure 7 above. The parallel framework makes provisions for the encoded video stream being decodable starting at various offsets, thereby enabling parallel decoding, but the positions of these offsets do not guarantee that computational load on multiple bin decoders is distributed evenly.  In fact, it is likely in practical scenarios that most of the bin decoders will sit idly, having already done their chores, while the rest of them are still busy decoding the appropriate encoded source streams.  In the present section we provide for the parallel framework an improvement, which aims at balancing the computational load at decoding time, for an arbitrary (possibly unknown) number of bin decoders.

The solution has two parts: the encoding and the decoding of the data. In this section all length values are expressed in bits, unless stated otherwise.

The encoding part creates the encoded bit streams S’k of length M’k for each of the input bitstreams Sk, k=0,…,63, with the given entropy coder.  If the entropy coder is properly designed, M’k should be close enough to Mk= Lk*(pk*log(1/pk) + (1-pk)*log(1/(1-pk))).

The output of the load balancing extension to the parallel framework is the concatenation of the following:

- [First part of the header] C(M’k) for k = 0,…,63,

- [Second part of the header, which depends on the V2V codes]

- [Payload] S’k (k=0,…,63)

Part 1 is identical to Part 1 in Section 1.1.2.  Part 3 is identical to Part 2 in Section 1.1.2. The code C() is the same as defined in Section 1.1.2.

The format and content of the second part of the header depends on the V2V codes used to encode the Sk sequences, and a distributor function.  This part is a self-delimiting sequence.

Let D(a, b) be the set of all (v1,…,vb) vectors, for which vk is a positive integer (k=1,…,b), and v1 + v2 + … + vb = a.  Let S’ = S’0 + … + S’63.  Then, using the notations above, the distributor function is a function of the form F : {0,1}^K -> D(K, b).  The distributor partitions the concatenated encoded sequences into b units, such that decoding each unit would take approximately the same time.

In this form, the distribution function gives the lengths of the decodable segments.  We shall also make use of the starting positions of these segments, given by the associated function F’(x), where for F(x)= (v1,…,vb), F’(x)=(0, v1, v1+v2, v1+v2+v3, …, v1+…+vb).  Obviously, the functions F and F’ mutually determine each other.

Example: the uniform distributor assumes that the decoding complexity of any S’k depends only on M’k, that is, the length of S’k.  Then, for the concatenated encoded bit stream x of length S’, F(x) = (S’/b, S’/b, …, S’/b) (the division may be rounded up or down to meet the final sum M’0+…+M’63).  This distributor may be applied to any entropy coding algorithm.

Example: the weighted distributor assumes that the decoding complexity of any S’k is uniform within S’k, but may vary according to k.  Let wk be this complexity for S’k, and W=w0*M’0 + … + w63*M’63.  Then the distributor assigns the values F’(x) = (v1,…,vb) such that for any k < b: the decoding complexity of the segment x[vk,…,vk+1-1] is W/b.  This distributor may be applied to any entropy coding algorithm.

We now describe the format of the second part of the header. Consider a V2V bin encoder.  For all k=1,…,b-1, let Bk be the number of bits, which we have to read ahead from F’(x)[k] to arrive at the start of the next codeword.  That is, Bk=0 if a codeword starts right at F’(x)[k], Bk=1 if the next codeword starts at F’(x)[k+1], and so on.  Then Part 2 of the header is the concatenation of the values C(Bk) for k=1,…,b-1.

After this, the encoder output is fully specified.

The decoder first decodes the M’k values from the header, then uses these values and the known distributor function to compute F(x)[k], k=1,…,b.  Once these values are known, the decoder distributes the F(x) segments among the d available bin decoders, using the following load balancing algorithm.  Let Gk be the number of F(x) segments given to the kth processor, k=1,…,d.  Then

- if d >= b (there is at least one processor for each segment):

Gk = 1 for k=1,…,b and Gk=0 for k>b (the last d-b processors are unused)

- if d < b (there are more segments than processors):

B0=b

G0=0

Bk=Bk-1-Gk-1

Gk=[Bk/(d-k+1)] (here the [] operator means rounding)

Using the output of the algorithm, the first G1 segments are assigned to the first bin decoder, the next G2 segments to the second bin decoder, and so on.  This algorithm creates the best equalized load for d bin decoders, provided the b segments are already equalized for decoding complexity.

The starting points for the encoded segments within the encoded bitstream x are identified using the contents of Part 2 of the header.  After this, each bin decoder is given a pointer to the start of its respective stream, its variables are initialized (optionally, as is in the case of arithmetic coding, from the header), and simultaneous, independent decoding of x can commence.

The load balancing algorithm provides optimal performance for any specific set of b segments and d bin decoders; however, some sets obviously perform better than others.  The best performance among all (b, d) values is reached when b is a multiple of d.

Example. If d divides b, then the load balancing algorithm simplifies to Gk=b/d for all k=1,…,d.

The general method works with the case when the encoder has no information about the number d of bin decoders at the side of the decoder.  In the case that the encoder has a knowledge of some potential candidates d1, d2, …, dk for the number of bin decoders, we present the following improvement: let b be a number which is a multiple of all of d1, …, dk (the least common multiple of the numbers d1, …, dk is adequate).  In this case, for all the candidates the load distribution algorithm simplifies to the previous example, while for all other bin decoder numbers it resorts to the general algorithm.

Example. Suppose that the encoder knows that target decoders may be equipped with any one of 1, 2, 3, 4, 6, 8, 10, 12 or 16 bin decoders.  The least common multiple of these numbers is 240, so the encoder should set b=240.

Example. If the encoder sets b=2t, then optimal load balancing is achieved for each of 1, 2, 4, 8, …, 2t bin decoders.

### Binarization and context modeling

As already mentioned above, we have reused the binarization and context modeling schemes of AVC CABAC for most syntax elements in this video coding algorithm, especially for those which have already been present in AVC like, e.g., syntax elements related to intra prediction modes or motion parameters. For entropy coding of novel syntax elements like, e.g., flags indicating the quadtree structure, suitable context models have been designed. In addition, for coding of syntax elements related to transform coefficients of larger block sizes, an improved context modeling scheme has been employed.

### Improved context modeling scheme for coding of transform coefficients

The novel context modeling scheme applies to the syntax elements significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus_one of blocks of size 8x8 and larger, and it is complemented by an adaptive switching of the scan during encoding/decoding of the significance map (specifying the locations of non-zero transform coefficient levels).

Coding of the significance map

Coding of the syntax elements significant_coeff_flag and the last_significant_coeff_flag, as constituting the significance map, is improved by an adaptive scan and a new context modeling based on a defined neighborhood of already coded scan positions. This turned out to be beneficial especially for large block sizes.

Fig. 9  Illustration of the two scanning patterns for the significance map

Scanning order

The scanning order for coding the significance map is adapted by switching between two predefined scan patterns. For the first scanning pattern, the diagonal sub-scans are scanned from bottomleft to topright (left illustration of Fig. 9), and for the second scanning pattern, the diagonal subscans are scanned from topright to bottomleft (right illustration of Fig. 9). The coding of the significance map starts with the second scanning pattern. While coding the syntax elements, the number of significant transform coefficient values is counted by two counters c1 and c2. The first counter c1 counts the number of significant transform coefficients that are located in the bottom-left part of the transform block; i.e., this counter is incremented by one when a significant transform coefficient level is coded/decoded for which the horizontal coordinate x inside the transform block is less than the vertical coordinate y. The second counter c2 counts the number of significant transform coefficients that are located in the top-right part of the transform block; i.e., this counter is incremented by one when a significant transform coefficient level is coded/decoded for which the horizontal coordinate x inside the transform block is greater than the vertical coordinate y. At the end of each diagonal sub-scan, it is decided whether the first or the second of the predefined scanning patterns is used for the next diagonal sub-scan. This decision is based on the values of the counters c1 and c2. When the counter for the bottom-left part of the transform block is greater than the counter for the bottom-left part, the scanning pattern that scans the diagonal sub-scans from bottom-left to top-right is used; otherwise (the counter for the bottom-left part of the transform block is less than or equal to the counter for the bottom-left part), the scanning pattern that scans the diagonal sub-scans from topright to bottom-left is used.

Context modeling

For 4x4 blocks, the context modeling for the syntax elements significant_coeff_flag is done as specified in AVC. For 8x8 blocks, the transform block is decomposed into 16 sub-blocks of 2x2 samples, and each of these sub-blocks is associated with a separate context. The context model selection for larger transform blocks (e.g., for blocks greater than 8x8) is based on the number of already coded significant transform coefficients in a predefined neighborhood (inside the transform block). For coding of the last_significant_coeff_flag, a context modeling has been designed that depends on a distance measure of the current scan position to the top-left corner of the given transform block. To be more specific, the context model for coding the last_significant_coeff_flag is chosen based on the scan diagonal on which the current scan position lies (i.e., it is chosen based on x + y, where x and y represent the horizontal and vertical location of a scan position inside the transform block, respectively). To avoid overfitting, the distance measure x + y is mapped on a reduced set of context models in a certain way (e.g. by quantizing x + y).

Coding of absolute values of transform coefficient levels

The coding process for absolute transform coefficient levels maps each quadratic (or rectangular) block of size 8x8 and larger onto an ordered set (vector) of 4x4 sub-blocks by using a forward zig-zag scan; while the transform coefficient levels inside a sub-block are processed in a reverse zig-zag scan. Following the handling of 4x4 blocks in AVC CABAC, the context model set for each sub-block consists of two times five context models with five models for both the first bin and all remaining bins (up to and including the 14. bin) of the coeff_abs_level_minus_one syntax element, where the selection of context models is done exactly as in the original CABAC. However, as a novel feature, different sub-blocks may select different sets of context models, where the choice of the context model set for a sub-block depends on certain statistics of one or more already coded sub-blocks.