| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A301 |
| --- | --- |

| Title: | Tool Experiment 1: Decoder-Side Motion Vector Derivation (draft version) |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Description |  |  |
| Author(s) or Contact(s): | Mathias Wien RWTH Aachen University Yi-Jen Chiu Intel Corp. | Tel: Email: | +49-241-8027672 wien@ient.rwth-aachen.de yi-jen.chiu@intel.com |
| Source: | RWTH Aachen University, Intel |  |  |

_____________________________

# Introduction

The tool experiment targets at exploring the performance and value of DMVD and at analyzing the interaction of DMVD with other tools under consideration for HEVC.

The basic principle for Decoder-side Motion Vector Derivation (DMVD) is the prediction technique to derive motion vector information on a set of motion candidates. The prediction technique such as template matching and motion estimation can be applied at video decoder side to improve the coding efficiency without the requirement of motion vector transmission from video encoder side to video decoder side. A motion candidate is composed of a 2D motion vector and the associated reference picture index. The template matching cost of a candidate is obtained by calculating a cost criterion between the template region in the current picture and the template region referenced by the motion candidate. The template and the reference region in the reference picture is of identical shape; its position is offset by the displacement given by the 2D motion vector of the candidate.

# Participants List

| Participant | Person, Email address |
| --- | --- |
| Huawei | Haoping Yu, haopingyu@huawei.com Mingyuan Yang, myyang@huawei.com Sixin Lin, linsx@huawei.com |
| Intel | Yi-Jen Chiu, yi-jen.chiu@intel.com Lidong Xu, lidong.xu@intel.com |
| JVC | Sakazume Satoru, sakazume.satoru@jk-holdings.com Fukushima Shigeru, fukushima.shigeru@jk-holdings.com |
| Leibniz Univ. Hannover | Sven Klomp, klomp@tnt.uni-hannover.de Marco Munderloh, munderl@tnt.uni-hannover.de |
| LGE | Seung Wook Park, seungwook.park@lge.com |
| MediaTek | Shamin Lei, shawmin.lei@mediatek.com Yu-Wen Huang, yuwen.huang@mediatek.com |
| Qualcomm | Rajan Joshi, rajanj@qualcomm.com |
| RWTH | Mathias Wien, wien@ient.rwth-aachen.de Steffen Kamp, kamp@ient.rwth-aachen.de |
| Skype | Lazar Bivolarsky, lazar.bivolarsky@skype.net |
| Sungkyunkwan University | Byeungwoo Jeon, bjeon@skku.edu |

# Tools under Test

The tools in the table below have been contributed in proposals to the CfP.

| Proponent | Document | Codebase | Tool description |
| --- | --- | --- | --- |
| Huawei | JCTVC-A111 | KTA2.6r1 | Decoder motion derivation with a template and symmetrical motion vectors, MB size 16x16, P- and B-pictures including B-Direct-modes. |
| Intel | JCTVC-A106 | KTA2.6r1 | Mirror-based motion estimation for B-pictures including template match metric |
| JVC | JCTVC-A108 | JM16.2 | Second MV derivation using decoder-side inter-reference ME supported block size 16x16 P- and B-pictures |
| MediaTek | JCTVC-A109 | KTA2.6 | Apply DMVD on P16x16, P32x32, P64x64 Support DMVD Skip or DMVD NonSkip Target size same as transform size (4x4, 8x8, 16x16, 32x32) for DMVD NonSkip Target size same as motion partition size (16x16, 32x32, 64x64) for DMVD Skip Fast search with EPZS predictors and fractional pixel refinement Adaptive MV precision P- and B-pictures supported Syntax always transmitted for DMVD Skip and DMVD NonSkip, no parsing problem |
| RWTH | JCTVC-A112 | KTA2.6r1 | Candidate-based fast DMVD, maximum supported block size 32x32, P- and B-pictures |

[TBD: List additional tools/variations to be explored or keep it as is for the moment?]

## Tool  Experiment Tasks

The following tasks shall be performed in this core experiment.

- DMVD in new codebase (not likely to be performed in this period)

- Identify performance of DMVD tools and suitable tool configurations for CS1 and CS2

- Analyze DMVD interaction with tools listed in the TMuC [4] (as present in the software used for the evaluation)

# Test Sequences and Test Points

Simulation results will be generated for the sequences specified in the call for proposals [1]. For each sequence, four quantizer settings shall be used. The proposed quantizer step sizes for the JM are QPI=27, 30, 34, 38 and QPP=QPI+1, QPB=QPI+2. Results according to the CfP coding conditions as given by constraint set 1 and constraint set 2 shall be provided.

Proposed test points:

Class B1:	1920x1080p 24 fps: "ParkScene", "Kimono"

Class B2: 	1920x1080p 50-60 fps: "Cactus", "BasketballDrive", "BQTerrace"

Class C :	832x480p 30-60 fps (WVGA): "BasketballDrill", "BQMall", "PartyScene", "RaceHorses"

Class D :	416x240p 30-60 fps (WQVGA): "BasketballPass", "BQSquare", "BlowingBubbles",

"RaceHorses"

# Time-line and Responsibilities

TBD

- 2010-May-7:	Final CE-description, selection of software basis, and description of proposals

- 2010-May-21:	Final definition of common test configuration

- 2010-June-18	Tool integration and testing

- 2010-July-7: 	Cross-check

- 2010-July-14:	Input Document upload

# Software and Configuration

The selection of the codebase for the tool experiment depends on the further development of the common JCT-VC activity. Here, KTA is used to for evaluation.

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this tool experiment.

## Coding Performance Measurements

Objective rate-distortion measurements using PSNR, BD-PSNR, BD-Rate [2][3].

The visual quality will be assessed by the involved experts.

## Complexity Considerations

For assessment of the complexity impact, the following numbers should be provided:

- Algorithm description including tool configuration

- Number of operations (adds, multiplications, …) on number of reference frames

- Memory bandwidth consumption

- Encoding time (relative to the tool switched off)

- Decoding time (relative to the tool switched off)

- …

# References

- ITU-T Q6/16 and ISO/IEC JTC1/SC29/WG11, Joint call for proposals on video compression technology, MPEG Document N11113, Kyoto, Japan, January 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-A205, 1. JCT-VC Meeting, Dresden, April 2010.