| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A302 |
| --- | --- |

| Title: | Tool Experiment 2 on IBDI and memory compression |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | TE description |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Participants

Coordinator: Takeshi Chujoh (TOSHIBA Corporation)

| Proponent | Contact | Email |
| --- | --- | --- |
| TOSHIBA | Takeshi Chujoh | takeshi.chujoh@toshiba.co.jp |
| Texas Instruments | Madhukar Budagavi | madhukar@ti.com |
| NEC | Kenichi Chono | chono@ct.jp.nec.com |
| LG | Seungwook Park | seungwook.park@lge.com |
| Sharp | Andrew Segall | asegall@sharplabs.com |
| Qualcomm | Rajan Joshi | rajanj@qualcomm.com |
| Intel | Yi-jen Chiu | yi-jen.ciu@intel.com |

# Functionality addressed

- Improvement of coding efficiency by increasing internal process of video codec while minimizing reference frame memory access bandwidth

- Reduction of reference frame memory access bandwidth and reference frame memory size.

# Description of tool

Details are provided in JCTVC-A101, JCTVC-A104 and JCTVC-A117.

## JCTVC-A101 (Texas Instruments)

Consumer electronics video devices such as camera phones, digital still cameras, digital camcorders, personal media players, video conferencing systems, set-top boxes etc. have limited system memory bandwidth available because of cost and power consumption constraints. Video coding consumes a significant amount of this limited system memory bandwidth especially at high-definition resolution and beyond. Techniques that reduce memory bandwidth in video coding are crucial for implementing video coding at HD resolutions and beyond in these devices. Memory bandwidth reduction is also desirable from a power consumption point of view since memory accesses consume a significant amount of power. Therefore, in addition to coding efficiency improvements, memory access bandwidth reduction is important for the new video coding standard.

In the algorithm proposed in JCTVC-A101, the reference frames are compressed before being stored in memory and they are decompressed after being read from the memory. Figure 1 illustrates where the compressed reference frame buffer (CRFB) algorithm is executed within the simplified video encoding block diagram. Decoder implementation is similar. Two key steps in the proposed technique are: (1) Using fixed-length compression (potentially lossy) to compress reference frames in order to maintain random access for predefined blocks of pixels in memory. (2) Carrying out reference frame compression in the core video coding loop so that quantization errors encountered during fixed-length compression show up in the residual after motion compensation thereby preventing drift between the encoder and the decoder.

Figure 1 - Video coding using compressed reference frame buffers (CRFB).

Reference frame compression algorithm is designed based on the following goals:

- Random memory access to predefined pixel blocks.

- Low-complexity implementation.

- Minimal quality degradation.

## JCTVC-A117 (TOSHIBA Corporation)

IBDI (Internal Bit Depth Increase) is a coding tool that expands the bit depth of input picture from M bits to N bits (HAIP: High Accuracy Internal Process) and shrinks the bit depth of decoded picture at DPB input (DPC: Decoded Picture Compression) from N bits to M bits at encoder side (M < N ). At decoder side, the decoded picture with the bit depth of N is rounded to M bits for output (Error: Reference source not found).

By applying HAIP, the accuracy of all internal processes is increased (N-M) bits. This intends to reduce the rounding error of intra-fame prediction, spatial transform, in-loop filtering in order to improve coding efficiency.

DPC intends to maintain the capacity of DPB and bandwidth of DPB access to the ones for the picture with the original bit depth. In the case of the submitted material, M equals to 8 and N equals to 12. Storing the 12 bit decoded picture to DPB will require 1.5 times of DPB capacity and 1.5 times of memory bandwidth compared to storing the 8 bit decoded picture. Therefore, the bit depth compression process of the decoded picture from 12 bits to 8 bits is performed to each color component macroblock by macroblock as follows:

- Obtain the minimum pixel value Min and the maximum pixel value Max of the component

- Calculate a 8 bit minimum pixel value MB_min = Min >> 4

- Calculate a dynamic range R between Max and (MB_min<<4)

- Decide the right shift value S so that R becomes less than (256 << S)

- Calculate the storing pixel P (= ((pixel value) – (MB_min<<4)) >> S) and store P

- MB_min and S are also stored to DPB

The bit depth decompression process is performed to each color component macroblock by macroblock as follows:

- Obtain MB_min, S and P from DPB

- Calculate the decompressed pixel value D (= P << S + MB_min) and use D as the pixel value

Figure 2 explains the relationship of above values.

0

28+4 - 1

Pixel value

Count

(1) Min

(1) Max

(3) R = Max – (MB_min<<4)

(MB_min<<4)

(2) MB_min = Min>>4

Histogram of MB pixels

(4) R < (256<<S)

(5) P (= ((pixel value) – (MB_min<<4)) >> S)

Figure 2 Bit depth compression process

## JCTVC-A104 (NEC Corporation)

The contribution presented that a pixel quantization process for shrinking increased bit-depth generates banding-artifacts on IBDI output images, and that the banding artifacts can be successfully masked by comfort noise. Since a bit-depth compression scheme and a reference frame memory compression scheme are based on the quantization process, those schemes may generate banding-artifacts on the output images. Therefore in addition to compression performance improvements by the IBDI and reference frame memory access bandwidth reduction by the CRFB, their visual impacts on the output images shall be cross-checked by the core experiment proponents.

As a memory compression scheme, we developed an H.264/MPEG-4 AVC decoder using an 1-D DPCM as in [1]. Figure 3 shows the proposed 1-D DPCM prediction structure and the number of bits allocated to luminance pixels. (Chroma pixels are compressed in a similar way.) A 1-D DPCM block consists of eight consecutive pixels. The right-most pixel is first coded with PCM and serves as a base point. The other seven pixels are then coded with DPCM from right to left, using a five-bit nonlinear quantizer, and they contain distortion. In the core experiment, we use a different version of the 1-D DPCM, in which the base point is placed in the left-most pixel and the other seven pixels are DPCM coded by the left-most pixel.

Figure 3 1-D DPCM prediction structure and bit allocation.

As JCTVC-A101, the proposed reference frame compression algorithm is designed based on the following goals:

- Random memory access to predefined pixel blocks.

- Low-complexity implementation.

- Minimal quality degradation.

[1] J. Tajime and Y. Miyamoto, "Memory Compression Method for Avoiding Perceivable Distortion Due to Intra-Prediction in H.264 Decoders," Proc. of IEEE International Conference on Consumer Electronics, 2007.

# Expected gains

- Improvement of coding efficiency by increasing internal process of video codec codec while minimizing reference frame memory access bandwidth

- Reduction of reference frame memory access bandwidth and reference frame memory size.

# Tool experiments conditions

## Software

JM-KTA 2.6r1

## Coding conditions

Cfg files from VCEG-AJ10 common conditions.

- CS1 will use Hierarchical-B frames with “GOP length 8”.

- CS2 will use IPPP coding.

Following KTA tools enabled for IBDI off:

- MVCompetition           	= 1  # Enabled with default parameters

- UseIntraMDDT            	= 1  # Use MDDT for intra blocks

- UseHPFilter             	= 1  # Use High Precision H.264 filter

- UseAdaptiveLoopFilter   	= 1  # Use adaptive loop filtering

- UseExtMB                	= 2  # Use extended block size (64x64)

Following KTA tools enabled for IBDI on:

- InputBitDepth          	= 8  # InputBitDepth for IBDI

- BitDepthLuma          	= 12 # Bit Depth for Luminance

- BitDepthChroma        	= 12 # Bit Depth for Chrominance

## Test set

Class A: Size 2560x1600 (pixel resolution as in original 4Kx2K) 30 fps

| Sxx | Name | Duration | QP values for CS1 | QP values for CS2 |
| --- | --- | --- | --- | --- |
| S01 | Traffic | 0-73 frames | 22, 25, 28, 31, 33 | - |
| S02 | People on Street | 0-73 frames | 29, 33, 37, 41, 43 | - |

Class B1: Size 1920x1080p 24 fps

| Sxx | Name | Duration | QP values for CS1 | QP values for CS2 |
| --- | --- | --- | --- | --- |
| S03 | Kimono | 72-193 frames | 21, 24, 27, 30, 34 | 22, 25, 28, 32, 35 |
| S04 | ParkScene | 0-121 frames | 24, 27, 30, 33, 36 | 25, 27, 30, 32, 35 |

Class B2: Size 1920x1080p 50-60 fps

| Sxx | Name | Duration | QP values for CS1 | QP values for CS2 |
| --- | --- | --- | --- | --- |
| S05 | Cactus | 0-121 frames | 25, 27, 30, 32, 35 | 26, 28, 31, 33, 36 |
| S06 | BasketballDrive | 0-121 frames | 25, 27, 30, 33, 36 | 27, 29, 32, 35, 38 |
| S07 | BQTerrace | 0-121 frames | 26, 28, 30, 31, 34 | 28, 29, 30, 32, 34 |

Class E: Size 1280x720p 60 fps

| Sxx | Name | Duration | QP values for CS1 | QP values for CS2 |
| --- | --- | --- | --- | --- |
| S16 | Vidyo1 | 0-121 frames | - | 24, 28, 31, 33, 36 |
| S17 | Vidyo3 | 0-121 frames | - | 26, 29, 32, 34, 37 |
| S18 | Vidyo4 | 0-121 frames | - | 25, 27, 30, 32, 35 |

## Evaluation criteria

- Measure impact on bitrate/PSNR using provided data. Use 5-point BD-PSNR and BD-Rate.

- Memory compression ratio.

- Complexity (encoding and decoding times)

- Subjective quality (informal comments)

## Time lines

+2W	: Deadline for sending email to coordinator expressing interest in participating in tool experiment.

+2W	: Deadline of formal description of all proposals.

+2W	: Distribution of document of core experiment.

Geneva-4W	: Distribution of the experimental software

Geneva-2W	: Check of experimental results and check subjective quality

Geneva-1W	: Upload contributions

Geneva		: