| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A303 |
| --- | --- |

| Title: | Tool Experiment 4: Inter Prediction in HEVC |  |
| --- | --- | --- |
| Status: | Output Document to JCT-VC |  |
| Purpose: | TE description |  |
| Authors: | Andreas Krutz1, Alexander Glantz1, Thomas Sikora1, Joonyoung Park2, Edouard Francois3, Yin Peng3, Peisong Chen4 | krutz@nue.tu-berlin.de jy.park@lge.com edouard.francois@technicolor.com peisongc@qualcomm.com |
| Source: | TU Berlin1, LG2, Technicolor3, Qualcomm4 |  |

_____________________________

# Introduction

The aim of this Tool Experiment is to improve temporal prediction and geometric block partitioning in HEVC test model.

# Participants

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 | Andreas Krutz (TU Berlin) – coordinator Thomas Sikora (TU Berlin) - coordinator Alexander Glantz (TU Berlin) Seungwook Park (LG) Byeong Moon Jeon (LG) Joonyoung Park (LG) Yi-Jen Chiu (Intel) Edouard Francois (Technicolor) Yin Peng (Technicolor) Yung-Lyul Lee (Sejong University/SKT) Martin Winken (HHI) Peisong Chen (Qualcomm) Xiaozhen Zheng (Huawei) Haoping Yu (Huawei) Jan de Cock (Universiteit Gent) Krit Panusopone (Motorola) Patrice Onno (Canon) Shawmin Lei (MediaTek) YW Huang (MediaTek) Lazar Bivolarsky (Skype) Xing Wen (Hong Kong University of Science & Technology) |
| --- | --- |

# Experimental Conditions

## Software

Since there is currently no common Test Software available, the subtests of this TE will be implemented into latest KTA Software.

Proposed modifications from technicolor and Qualcomm:

In the specific case of TExx related to Geometric Block Partitioning, the Qualcomm software (see contribution [5]) that supports an implementation of this tool will be used.

## Test Sequences and Bit Rates

The TE algorithms will be tested on a subset of the test sequences and bit rates for constraint set 1 and 2 as defined in the CfP [1].

Proposed modifications from technicolor:

The TE algorithms will be tested on all test sequences and for constraint set 1 and 2 as defined in the CfP document w11113. The following additional test conditions will be used for TExx (Geometric Block Partitioning):

- Number of frames: 100 frames

- QP values: for each test case, the QP values as used in [5] will be used

## Evaluation of TE Results

Results of the TEs will be evaluated on the Basis of BD-measures as defined in the CfP document [1]. That also means that the results will be compared against the anchor as defined in the CfP. In addition subjective viewing will be done during MPEG July 2010 Geneva meeting.

## Evaluation of Complexity

# Description of Tool Experiment

## Subtest 1: Adaptive Warped Reference (AWR [1]) (LG, TUB)

### General Concept

AWR algorithm generates warped reference pictures that compensate complex motions between a reference image and a current image to encode, where the complex motions are modeled as a parametric image transformation function.

The warped reference pictures are inserted to the reference picture list to be used in the motion estimation and motion compensation processes. To increase the selection ratio of warped reference pictures, we reorder the reference pictures as follows. If n warped reference pictures are generated from i’th reference picture, then the n warped reference pictures are inserted right after the i’th reference picture. Figure 1 shows an example of reordered reference picture list when the first (0th) reference picture is used to generate one warped reference picture.

Figure 1 - Reordering of the reference picture list

The more detailed technical description is described in [1].

### Bitstream organization

Besides the common bitstream, the following side information is send to the decoder:

| Location | Description |
| --- | --- |
| Frame Header | A flag that indicates whether AWR is applied or not |
| Frame Header | 4 motion vectors of the 4 corner points instead of the 8 homography parameters |

## Subtest 2: Adaptive Global Motion Temporal Prediction (Adaptive GMTP) (TUB, LG)

### General Concept

The core of the TE is a refined motion prediction based on short-term and long-term global motion estimation. Multiple previously decoded reference frames from the past and future can be used in combination in order to arrive at a precise prediction signal. Figure 1 shows a decoder that is based on the proposed adaptive global motion temporal prediction.

Figure 1 - Decoder based on Adaptive Global Motion Temporal Prediction

For prediction signal generation, global motion parameters are estimated between the current frame and a number N of previously decoded frames at the encoder, resulting in a set of short-term global motion parameters, e.g. based on an 8-parameter perspective motion model, which can then be combined to long-term parameters. These long-term parameters can then be used to compensate the global motion between those N frames and the current frame, which is illustrated in Figure 2.

Figure 2 - Generation of a prediction signal It,pred for the current picture It. The pictures inside the picture buffer can be past and/or future pictures in diplay order.

For each pixel in a block of the current frame the N related pixels in the N decoded frames are averaged to generate a predicted value with reduced coding noise. Figure 3 illustrates the averaging concept.

Figure 3 - Temporal prediction process to form a prediction signal It,pred for the current picture It, depicted is one line yk from a set of aligned pictures

### Motion model and motion parameter estimation

Since many video sequences have been recorded with a moving and zooming camera, resulting in complex motion in sequences to be coded. Higher-order motion models can efficiently account for the motion a camera. In the TE an 8-parametric higher-order motion model (the well-known perspective motion model) is used.

xq=m0xp+m1yp+m2m6xp+m7yp+1yq=m3xp+m4yp+m5m6xp+m7yp+1

In the above equation, (xp,yp)T is the location of a pixel in picture Ip and (xq,yq)T is its corresponding position in picture Iq. The parameters m0 to m7 describe the motion by means of translation, scaling, rotation, and perspective transformation. The matrix below describes these parameters between frame p and q.

Wp,q=m0,p,qm1,p,qm2,p,qm3,p,qm4,p,qm5,p,qm6,p,qm7,p,q1.

In order to keep memory requirements and complexity during motion estimation low, global motion estimation is only performed between two consecutive frames in display order.

Powerful low complex motion estimation algorithms appeared recently that allow global estimation to be performed on transmitted motion vectors [1][2]. For the purpose of the TE the global motion parameters for each frame of each sequence will be provided.

### Concatenation of short-term global motion parameters

Long-term motion parameters are then derived by accumulating previously estimated/decoded short-term motion parameters of several frames

T=W0,1,W1,2,K,WN−2,N−1

as is exemplary shown in . The accumulation is done by simple matrix multiplication. In that way, the motion between any arbitrary pair of pictures from the set is obtained.

Figure 4 - Generation of long-term global motion parameters

### Prediction signal generation

At the encoder, the final prediction signal is generated on a 16x16 macroblock basis. First, for each possible N, one preliminary prediction signal picture is computed. For that, each macroblock MSE, measured between prediction and original, is minimized. If the MSE of GMTP is less or equal than the MSE of the other modes, the GMTP block is taken for the preliminary prediction signal picture. When all preliminary prediction signals have been generated, the picture with minimum MSE, measure on picture-basis, is taken as the final prediction signal.

### Bitstream organization

Besides the common bitstream, the following side information is send to the decoder:

| Location | Description |
| --- | --- |
| Frame Header | 24 byte global motion parameter set (3 byte floating point precision per parameter, 8 parameters in perspective motion model) |
| Frame Header | 4 bit number N of pictures used for filtering GMTP blocks in current picture (FLC based on the assumption of maximum decoded picture buffer size of 16=24) |
| Macroblock Header | List of possible mb_type is extended to GMTP16x16 (size depends on entropy coding method) |

## Subtest 3: Geometric Block Partitioning (Technicolor (coordinator Edouard Francois, Qualcomm)

### General Concept

The general concept of Geometric block partitioning (GEO mode) was proposed to VCEG as document VCEG-AF10. In addition to the classical horizontal and vertical motion partitions, GEO mode consists in another kind of motion partition. This motion partition divides the block into 2 regions. The boundary separating the 2 regions is defined by a straight line. One motion vector is sent for each region. Now we will describe how the various geometry partitions are created. The origin is assumed to be at the center of the block. Then, each geometry partition is defined by a line passing through the origin that is perpendicular to the line defining the partition boundary. This is shown in Fig. 1. The geometry partition is defined by the angle subtended by the perpendicular line with the X axis ()  and the distance of the partition line from the origin ().

Figure 5: Parameters defining a geometry motion partition

### Overlapped motion compensation

For a block that is predicted in a manner described above, overlapped block motion compensation (OBMC) is applied. Let the two regions created by a geometry partition be denoted by region 1 and region 2. Let the corresponding motion vectors be denoted by  and , respectively. A pixel from region 1 (2) is defined to be a boundary pixel if any of its four connected neighbors (left, top, right, and bottom) belongs to region 2 (1). Fig. 6 shows an example where light blue pixels belong to the boundary of region 1 and white pixels belong to the boundary of region2. If a pixel is not a boundary pixel, normal motion compensation is performed using the appropriate motion vector. If a pixel is a boundary pixel, the motion compensation is performed using a weighted sum of the motion predictions from the two motion vectors,  and . The weights are   for the region containing the boundary pixel and   for the other region.

Figure 6: Overlapped motion compensation for geometry partitions

### Motion vector prediction for geometric block partitions

The process of forming motion vector prediction for a geometric block partition is shown Figure 7. Motion vectors of neighboring blocks are used to form the motion vector prediction. Depending on the partition, a subset of blocks from  are used to form motion vector prediction. The choice of block for motion vector prediction depends upon whether they lie on the same side or opposite side of the geometry partition boundary.

Figure7: Motion vector prediction for a block having geometry partition

A more detailed description of the tool and of its implementation can be found in [5 to 8].

### Proposed works/tools

Technicolor has made several works related to GEO mode, in particular [6 to 8].

Qualcomm’s submission to the Call for Proposal with software implementation [5] provides a good reference point.

The goal is to evaluate other tools within the reference implementation to further improve the coding efficiency of geometry partition.

### Participants

| Participant | Contact |
| --- | --- |
| Technicolor | peng.yin@technicolor.com |
| Qualcomm | peisongc@qualcomm.com |

# References

[1] ISO/IEC JTC1/SC29/WG11, “Joint Call for Proposals on Video Compression Technology”, MPEG 
     Document N11113, January 2010

[2] Joonyoung Park, Seungwook Park, and Byeongmoon Jeon, “Coding tools using parametric   
     representations to improve coding efficiency”, JCTVC-A021

[3] Michael Tok, Alexander Glantz, Marina Georgia Arvanitidou, Andreas Krutz, Thomas
     Sikora, “COMPRESSED DOMAIN GLOBAL MOTION ESTIMATIONUSING THE     
     HELMHOTZ TRADEOFF ESTIMATOR”, submitted to ICIP 2010

[4] Martin Haller, Andreas Krutz, Thomas Sikora, “ROBUST GLOBAL MOTION 
     ESTIMATION USING MOTION VECTORS OF VARIABLE BLOCK SIZES AND
     AUTOMATIC MOTION MODEL SELECTION”, submitted to ICIP 2010

[5] Marta Karczewicz, Peisong Chen, Rajan Joshi, Xianglin Wang, Wei-Jung Chien, Rahul Panchal, “Video coding technology proposal by Qualcomm Inc” JCTVC Contribution JCTVC-A121, Dresden, Germany, April 2010.

[6] O. Divorra, P. Yin and C. Gomila, “Geometry-Adaptive Block Partitioning”, ITU-TQ.6/SG16 VCEG, VCEG-AF10, April 2007.

[7] O. Divorra, P. Yin and C. Gomila, “Geometry-Adaptive Block Partitioning On B-Frames”, ITU-TQ.6/SG16 VCEG, VCEG-AG13, October 2007.

[8] O. Divorra Escoda, P. Yin and C. Gomila, Geometry-adaptive Block Partitioning; ITU-TQ.6/SG16 VCEG-AH16 January 2008.