| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0002 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Chris Rosewarne (Canon) Matteo Naccari (BBC) Jens-Rainer Ohm (RWTH Aachen) Karl Sharman (Sony) Gary J. Sullivan (Microsoft) Ye-Kui Wang (Qualcomm) | Email: | benjamin.bross@hhi.fraunhofer.de Chris.Rosewarne@cisra.canon.com.au Matteo.Naccari@bbc.co.uk ohm@ient.rwth-aachen.de Karl.Sharman@eu.sony.com garysull@microsoft.com yekuiw@qti.qualcomm.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 26th meeting in Geneva, CH (January 2017) and the 27th meeting in Hobart, AU (March/April 2017).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Propose improvements to the JCTVC-Y1002 HEVC Test Model 16 (HM 16) Update 7 of Encoder Description - Collect reports of errata for the HEVC specification. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |

# Status of AHG2 Work

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

Regarding the document ‘High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 7 of Encoder Description’ [1], the following (non-exhaustive list of) modifications are suggested:

- Update the Random Access GOP structure description to reflect the 16 picture GOP adopted in recent HM versions.

- When the HM SCC codebase is merged into the HM codebase, it is recommended to also merge the SCC Test Model Encoder Description with the HM Encoder Description.

  - From a review of the most recent Screen Content Coding Test Model Encoder Description (JCTVC-W1014), merging the specific described tools into the structure of the HM Encoder Description is suggested, with review to ensure that SCC encoder method description reflects current SCM software behaviour.

- Section ‘3.1.2	Coding tool configuration options’ contains a partial description of coding tool options. As these options are described in the ‘HM software manual’ (a PDF document autogenerated from the HM source code), it is suggested to replace this section with instructions for generating the HM software manual.

- The number of intra prediction example pictures can be reduced.

- Wavefront parallel processing description can be expanded.

- More detail on high throughput operation and associated profiles should be included (including update of Table 5-1).

Regarding subclause 7.4.4 Profile, tier and level semantics, the semantics of HEVC version 4 were modified as follows (relative to JCTVC-V1005):

general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_8bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag, general_max_monochrome_constraint_flag, general_intra_constraint_flag, general_one_picture_only_constraint_flag general_lower_bit_rate_constraint_flag, and general_max_14bit_constraint_flag, when present, have semantics specified in Annex A when the profile indicated by general_profile_idc or general_profile_compatibility_flag[ j ] is a profile specified in Annex A. When profilePresentFlag is equal to 1, general_profile_idc is not in the range of 4 to 7, inclusive, and general_profile_compatibility_flag[ j ] is not equal to 1 for any value of j in the range of 4 to 7, inclusive, these syntax elements are not present and are inferred to be equal to 0. When general_profile_idc is not equal to 10 and is not in the range of 4 to 7, inclusive, and general_profile_compatibility_flag[ 10 ] is not equal to 1 and general_profile_compatibility_flag[ j ] is not equal to 1 for any value of j in the range of 4 to 7, inclusive, the value of each of these syntax elements shall be equal to 0.

Additionally, in Table 9-48 binIdx = 2 is unreachable for sao_eo_class_luma/chroma, and thus was marked as ‘na’ rather than ‘bypass’ in HEVC version 4.

Additionally, the following changes were also made and included in the recently published version 4 of the HEVC specification by ITU-T:

- Search “monchrome_palette_flag” and change to “monochrome_palette_flag”.

- Search “When slice_temporal_mvp_enabled_flag is equal to 0 in a slice with TemporalId are equal to 0” and remove “are”.

- Search “referenc"”, there should be 8 instances, replace with “reference"” (add the missing ‘e’).

- Search “orderd” and change to “ordered”.

- Search “is equal 4” and change to “is equal to 4”.

- Search “an 4-tap filter” and change to “a 4-tap filter”.

- Search “the both the” and change to “both the” (remove the first “the”).

- Search “up to be not including” and change to “up to but not including”.

- Search “decoders implementing the IDCT on 8x8 blocks, should down-convert” and change to “decoders implementing the IDCT on 8x8 blocks should down-convert” (remove comma).

- Search “is well known in industry and” and change to “is well known in the industry and”.

- Search “knee function process that are” and change to “knee function processes that are”.

- In the mode of “No Markup”, search “the the” and remove one of the two.

- Search “withtout” and change to “without”.

- Search “picturea” and change to “picture” (but don’t remove ‘A’ from two instances of “pictureA”).

- In F.11.2, change “NOTE 1” to “NOTE”, as there is only one NOTE in that subclause.

- In F.11.2, the 2nd bullet item starting with “The profile to which bLayer[ j ] is indicated to conform”, the following changes should be made:

  - The bullet item should be indented to be on the same level as the next bullet item, which starts with “The tier to which bLayer[ j ] is indicated to conform”.

  - The line change between “as specified in” and “Table F.3” should be removed.

- Search “an partition” and change to “a partition”.

- Search “inofrmaiton” and change to “information”.

- Search “loop filtering and resampling filter is applied” and change to “loop filtering and resampling filter are applied”.

- Search “data that follow” and change “data that follows”.

For the third edition to be published by ISO/IEC, we have reviewed the text from the ITTF editors and suggested the following changes to be made:

- In 3.5, 3.13 and 3.19, the words in the note should be part of the normative definition (basically, revoke the change of making the end part into a note). The current way of splitting those words away and putting them in notes makes the definition incomplete and technically incorrect.

- In 3.22 and 3.82, the word “adjective” should not be part of the term itself, it’s better to just remove “, adjective”.

- In 3.39, the words “using decoding process (3.44) on” should be changed to “applying the decoding process (3.44) to”.

- In 3.60, the last row with words enclosed in “[]” should be removed.

- In 3.82, remove the word “is” before the word “representing”.

- In 3.128, in the NOTE, change “shall not” to “cannot”, and you may also consider changing “may” to “can”.

- In 3.160, a space is missing immediately after “picture (3.97)”.

- In 3.168, the words “in this third edition of this document” in the note should be changed to “in this document”, as unspecified values are intended to have specified meaning in any future version of this document as well.

- In 3.171, the definition of “z-scan order” in its currently changed form is pretty messed up and impossible to be understood. Either a complete rewording or reverting back to the original wording is needed (for simplicity, the latter is preferred).

- In 7.1, paragraph 2, change “implements” (was “should implement”) to “is expected to implement”.

- In 7.3.5, the last two rows of the syntax table got removed (one containing “sei_payload( payloadType, payloadSize”, one containing “}”); they need to be added back.

- In 7.4.2.2, as part of the semantics of nuh_temporal_id_plus1, Equation 7-1 was changed to be in all capital letters. This change needs to be revoked: “TemporalId = nuh_temporal_id_plus1 − 1” should be changed to “TemporalId = nuh_temporal_id_plus1 − 1”, as the case of letters matters significantly.

- In 7.4.2.4.2, 2nd paragraph after NOTE 2, last sentence: Change “At most, one VPS …” back to “At most one VPS …” (remove the added comma).

- Throughout the document, originally the period symbol ‘.’ was used as the decimal point or decimal mark, but now both period and comma are used in a mixed manner, e.g., there are a lot of mixes of these two in Tables E.1 and E.3 in subclause E.3.1. I’d prefer to revoke all these changes to still use period. If comma is strongly preferred, then at least use it consistently.

- In 7.4.3.2.1, change “log2_diff_max_min_pcm_luma__block_size” back to “log2_diff_max_min_pcm_luma_coding_block_size” (adding back “coding” as highlighted; but don’t add the highlighting to the text).

- In 7.4.3.2.1, NOTE 5, change “allocates” (was “should allocate”) to “is expected to allocate”, and you may also consider changing “may” to “can”.

- Throughout the document, originally ‘:’ was used when describing a list of items using “the following” such as “The following variables blah blah:” Now, in some of such places ‘:’ was changed to ‘.’, but not in all places, one exception for example is in the semantics of row_height_minus1[ i ] in 7.4.3.3.1 and other such examples exist in 7.4.8, and in clause 8 most if not all places are exceptions. Please be consistent unless there is a special reason for the inconsistency.

- In 7.4.4, 1st paragraph after NOTE 4, change “general_max_12bit__flag, general_max_10bit__flag, general_max_8bit__flag, general_max_422chroma_constraint_flag, general_max_420chroma__flag, general_max_monochrome__flag, general_intra__flag, general_one_picture_only_constraint_flag, general_lower_bit_rate__flag”

back to

“general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_8bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag, general_max_monochrome_constraint_flag, general_intra_constraint_flag, general_one_picture_only_constraint_flag, general_lower_bit_rate_constraint_flag” (adding back the multiple instances of “constraint” as highlighted, but don’t add the highlighting to the text).

- In 7.4.4, NOTE 6, change “indicates” (was “should indicate”) to “is expected to indicate”.

- In 7.4.4, 2nd last paragraph before NOTE 8, change “sub_layer_non_packed__flag[ i ], sub_layer_frame_only__flag[ i ], sub_layer_max_12bit__flag[ i ], sub_layer_max_10bit__flag[ i ], sub_layer_max_8bit__flag[ i ], sub_layer_max_422chroma__flag[ i ], sub_layer_max_420chroma__flag[ i ], sub_layer_max_monochrome__flag[ i ], sub_layer_intra__flag[ i ], sub_layer_one_picture_only_constraint_flag[ i ], sub_layer_lower_bit_rate__flag[ i ]”

back to

“sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ i ], sub_layer_max_12bit_constraint_flag[ i ], sub_layer_max_10bit_constraint_flag[ i ], sub_layer_max_8bit_constraint_flag[ i ], sub_layer_max_422chroma_constraint_flag[ i ], sub_layer_max_420chroma_constraint_flag[ i ], sub_layer_max_monochrome_constraint_flag[ i ], sub_layer_intra_constraint_flag[ i ], sub_layer_one_picture_only_constraint_flag[ i ], sub_layer_lower_bit_rate_constraint_flag[ i ]”(adding back the multiple instances of “constraint” as highlighted, but don’t add the highlighting to the text).

- In 7.4.7.1, NOTE 4, remove “is” from “When slice_temporal_mvp_enabled_flag is equal to 0 in a slice with TemporalId is equal to 0,”.

- In 7.4.7.1, semantics of entry_point_offset_minus1[ i ], 2nd sentence, change “data that follows” to “data that follow”.

- In 7.4.7.1, NOTE 6, change the first instance of “includes” (was “must include”) to “needs to include”, and remove the second instance of “includes”.

- In 7.4.8, very first sentence, add back ‘A’ at the beginning, or add “The” instead.

- In 8.1.3, NOTE2, change “the current picturea” to “the current picture” (remove ‘a’).

- In 8.1.3, 1st paragraph after NOTE 2, change “c8.3.3” to “8.3.3”.

- In 8.4.3, Table 8-2, two instances of “≤” are used, and in E.3.1 Formulae E-51 to E-54, six instances of “≤” are used. However, “<=” defined in 5.4 and used in 374 places (including formulae). Shouldn’t “<=” be used consistently?

- In 9.3.5.2, the NOTE in the last paragraph, change “is” (was “should be”) to “is expected to be”.

- In A.2, the NOTE in the last paragraph, change “do not infer” (was “should not infer”) to “are not expected to infer”, and change “do infer” (was “should infer”) to “are expected to infer”.

- In A.3.2, the NOTE still contains two instances of “should also be”. If you would like to avoid them, please change them to “is also expected to be”.

- In A.3.4, the following paragraph was a NOTE and now is a normative text. Please revoke this change, and to avoid the use of the two instances of “should also be”, change them to “are also expected to be”.

When general_profile_compatibility_flag[ 3 ] is equal to 1, general_profile_compatibility_flag[ 1 ] and general_profile_compatibility_flag[ 2 ] should also be equal to 1. When sub_layer_profile_compatibility_flag[ i ][ 3 ] is equal to 1 for a value of i, sub_layer_profile_compatibility_flag[ i ][ 1 ] and sub_layer_profile_compatibility_flag[ i ][ 2 ] should also be equal to 1.

- In C.1, Figure C.1, remove “.EPS” at the bottom right part of the figure, which I guess was added unintentionally.

- In D.3.1, NOTE 4, change “are set” (was “should be set”) to “are expected to be set”.

- In D.3.3, NOTE 5, change “is used to” (was “should be used to”) to “is expected to be used to”.

- In D.3.3, NOTE 6, change “is interpreted as” (was “should be interpreted as”) to “is expected to be interpreted as”.

- In D.3.4, 2nd paragraph, change the two instances of “one or more …” back to “the one or more”, as herein “one or more …” refers to the instance of “one or more …” defined in the first paragraph.

- In D.3.8, NOTE 2, change “is associated with” (was “should be associated with”) to “is expected to be associated with”.

- In D.3.8, NOTE 3, change “decoders infer” (was “decoders should infer”) to “decoders are expected to infer”.

- In D.3.13, NOTE 8, change “down-convert” (was “should down-convert”) to “are expected to down-convert”.

- In D.3.16, NOTE 10, change “parameters indicate” (was “parameters should indicate”) to ““parameters are expected to indicate”.

- In D.3.23, in the NOTE, change “Encoders are cautious” (was “Encoders should be cautious”) to “Encoders are expected to be cautious”.

- In D.3.27, NOTE 2, change both instances of “units_field_based_flag[ i ] are” (were “units_field_based_flag[ i ] should be”) to “units_field_based_flag[ i ] is expected to be”.

- In D.3.27, NOTE 3, change “counting_type[ i ] are” (were “counting_type[ i ] should be”) to “counting_type[ i ] is expected to be”.

- In D.3.27, NOTE 4, change “time_offset_value[ i ] it is” (were “time_offset_value[ i ] should be”) to “time_offset_value[ i ] is expected to be”.

- In D.3.27, NOTE 7, change “the ndustry” to “the industry”.

- In D.3.28, NOTE 9, change “is the same” (was “should be the same”) to “is expected to be the same”.

- In D.3.29, NOTE 3, change “are used by” (was “should be used by”) to “are expected to be used by”.

- In D.3.35, NOTE 3, change “is performed” (was “should be performed”) to “is expected to be performed”.

- In F.1, change “Clauses 2 to9” to “Clauses 2 to 9” (add a space).

- In F.7.4.3.3.6, in the NOTE (at the end), change “selects” (was “should select”) to “is expected to select”.

- In F.11.2, change “NOTE 1” to “NOTE”, as there is only one NOTE in that subclause.

- In F.11.2, replace all texts between the NOTE paragraph (leave that paragraph as is) and Table F.3 (leave the table as is) as follows (There was a major error in the original text: multiple paragraphs of shaded text were there but should not, due to an issue related to a field update. I apologize for that, and please let me know if you’d prefer to the following text sent in a Word document):

A decoder that conforms to a list ptliList consisting of profile, tier, level and INBLD capability quadruplets ( Pi, Ti, Li, InbldFlagi ) for i in the range of 0 to d − 1, inclusive, where d is a positive integer, shall be capable of decoding any output operation point containing b necessary layers, where b is less than or equal to d, when the following condition applies:

–      There exists a reordered list, orderedPtliList, of the profile, tier, level and INBLD capability quadruplets ( orderedPj, orderedTj, orderedLj, orderedInbldFlagj ) of the list ptliList such that all the following conditions apply for each value of j in the range of 0 to b − 1, inclusive, where the variable bLayer[ j ] represents the j-th necessary layer in the output operation point:

–      The profile to which bLayer[ j ] is indicated to conform is included in CompatibleProfileList for the profile orderedPj as specified in Table F.3.

–      The tier to which bLayer[ j ] is indicated to conform is lower than or equal to the tier orderedTj.

–      The level to which bLayer[ j ] is indicated to conform is lower than or equal to the level orderedLj.

–      When the profile to which bLayer[ j ] is indicated to conform is a profile specified in Annex A, the value of general_inbld_flag (when OpTid of the output operation point is equal to vps_max_sub_layers_minus1) or sub_layer_inbld_flag[ OpTid ] (when OpTid of the output operation point is less than vps_max_sub_layers_minus1) of bLayer[ j ] is less than or equal to orderedInbldFlagj.

For each particular format range extensions profile specified in A.3.5, the compatible format range extensions profiles are defined as the profiles that are indicated by both of the following conditions being true:

–      The value of general_profile_idc is equal to 4 or general_profile_compatibility_flag[ 4 ] is equal to 1.

–      The value of each constraint flag listed in Table A.2 is less than or equal to the corresponding value specified in the row of in Table A.2 for the particular format range extensions profile.

For each particular scalable format range extensions profile specified in H.11.1.2, the compatible scalable format range extensions profiles are defined as the profiles that are indicated by all of the following conditions being true:

–      The value of general_profile_idc is equal to 10 or general_profile_compatibility_flag[ 10 ] is equal to 1.

–      The value of each constraint flag listed in Table H.4 is less than or equal to the corresponding value specified in the row of Table H.4 for the particular scalable format range extensions profile.

- In F.13.3.4, in the NOTE, change “decoders start” (was “decoders should start”) to “decoders are expected to start”.

- In F.13.4, NOTE 1, change “Encoders take this into account to ensure …” (was “Encoders must take this into account to ensure …”) to “Encoders need to take this into account to ensure …”.

- In F.14.3.2.4, NOTE 2, change “will be” (was “should be”) to “is expected to be”.

- In F.14.3.2.4, NOTE 3, change “decoders infer” (was “decoders should infer”) to “decoders are expected to infer”.

- In G.1, change “Clauses 2 to10” to “Clauses 2 to 10” (add a space).

# Recommendations

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Decide whether to issue a further update to the ‘Encoder Description’ document incorporating any suggested changes. In particular, decide a plan for potential merging of the HM and SCC Encoder Description documents.

# References

- JCTVC-Y1002, “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 7 of Encoder Description”, 25th JCT-VC meeting, Chengdu, CN, 14–21 October 2016.