| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0009 |
| --- | --- |

| Title: | JCT-VC AHG report: Report development for HDR/WCG signalling, backward compatibility, and display adaptation (AHG9) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | E. François (chair), W. Husak, D. Rusanovskyy, P. Topiwala, P. Wu (vice-chairs) | Tel: Email: | +33-649225687 edouard.francois@technicolor.com WJH@dolby.com dmytror@qti.qualcomm.com pankajtva@gmail.com ping.wu@zte.com.cn |
| Source: | AHG9 |  |  |

_____________________________

# Abstract

This document provides a report of the AHG9 activity on HDR/WCG technology for backward compatibility, display adaptation, and quality enhancement post-processing, conducted between the 26th and 27th JCT-VC meetings. It is reported that in the context of the AhG mandates, draft 2 of a technical report on "Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG" was uploaded. It is also reported that updated HDR CTCs and related anchor results have been produced. Two new input contributions are reported to be related to the AhG. The AhG recommended to review these input contributions and the technical report draft 2.

# Mandates

The mandates of this AHG are:

- Study technology for HDR/WCG signalling, backward compatibility, display adaptation, and quality enhancement post-processing

- Produce the draft technical report JCTVC-Z1012.

- Study and propose test conditions for associated experiments.

# Activity

The e-mail reflector is [jct-vc@lists.rwth-aachen.de]. To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG9] in the subject line for easy grouping.

A kick-off message was sent to the e-mail reflector on March 3, 2017, listing the AHG mandates and suggesting emails discussion on these issues to take place on the JCT-VC reflector. 3 emails have been sent, reporting the results of the new JCT-VC HDR anchors, using HM16.14/HDRTools-v0.14, then HM16.15/HDRTools-v0.15. Note that a wrong AHG number (14, instead of 9) has been used in the e-mail exchange.

Regarding mandates 1 and 2, the document JCTVC-Z1012 “Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video (Draft 2)” has been produced during the editing period and uploaded to the JCT-VC document repository by March 27, 2017.

# Related input contributions

Two input contributions appear to be directly related to AHG9.

| Doc.# | Title | Authors | Summary |
| --- | --- | --- | --- |
| JCTVC-AA0037 | Backward Compatible HDR Coding with User Data SEI | P. Topiwala, M. Krishnan, W. Dai | usage of User Data Unregistered SEI for transmitting metadata related to backward compatible solutions to coding HDR video |
| JCTVC-AA0038 | An Update to the Tone Mapping Information SEI | P. Topiwala, M. Krishnan, W. Dai | Suggests a simple modification of the TMI SEI to permit tone mappings in either linear or nonlinear light |

# JCT-VC HDR anchors

The JCT-VC new anchor document for HDR content has been produced and uploaded as contribution JCTVC-Z1020 on February 11, 2017.

Two sets of JCT-VC HDR anchors have been produced, and shared on the JCT-VC reflector. The first set corresponding to anchors using HDRTools-v0.14 and HM16.14, has been distributed on February 20, 2017. The second set corresponding to anchors using HDRTools-v0.15 and HM16.15, has been distributed on March 17, 2017. The results have been produced and cross-checked by Dolby, Qualcomm and Technicolor.

## JCT-VC HDR anchors with HDRTools_v0.14, and HM16.14

Previous anchors were generated with HDRTools_v0.12, and HM16.12 (JCTVC-X1020), with gop size set to 8 and search range set to 64.

A first set of new anchors, aligned with the JCT-VC test conditions (JCTVC-Z1020), has been generated using HDRTools_v0.14, and HM16.14, with gop size set to 16, adaptive search range (ASR) mode on, search range set to 256, and MinSearchWindow set to 64.

The corresponding anchors are attached to the contribution, in file 
JCTVC-CTCs_HDR_HDRToolsv0.14_HM16.14.xls.

Table 1 and Table 2 depict the BD-rate variations compared to the previous anchors (HDRTools_v0.12, HM16.12, gop size 8, search range 64), for Y'CbCr and ICtCp cases respectively.

Table 1. YCbCr BD-rate variations between HDRTools_v0.12, HM16.12, gop size 8, search range 64, and HDRTools_v0.14, HM16.14, gop size 16, ASR on, search range 256.

|  | YCbCr | X | Y | Z | XYZ | DE100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -8.3% | -7.4% | -13.3% | -9.3% | -8.5% | -5.2% |
|  | Market3Clip4000r2 | -12.7% | -12.0% | -15.5% | -13.6% | -33.6% | -17.8% |
|  | SunRise | -24.4% | -23.1% | -27.6% | -25.2% | -53.3% | -26.4% |
| class B | BikeSparklers cut 1 | -10.4% | -9.9% | -10.0% | -10.2% | -19.6% | -10.6% |
|  | BikeSparklers cut 2 | -9.5% | -9.0% | -8.3% | -9.2% | -18.0% | -9.2% |
|  | GarageExit | -13.4% | -12.7% | -15.2% | -13.8% | -20.4% | -13.1% |
| class C | ShowGirl2Teaser | -8.5% | -7.7% | -11.2% | -9.1% | -20.2% | -9.2% |
| class D | StEM_MagicHour cut 1 | -17.3% | -15.4% | -21.0% | -18.6% | -30.4% | -17.6% |
|  | StEM_MagicHour cut 2 | -5.4% | -4.9% | -6.7% | -5.9% | -9.0% | -4.8% |
|  | StEM_MagicHour cut 3 | -5.5% | -4.9% | -7.1% | -6.2% | -9.6% | -4.8% |
|  | StEM_WarmNight cut 1 | -17.2% | -16.0% | -19.0% | -17.6% | -28.1% | -17.8% |
|  | StEM_WarmNight cut 2 | -22.9% | -21.3% | -24.9% | -23.4% | -30.1% | -22.3% |
| class G | BalloonFestival | -9.5% | -8.7% | -12.2% | -10.5% | -27.3% | -14.4% |
| class H | EBU_04_Hurdles | -10.1% | -10.2% | -12.8% | -11.3% | -25.0% | -14.5% |
|  | EBU_06_Start | -12.7% | -10.3% | -15.7% | -13.1% | -29.0% | -12.3% |
|  | Overall | -12.5% | -11.6% | -14.7% | -13.1% | -24.1% | -13.3% |

Table 2. ICtCp BD-rate variations between HDRTools_v0.12, HM16.12, gop size 8, search range 64, and HDRTools_v0.14, HM16.14, gop size 16, ASR on, search range 256.

|  | ICtCp | X | Y | Z | XYZ | DE100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -7.1% | -6.9% | -13.1% | -8.8% | -7.7% | -4.7% |
|  | Market3Clip4000r2 | -12.6% | -12.2% | -14.6% | -13.2% | -30.9% | -17.9% |
|  | SunRise | -24.1% | -23.5% | -25.3% | -24.4% | -42.7% | -26.8% |
| class B | BikeSparklers cut 1 | -9.6% | -9.3% | -11.0% | -10.2% | -15.1% | -10.0% |
|  | BikeSparklers cut 2 | -8.5% | -7.8% | -9.9% | -9.0% | -15.0% | -8.2% |
|  | GarageExit | -12.7% | -12.2% | -13.0% | -12.7% | -16.0% | -12.8% |
| class C | ShowGirl2Teaser | -8.3% | -7.7% | -11.0% | -9.0% | -17.4% | -9.1% |
| class D | StEM_MagicHour cut 1 | -16.6% | -16.0% | -17.2% | -16.9% | -23.7% | -18.1% |
|  | StEM_MagicHour cut 2 | -4.9% | -4.5% | -6.7% | -5.7% | -9.6% | -4.5% |
|  | StEM_MagicHour cut 3 | -5.5% | -5.0% | -6.0% | -5.8% | -8.5% | -5.0% |
|  | StEM_WarmNight cut 1 | -16.6% | -16.2% | -17.1% | -16.8% | -21.8% | -18.0% |
|  | StEM_WarmNight cut 2 | -22.0% | -21.7% | -23.0% | -22.3% | -25.2% | -22.6% |
| class G | BalloonFestival | -9.7% | -8.9% | -11.3% | -10.2% | -25.3% | -14.5% |
| class H | EBU_04_Hurdles | -9.2% | -9.4% | -11.4% | -10.1% | -20.2% | -13.0% |
|  | EBU_06_Start | -11.8% | -10.5% | -13.2% | -11.9% | -25.6% | -12.4% |
|  | Overall | -11.9% | -11.5% | -13.6% | -12.5% | -20.3% | -13.2% |

## JCT-VC HDR anchors HDRTools_v0.15, and HM16.15

A second set of new anchors has been generated using HDRTools_v0.15, and HM16.15, with gop size set to 16, adaptive search range (ASR) mode on, search range set to 256, and MinSearchWindow set to 64.

The corresponding anchors are attached to the contribution, in file 
JCTVC-CTCs_HDR_HDRToolsv0.15_HM16.15.xls.

Table 3 and Table 4 depict the BD-rate variations compared to the anchors using HDRTools_v0.14, and HM16.14, and the anchors using HDRTools_v0.15, and HM16.15 (in both cases, gop size=16, search range=256, ASR on), for Y'CbCr and ICtCp cases respectively.

Table 3. YCbCr BD-rate variations between HDRTools_v0.14, HM16.14 and HDRTools_v0.15, HM16.15 (for both cases, gop size 16, ASR on, search range 256).

|  | YCbCr | X | Y | Z | XYZ | DE100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | Market3Clip4000r2 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | SunRise | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.2% |
| class B | BikeSparklers cut 1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | BikeSparklers cut 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | GarageExit | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| class C | ShowGirl2Teaser | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| class D | StEM_MagicHour cut 1 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | StEM_MagicHour cut 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | StEM_MagicHour cut 3 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | StEM_WarmNight cut 1 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | StEM_WarmNight cut 2 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| class G | BalloonFestival | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| class H | EBU_04_Hurdles | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | EBU_06_Start | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
|  | Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |

Table 4. ICtCp BD-rate variations between HDRTools_v0.14, HM16.14 and HDRTools_v0.15, HM16.15 (for both cases, gop size 16, ASR on, search range 256).

|  | ICtCp | X | Y | Z | XYZ | DE100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -0.1% | -0.1% | 2.7% | 0.7% | 0.9% | 0.1% |
|  | Market3Clip4000r2 | 0.3% | 0.3% | 0.2% | 0.3% | 0.2% | 0.2% |
|  | SunRise | 0.3% | 0.4% | 0.3% | 0.3% | 0.0% | 0.3% |
| class B | BikeSparklers cut 1 | 0.4% | 0.4% | 0.2% | 0.3% | 0.7% | 0.5% |
|  | BikeSparklers cut 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | GarageExit | -0.6% | -0.5% | -0.4% | -0.5% | -0.1% | -0.5% |
| class C | ShowGirl2Teaser | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| class D | StEM_MagicHour cut 1 | 0.2% | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% |
|  | StEM_MagicHour cut 2 | 0.2% | 0.2% | -0.1% | 0.0% | 0.2% | 0.2% |
|  | StEM_MagicHour cut 3 | 0.0% | 0.0% | -0.3% | -0.1% | 0.2% | 0.0% |
|  | StEM_WarmNight cut 1 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | StEM_WarmNight cut 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| class G | BalloonFestival | 0.0% | 0.0% | -0.2% | -0.1% | -0.4% | -0.3% |
| class H | EBU_04_Hurdles | -0.3% | -0.5% | -0.7% | -0.5% | -1.1% | -1.3% |
|  | EBU_06_Start | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.2% |
|  | Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |

# Recommendations

The ad hoc group recommends:

- to review the input contribution falling under the AHG9 mandates;

- to review and revise the draft 2 of the Technical report version on "HDR/WCG technology for backward compatibility, display adaptation, and quality enhancement post-processing".