| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0027 |
| --- | --- |

| Title: | AHG7: MCTS Context information SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael Ouedraogo Jonathan Taquet Franck Denoual Frederic Maze | Email: | nael.ouedraogo@crf.canon.fr jonathan.taquet@crf.canon.fr franck.denoual@crf.canon.fr frederic.maze@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This draft document proposes modifications to the on-going Amd3 on HEVC specification. It describes the prediction context of motion-constrained tile sets (MCTS). The proposal specifies a SEI message for signaling the prediction context of motion-constrained tile sets. The purpose of this SEI is to facilitate the extraction process of tiles and their use in other decoding contexts.

# Problem Statement

The current SEI messages for motion constrained tile sets do not guarantee that a tile set, even when mc_all_tiles_exact_sample_value_match_flag=1, will always reconstruct correctly. Indeed, current MCTS SEI assumes that the tile set is decoded alone. But there exist use cases combining tile sets from different video sequences. In such case, depending on encoder choices, the reconstruction may not always be correct. This contribution proposes to signal more information on the encoding context, in a dedicated SEI message, so that decoder and application using a tile set is informed on encoder choices.

# Proposal

In Clause D.2.1 and D.3.1 insert the following syntax modifications given in red font:

General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| … |  |
| else if( payloadType = = 153 ) |  |
| mcts_extraction_info_nesting( payloadSize ) |  |
| else if( payloadType = = 154 ) |  |
| mcts_context_info ( payloadSize ) |  |
| … |  |
| else if( payloadType = = 160 ) |  |
| layers_not_present( payloadSize ) /* specified in Annex F */ |  |
| } |  |
| } |  |

D.3.1 General SEI message semantics

The list SingleLayerSeiList is set to consist of the payloadType values 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, and 134 to 154, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, and 134 to 154, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, and 135 to 154, inclusive.

In Table D.1, append the following rows to the end of the table:

| mcts_context_info | - The CLVS containing the SEI message |
| --- | --- |

Furthermore, in Clause D.2 and D.3 insert the following additional syntax and semantics:

D.2.45 MCTS context information SEI message syntax

| mcts_context_info( payloadSize ) { | Descriptor |
| --- | --- |
| num_mcts_context_in_minus1 | ue(v) |
| for( i = 0; i <= num_mcts_context_minus1; i++ ) { |  |
| mcts_context_id [i ] | ue ( v ) |
| mcts_slice_encapsulated_flag [ i ] | u(1) |
| mcts_motion_vectors_over_pic_boundaries [ i ] | u(1) |
| if ( loop_filter_across_tiles_enabled_flag ) { |  |
| mcts_exclude_loop_filter_margin[ i ] | u (1) |
| } |  |
| mcts_temporal_motion_vector_predictor_flag [ i ] | u(1) |
| } |  |

D.3.45 MCTS context information SEI message syntax

The motion-constrained tile sets context information SEI message indicates context information for motion-constrained tile sets. In particular, it specifies the number of slices used for motion-constrained tile sets and inter prediction constraints.

Let a set of pictures associatedPicSet be the pictures from the access unit containing the motion-constrained context information SEI message, inclusive, up to but not including the first of any of the following in decoding order:

–	The next access unit, in decoding order, that contains a temporal motion-constrained tile sets SEI message

–	The next IRAP picture with NoRaslOutputFlag equal to 1, in decoding order.

–	The next IRAP access unit, in decoding order, with NoClrasOutputFlag equal to 1.

The scope of the motion-constrained tile set context information SEI message is the set of pictures associatedPicSet.

When a motion-constrained tile set context information SEI message is present for any picture in associatedPicSet, a temporal motion-constrained tile set SEI message shall be present for the first picture of associatedPicSet in decoding order and may also be present for other pictures of associatedPicSet.  The mcts_context_id[ ] values of the  motion-constrained tile sets context information SEI message shall have a corresponding equal mcts_id[] value in the associated temporal motion-constrained tile set context SEI message with either mc_all_tiles_exact_sample_value_match_flag or mc_exact_sample_value_match_flag equal to 1.

The motion-constrained tile set context information SEI message shall not be present for any picture in associatedPicSet when tiles_enabled_flag is equal to 0 for any PPS that is active for any picture in associatedPicSet.

The motion-constrained tile set context information SEI message shall not be present for any picture in associatedPicSet unless every PPS that is active for any picture in associatedPicSet has the same values of the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width_minus1[ i ], and row_height_minus1[ i ].

NOTE 1 – This constraint is similar to the constraint associated with tiles_fixed_structure_flag equal to 1, and it may be desirable for tiles_fixed_structure_flag to be equal to 1 when the motion-constrained tile sets context information SEI message is present (although this is not required).

When more than one motion-constrained tile sets context information SEI message is present for the pictures of associatedPicSet, they shall contain identical content.

The number of temporal motion-constrained tile sets SEI messages in each access unit shall not exceed 5.

num_mcts_context_minus1 plus 1 indicates the number of context information specified in the motion constrained tile sets context information SEI message. The value of num_mcts_context_minus1 shall be in the range of 0 to 232 – 2, inclusive.

mcts_context_id [ i ] identifies the i-th tile set with mcts_id equal to mcts_context_id [ i ] associated to the i-th context information set. The value of mcts_context_id[ i ] shall be in the range of 0 to 232 − 2, inclusive.

mcts_slice_encapsulated_flag [ i ] equal to 1 specifies that all the coding tree units of the tile set with mcts_id equal to mcts_context_id [ i ] are contained in one or more slice. In addition, this one or more slice contains only coding tree units of the tile sets with mcts_id equal to mct_context_id [ i ]. Otherwise when equal to 0, the one or more slice may include coding unit of tile set with mcts_id different than mcts_context_id [ i ]

NOTE – When mcts_slice_encapsulated_flag [ i ] is equal to 0, the tile set extraction process may remove bytes corresponding to coding tree unit belonging to another tile set to obtain a NAL unit containing only data related to the tile set with mcts_id equal to mcts_context_id [ i ].

mcts_motion_vectors_over_pic_boundaries [ i ] when equal to 0 indicates that no sample outside the picture boundaries and no sample at a fractional sample position for which the sample value is derived using one or more samples outside the picture boundaries is used for inter prediction of the i-th tile set with mcts_id equal to mcts_context_id [ i]. Otherwise when equal to 1 the inter prediction may use sample outside one picture boundary and sample at a fractional sample position for which the sample value is derived using one or more samples outside one picture boundary when the picture boundary is shared with the i-th tile set with mcts_id equal to mcts_context_id[ i ].

NOTE: When mcts_motion_vectors_over_pic_boundaries [ i ] is equal to 1, tile set extraction process shall not use a tile setup for which the tile set with mcts_id equal to mcts_context_id[i] is not sharing the same boundaries with the picture boundaries.

mcts_exclude_loop_filter_margin[ i ] when equal to 1 indicates that no sample inside the margin around the tile boundaries which are modified by loop filters and no sample at a fractional sample position for which the sample value is derived using one or more samples from this margin is used for inter prediction of the i-th tile set with mcts_id eqal to mcts_context_id [ i ]. Otherwise when equal 0, the inter prediction may use sample within this margin.

NOTE: When mcts_exclude_loop_filter_margin[i] is equal to 0, tile set extraction process shall not use a tile setup for which the tile set with mcts_id equal to mcts_context_id[i] is decoded with other tile set different from the tile set at encoding.

mcts_temporal_motion_vector_predictor_flag [ i ] when equal to 0 indicates that the candidates list generation process for temporal motion vector prediction is constrained such that the predictor selected in the list of candidates is never the bottom right motion vector when the bottom right motion vector of the collocated block is outside the i-th tile set with mcts_id equal to mcts_context_id [ i ]. Otherwise when equal to 1 indicates that the derivation process for temporal motion vector prediction is constrained such that the decoding of the motion predictor is the same when decoded with or without the i-th tile set with mcts_id equal to mcts_context_id [ i ].

NOTE: When mcts_temporal_motion_vector_predictor[ i ] is equal to 1, tile set extraction process shall not use a tile setup for which the tile set with mcts_id equal to mcts_context_id[i] is decoded with other tile sets different from the tile sets at the encoding.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).