| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0029 |
| --- | --- |

| Title: | MCTS extraction with slice reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin, Y. Sanchez | Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

360-degree video typically wraps around the vertical picture boundary and extraction of spatial subsections spanning across the vertical picture boundaries is desirable. Furthermore, using projections such as the cubemap projection may lead to scattered video subsections that represent continuous content when ordered in the correct spatial arrangement. Extraction of such bitstream subsets in this correct spatial arrangement is not possible with the current draft MCTS extraction design and hence, applications need to rely on post-decoding rendering modules.

This document proposes to extent the current draft MCTS extraction in order to allow and control extraction of non-neighbouring video subsections. This is achieved by

- Adding signalling of slice segment addresses of the extracted slice segments into the MCTS extraction info SEI message design.

- Incorporating a reordering step into the MCTS extraction process.

# Problem description

In panorama or 360 video applications, it is typical that only a subsection of the picture plane needs to be presented to the user. When video content is encoded using MCTS to support extraction of HEVC compliant sub-bitstreams depicting a video subsection, means for control of the VCL NAL unit order in the extracted sub-bitstream are desirable.

For instance, in 360-degree cylindrical video, the content wraps around at the vertical picture boundaries as indicated in the top of Figure 1. Current draft MCTS extraction design maintains the VCL NAL unit order of the original bitstream, i.e. {a,b,c,d}, also in the extracted sub-bitstream which would require regions to be rearranged post-decoding at the rendering stage. Note that for interactive panorama streaming, low-tier end devices may lack a rendering module that allows rearranging spatial subsets. It is therefore desirable to reorder the VCL NAL units of the extracted sub-bitstream to {b,a,d,c} in order to achieve continuous content across decoded picture plane of the extracted sub-bitstream as illustrated on the bottom of Figure 1.

Figure 1: 360-degree cylindrical video and a picture boundary crossing video subsection.

A further example is 360-degree video using the cubemap projection (CMP) where the samples of each faces are arranged on the coded picture plane, for instance as illustrated on the left-hand side of Figure 2. However, a post-decoder rendering stage is required in systems using CMP to render an undistorted viewport from the content (or subsections thereof). Such a rendering module may come at varying complexity/capability tradeoffs, i.e. some ready-made modules may expect a given arrangement of content (or subsections thereof).

For instance, the tiles associated to a continuous subsection {a,b,c,d,e,f} may be scattered over the picture plane. In this case, it is also desirable to allow for reordering of the associated VCL NAL units, e.g. into {e,f,a,b,c,d}, to achieve continuous content across the extracted picture plane as illustrated at the right-hand side of Figure 2.

Figure 2: 360-degree CMP video with a scattered subsection.

# Proposal

This document proposes to extent the MCTS EIS SEI message of HEVC PDAM3 to solve the problem identified above by

- Adding signalling of output slice segment addresses of the extracted slice segments into the MCTS extraction info SEI message.

- Incorporating a reordering step into the MCTS extraction process.

The following highlighted changes with respect to the syntax and semantics in JCTVC-Z1005 are proposed.

| mcts_extraction_info_set( ) { | Descriptor |
| --- | --- |
| num_extraction_info_sets_minus1 | ue(v) |
| for( i = 0; i <= num_extraction_information_sets_minus1; i++ ) { |  |
| num_associated_tile_set_identifiers_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_associated_tile_set_identifiers_minus1[ i ]; j++ ) |  |
| mcts_identifier[ i ][ j ] | ue(v) |
| num_slice_segments_minus1[ i ] | ue(v) |
| for( k = 0; k <= num_slice_segments_minus1[ i ]; k++ ) |  |
| output_slice_segment_address[ i ][ k ] | u(v) |
| [ … ] |  |
| } |  |
| } |  |

num_slice_segments_minus1[ i ] plus 1 indicates the number of slice segments belonging to an MCTS with mcts identifier equal to any value of the list mcts_identifier[ i ][ j ]. The value of num_slice_segments_minus1[ i ] shall be in the range of 0 to 4096, inclusive.

output_slice_segment_address[ i ][ k ] identifies the slice segment address of the k-th slice segment in bitstream order belonging to an MCTS with mcts identifier equal to any value within the list mcts_identifier[ i ][ j ]. The length of the output_slice_segment_address syntax element is Ceil( Log2( PicSizeInCtbsY ) ) bits. The value of output_slice_segment_address shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive and no value of output_slice_segment_address shall be in output_slice_segment_address[ i ][ k ] more than once for one extraction information set.

[ … ]

- For the k-theach remaining VCL NAL units of each access unit in outBitstream, adjust the slice segment header as follows:

–	For the first VCL NAL unit within each access unit, set the value of first_slice_segment_in_pic_flag equal to 1, otherwise 0.

–	sSet the value of first_slice_segment_in_pic_flag equal to 0 and slice_segment_address equal to output_slice_segement_address[ mctsEISIdTarget ][ k ]. according to the tile setup defined in the PPS with pps_pic_parameter_set_id equal to slice_pic_parameter_set_id.

- Reorder the VCL NAL units in each access unit for ascending values of slice_segment_address.

- For the first VCL NAL unit within each access unit, set the value of first_slice_segment_in_pic_flag equal to 1., otherwise 0.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).