| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0030 |
| --- | --- |

| Title: | Region wise quality indication SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyun Mook Oh Sejin Oh | Email: | hyunmook.oh@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In 360 video applications, a large scale picture for the entire view is divided into multiple regions and then down-sampled with different spatial resolutions or encoded with different quantization parameters. The purpose of them is to reduce the overall bitrate while increasing the image quality of interesting area. When the region wise quality difference is applied, however, the artificial region boundary is noticeable in the regions with details. In order to resolve this on the decoded pictures, a region wise picture quality indication method is proposed to describe the type, the relative level, and the amount of quality degradation in each region. With the information, the unintended region boundary shown in the active area of viewing window could be alleviated.

# Problem Statement

In 360 video applications, devices (e.g., HMD) display not the entire active video area but a partial region in the area. In this usage scenario, the region covered by viewing window could be changed within the possible image area according to the viewer navigation. In order to provide a sufficient resolution of an image in the user viewport, the high spatial resolution of the entire video is considered, which is a burden for bandwidth limited delivery or interfaces.

In order to alleviate the bandwidth issue, the region wise encoded videos with different compression quality can be applied [1, 2]. As an alternative approach, a picture could be divided into multiple regions and then packed with different spatial resolutions so that the less important region is down-sampled while the most important region has higher image quality in terms of spatial resolution. The other approaches to reduce the bandwidth for large scale video are also in discussion for 360 video applications [3]. In Figure 1, examples of multi-regional image with different qualities factors, i.e. compression quality and spatial scaling, are described.

Figure 1. Examples of region wise quality difference caused by (a) different compression quality (b) different spatial scaling

When those approaches are used, the restored full sized picture output will have region-wise different image quality, due to different compression or different spatial down-scaling. As a user targets to see one specific region of interesting or importance, the high quality of image could be provided when the viewing window is located in the high quality region. However, when the viewer does not follow the expectation and the viewing window covers the regions with different image qualities, the viewer might feel uncomfortable and less realism due to the unwanted boundary artifact.

One possible approach to reduce the boundary artifact is to enhance the picture quality of the degraded region in the viewing window with post processing. Another approach would be to make smoothing the boundary to minimize the abrupt transition between regions. For those processes, a region wise quality indication SEI message is proposed to provide region based quality information and details. In the proposed SEI message, two types of quality indication are considered: spatial scaling and quantization error. According to the quality indication types, relative quality level in a picture and the detailed factors of picture quality degradation are described to support the possible post processing.

# Proposed region-wise quality indication SEI message syntax

| region_wise_quality_indication( payloadSize ) { | Descriptor |
| --- | --- |
| region_wise_quality_indication_id | ue(v) |
| region_wise_quality_indication_cancel_flag | u(1) |
| if( !region_wise_quality_indication_cancel_flag ) { |  |
| region_wise_quality_indication_persistence_flag | u(1) |
| region_wise_spherical_coordinate_flag | u(1) |
| quality_indication_type | u(4) |
| number_of_region_minus1 | u(8) |
| for( i = 0; i <= number_of_region_minus1; i++ ) { |  |
| region_top_offset[ i ] | u(8) |
| region_left_offset[ i ] | u(8) |
| region_bottom_offset[ i ] | u(8) |
| region_right_offset[ i ] | u(8) |
| if( region_wise_spherical_coordinate_flag ) { |  |
| region_yaw_center[ i ] | i(16) |
| region_pitch_center[ i ] | i(16) |
| region_roll_center[ i ] | i(16) |
| region_yaw_range[ i ] | u(16) |
| region_pitch_range[ i ] | u(16) |
| } |  |
| region_quality_indication_level[ i ] | u(8) |
| region_quality_indication_info[ i ] | u(8) |
| } |  |
| } |  |
| } |  |

region_wise_quality_indication_id contains an identifying number that may be used to identify the purpose of the one or more region-wise quality indication SEI messages. The value of region_wise_quality_indication_id shall be in the range of 0 to 216 − 1, inclusive.

Values of region_wise_quality_indication_id from 0 to 255 and from 512 to 215 − 1 may be used as determined by the application. Values of region_wise_quality_indication_id from 256 to 511 and from 215 to 216 − 1 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of region_wise_quality_indication_id in the range of 256 to 511, inclusive, or in the range of 215 to 216 − 1, inclusive, shall ignore it.

region_wise_quality_indication_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous region-wise quality indication SEI message in output order that applies to the current layer. region_wise_quality_indication_cancel_flag equal to 0 indicates that region-wise quality indication information follows.

region_wise_quality_indication_persistence_flag specifies the persistence of the region-wise quality indication SEI message for the current layer.

region_wise_quality_indication_persistence_flag equal to 0 specifies that the region-wise quality indication information applies to the current decoded picture only.

Let picA be the current picture. region_wise_quailty_indication_persistence_flag equal to 1 specifies that the region-wise quality indication information persists for the current layer in output order until any of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture picB in the current layer in an access unit containing a region-wise quality indication SEI message with the same value of region_wise_quality_indication_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

region_wise_spherical_coordinate_flag equals to 1 indicates that the SEI message contains spherical surface coordinates corresponding to the region. region_wise_spherical_coordinate_flag equals to 0 indicates that the region location is represented only in rectangular coordinate.

quality_indication_type specifies the types of quality indication describing the quality difference between regions in the picture. In the current specification, two types of qualities are described by the SEI message.

| quality_indication_type | Description |
| --- | --- |
| 0 | reserved |
| 1 | - spatial scaling |
| 2 | - quantization |
| 3-15 | - reserved for future use |

number_of_region_minus1 indicates the total number of regions with quality indication level and information in the coded picture.

region_top_offset[ i ], region_left_offset[ i ], region_bottom_offset[ i ], and region_right_offset[ i ] specify the coordinates of the i-th rectangular region specified in the SEI message. The offsets for the rectangular region are specified in units of luma sample positions. The i-th rectangular region contains the luma samples with horizontal picture coordinates from SubWidthC * region_left_offset[ i ] to pic_width_in_luma_samples − ( SubWidthC * region_right_offset[ i ] + 1 ), inclusive, and vertical picture coordinates from SubHeightC * region_top_offset[ i ] to pic_height_in_luma_samples − ( SubHeightC * ‌region_bottom_offset[ i ] + 1 ), inclusive.

The value of SubWidthC * ( region_left_offset[ i ] + region_right_offset[ i ] ) shall be less than pic_width‌_in_luma_samples and the value of SubHeightC * ( region_top_offset[ i ] + region_bottom_offset[ i ] ) shall be less than pic_height_in_luma_samples.

region_yaw_center[ i ] specifies the value of yaw that corresponds to the center of the i-th region, in increments of 0.01 degrees. The values of region_yaw_center[ i ] shall be in the range of -18000 and 17999. When not present, the value of region_yaw_center[ i ] should be inferred to be equal to 0. When omnidirectional projection indication SEI message is present, region_yaw_center[ i ] should be in the range derived by yaw_center and yaw_range defined in omnidirectional projection indication SEI message.

region_pitch_center[ i ] specifies the value of pitch that corresponds to the center of the i-th region, in increments of 0.01 degrees. The values of region_pitch_center[ i ] shall be in the range of -9000 and 9000. When not present, the value of region_pitch_center[ i ] should be inferred to be equal to 0. When omnidirectional projection indication SEI message is present, region_pitch_center[ i ] should be in the range derived by pitch_center and pitch_range defined in omnidirectional projection indication SEI message.

region_roll_center[ i ] specifies the value of roll that corresponds to the center of the i-th region, in increments of 0.01 degrees. The values of region_roll_center[ i ] shall be in the range of -18000 and 17999. When not present, the value of region_roll_center[ i ] should be inferred to be equal to 0. When omnidirectional projection indication SEI message is present, region_roll_center[ i ] should be in the range derived by roll_center and roll_range defined in omnidirectional projection indication SEI message.

region_yaw_range[ i ] specifies the range of values of yaw that is covered by the i-th region, in increments of 0.01 degrees. The values of region_yaw_range[ i ] shall be in the range of 1 and 36000. When not present, the value of region_yaw_range[ i ] should be inferred to be equal to 36000.

region_pitch_range[ i ] specifies the range of values of pitch that is covered by the i-th region, in increments of 0.01 degrees. The values of region_pitch_range[ i ] shall be in the range of 1 and 18000. When not present, the value of pitch range should be inferred to be equal to 18000.

region_quality_indication_level[ i ] specifies the value of the quality indication level of the i-th region. The value is in the range of 0 to 255 where the minimum value 0 should be used for the highest quality region within the current picture.

region_quality_indication_info[ i ] specifies the detailed information for the quality of the i-th region with regards to quality_indication_type. When quality_indication_type equals 1, the detailed spatial scaling information is specified by region_quality_indication_info. When quality_indication_type equals 2, the detailed encoding quantization information is defined by region_quality_indication_info.

# References

- M38945. “Virtual reality video metadata in ISO base media file format”, Miska M. Hannuksela, Emre B. Aksu, Igor D. D. Curcio, October 2016, Chengdu.

- M39901, “OMAF ROI-VE: Quality ranking indication”, Miska M. Hannuksela, January 2017, Geneva.

- N16636, “Text of ISO/IEC 23000-20 CD Omnidirectional Media Application Format”, January 2017, Geneva

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).