| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0031 |
| --- | --- |

| Title: | Comments on and proposed changes to the draft text for omnidirectional projection indication SEI message in PDAM3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyun Mook Oh Sejin Oh | Email: | Hyunmook.oh@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

At the 26th JCT-VC meeting in Geneva, January 2017, the output document for ISO/IEC 23008-2:201x/PDAM3 was published which contained omnidirectional projection indication SEI message to support 360 video applications. In reviewing the semantics of the SEI message, some possible editorial issues are found. This contribution describes comments and proposed changes to the draft text for omnidirectional projection indication SEI message for those issues.

# Proposal

The proposed specification text is included in the accompanying document in the zip package. The change-marked text is relative to ISO/IEC 23008-2:201x/PDAM3.

# Comments on PDAM3

- roll_center

It is written that the range of roll_center shall be in -18000 and 18000. However, it seems that the coverage is overlapped when roll_center has maximum or minimum value. The following is suggested definition of roll_center.

- Current: 	roll_center specifies the value of roll that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of roll_center shall be in the range of -18000 and 18000. When not present, the value of roll_center should be inferred to be equal to 0.

- Suggested:	roll_center specifies the value of roll that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of roll_center shall be in the range of -18000 and 17999. When not present, the value of roll_center should be inferred to be equal to 0.

- Equations (D-60) and (D-67)

Following the equation (D-60), max matches with j’= croppedHeightY -0.5 while min matches with j’= 0 (assuming that the roll angle  = 0). When considering Figure D.14, however, the matching is described in the reversed direction, that is, max matches with j’= 0 while min matches with j’= croppedHeightY -0.5. To be aligned with the Figure D.14, the following correction is suggested.

- Current: 	 = ( j′ + 0.5 ) * ( min − max ) ÷ croppedHeightY − min			(D-1)

- Suggested	 = ( j′ + 0.5 ) * ( max − min ) ÷ croppedHeightY − max

Also, if it makes sense, the corresponding equation for chroma samples is suggested as follows.

- Current:	′ = ( j′ + CenterTopOffsetC ) * ( min − max ) ÷ croppedHeightC − min	(D-2)

- Suggested:	′ = ( j′ + CenterTopOffsetC ) * ( max − min ) ÷ croppedHeightC − max

- Table D.22

In the consideration of chroma sub-sampling, the pairs of left and top offsets are recommended according to the chroma sampling positions. As chroma samples are down-sampled by 2 pixel resolutions in the horizontal direction (for both 4:2:2 and 4:2:0) and in the vertical direction (only for 4:2:0), respectively, the offsets in the chroma sample grid is thought as half of the offset values in luma sampling grid. For example, the left and top offsets for chroma sampling type 2 with chroma_format_idc = 1 are half of 0.5, i.e. 0.25. Assuming that this is valid, the following sets of CenterLeftOffsetC and CenterTopOffsetC are suggested for Table D.22.

- Current:

Table D.22 – Derivation of CenterLeftOffsetC and CenterTopOffsetC

| chroma_format_idc | ChromaLocType | CenterLeftOffsetC | CenterTopOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0.125 | 0.25 |
| 1 (4:2:0) | 1 | 0.25 | 0.25 |
| 1 (4:2:0) | 2 | 0.125 | 0.125 |
| 1 (4:2:0) | 3 | 0.25 | 0.125 |
| 1 (4:2:0) | 4 | 0.125 | 0.375 |
| 1 (4:2:0) | 5 | 0.25 | 0.375 |
| 2 (4:2:2) | – | 0.125 | 0.5 |
| 3 (4:4:4) | – | 0.5 | 0.5 |

- Suggested:

Table D.22 – Derivation of CenterLeftOffsetC and CenterTopOffsetC

| chroma_format_idc | ChromaLocType | CenterLeftOffsetC | CenterTopOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0.25 | 0.5 |
| 1 (4:2:0) | 1 | 0.5 | 0.5 |
| 1 (4:2:0) | 2 | 0.25 | 0.25 |
| 1 (4:2:0) | 3 | 0.5 | 0.25 |
| 1 (4:2:0) | 4 | 0.25 | 0.75 |
| 1 (4:2:0) | 5 | 0.5 | 0.75 |
| 2 (4:2:2) | – | 0.25 | 0.5 |
| 3 (4:4:4) | – | 0.5 | 0.5 |

# Patent rights declaration(s)

LG Electronics Inc. does not have any current or pending patent rights relating to the technology described in this contribution.