| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0032 |
| --- | --- |

| Title: | Multigeneration processing and Luma Adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A.M. Tourapis, Y. Su, D. Singer 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
| Source: | Apple Inc |  |  |

_____________________________

# Abstract

This contribution discusses the performance of luma adjustment in the context of multigeneration processing. The performance of video data previously processed using a luma adjustment method, converted back to a 4:4:4 representation, and subsequently converted again without the use of a luma adjustment method to a 4:2:0 representation is analyzed.

# Introduction

It has been suggested in several MPEG contributions [1][2][3] that the Y’CbCr non-constant luminance 4:2:0 representation may suffer from chroma leakage problems. This problem is exacerbated in HDR content by the use of the PQ transfer function.

A solution to this problem was presented in [4], where the value of the luma component was iteratively adjusted so as to minimize the distortion introduced into the luminance component. This method was named as the Luma adjustment method and considerably reduced the artifacts that were introduced using conventional downsampling methods. Slight variations of this method that did not require any iterations, and were therefore considerably simpler to implement, were also presented [5][6][7].

In several scenarios, e.g. for display processing or editing, it may be required to convert the signal back to a 4:4:4 representation, process it and then convert it again back to the subsampled representation. There have been concerns that performing these steps, and if the luma adjustment method is not anymore available, artifacts due to chroma leakage would again appear.

This contribution investigates what is the impact of reprocessing content previously downconverted using the luma adjustment method using, however, conventional processing methods for downconversion.

# Multi-generation processing of Luma adjusted content

In our experiment, we converted several sequences to a Y’CbCr non-constant luminance 4:2:0 representation using the luma adjustment method described in [4]. These sequences were then converted back to a 4:4:4 representation, e.g. 4:4:4 RGB or 4:4:4 Y’CbCr. Such a process could happen, for example, in an editing system or within a display processing pipeline, among others. These sequences were then processed and converted one more time to the Y’CbCr non-constant luminance 4:2:0 representation without, however, the use of the luma adjustment method (Table 3). We have explored several variations, including the use of different downsampling filters (Table 4), multiple processing repetitions (Table 5), as well as applying the same concept on non-luma adjusted material (Table 6).

Table 1. Basic conversion results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 57.069 | 64.509 | 53.071 | 56.155 | 58.733 | 64.376 | 53.392 | 56.778 | 54.855 | 48.421 | 55.05 |
| Tibul | 53.761 | 62.882 | 52.225 | 54.453 | 55.158 | 60.007 | 52.677 | 55.007 | 53.982 | 45.614 | 49.462 |
| Market3 | 49.253 | 58.645 | 45.978 | 48.918 | 53.475 | 62.051 | 46.238 | 50.163 | 49.751 | 36.767 | 46.848 |
| AutoWelding | 45.363 | 53.435 | 40.763 | 44.064 | 48.196 | 58.62 | 40.967 | 44.909 | 42.573 | 39.686 | 52.333 |
| BikeSparklers | 43.389 | 50.782 | 35.385 | 39.346 | 45.922 | 58.474 | 35.76 | 40.092 | 38.285 | 34.069 | 48.337 |
| ShowGirl | 49.756 | 57.221 | 44.669 | 48.08 | 52.04 | 62.225 | 44.834 | 48.767 | 47.653 | 39.357 | 50.957 |
| MagicHour | 52.004 | 57.396 | 39.785 | 44.228 | 51.87 | 63.761 | 39.951 | 44.434 | 43.119 | 37.419 | 51.504 |
| WarmNight | 50.437 | 57.737 | 39.985 | 44.314 | 52.762 | 62.637 | 40.293 | 44.8 | 43.672 | 38.101 | 51.494 |
| BalloonFestival | 50.1 | 57.822 | 45.015 | 48.443 | 52.784 | 62.062 | 45.614 | 49.541 | 51.911 | 40.623 | 46.98 |
| Balloon | 54.751 | 63.213 | 51.018 | 54.075 | 58.598 | 67.757 | 51.337 | 55.278 | 54.619 | 37.874 | 47.7 |
| Average | 50.588 | 58.364 | 44.789 | 48.208 | 52.954 | 62.197 | 45.106 | 48.977 | 48.042 | 39.793 | 50.067 |

Table 2. Luma Adjustment (mode 8 + LUT)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 60.86 | 59.463 | 51.976 | 55.574 | 63.184 | 70.214 | 52.23 | 56.601 | 55.014 | 49.155 | 58.381 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.258 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.8 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 |
| AutoWelding | 46.686 | 50.456 | 40.518 | 43.994 | 50.226 | 69.821 | 40.717 | 45.005 | 42.77 | 39.807 | 55.678 |
| BikeSparklers | 43.973 | 49.365 | 35.232 | 39.255 | 46.915 | 69.757 | 35.609 | 40.054 | 38.301 | 34.084 | 51.927 |
| ShowGirl | 50.359 | 55.131 | 44.746 | 48.141 | 53.308 | 66.839 | 44.913 | 49.045 | 48.003 | 39.411 | 53.01 |
| MagicHour | 52.875 | 55.355 | 39.842 | 44.285 | 53.222 | 70.641 | 40.017 | 44.58 | 43.289 | 37.446 | 53.887 |
| WarmNight | 51.9 | 55.315 | 39.756 | 44.155 | 54.552 | 70.767 | 40.056 | 44.671 | 43.585 | 38.145 | 54.147 |
| BalloonFestival | 50.649 | 56.034 | 44.503 | 48.091 | 53.726 | 69.592 | 45.062 | 49.266 | 51.725 | 40.679 | 48.243 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 |
| Average | 52.2001 | 55.4109 | 44.2598 | 47.9289 | 55.1982 | 69.711 | 44.5482 | 48.9016 | 48.087 | 39.9951 | 52.4173 |

Table 3. Reprocessed Luma Adjusted data ([1 6 1] filter)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 59.946 | 59.284 | 51.576 | 55.149 | 62.181 | 69.511 | 51.834 | 56.153 | 54.631 | 48.867 | 57.743 |
| Tibul | 58.59 | 55.54 | 49.496 | 52.887 | 60.756 | 68.857 | 49.793 | 54.174 | 53.445 | 46.35 | 52.683 |
| Market3 | 49.389 | 55.28 | 45.108 | 48.208 | 54.88 | 68.696 | 45.365 | 49.658 | 49.124 | 36.493 | 47.748 |
| AutoWelding | 46.056 | 50.133 | 40.032 | 43.495 | 49.539 | 67.768 | 40.234 | 44.498 | 42.001 | 39.425 | 55.113 |
| BikeSparklers | 43.573 | 49.073 | 34.911 | 38.923 | 46.489 | 68.21 | 35.289 | 39.725 | 37.728 | 33.811 | 51.449 |
| ShowGirl | 49.944 | 54.899 | 44.337 | 47.747 | 52.802 | 65.652 | 44.509 | 48.624 | 47.44 | 39.156 | 52.706 |
| MagicHour | 52.503 | 55.177 | 39.524 | 43.968 | 52.881 | 69.886 | 39.701 | 44.263 | 42.791 | 37.219 | 53.645 |
| WarmNight | 51.475 | 55.154 | 39.551 | 43.939 | 54.152 | 69.789 | 39.852 | 44.46 | 43.208 | 37.951 | 53.87 |
| BalloonFestival | 49.491 | 55.275 | 43.634 | 47.172 | 52.575 | 67.639 | 44.191 | 48.356 | 50.594 | 40.1 | 47.89 |
| Balloon | 54.936 | 61.568 | 50.165 | 53.456 | 59.292 | 69.524 | 50.474 | 54.662 | 54.262 | 37.809 | 47.918 |
| Average | 51.5903 | 55.1383 | 43.8334 | 47.4944 | 54.5547 | 68.5532 | 44.1242 | 48.4573 | 47.5224 | 39.7181 | 52.0765 |

Table 4. Reprocessed Luma Adjusted data (conventional filter)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 59.761 | 59.275 | 51.46 | 55.04 | 61.967 | 69.22 | 51.718 | 56.026 | 54.364 | 48.759 | 57.587 |
| Tibul | 58.263 | 55.515 | 49.398 | 52.782 | 60.369 | 68.181 | 49.694 | 54.046 | 53.245 | 46.222 | 52.436 |
| Market3 | 49.266 | 55.224 | 45.05 | 48.133 | 54.732 | 68.537 | 45.304 | 49.587 | 48.902 | 36.335 | 47.724 |
| AutoWelding | 45.925 | 50.067 | 39.931 | 43.391 | 49.396 | 67.478 | 40.13 | 44.391 | 41.838 | 39.333 | 55.036 |
| BikeSparklers | 43.468 | 49.001 | 34.82 | 38.831 | 46.382 | 68.027 | 35.194 | 39.629 | 37.582 | 33.722 | 51.402 |
| ShowGirl | 49.819 | 54.821 | 44.221 | 47.631 | 52.671 | 65.6 | 44.391 | 48.506 | 47.228 | 39.056 | 52.654 |
| MagicHour | 52.397 | 55.117 | 39.445 | 43.888 | 52.766 | 69.771 | 39.621 | 44.181 | 42.671 | 37.151 | 53.601 |
| WarmNight | 51.372 | 55.109 | 39.47 | 43.858 | 54.048 | 69.637 | 39.771 | 44.378 | 43.086 | 37.887 | 53.812 |
| BalloonFestival | 49.597 | 55.377 | 43.681 | 47.234 | 52.685 | 67.725 | 44.241 | 48.414 | 50.421 | 40.016 | 47.865 |
| Balloon | 54.822 | 61.494 | 50.102 | 53.381 | 59.162 | 69.506 | 50.409 | 54.59 | 54.073 | 37.715 | 47.913 |
| Average | 51.469 | 55.1 | 43.7578 | 47.4169 | 54.4178 | 68.3682 | 44.0473 | 48.3748 | 47.341 | 39.6196 | 52.003 |

Table 5. Reprocessed Luma Adjusted data – two repetitions ([1 6 1] filter)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 59.111 | 59.122 | 51.286 | 54.81 | 61.26 | 68.636 | 51.55 | 55.802 | 54.436 | 48.659 | 57.224 |
| Tibul | 58.019 | 55.511 | 49.327 | 52.704 | 60.119 | 67.836 | 49.629 | 53.964 | 53.282 | 46.223 | 52.463 |
| Market3 | 48.776 | 54.953 | 44.536 | 47.641 | 54.17 | 67.391 | 44.792 | 49.068 | 48.546 | 36.278 | 47.591 |
| AutoWelding | 45.459 | 49.83 | 39.558 | 43.011 | 48.887 | 65.787 | 39.761 | 44.004 | 41.391 | 39.134 | 54.575 |
| BikeSparklers | 43.166 | 48.783 | 34.572 | 38.576 | 46.054 | 66.479 | 34.946 | 39.374 | 37.253 | 33.583 | 50.93 |
| ShowGirl | 49.602 | 54.688 | 43.98 | 47.405 | 52.403 | 64.933 | 44.156 | 48.265 | 47.063 | 38.973 | 52.428 |
| MagicHour | 52.192 | 55.004 | 39.175 | 43.625 | 52.54 | 69.014 | 39.354 | 43.915 | 42.361 | 37.031 | 53.397 |
| WarmNight | 51.09 | 54.989 | 39.31 | 43.691 | 53.774 | 68.792 | 39.613 | 44.215 | 42.881 | 37.788 | 53.603 |
| BalloonFestival | 48.376 | 54.542 | 42.761 | 46.258 | 51.443 | 65.236 | 43.315 | 47.441 | 49.603 | 39.605 | 47.512 |
| Balloon | 54.534 | 61.357 | 49.731 | 53.04 | 58.919 | 69.389 | 50.046 | 54.243 | 54.025 | 37.759 | 47.896 |
| Average | 51.0325 | 54.8779 | 43.4236 | 47.0761 | 53.9569 | 67.3493 | 43.7162 | 48.0291 | 47.0841 | 39.5033 | 51.7619 |

Table 6. Reprocessed conventional data

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 56.535 | 64.167 | 52.621 | 55.686 | 58.195 | 63.886 | 52.942 | 56.306 | 54.484 | 48.186 | 54.77 |
| Tibul | 53.512 | 62.692 | 51.934 | 54.184 | 54.915 | 59.787 | 52.388 | 54.741 | 53.712 | 45.462 | 49.357 |
| Market3 | 48.667 | 58.104 | 45.352 | 48.307 | 52.914 | 61.504 | 45.611 | 49.548 | 48.999 | 36.474 | 46.728 |
| AutoWelding | 44.845 | 52.924 | 40.257 | 43.555 | 47.675 | 58.04 | 40.463 | 44.4 | 41.818 | 39.319 | 52.065 |
| BikeSparklers | 43.016 | 50.423 | 35.055 | 39.007 | 45.534 | 57.987 | 35.429 | 39.755 | 37.713 | 33.796 | 48.1 |
| ShowGirl | 49.378 | 56.89 | 44.275 | 47.692 | 51.629 | 61.643 | 44.445 | 48.371 | 47.124 | 39.107 | 50.806 |
| MagicHour | 51.679 | 57.143 | 39.471 | 43.914 | 51.585 | 63.368 | 39.64 | 44.123 | 42.634 | 37.194 | 51.342 |
| WarmNight | 50.106 | 57.49 | 39.772 | 44.091 | 52.453 | 62.275 | 40.081 | 44.583 | 43.293 | 37.909 | 51.331 |
| BalloonFestival | 48.966 | 56.88 | 44.087 | 47.466 | 51.668 | 60.869 | 44.678 | 48.571 | 50.751 | 40.054 | 46.713 |
| Balloon | 54.362 | 62.94 | 50.544 | 53.633 | 58.259 | 67.571 | 50.867 | 54.833 | 54.308 | 37.805 | 47.679 |
| Average | 50.1066 | 57.9653 | 44.3368 | 47.7535 | 52.4827 | 61.693 | 44.6544 | 48.5231 | 47.4836 | 39.5306 | 49.8891 |

It can be observed (Table 3, Table 4, and Table 5) that although the signal deteriorates compared to the original even further, luminance distortion is still lower than the distortion introduced when using the conventional processing method directly on the original method (Table 1). This is true even when using a different filter for downsampling (Table 4) or when performing the process multiple times (Table 5). Results suggest that preprocessing with a luma adjustment method makes the material more robust to chroma leakage from subsequent processing. This could be beneficial for some applications that may original expect 4:4:4 or 4:2:2 video content but will subsequently convert the data to 4:2:0, to preprocess the video data using a luma adjustment method (e.g. for broadcast encoder applications or for editing). This is especially true if it is unknown whether later conversion to 4:2:0 could support the luma adjustment method.

# Conclusion

The performance of luma adjustment in the context of multigeneration processing is discussed in this document. It is presented that content previously processed using the luma adjustment method appear to retain their “luminance preserving” characteristics. That is conversion back to a 4:4:4 representation followed by a secondary conversion to a 4:2:0 representation, does not appear to require processing for controlling luminance distortion using a luma adjustment method. This property may be useful for some applications, such as video editing and in legacy encoding/transcoding systems that might not be able to directly use the luma adjustment method.

# References

- E. François, P. Lopez, Y. Olivier, “About using a BT.2020 container for BT.709 content”, ISO/IEC (MPEG) document m35255, Sapporo, July 2014

- J. Stessen, R. Nijland, R. Brondijk, R. Goris, “Chromaticity Based Color Signals for Wide Color Gamut and High Dynamic Range”, ISO/IEC (MPEG) document m35065, Strasbourg, France, Oct. 2014.

- J. Ström, “Investigation of HDR color subsampling”, ISO/IEC (MPEG) document m35841, Geneva, Switzerland, Feb. 2015.

- J. Ström, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson's response to CfE for HDR and WCG”, ISO/IEC (MPEG) document m36184, Warsaw, Poland, June 2015.

- A. Norkin “Closed form HDR 4:2:0 chroma subsampling (HDR CE1 and AHG5 related)”, JCT-VC JCTVC-W0107, Geneva, Feb 2016.

- C. Rosewarne and V Kolesnikov, “CE1-related: LUT-based luma sample adjustment”, JCT-VC, JCTVC-W0056, San Diego 2016.

- C. Rosewarne and V Kolesnikov, “AHG13: Further results for LUT-based luma sample adjustment”, JCT-VC, JCTVC-X0054, Geneva 2016.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).