| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0033 |
| --- | --- |

| Title: | Suggested improvements to the omnidirectional projection indication portions of the HEVC Supplemental Enhancement Information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Chad Fogg Jill Boyce, Qian Xu | Email: | chadfogg@gmail.com jill.boyce@intel.com, qian.z.xu@intel.com |
| Source: | Movie Labs, Intel |  |  |

_____________________________

# Abstract

This document suggests improvements to the draft text (JCTVC-Z1005) for changes to the High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 | ISO/IEC 23008-2) supplemental enhancement information (SEI) message for omnidirectional 360° projection. The improvements include equations for stereo-vision support, and add equations for rotation in the Cartesian XYZ domain. To improve clarify of this proposal, text from other SEI messages in JCTVC-Z1005 (regional nesting, and motion-constrained tile sets extraction information) are not included in this input document. Changes relative to JCTVC-Z1005 are visible by enabling "All markup" Word change bar tracking in the Review tab.

# Proposed changes to JCTVC-Z1005

- D.3.41	Omnidirectional projection indication SEI message semantics

The omnidirectional projection indication SEI message provides information to enable remapping of the colour samples of the output decoded pictures onto an alternative coordinate space for use in panoramic video applications.

When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer, an omnidirectional projection indication SEI message shall be present for the first picture of the CLVS. The omnidirectional projection SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS.

The luma dimensions of the cropped output picture are computed as follows:

croppedWidthY = pic_width_in_luma_samples −
	SubWidthC * ( conf_win_right_offset + conf_win_left_offset ) /StereoWidthScale	(D-51)

croppedHeightY = pic_height_in_luma_samples −
	SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset ) /StereoHeightScale	(D-52)

croppedWidthC = croppedWidthY / SubWidthC		(D-53)

croppedHeightC = croppedHeightY / SubHeightC		(D-54)

NOTE: "when segmented_rect_content_interpretation_type is equal to 1 or 2, the cropped picture width shall be equal to r0w from equation D-32, and cropped picture height shall be equal to r0h from equation D-33.  The coordinates (i,j) are samples in the Constituent frame as defined in D.3.29.”

If frame_packing_arrangement_type present, StereoWidthScale and StereoHeightScale are determined according to the following table. Otherwise, if frame_packing_arrangement_type and segmented_rect_content_interpretation_type are not present, the value for StereoWidthScale and StereoHeightScale shall assumed to be 1.

| frame_packing_arrangement_type | StereoWidthScale | StereoHeightScale |
| --- | --- | --- |
| 3 | 2 | 1 |
| 4 | 1 | 2 |
| 5 | 1 | 1 |

Figure D.12 cropped output of decoded picture luma samples

omnidirectional_projection_information_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional projection indication SEI message in output order. omnidirectional_projection_information_cancel_flag equal to 0 indicates that omnidirectional projection indication information follows.

omnidirectional_projection_information_persistence_flag specifies the persistence of the omnidirectional projection indication SEI message for the current layer.

omnidirectional_projection_information_persistence_flag equal to 0 specifies that the omnidirectional projection indication SEI message applies to the current decoded picture only.

Let picA be the current picture. omnidirectional_projection_information_persistence_flag equal to 1 specifies that the omnidirectional projection indication SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a omnidirectional projection indication SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

geometry type indicates the mathematical convention where points within a space can be uniquely identified by a location in one or more dimensions. When geometry_type is equal to 1, the projection indicator is given in spherical coordinates, where  is the azimuth (longitude) and  is the elevation (latitude) as depicted in Figure D.13.

| (a) region with yaw and pitch offsets | (b) region (lighter shade) with yaw, pitch, and roll offsets |
| --- | --- |

Figure D.13 Spherical surface coordinates ,  with yaw, pitch, and roll of the projection image relative to the equator and 0 meridian

sub_geometry_flag indicates the presence of sub geometry elements that further describe or limit the geometry of this SEI message.

projection_type indicates the particular mapping of the rectangular decoder picture output samples onto the coordinate system specified by geometry_type.

yaw_center specifies the value of counter-clockwise rotation in degrees around the up vector, in increments of 0.01 degrees. The values of yaw_center shall be in the range of -18000 and 17999. When not present, the value of yaw_center should be inferred to be equal to 0.

pitch_center specifies the value of counter-clockwise rotation in degrees around the yaw_center, i.e., the right vector post yaw rotation, in increments of 0.01 degrees. The values of pitch_center shall be in the range of -9000 and 9000. When not present, the value of pitch_center should be inferred to be equal to 0.

roll_center specifies the value of clockwise rotation in degrees around the pitch_center, i.e., the forward vector post yaw and pitch rotation, in increments of 0.01 degrees. The values of roll_center shall be in the range of -18000 and 18000. When not present, the value of roll_center should be inferred to be equal to 0.

yaw_range specifies the range of values of yaw that are present in the projection image centered around (0, 0), in increments of 0.01 degrees. The values of yaw_range shall be in the range of 1 and 36000. When not present, the value of yaw_range should be inferred to be equal to 36000.

pitch_range specifies the range of values of pitch that are present in the projection image centered around (0, 0), in increments of 0.01 degrees. The values of pitch_range shall be in the range of 1 and 18000. When not present, the value of pitch range should be inferred to be equal to 18000.

When projection_type is equal to 1, sub_geometry_flag is equal to 1, and geometry_type is equal to 1, cropped output picture samples at locations (i, j) are indicated as corresponding to angular coordinates. For i equal to SubWidthC * conf_win_left_offset to SubWidthC * conf_win_left_offset + croppedWidthC − 1 and j equal to SubHeightC * conf_win_top_offset to SubHeightC * conf_win_top_offset + croppedHeightC − 1, the corresponding angular coordinates (, ) for the luma sample locations, in radians, are given by the following equirectangular mapping equations:

min = (− yaw_range ÷ 2 ) * 0.01 * π ÷ 180		(D-55)

max = (+ yaw_range ÷ 2 ) * 0.01 * π ÷ 180		(D-56)

min = (− pitch_range ÷ 2 ) * 0.01 * π ÷ 180		(D-57)

max = (+ pitch_range ÷ 2 ) * 0.01 * π ÷ 180		(D-58)

 = ( i′ + 0.5 ) * ( max − min ) ÷ croppedWidthY  + min	(D-59)

 = ( j′ + 0.5 ) * ( min − max ) ÷ croppedHeightY − min	(D-60)

-

-

where

i′ = i − SubWidthC * conf_win_left_offset		(D-63)

j′ = j − SubHeightC * conf_win_top_offset		(D-64)

Assume a right-handed coordinate system where -X is forward, -Z is right and +Y is up. 		-

The corresponding (X, Y, Z) coordinates for the luma sample locations are given by

X = cos()cos()		(D-63)

Y = sin()			(D-64)

Z = -cos()sin()		(D-65)

After the yaw, pitch, and roll rotation, the sample point (X, Y, Z) is rotated to (X′, Y′, Z′) with

(D-66)

The rotation matrix R is calculated as

R = Ry(yaw_center)*Rz(-pitch_center)*Rx(roll_center),				    		     (D-67)

where

𝑅𝑦(𝑦𝑎𝑤)=,

𝑅𝑧(-𝑝𝑖𝑡𝑐ℎ)=,

𝑅𝑥(𝑟𝑜𝑙𝑙)=.

The longitude and latitude (ϕ′, θ′) can be evaluated with (X′, Y′, Z′) coordinates with Equation (X-6)(X-7).

ϕ′ = tan-1(-Z′/X′) 											(D-68)

θ′ = sin-1(Y′/(X′2+Y′2+Z′2)1/2) 									(D-69)

When chroma_format_idc is not equal to 0 (monochrome), for i equal to conf_win_left_offset to conf_win_left_offset + croppedWidthC − 1 and j equal to conf_win_top_offset to conf_win_top_offset + croppedHeightC − 1, the corresponding angular coordinates (, ) for the chroma sample locations, in radians, are given by

 = ( i′ + CenterLeftOffsetC ) * ( max − min )  ÷ croppedWidthC + min	(D-70)

 = ( j′ + CenterTopOffsetC ) * ( min − max ) ÷ croppedHeightC − min	(D-71)

-

-

where

i′ = i − conf_win_left_offset		(D-72)

j′ = j − conf_win_top_offset		(D-73)

and CenterLeftOffsetC and CenterTopOffsetC are given by Table D.22 as a function of chroma_format_idc and ChromaLocType, which is set as follows:

- If j % 2 is equal to 0, ChromaLocType is equal to chroma_sample_loc_type_top_field

- Otherwise, ChromaLocType is equal to chroma_sample_loc_type_bottom_field

- Similarly to luma, the corresponding (X, Y, Z) coordinates for the chroma sample locations can be calculated inD.63~65. After the yaw, pitch, and roll rotation, the (X′, Y′, Z′) coordinates and the angular coordinates (ϕ′, θ′) are represented by equation X66~69.

Table D.22 – Derivation of CenterLeftOffsetC and CenterTopOffsetC

| chroma_format_idc | ChromaLocType | CenterLeftOffsetC | CenterTopOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0.125 | 0.25 |
| 1 (4:2:0) | 1 | 0.25 | 0.25 |
| 1 (4:2:0) | 2 | 0.125 | 0.125 |
| 1 (4:2:0) | 3 | 0.25 | 0.125 |
| 1 (4:2:0) | 4 | 0.125 | 0.375 |
| 1 (4:2:0) | 5 | 0.25 | 0.375 |
| 2 (4:2:2) | – | 0.125 | 0.5 |
| 3 (4:4:4) | – | 0.5 | 0.5 |

Figure D.14	Equirectangular mapping between cropped output picture luma samples and spherical coordinates

# Patent rights declaration(s)

MovieLabs and Intel do not have any current or pending patent rights relating to the technology described in this contribution.