| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0035 |
| --- | --- |

| Title: | Omnidirectional projection indication SEI message geometry_type and projection_type changes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

Several changes are proposed to add functionalities the omnidirectional projection indication SEI message in JCTVC-Z1005. It is proposed to add code values to the geometry_type and projection_type syntax elements, and change their semantics to clarify the distinction between a projection format used for coding a video sequence from the geometry used for rendering. This distinction is particularly relevant to distinguish cube map projection format used for encoding from cubic rendering. The “viewport mode” functionality proposed in JCTVC-Z0034 can also be provided with the proposed changes, without requiring any additional syntax.

# Background

The omnidirectional_projection_indication SEI message in JCTVC-Z1005 defines the equirectangular projection format for 360° video. The syntax of the SEI message from JCTVC-Z1005 is copied below. No changes to the syntax are proposed in this contribution.

| omnidirectional_projection_indication( payloadSize ) { | Descriptor |
| --- | --- |
| omnidirectional_projection_indication_cancel_flag | u(1) |
| if( !omnidirectional_projection_indication_cancel_flag ) { |  |
| omnidirectional_projection_information_persistence_flag | u(1) |
| geometry_type | u(5) |
| sub_geometry_flag | u(1) |
| projection_type | u(8) |
| if( geometry_type = = 1 && sub_geometry_flag = = 1 && projection_type = = 1 ) { |  |
| yaw_center | i(16) |
| pitch_center | i(16) |
| roll_center | i(16) |
| yaw_range | u(16) |
| pitch_range | u(16) |
| } |  |
| } |  |
| } |  |

# Proposal

## Additional code values for geometry_type and projection_type

Additional code values are proposed for the geometry_type and projection_type syntax elements.  For geometry_type, in addition to the current value indicating a spherical shape, it is proposed to add a code value for a cubical shape. For projection_type, in addition to the current value indicating equirectangular projection, it is proposed to add values for cube map projection and rectilinear projection.

The cube map projection format has been proposed for tile-based decoding approaches, using the temporal motion compensated tile sets SEI message, and benefits from more consistent pixel region sizes corresponding to viewports across all positions in a sphere, as compared to equirectangular projection, where regions near the poles are much larger in the coded picture. A particular cube map layout will need to be selected and defined.

Cubic rendering is commonly supported by modern GPUs.

Rectilinear projection format is used for viewports, as described in section 2.3.

## Distinction between projection format used for coding and rendering geometry

Changes to the semantics for geometry_type and projection_type are proposed to clarify the distinction between a projection format used for coding a video sequence from the geometry used for rendering. For example, a cube map projection format can be used for encoding the sequences, e.g. for coding efficiency benefits for some sequences, without requiring cubic rendering. The 360Lib software used in JVET assumes spherical rendering when PSNR-based calculations are made, even when cube map projection format is used for encoding. Also, when rectilinear viewports are generated from any of the projection formats used for encoding, a mapping back to a sphere is first performed, before creating the viewport using rectilinear projection. Were the intermediate mapping to a sphere not performed, it is expected that the objective and subjective video quality would be reduced, because of the warping at the edges of the cube faces.

Figure 1(a) illustrates a cube map, with 6 faces, labeled A through F. Figure 1b shows a possible layout of the cube map, where the 6 faces have been arranged into a 3x2 grid, with some of the faces rotated.

Figure 1(a) Cube map 4x3 layout with 6 cube faces, labeled A through F.

Figure 1(b) Compact cube map 3x2 layout, where faces D, E, F have been rotated.

Figure 2(a) shows an example equirectangular format image from an omnidirectional video, and Figure 2(b) an example of the same image mapped into a cube map, using the 3x2 layout of Figure 2(b).

Figure 3 shows an example rectilinear viewport generated from the equirectangular image, by first mapping the projection format to a sphere, and then forming a rectilinear projection at the selected viewport position.

Figure 2(a) Example equirectangular projection image

Figure 2(b) Example cube map projection  image

Figure 3. Example viewport

The viewer selects which region to view in the viewport, typically via movement of a head mounted display. When a cube map projection is used, typically a selected region will include portions of 3 different cube faces. For example, for the cube map illustrated in figure 2(a), a viewport may contain portions of faces A, E and F.

Omnidirectional video in a cube map format can be rendered as a cube, without an intermediate mapping to a sphere, as is done in the 360Lib software. This reduces computational complexity, but can cause artifacts when viewing a region containing a cube vertex and/or edges. An object that is split between two cube faces will show a discontinuity at the edge between the faces. For example, Figure 4(a) illustrates a cube, and Figure 4(b) illustrates a viewport which contains portions of 3 faces, with a straight narrow object split between two of the faces. A bend in the line occurs at the edge, with the angle of the line changing, which does not accurately represent a straight object, which can reduce the subjective view quality for the viewer.

| Figure 4(a) Cube illustration | Figure 4(b) Viewport from cube |
| --- | --- |

Content creators can reduce the subjective impact of this effect of cube map rendering by either rotating the content to arrange the cube face positions to avoid objects that are split across cube faces, or by pre-compensating for the effect, i.e. pre-warping the content in faces to avoid visible seams at the cube face edges.

Without this proposal, an omnidirectional video decoder/player will not be able to determine if the rectilinear viewport should be mapped via an intermediate sphere or not. With this proposal, the content creator can indicate to the decoder/player the recommended rendering type, separate from the projection type, which may be selected based on coding efficiency or for other reasons.

## Rectilinear viewport support

JCTVC-Z0034 proposed SEI message support for a “viewport mode” to indicate that a rectilinear viewport had been extracted from a spherical video, and the position and size of that rectilinear viewport. Figure 5 shows a system diagram with a Media Aware Network Element (MANE) that could be used to decode a spherical bitstream, extract a rectilinear viewport from it, and re-encode the rectilinear viewport.

The MANE decodes a high resolution video which represents the entire 360º x 180º spherical video, in any projection format (ERP, CMP, etc.) Based upon instantaneous HMD position data received from the client, and possibly the size of the field of view of the client display, the MANE then extracts a region of the spherical video and maps it to a rectangular region in a rectilinear projection format. The extracted region contains the viewport that would be viewed on the HMD for that position, possibly with a buffer region around it. That extracted region is re-encoded, and the omnidirectional projection information SEI message is used to indicate the position and size of the extracted region.

At the HMD, it is necessary to have information about the position and size of the extracted region in order to properly display the appropriate viewport, because the HMD position may change continuously. If the extracted region contains a buffer around the received HMD position data, allowing for some small movement during the period of latency, the displayed viewport may be cropped from the decoded picture.

The syntax of the omnidirectional projection indication SEI message included in JCTVC-Z1005 can be used to provide this functionality, with the addition of a rectilinear code value for the projection_type syntax element. The SEI message in Z1005 already provides syntax elements to indicate a center (yaw, pitch, roll) position and a (yaw, pitch) range. Figure 6 from JCTVC-Z1005 shows a region with yaw and pitch effects, which represents a region that could be coded in a rectilinear projection format. It is straightforward to use these syntax elements to describe the size and position of a rectilinear viewport that had previously been extracted from spherical video.

When rectilinear projection is used, the yaw_range and pitch_range must represent less than 180°, as rectilinear projection cannot capture more than a hemisphere.

Figure 5. MANE-assisted omnidirectional video distribution system (from JCTVC-Z0034)

Figure 6. region with yaw and pitch offsets (from JCTVC-Z1005)

## Summary of proposed types

| geometry_type | sub_geometry_flag | projection_type | description |
| --- | --- | --- | --- |
| 0 (sphere) | 0 | 0 (ERP) | ERP content, rendered on a sphere, filling the entire sphere |
| 0 (sphere) | 1 | 0 (ERP) | ERP content, rendered on a sphere, filling a portion of the sphere |
| 0 (sphere) | 1 | 2 (rectilinear) | rectilinear region which had been extracted from a sphere (“viewport mode” from JCTVC-Z0034), rendered on a sphere, filling a portion of the sphere |
| 1 (cube) | 0 | 1 (cube map) | cube map projection content, should be rendered on a cube |
| 0 (sphere) | 0 | 1 (cube map) | cube map projection content, should be rendered on a sphere |

# Proposed semantics

Changes relative to the Omnidirectional projection indication SEI message in JCTCV-Z1005 are shown when change marks are shown. Some of the equations are not yet provided, because they depend upon the exact cube map layout to be used.

- D.3.41	Omnidirectional projection indication SEI message semantics

…

geometry type indicates the mathematical convention where points within a space can be uniquely identified by a location in one or more dimensions. When geometry_type is equal to 0, the projection indicator is mapped to spherical coordinates, where  is the azimuth (longitude) and  is the elevation (latitude) as depicted in Figure D.13. When geometry_type is equal to 1, the projection indicator is mapped to cubic coordinates in the Cartesian (X, Y, Z) space.

Table D.X – Definition of geometry_type

| Value | Interpretation |
| --- | --- |
| 0 | Spherical coordinates (, ) |
| 1 | Cartesian coordinates (X, Y, Z) |

sub_geometry_flag indicates the presence of sub geometry elements that further describe or limit the geometry of this SEI message.

projection_type indicates the particular mapping of the rectangular decoder picture output samples onto the coordinate system specified by geometry_type., as specified in Table D.Y.

Table D.Y – Definition of projection_type

| Value | Interpretation |
| --- | --- |
| 0 | Equirectangular projection |
| 1 | Cube map projection |
| 2 | Rectilinear projection |

When projection_type equal to 3, sub_geometry_flag shall be equal to 1.

yaw_center specifies the value of yaw that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of yaw_center shall be in the range of -18000 and 17999. When not present, the value of yaw_center should be inferred to be equal to 0.

pitch_center specifies the value of pitch that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of pitch_center shall be in the range of -9000 and 9000. When not present, the value of pitch_center should be inferred to be equal to 0.

roll_center specifies the value of roll that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of roll_center shall be in the range of -18000 and 18000. When not present, the value of roll_center should be inferred to be equal to 0.

yaw_range specifies the range of values of yaw that are present in the projection image, in increments of 0.01 degrees. The value of yaw_range shall be in the range of 1 and 36000. When not present, the value of yaw_range should be inferred to be equal to 36000.

When projection_type equal to 2, the value of yaw_range shall be in the range of 1 and 18000.

pitch_range specifies the range of values of pitch that are present in the projection image, in increments of 0.01 degrees. The values of pitch_range shall be in the range of 1 and 18000. When not present, the value of pitch range should be inferred to be equal to 18000.

When projection_type equal to 2, the value of pitch_range shall be in the range of 1 and 18000.

When projection_type is equal to 0, sub_geometry_flag is equal to 1, and geometry_type is equal to 1, cropped output picture samples at locations (i, j) are indicated as corresponding to angular coordinates on the sphere. For i equal to SubWidthC * conf_win_left_offset to SubWidthC * conf_win_left_offset + croppedWidthC − 1 and j equal to SubHeightC * conf_win_top_offset to SubHeightC * conf_win_top_offset − 1, the corresponding angular coordinates (, ) for the luma sample locations, in radians, are given by the following equirectangular mapping equations:

min = ( yaw_center − yaw_range ÷ 2 ) * 0.01 * π ÷ 180		(D-1)

max = ( yaw_center + yaw_range ÷ 2 ) * 0.01 * π ÷ 180	(D-2)

min = ( pitch_center − pitch_range ÷ 2 ) * 0.01 * π ÷ 180	(D-3)

max = ( pitch_center + pitch_range ÷ 2 ) * 0.01 * π ÷ 180	(D-4)

 = ( i′ + 0.5 ) * ( max − min ) ÷ croppedWidthY  + min	(D-5)

 = ( j′ + 0.5 ) * ( min − max ) ÷ croppedHeightY − min	(D-6)

 = Cos(  ) *  + Sin(  ) * 		(D-7)

 = Sin(  ) *  − Cos(  ) * 		(D-8)

where

i′ = i − SubWidthC * conf_win_left_offset		(D-9)

j′ = j − SubHeightC * conf_win_top_offset		(D-10)

 = roll_center * 0.01 *  π ÷ 180		(D-11)

When chroma_format_idc is not equal to 0 (monochrome), for i equal to conf_win_left_offset to conf_win_left_offset + croppedWidthC − 1 and j equal to conf_win_top_offset to conf_win_top_offset + croppedHeightC − 1, the corresponding angular coordinates (, ) for the chroma sample locations, in radians, are given by

′ = ( i′ + CenterLeftOffsetC ) * ( max − min )  ÷ croppedWidthC + min	(D-12)

′ = ( j′ + CenterTopOffsetC ) * ( min − max ) ÷ croppedHeightC − min	(D-13)

′ = Cos(  ) * ′ + Sin(  ) * ′		(D-14)

′ = Sin(  ) * ′ − Cos(  ) * ′		(D-15)

where

i′ = i − conf_win_left_offset		(D-16)

j′ = j − conf_win_top_offset		(D-17)

and CenterLeftOffsetC and CenterTopOffsetC are given by Table D.22 as a function of chroma_format_idc and ChromaLocType, which is set as follows:

- If j % 2 is equal to 0, ChromaLocType is equal to chroma_sample_loc_type_top_field

- Otherwise, ChromaLocType is equal to chroma_sample_loc_type_bottom_field

Table D.22 – Derivation of CenterLeftOffsetC and CenterTopOffsetC

| chroma_format_idc | ChromaLocType | CenterLeftOffsetC | CenterTopOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0.125 | 0.25 |
| 1 (4:2:0) | 1 | 0.25 | 0.25 |
| 1 (4:2:0) | 2 | 0.125 | 0.125 |
| 1 (4:2:0) | 3 | 0.25 | 0.125 |
| 1 (4:2:0) | 4 | 0.125 | 0.375 |
| 1 (4:2:0) | 5 | 0.25 | 0.375 |
| 2 (4:2:2) | – | 0.125 | 0.5 |
| 3 (4:4:4) | – | 0.5 | 0.5 |

When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer and projection_type is equal to 1 and geometry_type is equal to 1, no frame packing arrangement SEI messages and segmented rectangular frame packing arrangement SEI messages shall be present in the CLVS.

Figure D.14	Equirectangular mapping between cropped output picture luma samples and spherical coordinates

When geometry_type is equal to 0 and projection_type is equal to 1, the angular coordinates  and  are defined … [Ed.: Use text provided in JCTVC-A0034]

When geometry_type is equal to 0 and projection_type is equal to 2, the angular coordinates  and  are defined … [Ed.: Use text provided in JCTVC-A0034 for the front face]

When geometry_type is equal to 1 and projection_type is equal to 1, the (X, Y, Z) coordinates are defined …[Ed.: Need to select a particular cube map projection layout to determine the equations.]

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).