| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0035r1 |
| --- | --- |

| Title: | Omnidirectional projection indication SEI message geometry_type and projection_type changes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

Several changes are proposed to add functionalities the omnidirectional projection indication SEI message in JCTVC-Z1005.

Based on discussion with experts during the Hobart meeting, one aspect of the original contribution is withdrawn. The revised contribution with that aspect removed is now contained in this document, using change marks from the original version of the contribution.

It is proposed to add code values to the projection_type syntax element for cube map projection and rectilinear projection. The “viewport mode” functionality proposed in JCTVC-Z0034 can also be provided with the proposed changes, without requiring any additional syntax.

# Background

The omnidirectional_projection_indication SEI message in JCTVC-Z1005 defines the equirectangular projection format for 360° video. The syntax of the SEI message from JCTVC-Z1005 is copied below. No changes to the syntax are proposed in this contribution.

| omnidirectional_projection_indication( payloadSize ) { | Descriptor |
| --- | --- |
| omnidirectional_projection_indication_cancel_flag | u(1) |
| if( !omnidirectional_projection_indication_cancel_flag ) { |  |
| omnidirectional_projection_information_persistence_flag | u(1) |
| geometry_type | u(5) |
| sub_geometry_flag | u(1) |
| projection_type | u(8) |
| if( geometry_type = = 1 && sub_geometry_flag = = 1 && projection_type = = 1 ) { |  |
| yaw_center | i(16) |
| pitch_center | i(16) |
| roll_center | i(16) |
| yaw_range | u(16) |
| pitch_range | u(16) |
| } |  |
| } |  |
| } |  |

# Proposal

## Additional code values for projection_type

Additional code values are proposed for the projection_type syntax element.  In addition to the current value indicating equirectangular projection, it is proposed to add values for cube map projection and rectilinear projection.

The cube map projection format has been proposed for tile-based decoding approaches, using the temporal motion compensated tile sets SEI message, and benefits from more consistent pixel region sizes corresponding to viewports across all positions in a sphere, as compared to equirectangular projection, where regions near the poles are much larger in the coded picture. A particular cube map layout will need to be selected and defined.

Cubic rendering is commonly supported by modern GPUs.

Rectilinear projection format is used for viewports, as described in section 2.3.

## Cube map background

Figure 1(a) illustrates a cube map, with 6 faces, labeled A through F. Figure 1b shows a possible layout of the cube map, where the 6 faces have been arranged into a 3x2 grid, with some of the faces rotated.

Figure 1(a) Cube map 4x3 layout with 6 cube faces, labeled A through F.

Figure 1(b) Compact cube map 3x2 layout, where faces D, E, F have been rotated.

Figure 2(a) shows an example equirectangular format image from an omnidirectional video, and Figure 2(b) an example of the same image mapped into a cube map, using the 3x2 layout of Figure 2(b).

Figure 3 shows an example rectilinear viewport generated from the equirectangular image, by first mapping the projection format to a sphere, and then forming a rectilinear projection at the selected viewport position.

Figure 2(a) Example equirectangular projection image

Figure 2(b) Example cube map projection  image

Figure 3. Example viewport

The viewer selects which region to view in the viewport, typically via movement of a head mounted display. When a cube map projection is used, typically a selected region will include portions of 3 different cube faces. For example, for the cube map illustrated in figure 2(a), a viewport may contain portions of faces A, E and F.

  - Rectilinear viewport support

JCTVC-Z0034 proposed SEI message support for a “viewport mode” to indicate that a rectilinear viewport had been extracted from a spherical video, and the position and size of that rectilinear viewport. Figure 5 shows a system diagram with a Media Aware Network Element (MANE) that could be used to decode a spherical bitstream, extract a rectilinear viewport from it, and re-encode the rectilinear viewport.

The MANE decodes a high resolution video which represents the entire 360º x 180º spherical video, in any projection format (ERP, CMP, etc.) Based upon instantaneous HMD position data received from the client, and possibly the size of the field of view of the client display, the MANE then extracts a region of the spherical video and maps it to a rectangular region in a rectilinear projection format. The extracted region contains the viewport that would be viewed on the HMD for that position, possibly with a buffer region around it. That extracted region is re-encoded, and the omnidirectional projection information SEI message is used to indicate the position and size of the extracted region.

At the HMD, it is necessary to have information about the position and size of the extracted region in order to properly display the appropriate viewport, because the HMD position may change continuously. If the extracted region contains a buffer around the received HMD position data, allowing for some small movement during the period of latency, the displayed viewport may be cropped from the decoded picture.

The syntax of the omnidirectional projection indication SEI message included in JCTVC-Z1005 can be used to provide this functionality, with the addition of a rectilinear code value for the projection_type syntax element. The SEI message in Z1005 already provides syntax elements to indicate a center (yaw, pitch, roll) position and a (yaw, pitch) range. Figure 6 from JCTVC-Z1005 shows a region with yaw and pitch effects, which represents a region that could be coded in a rectilinear projection format. It is straightforward to use these syntax elements to describe the size and position of a rectilinear viewport that had previously been extracted from spherical video.

When rectilinear projection is used, the yaw_range and pitch_range must represent less than 180°, as rectilinear projection cannot capture more than a hemisphere.

Figure 5. MANE-assisted omnidirectional video distribution system (from JCTVC-Z0034)

Figure 6. region with yaw and pitch offsets (from JCTVC-Z1005)

## Summary of proposed types

| geometry_type | sub_geometry_flag | projection_type | description |
| --- | --- | --- | --- |
| 0 (sphere) | 0 | 0 (ERP) | ERP content, filling the entire sphere |
| 0 (sphere) | 1 | 0 (ERP) | ERP content, filling a portion of the sphere |
| 0 (sphere) | 1 | 2 (rectilinear) | rectilinear region which had been extracted from a sphere (“viewport mode” from JCTVC-Z0034), filling a portion of the sphere |
| 0 (sphere) | 0 | 1 (cube map) | cube map projection content, filling the entire sphere |

# Proposed semantics

Changes relative to the Omnidirectional projection indication SEI message in JCTCV-Z1005 are shown when change marks are shown. Some of the equations are not yet provided, because they depend upon the exact cube map layout to be used.

- D.3.41	Omnidirectional projection indication SEI message semantics

…

geometry type indicates the mathematical convention where points within a space can be uniquely identified by a location in one or more dimensions. When geometry_type is equal to 0, the projection indicator is given in spherical coordinates, where  is the azimuth (longitude) and  is the elevation (latitude) as depicted in Figure D.13.

sub_geometry_flag indicates the presence of sub geometry elements that further describe or limit the geometry of this SEI message.

projection_type indicates the particular mapping of the rectangular decoder picture output samples onto the coordinate system specified by geometry_type.

Table D.Y – Definition of projection_type

| Value | Interpretation |
| --- | --- |
| 0 | Equirectangular projection |
| 1 | Cube map projection |
| 2 | Rectilinear projection |

When projection_type equal to 3, sub_geometry_flag shall be equal to 1.

yaw_center specifies the value of yaw that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of yaw_center shall be in the range of -18000 and 17999. When not present, the value of yaw_center should be inferred to be equal to 0.

pitch_center specifies the value of pitch that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of pitch_center shall be in the range of -9000 and 9000. When not present, the value of pitch_center should be inferred to be equal to 0.

roll_center specifies the value of roll that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of roll_center shall be in the range of -18000 and 18000. When not present, the value of roll_center should be inferred to be equal to 0.

yaw_range specifies the range of values of yaw that are present in the projection image, in increments of 0.01 degrees. The value of yaw_range shall be in the range of 1 and 36000. When not present, the value of yaw_range should be inferred to be equal to 36000.

When projection_type equal to 2, the value of yaw_range shall be in the range of 1 and 18000.

pitch_range specifies the range of values of pitch that are present in the projection image, in increments of 0.01 degrees. The values of pitch_range shall be in the range of 1 and 18000. When not present, the value of pitch range should be inferred to be equal to 18000.

When projection_type equal to 2, the value of pitch_range shall be in the range of 1 and 18000.

When projection_type is equal to 0, sub_geometry_flag is equal to 1, and geometry_type is equal to 1, cropped output picture samples at locations (i, j) are indicated as corresponding to angular coordinates on the sphere. For i equal to SubWidthC * conf_win_left_offset to SubWidthC * conf_win_left_offset + croppedWidthC − 1 and j equal to SubHeightC * conf_win_top_offset to SubHeightC * conf_win_top_offset − 1, the corresponding angular coordinates (, ) for the luma sample locations, in radians, are given by the following equirectangular mapping equations:

min = ( yaw_center − yaw_range ÷ 2 ) * 0.01 * π ÷ 180		(D-1)

max = ( yaw_center + yaw_range ÷ 2 ) * 0.01 * π ÷ 180	(D-2)

min = ( pitch_center − pitch_range ÷ 2 ) * 0.01 * π ÷ 180	(D-3)

max = ( pitch_center + pitch_range ÷ 2 ) * 0.01 * π ÷ 180	(D-4)

 = ( i′ + 0.5 ) * ( max − min ) ÷ croppedWidthY  + min	(D-5)

 = ( j′ + 0.5 ) * ( min − max ) ÷ croppedHeightY − min	(D-6)

 = Cos(  ) *  + Sin(  ) * 		(D-7)

 = Sin(  ) *  − Cos(  ) * 		(D-8)

where

i′ = i − SubWidthC * conf_win_left_offset		(D-9)

j′ = j − SubHeightC * conf_win_top_offset		(D-10)

 = roll_center * 0.01 *  π ÷ 180		(D-11)

When chroma_format_idc is not equal to 0 (monochrome), for i equal to conf_win_left_offset to conf_win_left_offset + croppedWidthC − 1 and j equal to conf_win_top_offset to conf_win_top_offset + croppedHeightC − 1, the corresponding angular coordinates (, ) for the chroma sample locations, in radians, are given by

′ = ( i′ + CenterLeftOffsetC ) * ( max − min )  ÷ croppedWidthC + min	(D-12)

′ = ( j′ + CenterTopOffsetC ) * ( min − max ) ÷ croppedHeightC − min	(D-13)

′ = Cos(  ) * ′ + Sin(  ) * ′		(D-14)

′ = Sin(  ) * ′ − Cos(  ) * ′		(D-15)

where

i′ = i − conf_win_left_offset		(D-16)

j′ = j − conf_win_top_offset		(D-17)

and CenterLeftOffsetC and CenterTopOffsetC are given by Table D.22 as a function of chroma_format_idc and ChromaLocType, which is set as follows:

- If j % 2 is equal to 0, ChromaLocType is equal to chroma_sample_loc_type_top_field

- Otherwise, ChromaLocType is equal to chroma_sample_loc_type_bottom_field

Table D.22 – Derivation of CenterLeftOffsetC and CenterTopOffsetC

| chroma_format_idc | ChromaLocType | CenterLeftOffsetC | CenterTopOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0.125 | 0.25 |
| 1 (4:2:0) | 1 | 0.25 | 0.25 |
| 1 (4:2:0) | 2 | 0.125 | 0.125 |
| 1 (4:2:0) | 3 | 0.25 | 0.125 |
| 1 (4:2:0) | 4 | 0.125 | 0.375 |
| 1 (4:2:0) | 5 | 0.25 | 0.375 |
| 2 (4:2:2) | – | 0.125 | 0.5 |
| 3 (4:4:4) | – | 0.5 | 0.5 |

When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer and projection_type is equal to 1 and geometry_type is equal to 1, no frame packing arrangement SEI messages and segmented rectangular frame packing arrangement SEI messages shall be present in the CLVS.

Figure D.14	Equirectangular mapping between cropped output picture luma samples and spherical coordinates

When geometry_type is equal to 0 and projection_type is equal to 1, the angular coordinates  and  are defined … [Ed.: Use text provided in JCTVC-A0034]

When geometry_type is equal to 0 and projection_type is equal to 2, the angular coordinates  and  are defined … [Ed.: Use text provided in JCTVC-A0034 for the front face]

[Ed.: Need to select a particular cube map projection layout to determine the equations.]

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).