| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0036r1 |
| --- | --- |

| Title: | Omnidirectional recommended viewport SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

An omnidirectional recommended viewport SEI message is proposed for HEVC and AVC to allow the encoder to indicate a “director’s view,” which is a recommended rectangular viewport in a rectilinear projection from the coded spherical video, to align with the functionality provided in the draft OMAF specification in N16636. This functionality was previously proposed in JCTVC-Z0034, as the “ROI mode”.

The r1 version of this document adds capability to send multiple recommended regions with ids, and to add a roll center syntax element, based upon discussion during the initial review of the contribution. Some editorial improvements to the semantics are also provided.

# Proposal

360 degree video can be coded using existing HEVC and AVC profiles, using a projection format to map spherical video data to a rectangular format used for coding. Only a portion of the 360 x 180 degree coded video is typically viewed at a given time, with the particular position viewed determined at the client, and then projecting the coded data from its coding projection (such as ERP) to a sphere, and then from the sphere to a rectilinear viewport. If a head mounted display (HMD) is used, the position of the HMD is used to determine which region is used for the viewport. If a 2D display is used, such as on a computer, an input interface is provided for the user to indicate the desired viewport.

When viewing omnidirectional video, the viewer typically views only a small portion of the full 360º x 180º video at a given time. A viewer may miss the “most interesting” area of the video. The content creator/director may provide information indicating the suggested viewing angle and size, which can be sent as metadata in an SEI message. The client viewer may choose to follow the recommendation or to override it.

The OMAF draft specification in N16636 includes a recommended viewport, with a (yaw, pitch) center and (yaw, pitch) range provided. This corresponds to a rectangular region on a rectilinear viewport. The region is bounded by two horizontal “great circles” and two vertical “great circles,” where a great circle is a diameter of the sphere. Figure 1 was provided by Emmanuel Thomas on the email reflector of the OMAF AHG on ROI, to illustrate a rectangular region of a rectilinear projection as mapped to a sphere.

In this contribution, it is proposed to include this information in HEVC and AVC SEI messages, to align with the information provided in the OMAF draft.

Figure 1. Rectangular region of a rectilinear projection mapped to a sphere

[Source: Emmanuel Thomas]

## Proposed Syntax

| omnidirectional_viewport( payloadSize ) { | Descriptor |
| --- | --- |
| omnidirectional_ viewport _id | u(3) |
| omnidirectional_ viewport _cancel_flag | u(1) |
| if( !omnidirectional_viewport_cancel_flag ) { |  |
| omnidirectional_viewport_persistence_flag | u(1) |
| omnidirectional_ viewport _cnt_minus1 | u(3) |
| for( i = 0; i <= omnidirectional_ cnt_minus1; i++ ) { |  |
| omnidirectional_viewport_yaw_center[ i ] | i(16) |
| omnidirectional_viewport_pitch_center[ i ] | i(16) |
| omnidirectional_viewport_roll_center[ i ] | i(16) |
| omnidirectional_viewport_yaw_range[ i ] | i(16) |
| omnidirectional_viewport_pitch_range[ i ] | i(16) |
| } |  |
| } |  |
| } |  |

## Proposed Semantics

When the associated picture has PicOutputFlag equal to 1, the omnidirectional viewport SEI message informs the decoder of a mode which indicates one or more viewport regions of interest of a spherical projection, bounded by two great circles and two small circles on the sphere.

The omnidirectional_viewport SEI message shall follow the omnidirectional_projection_indication SEI message in the CVS. [Ed.(JB): Improve this sentence to clarify scope and ordering.]

omnidirectional_ viewport _id contains an identifying number that may be used to identify which viewport region of interest is recommended to be displayed.

omnidirectional_viewport_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional viewport SEI message in output order. omnidirectional_viewport_cancel_flag equal to 0 indicates that omnidirectional viewport information follows.

omnidirectional_viewport_persistence_flag specifies the persistence of the omnidirectional viewport SEI message for the current layer.

omnidirectional_viewport_persistence_flag equal to 0 specifies that the omnidirectional viewport SEI message applies to the current decoded picture only.

Let picA be the current picture. omnidirectional_viewport_persistence_flag equal to 1 specifies that the omnidirectional viewport SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a spherical viewport SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

omnidirectional_viewport_yaw_center[ i ], specifies the center of the i-th recommended viewport region in degrees, around the up vector, in increments of 0.01 degrees. The values of omnidirectional_viewport_yaw_center[ i ] shall be in the range of -18000 and 17999.

omnidirectional_viewport_pitch_center[ i ], specifies the center of the i-th recommended viewport region in degrees, around the yaw_center, i.e., the right vector post yaw rotation, in increments of 0.01 degrees. The values of omnidirectional_viewport_pitch_center[ i ] shall be in the range of -9000 and 9000.

omnidirectional_viewport_roll_center[ i ], specifies the center of the i-th recommended viewport region in degrees, around the pitch_center, i.e., the forward vector post yaw and pitch rotation in increments of 0.01 degrees. The values of omnidirectional_viewport_roll_center[ i ] shall be in the range of -18000 and 18000.

omnidirectional_viewport_yaw_range[ i ], specifies the size of the i-th recommended viewport region of the projection mapped decoded picture centered around (0, 0), in range of values of yaw in increments of 0.01 degrees. The values of omnidirectional_viewport_yaw_range[ i ] shall be in the range of 1 and 36000..

omnidirectional_viewport_yaw_range[ i ] shall be less than or equal to yaw_range.

omnidirectional_viewport_pitch_range[ i ], specifies the size of the i-th recommended viewport region of the projection mapped decoded picture centered around (0, 0), in range of values of pitch in increments of 0.01 degrees. The values of omnidirectional_viewport_pitch_range[ i ]  shall be in the range of 1 and 18000.

omnidirectional_viewport_pitch_range[ i ] shall be less than or equal to pitch_range.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).