| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0039 |
| --- | --- |

| Title: | Additional methods for Luma Adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A.M. Tourapis, Y. Su, D. Singer 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
| Source: | Apple Inc |  |  |

_____________________________

# Abstract

This contribution presents two additional methods for luma adjustment. The first method uses an interpolation method instead of the binary search technique to determine the luma adjustment solution that optimizes luminance. In the second method, the solution is derived by offsetting the conventionally computed luma by the distortion introduced to the luminance information. The performance of these two methods on a variety of sequences is presented.

# Introduction

It has been suggested in several MPEG contributions [1][2][3] that the Y’CbCr non-constant luminance 4:2:0 representation may suffer from chroma leakage problems. This problem is exacerbated in HDR content by the use of the PQ transfer function.

A solution to this problem was presented in [4][9], where the value of the luma component was iteratively adjusted so as to minimize the distortion introduced into the luminance component. This method was named as the Luma adjustment method and considerably reduced the artifacts that were introduced using conventional downsampling methods. Slight variations of this method that did not require any iterations, and were therefore considerably simpler to implement, were also presented [5][6][7][8].

This contribution presents additional methods for luma adjustment that do not involve any iterations and may be suitable for some implementations.

# Simplified Luma Adjustment Methods

The original luma adjustment method presented in [4] employed a binary search technique to determine the best luma value that would minimize luminance distortion. This method was criticized due to the iterative steps needed. Although [9] introduced methods to reduce the number of iterations, i.e. by initially determing the bounds of the binary search given the pixel characteristics, and thus reduce the complexity of the method, the iterative nature of the search was still deemed a problem especially for hardware implementations. Several alternative methods that approximate the iterative solution were therefore presented in [5][6][7][8], with each one using a different approximation such as using a Taylor Series approximation [5], or by using a variety of LUT methods [6][7][8].

A similar strategy is used also in our first method. As we have discussed earlier, in  [9] the number of iterations of the binary search was substantially reduced by first computing the luma boundaries for the search. In particular, it was computed that the solution for luma adjustment of a signal with a luminance value of , original non-linear color component values of R', G', and B', and reconstructed chroma values (after downsampling and upsampling), equal to  and , should be within the values , computed as:

where,

and

.

These two bounds were then used for the binary search method.

Given the proximity of these two boundary values, we can assume that although the transfer function might not be linear, it is piecewise linear within these two boundary values. Given that assumption, we can then simply interpolate to determine the adjusted luma value. In particular, we can compute the adjusted luma value as:

where Ylowbound and Yhigh_bound now correspond to the values of luminance for the low bound and high bound luma values respectively.

In an alternative method, instead of computing the boundary values, we first compute a first approximation to the luma value using the conventional method, i.e. by directly computing luma using the form:

Then, this value is adjusted to compute the adjusted luma value using the simple formula of:

where Ydirect is the corresponding luminance value given ,  , and .  The idea behind this method is that although luma might not have a linear relationship with luminance, the error between two luma values might be directly related to the error of their corresponding luminance values. Therefore, if we compute the introduced luminance error, we can compensate for it by adding it back to the computed luma.

The objective performance results using these two methods compared to the original/conventional method (Table 1) and the binary search method (Table 2) are presented in  and Table 4, respectively.

It can be seen that both methods, and especially the interpolative technique, result in objective performance very similar to that of the binary search method. The interpolative method has an average loss of 0.046dB, with a worse loss of 0.172dB for the Tibul sequence in terms of tPSNR(Y), while the offset method has an average loss of 0.077dB, with a worse loss of 0.385dB in tPSNR(Y) again for the Tibul sequence. These results appear to be considerably better than the results presented for the techniques in [5] (Table 5). These seem to suggest that these two methods may be good alternatives to the binary search method, for performing luma adjustment.

Table 1. Basic conversion results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 57.069 | 64.509 | 53.071 | 56.155 | 58.733 | 64.376 | 53.392 | 56.778 | 54.855 | 48.421 | 55.050 |
| Tibul | 53.761 | 62.882 | 52.225 | 54.453 | 55.158 | 60.007 | 52.677 | 55.007 | 53.982 | 45.614 | 49.462 |
| Market3 | 49.253 | 58.645 | 45.978 | 48.918 | 53.475 | 62.051 | 46.238 | 50.163 | 49.751 | 36.767 | 46.848 |
| AutoWelding | 45.363 | 53.435 | 40.763 | 44.064 | 48.196 | 58.620 | 40.967 | 44.909 | 42.573 | 39.686 | 52.333 |
| BikeSparklers | 43.389 | 50.782 | 35.385 | 39.346 | 45.922 | 58.474 | 35.760 | 40.092 | 38.285 | 34.069 | 48.337 |
| ShowGirl | 49.756 | 57.221 | 44.669 | 48.080 | 52.040 | 62.225 | 44.834 | 48.767 | 47.653 | 39.357 | 50.957 |
| MagicHour | 52.004 | 57.396 | 39.785 | 44.228 | 51.870 | 63.761 | 39.951 | 44.434 | 43.119 | 37.419 | 51.504 |
| WarmNight | 50.437 | 57.737 | 39.985 | 44.314 | 52.762 | 62.637 | 40.293 | 44.800 | 43.672 | 38.101 | 51.494 |
| BalloonFestival | 50.1-- | 57.822 | 45.015 | 48.443 | 52.784 | 62.062 | 45.614 | 49.541 | 51.911 | 40.623 | 46.980 |
| Balloon | 54.751 | 63.213 | 51.018 | 54.075 | 58.598 | 67.757 | 51.337 | 55.278 | 54.619 | 37.874 | 47.700 |
| Average | 50.588 | 58.364 | 44.789 | 48.208 | 52.954 | 62.197 | 45.106 | 48.977 | 48.042 | 39.793 | 50.067 |

Table 2. Luma Adjustment (mode 8 + LUT)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 60.86 | 59.463 | 51.976 | 55.574 | 63.184 | 70.214 | 52.230 | 56.601 | 55.014 | 49.155 | 58.381 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.258 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.800 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 |
| AutoWelding | 46.686 | 50.456 | 40.518 | 43.994 | 50.226 | 69.821 | 40.717 | 45.005 | 42.770 | 39.807 | 55.678 |
| BikeSparklers | 43.973 | 49.365 | 35.232 | 39.255 | 46.915 | 69.757 | 35.609 | 40.054 | 38.301 | 34.084 | 51.927 |
| ShowGirl | 50.359 | 55.131 | 44.746 | 48.141 | 53.308 | 66.839 | 44.913 | 49.045 | 48.003 | 39.411 | 53.010 |
| MagicHour | 52.875 | 55.355 | 39.842 | 44.285 | 53.222 | 70.641 | 40.017 | 44.580 | 43.289 | 37.446 | 53.887 |
| WarmNight | 51.900 | 55.315 | 39.756 | 44.155 | 54.552 | 70.767 | 40.056 | 44.671 | 43.585 | 38.145 | 54.147 |
| BalloonFestival | 50.649 | 56.034 | 44.503 | 48.091 | 53.726 | 69.592 | 45.062 | 49.266 | 51.725 | 40.679 | 48.243 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 |
| Average | 52.2001 | 55.4109 | 44.2598 | 47.9289 | 55.1982 | 69.711 | 44.5482 | 48.9016 | 48.087 | 39.9951 | 52.4173 |

Table 3. Interpolation Based Luma Adjustment

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 60.870 | 59.447 | 51.972 | 55.570 | 63.195 | 70.207 | 52.226 | 56.599 | 55.013 | 49.156 | 58.380 |
| Tibul | 59.310 | 55.545 | 49.703 | 53.099 | 61.576 | 70.086 | 49.995 | 54.427 | 53.724 | 46.559 | 53.070 |
| Market3 | 50.034 | 55.627 | 45.704 | 48.800 | 55.600 | 69.611 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 |
| AutoWelding | 46.688 | 50.453 | 40.517 | 43.994 | 50.228 | 69.798 | 40.717 | 45.005 | 42.771 | 39.807 | 55.676 |
| BikeSparklers | 43.973 | 49.365 | 35.232 | 39.255 | 46.915 | 69.749 | 35.609 | 40.055 | 38.301 | 34.084 | 51.925 |
| ShowGirl | 50.369 | 55.147 | 44.735 | 48.137 | 53.283 | 66.758 | 44.901 | 49.030 | 47.989 | 39.410 | 53.007 |
| MagicHour | 52.875 | 55.361 | 39.841 | 44.284 | 53.218 | 70.595 | 40.016 | 44.579 | 43.289 | 37.446 | 53.886 |
| WarmNight | 51.885 | 55.333 | 39.756 | 44.155 | 54.535 | 70.650 | 40.056 | 44.671 | 43.585 | 38.145 | 54.146 |
| BalloonFestival | 50.649 | 56.035 | 44.503 | 48.091 | 53.726 | 69.586 | 45.063 | 49.266 | 51.725 | 40.679 | 48.243 |
| Balloon | 55.356 | 61.794 | 50.614 | 53.887 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 |
| Average | 52.2009 | 55.4107 | 44.2577 | 47.9272 | 55.1947 | 69.6649 | 44.5462 | 48.8993 | 48.0854 | 39.9953 | 52.4161 |

Table 4. Offset based Luma Adjustment

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 60.556 | 59.747 | 52.051 | 55.638 | 62.833 | 70.128 | 52.309 | 56.645 | 55.027 | 49.088 | 58.351 |
| Tibul | 58.801 | 55.944 | 49.843 | 53.231 | 60.974 | 69.873 | 50.142 | 54.518 | 53.793 | 46.497 | 53.025 |
| Market3 | 50.027 | 55.665 | 45.706 | 48.802 | 55.582 | 69.604 | 45.963 | 50.267 | 49.885 | 36.788 | 47.891 |
| AutoWelding | 46.647 | 50.544 | 40.527 | 44.001 | 50.166 | 69.717 | 40.726 | 45.007 | 42.768 | 39.802 | 55.654 |
| BikeSparklers | 43.969 | 49.380 | 35.233 | 39.255 | 46.908 | 69.741 | 35.610 | 40.055 | 38.300 | 34.084 | 51.923 |
| ShowGirl | 50.361 | 55.256 | 44.734 | 48.144 | 53.263 | 66.788 | 44.900 | 49.028 | 47.974 | 39.408 | 52.996 |
| MagicHour | 52.860 | 55.407 | 39.843 | 44.286 | 53.201 | 70.609 | 40.017 | 44.579 | 43.288 | 37.446 | 53.881 |
| WarmNight | 51.857 | 55.421 | 39.761 | 44.160 | 54.497 | 70.688 | 40.060 | 44.674 | 43.587 | 38.144 | 54.138 |
| BalloonFestival | 50.645 | 56.055 | 44.506 | 48.093 | 53.720 | 69.588 | 45.065 | 49.268 | 51.727 | 40.679 | 48.242 |
| Balloon | 55.355 | 61.798 | 50.615 | 53.887 | 59.669 | 69.609 | 50.919 | 55.095 | 54.571 | 37.879 | 47.936 |
| Average | 52.1078 | 55.5217 | 44.2819 | 47.9497 | 55.0813 | 69.6345 | 44.5711 | 48.9136 | 48.092 | 39.9815 | 52.4037 |

Table 5. Performance of Algorithm 2 in JCTVC-W0107

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 |
| FireEater | 60.76 | 59.531 | 51.998 | 55.591 | 63.070 | 70.119 | 52.251 | 56.611 | 55.016 | 49.149 | 58.303 |
| Tibul | 59.193 | 55.666 | 49.693 | 53.106 | 61.434 | 69.538 | 49.980 | 54.400 | 53.718 | 46.546 | 53.009 |
| Market3 | 50.028 | 55.395 | 45.689 | 48.772 | 55.679 | 68.693 | 45.946 | 50.257 | 49.886 | 36.788 | 47.817 |
| AutoWelding | 46.652 | 49.971 | 40.511 | 43.944 | 50.313 | 64.797 | 40.711 | 45.000 | 42.758 | 39.813 | 54.902 |
| BikeSparklers | 43.910 | 48.769 | 35.258 | 39.254 | 47.089 | 63.049 | 35.640 | 40.090 | 38.322 | 34.083 | 50.399 |
| ShowGirl | 50.330 | 54.772 | 44.758 | 48.114 | 53.360 | 65.605 | 44.926 | 49.053 | 48.011 | 39.411 | 52.718 |
| MagicHour | 52.809 | 55.136 | 39.850 | 44.284 | 53.275 | 69.044 | 40.026 | 44.590 | 43.299 | 37.447 | 53.654 |
| WarmNight | 51.764 | 54.987 | 39.790 | 44.170 | 54.638 | 68.037 | 40.093 | 44.707 | 43.598 | 38.144 | 53.977 |
| BalloonFestival | 50.645 | 55.757 | 44.485 | 48.061 | 53.778 | 68.767 | 45.043 | 49.253 | 51.717 | 40.679 | 48.136 |
| Balloon | 55.377 | 61.587 | 50.586 | 53.860 | 59.740 | 69.439 | 50.889 | 55.075 | 54.570 | 37.879 | 47.924 |
| Average | 52.147 | 55.157 | 44.262 | 47.916 | 55.238 | 67.709 | 44.551 | 48.904 | 48.090 | 39.994 | 52.084 |

# Conclusion

This contribution presented two alternative methods for luma adjustment. The first method computed the adjusted luma value through an interpolation method, while the second derives the corresponding adjusted value by computing and adjusting for the distortion introduced to the luminance component. The two methods result in 0.046dB and  0.077dB worse tPSNR(Y) performance respectively compared to the binary search luma adjusted method, while having considerably lower complexity.

# References

- E. François, P. Lopez, Y. Olivier, “About using a BT.2020 container for BT.709 content”, ISO/IEC (MPEG) document m35255, Sapporo, July 2014

- J. Stessen, R. Nijland, R. Brondijk, R. Goris, “Chromaticity Based Color Signals for Wide Color Gamut and High Dynamic Range”, ISO/IEC (MPEG) document m35065, Strasbourg, France, Oct. 2014.

- J. Ström, “Investigation of HDR color subsampling”, ISO/IEC (MPEG) document m35841, Geneva, Switzerland, Feb. 2015.

- J. Ström, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson's response to CfE for HDR and WCG”, ISO/IEC (MPEG) document m36184, Warsaw, Poland, June 2015.

- A. Norkin “Closed form HDR 4:2:0 chroma subsampling (HDR CE1 and AHG5 related)”, JCT-VC JCTVC-W0107, Geneva, Feb 2016.

- C. Rosewarne and V Kolesnikov, “CE1-related: LUT-based luma sample adjustment”, JCT-VC, JCTVC-W0056, San Diego 2016.

- C. Rosewarne and V Kolesnikov, “AHG13: Further results for LUT-based luma sample adjustment”, JCT-VC, JCTVC-X0054, Geneva 2016.

- J. Ström, “AhG-13 related: Multi-LUT Luma Adjustment Implementation”, JCT-VC, JCTVC-Y0030, Chengu, CN, Oct. 2016.

- J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Ström, G. J. Sullivan, P. Topiwala, and A. Tourapis, “Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics”, JCT-VC, JCTVC-Z1017, Geneva, Jan 2017

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).