| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA0042_r2 |
| --- | --- |

| Title: | Suggested fix for MCTS SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Bross R. Skupin Y. Sanchez | Email: | benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The semantics of the temporal motion-constrained tile sets SEI message constrain inter-picture prediction for a tile set to not use sample values outside the tile set. In case temporal motion vector prediction is used, the temporal motion vector predictor (TMVP) can be derived from a PU that lies outside the tile boundary in the collocated picture, i.e. the bottom-right candidate. Depending on the extraction process, the availability of this TMVP candidate can change and lead to different TMVPs before and after extraction. This problem was also reported in JCTVC-AA0027. In this document, it is proposed to prevent that by adding a restriction for a tile set to not use the TMVP for affected PUs.

# Problem statement

In HEVC the TMVP is derived using the motion vectors from the PU that is located at the bottom right of the current PU in the collocated picture (candidate C0). If one of the following conditions is true, a second candidate at the center of the collocated PU (candidate C1) is used for the derivation:

- The bottom-right candidate PU is coded in intra-picture prediction mode,

- The bottom-right candidate PU is located outside the picture boundary.

- The bottom-right candidate PU is located inside the next CTU line.

If the bottom-right candidate PU is located outside a tile boundary, it can still be used for TMVP derivation. Consider the case that a tile set is extracted, this PU could be located outside the picture boundary after extraction. This case is illustrated in Figure 1 Scenario A where TMVP candidate C0 is not available anymore after extraction.

In another case, two consecutive tiles (in raster scan order) of a temporal motion-constrained tile set from different rows are extracted, e.g. the last tile (2) in the first row and the first tile (3) in the second row. After extraction, the left picture boundary of tile 2 becomes a tile boundary and the left tile boundary of tile 3 becomes a picture boundary. In that case, the bottom-right candidate is excluded from the TMVP derivation process because the associated PU is outside the picture boundary. When the picture boundary becomes a tile boundary, this does not apply anymore and the bottom right candidate can be a valid TMVP candidate. This case is illustrated in Figure 1 Scenario B where TMVP candidate C0 becomes available after extraction. Candidate C0 becoming available after extraction can cause various mismatches.

Example 1: In AMVP for example, consider the case when no spatial candidates are available. When TMVP candidates are also not available, zero motion vectors are added to the list. Index 0 would then point to a zero MV. After extraction, C0 would become available and added as first candidate. Index 0 would then point to C0 and not to a zero MV anymore.

Example 2: In merge mode, when spatial and TMVP candidates are not available, no bi-predicted but only zero MV merging candidates are added to the list. Here, the first zero MV merging candidate with merge_idx equal to 0 has a reference picture index equal to 0 and the second zero MV merging candidate has a reference picture index equal to 1 and so on. After extraction, C0 would become available and added as first candidate. merge_idx equal to 0 would then point to C0 and merge_idx equal to 1 would relate to the zero MV merging candidate with reference picture index equal to 0. In that case, merge mode cannot be selected.

An encoder can prevent the mismatches by applying the following restriction on PUs located directly left to the right tile boundary of each identified tile set except the last one at the bottom right, where C0 is not available because its located in another CTU line:

- If spatial candidates are available for AMVP and merge mode, only use spatial merge and AMVP candidates, which are inserted in the lists before the TMVP candidate

- Otherwise (spatial candidates are not available), the second candidate (index 1) is used for AMVP, which always points to a zero MV candidate. Merge mode is not selected in that case because the zero MV merging candidates can have different reference indices.

The proposed solution in the next section incorporates that restriction in the temporal motion-constrained tile sets SEI message semantics.

Figure 1: Two scenarios when TMVP can change after MCTS extraction.

# Proposed solution and modified semantics

This following modification of the temporal motion-constrained tile sets SEI message semantics prevents the TMVP of a temporal motion-constrained tile set from referring to different motion vectors before and after extraction by adding a restriction for a tile set to not use the TMVP for affected PUs.

    - Temporal motion-constrained tile sets SEI message semantics

The temporal motion-constrained tile sets SEI message indicates that the inter prediction process is constrained such that no sample value outside each identified tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set, is used for inter prediction of any sample within the identified tile set.following encoder restrictions impact the decoding process for coding units coded in inter prediction mode inside any tile in the motion-constrained tile sets:

–	No sample values outside each identified tile set or outside the picture are referenced.

–	For PUs located directly left to the right tile boundary of each identified tile set except the last one at the bottom right and ( xPb, yPb ) specifies the top-left sample of the corresponding luma prediction block relative to the top-left sample of the current picture, the following applies if CuPredMode[ xPb ][ yPb ] is equal to MODE_INTER:

–	The number of spatial merging candidates numSpatialMergeCand is derived as follows:

numSpatialMergeCand = 	availableFlagA0 + availableFlagA1 + 	(D-1)
			availableFlagB0 + availableFlagB1 + availableFlagB2

with availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 being the output of the derivation process for spatial merging candidates specified in clause  and the following applies:

–	If numSpatialMergeCand is equal to 0, merge_flag[ xPb ][ yPb ] shall be equal to 0.

–	Otherwise ( numSpatialMergeCand is greater than 0 ), merge_idx[ xPb ][ yPb ] shall be in the range of 0 to numSpatialMergeCand − 1, inclusive.

–	The number of spatial motion vector predictor candidates numSpatialMvpCand is derived as follows:

if ( availableFlagLXA )
	numSpatialMvpCand = availableFlagLXA + ( mvLXA !=  mvLXB ) ?  availableFlagLXB : 0
else 			(D-2)
	numSpatialMvpCand = availableFlagLXB

with availableFlagLXA, availableFlagLXB, mvLXA and mvLXB being the output of the derivation process for motion vector predictor candidates from neighbouring prediction unit partitions specified in clause Error: Reference source not found and following applies:

–	If numSpatialMvpCand is equal to 0, mvp_l0_flag[ xPb ][ yPb ] and mvp_l1_flag[ xPb ][ yPb ] shall be equal to 1.

–	Otherwise ( numSpatialMvpCand is greater than 1 ), mvp_l0_flag[ xPb ][ yPb ] and mvp_l1_flag[ xPb ][ yPb ] shall be in the range of 0 to numSpatialMvpCand − 1, inclusive.

NOTE 1 – The first constraint restricts motion vectors to point to full-sample locations inside each identified tile set and to fractional-sample locations that require only full-sample locations inside each identified tile set for interpolation.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).