| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JCTVC-AA1005 |
| --- | --- |

| Title: | HEVC Additional Supplemental Enhancement Information (Draft 2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JCT-VC |  |  |
| Purpose: | Draft text for standardization |  |  |
| Author(s) or Contact(s): | Jill Boyce Adarsh Ramasubramanian Robert Skupin Gary J. Sullivan Alexis Tourapis | Email: | jill.boyce@intel.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is the draft text for changes to the High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 | ISO/IEC 23008-2) to specify additional supplemental enhancement information (SEI) messages for content colour volume, omnidirectional 360° projection, omnidirectional viewport, regional nesting, and motion-constrained tile sets extraction information, along with some corrections to existing specification texts.

- Changes to the specification text:

- Throughout the specification, replace all single-word instances of "nested" with "scalable-nested" (e.g., there is no single-word instance of "nested" in "non-nested"), and all instances of "non-nested" with "non-scalable-nested", except in 3.158 (which is kept unchanged) and Table F.4 in F.14.3.1 (for which a change is provided below).

- In in clause 4, add the following to the list of abbreviations (in alphabetical order):

MCTS	Motion-Constrained Tile Set

- In 5.8, renumber formulae 5-16 and 5-17 as 5-17 and 5-18, and add the following function definition:

Sin( x )	the trigonometric sine function operating on an argument x in units of radians	(5-16)

- In NOTE 2 of 7.4.2.4.4, delete the sentence that says “Consequently, hypothetical reference decoder (HRD) parameters carried in non-nested buffering period, picture timing and decoding unit information SEI messages apply to access units based on such access unit boundary detection.”

- In 7.4.4, replace the following paragraph:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages or segmented rectangular frame packing arrangement SEI messages present in the CVS.

- with the following:

general_non_packed_constraint_flag equal to 1 specifies that there are no frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages, and omnidirectional projection indication SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages, or omnidirectional projection indication SEI messages present in the CVS.

- In 8.7.2.1, replace the following paragraph:

The deblocking filter process is applied to all prediction block edges and transform block edges of a picture, except the edges that are at the boundary of the picture, for which the deblocking filter process is disabled by slice_deblocking_filter_disabled_flag, that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0, or that coincide with upper or left slice boundaries of slices with slice_loop_filter_across_slices_enabled_flag equal to 0. For the transform units and prediction units with block edges less than 8 samples in either the vertical or horizontal direction, only the edges lying on the 8x8 sample grid of the considered component are filtered.

- with the following:

The deblocking filter process is applied to all prediction block edges and transform block edges of a picture, except the following types of edges:

–	Edges that are at the boundary of the picture,

–	Edges that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0,

–	Edges that coincide with upper or left boundaries of slices with slice_loop_filter_across_slices_enabled_flag, equal to 0 or slice_deblocking_filter_disabled_flag equal to 1,

–	Edges within slices with slice_deblocking_filter_disabled_flag equal to 1,

–	Edges that do not correspond to 8x8 sample grid boundaries of the considered component,

–	Edges within chroma components for which both sides of the edge use inter prediction,

–	Edges of chroma transform blocks that are not edges of the associated transform unit.

- In C.1, immediately before the sentence that says “Figure C.1 shows the types of bitstream conformance points checked by the HRD.”, add the following NOTE 1, and renumber the existing NOTEs in the clause accordingly:

NOTE 1 – Decoders conforming to profiles specified in Annex A do not use NAL units with nuh_layer_id greater than 0 (e.g., access unit delimiter NAL units with nuh_layer_id greater than 0) for access unit boundary detection, except for identification of whether a NAL unit is a VCL or non-VCL NAL unit. Consequently, hypothetical reference decoder (HRD) parameters carried in non-scalable-nested buffering period, picture timing and decoding unit information SEI messages apply to access units that are identified based on such access unit boundary detection.

- Replace D.2.1 with the following:

- D.2.1	General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| picture_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 56 ) |  |
| green_metadata( payloadsize )/* specified in ISO/IEC 23001-11 */ |  |
| else if( payloadType = = 128 ) |  |
| structure_of_pictures_info( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| active_parameter_sets( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 131 ) |  |
| temporal_sub_layer_zero_index( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 134 ) |  |
| region_refresh_info( payloadSize ) |  |
| else if( payloadType = = 135 ) |  |
| no_display( payloadSize ) |  |
| else if( payloadType = = 136 ) |  |
| time_code( payloadSize ) |  |
| else if( payloadType = = 137 ) |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 138 ) |  |
| segmented_rect_frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 139 ) |  |
| temporal_motion_constrained_tile_sets( payloadSize ) |  |
| else if( payloadType = = 140 ) |  |
| chroma_resampling_filter_hint( payloadSize ) |  |
| else if( payloadType = = 141 ) |  |
| knee_function_info( payloadSize ) |  |
| else if( payloadType = = 142 ) |  |
| colour_remapping_info( payloadSize ) |  |
| else if( payloadType = = 143 ) |  |
| deinterlaced_field_identification( payloadSize ) |  |
| else if( payloadType = = 144 ) |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 146 ) |  |
| coded_region_completion( payloadSize ) |  |
| else if( payloadType = = 147 ) |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) |  |
| omni_projection_indication( payloadSize ) |  |
| else if( payloadType = = 151 ) |  |
| omni_viewport( payloadSize ) |  |
| else if( payloadType = = 152 ) |  |
| regional_nesting( payloadSize ) |  |
| else if( payloadType = = 153 ) |  |
| mcts_extraction_info_sets( payloadSize ) |  |
| else if( payloadType = = 154 ) |  |
| mcts_extraction_info_nesting( payloadSize ) |  |
| else if( payloadType = = 160 ) |  |
| layers_not_present( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 161 ) |  |
| inter_layer_constrained_tile_sets( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 162 ) |  |
| bsp_nesting( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 163 ) |  |
| bsp_initial_arrival_time( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 164 ) |  |
| sub_bitstream_property( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 165 ) |  |
| alpha_channel_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 166 ) |  |
| overlay_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 167 ) |  |
| temporal_mv_prediction_constraints( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 168 ) |  |
| frame_field_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 176 ) |  |
| three_dimensional_reference_displays_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 177 ) |  |
| depth_representation_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 178 ) |  |
| multiview_scene_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 179 ) |  |
| multiview_acquisition_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 180 ) |  |
| multiview_view_position( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 181 ) |  |
| alternative_depth_info( payloadSize ) /* specified in Annex I */ |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 146 ) |  |
| coded_region_completion( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

- Renumber clause D.2.40 (Reserved SEI message syntax) as D.2.46.

- Add clauses D.2.40 through D.2.45, as follows:

- D.2.40	Content colour volume SEI message syntax

| content_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| ccv_cancel_flag | u(1) |
| if( !ccv_cancel_flag ) { |  |
| ccv_persistence_flag | u(1) |
| ccv_primaries_present_flag | u(1) |
| ccv_min_luminance_value_present_flag | u(1) |
| ccv_max_luminance_value_present_flag | u(1) |
| ccv_avg_luminance_value_present_flag | u(1) |
| ccv_reserved_zero_2bits | u(2) |
| if( ccv_primaries_present_flag ) |  |
| for( c = 0; c < 3; c++ ){ |  |
| ccv_primaries_x[ c ] | i(16) |
| ccv_primaries_y[ c ] | i(16) |
| } |  |
| if( ccv_min_luminance_value_present_flag ) |  |
| ccv_min_luminance_value | u(32) |
| if( ccv_max_luminance_value_present_flag ) |  |
| ccv_max_luminance_value | u(32) |
| if( ccv_avg_luminance_value_present_flag ) |  |
| ccv_avg_luminance_value | u(32) |
| } |  |
| } |  |

- D.2.41	Omnidirectional projection indication SEI message syntax

| omni_projection_indication( payloadSize ) { | Descriptor |
| --- | --- |
| omni_projection_indication_cancel_flag | u(1) |
| if( !omni_projection_indication_cancel_flag ) { |  |
| omni_projection_information_persistence_flag | u(1) |
| omni_projection_type | u(5) |
| sub_geometry_flag | u(1) |
| if( projection_type = = 0 && sub_geometry_flag = = 1 ) { |  |
| omni_projection_yaw_center | i(32) |
| omni_projection_pitch_center | i(32) |
| omni_projection_roll_center | i(32) |
| omni_projection_yaw_range | u(32) |
| omni_projection_pitch_range | u(32) |
| } |  |
| } |  |
| } |  |

- D.2.42	Omnidirectional viewport SEI message syntax

| - omni_viewport( payloadSize ) { | Descriptor |
| --- | --- |
| - omni_viewport_id | u(10) |
| - omni_viewport_cancel_flag | u(1) |
| - if( !omni_viewport_cancel_flag ) { |  |
| - omni_viewport_persistence_flag | u(1) |
| - omni_viewport_cnt_minus1 | u(4) |
| - for( i = 0; i <= omni_viewport_cnt_minus1; i++ ) { |  |
| - omni_viewport_yaw_center[ i ] | i(32) |
| - omni_viewport_pitch_center[ i ] | i(32) |
| - omni_viewport_roll_center[ i ] | i(32) |
| - omni_viewport_yaw_range[ i ] | u(32) |
| - omni_viewport_pitch_range[ i ] | u(32) |
| - } |  |
| - } |  |
| - } |  |

- D.2.43	Regional nesting SEI message syntax

| regional_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| regional_nesting_id | u(16) |
| regional_nesting_num_rect_regions | u(8) |
| for( i = 0; i < regional_nesting_num_rect_regions; i++ ) { |  |
| regional_nesting_rect_region_id[ i ] | u(8) |
| regional_nesting_rect_left_offset[ i ] | u(16) |
| regional_nesting_rect_right_offset[ i ] | u(16) |
| regional_nesting_rect_top_offset[ i ] | u(16) |
| regional_nesting_rect_bottom_offset[ i ] | u(16) |
| } |  |
| num_sei_messages_in_regional_nesting_minus1 | u(8) |
| for( i = 0; i <= num_sei_messages_in_regional_nesting_minus1; i++ ) { |  |
| num_regions_for_sei_message[ i ] | u(8) |
| for(j = 0; j < num_regions_for_sei_message[ i ]; j++ ) |  |
| regional_nesting_sei_region_idx[ i ][ j ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

- D.2.44	Motion-constrained tile sets extraction information sets SEI message syntax

| mcts_extraction_info_sets( ) { | Descriptor |
| --- | --- |
| num_info_sets_minus1 | ue(v) |
| for( i = 0; i <= num_info_sets_minus1; i++ ) { |  |
| num_mcts_sets_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_mcts_sets_minus1[ i ]; j++ ) { |  |
| num_mcts_in_set_minus1[ i ][ j ] | ue(v) |
| for( k = 0; k <= num_mcts_in_set_minus1[ i ][ j ]; k++ ) |  |
| idx_of_mcts_in_set[ i ][ j ][ k ] | ue(v) |
| } |  |
| num_vps_in_info_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_vps_in_info_set_minus1[ i ]; j++ ) |  |
| vps_rbsp_data_length[ i ][ j ] | ue(v) |
| num_sps_in_info_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_sps_in_info_set_minus1[ i ]; j++ ) |  |
| sps_rbsp_data_length[ i ][ j ] | ue(v) |
| num_pps_in_info_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_pps_in_info_set_minus1[ i ]; j++ ) { |  |
| pps_nuh_temporal_id_plus1[ i ][ j ] | u(3) |
| pps_rbsp_data_length[ i ][ j ] | ue(v) |
| } |  |
| while( !byte_aligned( ) ) |  |
| mcts_alignment_bit_equal_to_zero | f(1) |
| for( j = 0; j <= num_vps_in_info_set_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= vps_rbsp_data_length[ i ][ j ]; k++ ) |  |
| vps_rbsp_data_byte[ i ][ j ][ k ] | u(8) |
| for( j = 0; j <= num_sps_in_info_set_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= sps_rbsp_data_length[ i ][ j ]; k++ ) |  |
| sps_rbsp_data_byte[ i ][ j ][ k ] | u(8) |
| for( j = 0; j <= num_pps_in_info_set_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= pps_rbsp_data_length[ i ][ j ]; k++ ) |  |
| pps_rbsp_data_byte[ i ][ j ][ k ] | u(8) |
| } |  |
| } |  |

- D.2.45	Motion-constrained tile sets extraction information nesting SEI message syntax

| mcts_extraction_info_nesting( ) { | Descriptor |
| --- | --- |
| all_mcts_flag | u(1) |
| if( !all_mcts_flag ) { |  |
| num_associated_mcts_minus1 | ue(v) |
| - for( i = 0; i <= num_associated_mcts_minus1; i++ ) |  |
| - idx_of_associated_mcts[ i ] | ue(v) |
| - } |  |
| - num_sei_messages_in_mcts_extraction_nesting_minus1 | ue(v) |
| - while( !byte_aligned( ) ) |  |
| - mcts_nesting_zero_bit /* equal to 0 */ | u(1) |
| - for( i = 0; i <= num_sei_messages_in_mcts_extraction_nesting_minus1; i++ ) |  |
| - sei_message( ) |  |
| - } |  |

- In D.3.1, replace the following paragraphs:

The list SingleLayerSeiList is set to consist of the payloadType values 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, and 134 to 148, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, and 134 to 148, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, and 135 to 148, inclusive.

- with the following:

The list SingleLayerSeiList is set to consist of the payloadType values 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, and 134 to 154, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, and 134 to 154, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, and 135 to 154, inclusive.

- In D.3.1, in Table D.1, replace the following rows of the table:

| Scene information | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a scene information SEI message |
| --- | --- |
| Green metadata | The CLVS containing the SEI message |
| Temporal motion-constrained tile sets | - The CLVS containing the SEI message |

- with the following:

| Scene information | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a scene information SEI message or starts a new CLVS |
| --- | --- |
| Green metadata | Specified by the syntax of the SEI message |
| Temporal motion-constrained tile sets | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains an SEI message of the same type or starts a new CLVS |

- In D.3.1, in Table D.1, append the following rows to the end of the table:

| Content colour volume | Specified by the syntax of the SEI message |
| --- | --- |
| Omnidirectional projection information | Specified by the syntax of the SEI message |
| Omnidirectional viewport | Specified by the syntax of the SEI message |
| Regional nesting | Depending on the region-nested SEI messages; each region-nested SEI message has the same persistence scope as if the SEI message was non-region-nested |
| Motion-constrained tile sets extraction information sets | The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains an SEI message of the same type or starts a new CLVS |
| Motion-constrained tile sets extraction information nesting | - The access unit containing the SEI message |

- In D.3.1, immediately after Table D.1, add the following:

The values of some SEI message syntax elements, including pan_scan_rect_id, scene_id, second_scene_id, snapshot_id, progressive_refinement_id, tone_map_id, frame_packing_arrangement_id, mcts_id[ i ], knee_function_id, colour_remap_id, ilcts_id[ i ], and regional_nesting_id, are split into two sets of value ranges, where the first set is specified as "may be used as determined by the application", and the second set is specified as "reserved for future use by ITU-T | ISO/IEC". Applications should be cautious of potential “collisions” of the interpretation for values of these syntax elements belonging to the first set of value ranges. Since different applications might use these IDs having values in the first set of value ranges for different purposes, particular care should be exercised in the design of encoders that generate SEI messages with these IDs having values in the first set of value ranges, and in the design of decoders that interpret SEI messages with these IDs having values in the first set of value ranges. This Specification does not define any management for these values. These IDs having values in the first set of value ranges might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of an SEI message with one of these IDs having the same value in the first set of value ranges) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

- In D.3.4 (Pan-scan rectangle SEI message semantics), replace the following:

Values of pan_scan_rect_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of pan_scan_rect_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of pan_scan_rect_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- with the following:

Values of pan_scan_rect_id from 0 to 255, inclusive, and from 512 to 231 − 1, inclusive, may be used as determined by the application. Values of pan_scan_rect_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of pan_scan_rect_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- In D.3.15 (Tone mapping information SEI message semantics), replace the following paragraph:

Values of tone_map_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of tone_map_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all tone mapping information SEI messages containing a value of tone_map_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

- with the following:

Values of tone_map_id from 0 to 255, inclusive, and from 512 to 231 − 1, inclusive, may be used as determined by the application. Values of tone_map_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of tone_map_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- In D.3.16 (Frame packing arrangement SEI message semantics), replace the following paragraph:

Values of frame_packing_arrangement_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of frame_packing_arrangement_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all frame packing arrangement SEI messages containing a value of frame_packing_arrangement_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

- with the following:

Values of frame_packing_arrangement_id from 0 to 255, inclusive, and from 512 to 231 − 1, inclusive, may be used as determined by the application. Values of frame_packing_arrangement_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of frame_packing_arrangement_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- In D.3.24, replace the following:

It is a requirement of bitstream conformance that the following restrictions apply on nesting of SEI messages:

- with the following:

It is a requirement of bitstream conformance that the following restrictions apply on the containing of SEI messages in a scalable nesting SEI message:

- In D.3.30 (Temporal motion-constrained tile sets SEI message semantics), replace the following paragraph:

- The temporal motion-constrained tile sets SEI message indicates that the inter prediction process is constrained such that no sample value outside each identified tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set, is used for inter prediction of any sample within the identified tile set.

- with the following:

The temporal motion-constrained tile sets SEI message indicates that the following constraints apply:

–	No sample values outside each identified tile set or outside the picture are referenced.

–	For PUs located directly left of the right tile boundary of each identified tile set except the last one at the bottom right, the following applies when CuPredMode[ xPb ][ yPb ] is equal to MODE_INTER, where ( xPb, yPb ) specifies the top-left sample of the corresponding luma prediction block relative to the top-left sample of the current picture:

–	With the number of spatial merging candidates numSpatialMergeCand derived as follows:

numSpatialMergeCand =	availableFlagA0 + availableFlagA1 + 	(D-42)
			availableFlagB0 + availableFlagB1 + availableFlagB2

where availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, and availableFlagB2 are the output of the derivation process for spatial merging candidates specified in clause 8.5.3.2.3, the following applies:

–	If numSpatialMergeCand is equal to 0, merge_flag[ xPb ][ yPb ] is equal to 0.

–	Otherwise (numSpatialMergeCand is greater than 0), merge_idx[ xPb ][ yPb ] is in the range of 0 to numSpatialMergeCand − 1, inclusive.

–	With the number of spatial motion vector predictor candidates numSpatialMvpCand derived as follows:

if ( availableFlagLXA )
	numSpatialMvpCand = availableFlagLXA + ( ( mvLXA  !=  mvLXB ) ? availableFlagLXB : 0 )
else			(D-43)
	numSpatialMvpCand = availableFlagLXB

where availableFlagLXA, availableFlagLXB, mvLXA, and mvLXB are the output of the derivation process for motion vector predictor candidates from neighbouring prediction unit partitions specified in clause 8.5.3.2.7, the following applies:

–	If numSpatialMvpCand is equal to 0, mvp_l0_flag[ xPb ][ yPb ] and mvp_l1_flag[ xPb ][ yPb ] is equal to 1.

–	Otherwise (numSpatialMvpCand is greater than 0), mvp_l0_flag[ xPb ][ yPb ] and mvp_l1_flag[ xPb ][ yPb ] is in the range of 0 to numSpatialMvpCand − 1, inclusive.

NOTE 1 – The first constraint restricts motion vectors to point to full-sample locations inside each identified tile set and to fractional-sample locations that require only full-sample locations inside each identified tile set for interpolation. The second constraint restricts the usage of motion vector candidates derived from blocks outside each identified tile set.

- In D.3.30 (Temporal motion-constrained tile sets SEI message semantics), renumber the subsequent NOTEs to account for the added NOTE.

- In D.3.30 (Temporal motion-constrained tile sets SEI message semantics) and subsequent subclauses of Annex D, renumber the subsequent formulae to account for the added formula.

- In D.3.30 (Temporal motion-constrained tile sets SEI message semantics), remove the following paragraph:

The number of temporal motion-constrained tile sets SEI messages applicable to the same nuh_layer_id value in each access unit shall not exceed 5.

- In D.3.30 (Temporal motion-constrained tile sets SEI message semantics), add the following paragraphs immediately before the semantics of mc_all_tiles_exact_sample_value_match_flag:

When a temporal motion-constrained tile sets SEI message is present, a slice segment that contains one or more tiles in any particular temporal motion-constrained tile set shall not be a dependent slice segment of an independent slice segment that contains one or more tiles that do not belong to that temporal motion-constrained tile set.

For purposes of referencing a particular temporal motion-constrained tile set that is identified in a temporal motion-constrained tile sets SEI message (e.g., for use with a motion-constrained tile sets extraction information sets SEI message or a motion-constrained tile sets extraction information nesting SEI message), an MCTS index is defined as follows:

–	If the value of each_tile_one_tile_set_flag of the temporal motion-constrained tile sets SEI message is equal to 0, the MCTS index is the value of the variable i within the loop of the num_sets_in_message_minus1 + 1 sets of MCTS information specified by the temporal MCTS SEI message.

–	Otherwise, the MCTS index of each MCTS is the tile position of the tile in tile raster scan order.

- In D.3.30 (Temporal motion-constrained tile sets SEI message semantics), replace the following sentence:

Decoders shall ignore (remove from the bitstream and discard) those SEI messages containing a value of mcts_id[ i ] in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive.

- with the following:

Decoders encountering a value of mcts_id[ i ] in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- In D.3.30 (Temporal motion-constrained tile sets SEI message semantics), after the paragraph providing the semantics of top_left_tile_index[ i ][ j ] and bottom_right_tile_index[ i ][ j ], add the following sentence:

The value of top_left_tile_index[ i ][ j ] and bottom_right_tile_index[ i ][ j ] shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive.

- In D.3.31 (Chroma resampling filter hint SEI message semantics), add the following paragraph before the semantics of hor_chroma_filter_idc:

All chroma resampling filter hint SEI messages that apply to the same CLVS shall have the same content.

- In D.3.32 (Knee function information SEI message semantics), replace the following:

Decoders shall ignore all knee function information SEI messages containing a value of knee_function_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

- with the following:

Decoders encountering a value of knee_function_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- In D.3.33 (Colour remapping information SEI message semantics), replace the following paragraph:

Values of colour_remap_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of colour_remap_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour remapping information SEI messages containing a value of colour_remap_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

- with the following:

Values of colour_remap_id from 0 to 255, inclusive, and from 512 to 231 − 1, inclusive, may be used as determined by the application. Values of colour_remap_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of colour_remap_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- In D.3.35 (Content light level information SEI message semantics), add the following paragraph before the semantics of max_content_light_level:

All content light level information SEI messages that apply to the same CLVS shall have the same content.

- Renumber clause D.3.40 (Reserved SEI message semantics) as D.3.46.

- Add subclauses D.3.40 through D.3.45, as follows:

- D.3.40	Content colour volume SEI message semantics

The content colour volume SEI message provides information about the colour volume characteristics of the associated picture. This SEI message describes the colour volume characteristics of the associated picture in terms of a nominal range, and deviations from this range may occur.

The variable transferCharacteristics is specified as follows:

–	If an alternative transfer characteristics SEI message is present for the CLVS, transferCharacteristics is set equal to preferred_transfer_characteristics;

–	Otherwise, (an alternative transfer characteristics SEI message is not present for the CLVS), transferCharacteristics is set equal to transfer_characteristics.

The content colour volume SEI message shall not be present, and decoders shall ignore it, when any of the following conditions is true:

–	Any of the values of transferCharacteristics, colour_primaries, and matrix_coeffs has a value defined as unspecified.

–	The value of transfer_characteristics is equal to 2, 4, or 5.

–	The value of colour_primaries is equal to 2.

The following applies when converting the signal from a non-linear to a linear representation:

–	If the value of transferCharacteristics is equal to 1, 6, 7, 14, or 15, the Rec. ITU-R BT.1886-0 reference electro-optical transfer function should be used to convert the signal back to its linear representation, where the value of screen luminance for white is set equal to 100 cd/m2, the value of screen luminance for black is set equal to 0 cd/m2, and the value of the exponent of the power function is set equal to 2.40.

–	Otherwise, if the value of transferCharacteristics is equal to 18, the hybrid log-gamma reference electro-optical transfer function specified in Rec. ITU-R BT.2100-0 should be used, where the value of nominal peak luminance of the display is set equal to 1000 cd/m2, the value of the display luminance for black is set equal to 0 cd/m2, and the value of system gamma is set equal to 1.2.

–	Otherwise (the value of transferCharacteristics is not equal to 1, 6, 7, 14, 15, or 18) when the content colour volume SEI message is present, the exact inverse of the transfer function specified in Table E.4 should be used to convert the non-linear signal to a linear representation.

ccv_cancel_flag equal to 1 indicates that the content colour volume SEI message cancels the persistence of any previous content colour volume SEI message in output order that applies to the current layer. ccv_cancel_flag equal to 0 indicates that content colour volume information follows.

ccv_persistence_flag specifies the persistence of the content colour volume SEI message for the current layer.

ccv_persistence_flag equal to 0 specifies that the content colour volume applies to the current decoded picture only.

Let picA be the current picture. ccv_persistence_flag equal to 1 specifies that the content colour volume SEI message persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a content colour volume SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for the picture order count of picB.

ccv_primaries_present_flag equal to 1 specifies that the syntax elements ccv_primaries_x[ c ] and ccv_primaries_y[ c ] are present. ccv_primaries_present_flag equal to 0 specifies that the syntax elements ccv_primaries_x[ c ] and ccv_primaries_y[ c ] are not present.

ccv_min_luminance_value_present_flag equal to 1 specifies that the syntax element ccv_min_luminance_value is present. ccv_min_luminance_value_present_flag equal to 0 specifies that the syntax element ccv_min_luminance_value is not present.

ccv_max_luminance_value_present_flag equal to 1 specifies that the syntax element ccv_max_luminance_value is present. ccv_max_luminance_value_present_flag equal to 0 specifies that the syntax element ccv_max_luminance_value is not present.

ccv_avg_luminance_value_present_flag equal to 1 specifies that the syntax element ccv_avg_luminance_value is present. ccv_avg_luminance_value_present_flag equal to 0 specifies that the syntax element ccv_avg_luminance_value is not present.

It is a requirement of bitstream conformance that the values of ccv_primaries_present_flag, ccv_min_luminance_value_present_flag, ccv_max_luminance_value_present_flag, and ccv_avg_luminance_value_present_flag shall not all be equal to 0.

ccv_reserved_zero_2bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for reserved_zero_2bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of reserved_zero_2bits[ i ].

ccv_primaries_x[ c ] and ccv_primaries_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c of the nominal content colour volume in normalized increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing colour volumes that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex E and Table E.3).

The values of ccv_primaries_x[ c ] and ccv_primaries_y[ c ] shall be in the range of −5 000 000 to 5 000 000, inclusive.

When ccv_primaries_x[ c ] and ccv_primaries_y[ c ] are not present, they are inferred to be equal to the normalized x and y chromaticity coordinates, respectively, specified by colour_primaries.

ccv_min_luminance_value specifies the normalized minimum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 according to the indicated transfer characteristics of the signal. The values of ccv_min_luminance_value are in normalized increments of 0.0000001.

ccv_max_luminance_value specifies the maximum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 according to the transfer characteristics of the signal. The values of ccv_max_luminance_value are in normalized increments of 0.0000001.

ccv_avg_luminance_value specifies the average luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 according to the transfer characteristics of the signal. The values of ccv_avg_luminance_value are in normalized increments of 0.0000001.

NOTE – The resulting domain from this conversion process may or may not represent light in a source or display domain – it is merely a gamut representation domain rather than necessarily being a representation of actual light in either the scene or display domain. Therefore, the values corresponding to ccv_min_luminance_value, ccv_max_luminance_value, and ccv_avg_luminance_value might not necessarily correspond to a true luminance value.

The value of ccv_min_luminance_value, when present, shall be less than or equal to ccv_avg_luminance_value, when present. The value of ccv_avg_luminance_value, when present, shall be less than or equal to ccv_max_luminance_value, when present. The value of ccv_min_luminance_value, when present, shall be less than or equal to ccv_max_luminance_value, when present.

When the visually relevant region does not correspond to the entire cropped decoded picture, such as for "letterbox" encoding of video content with a wide picture aspect ratio within a taller cropped decoded picture, the indicated ccv_min_luminance_value, ccv_max_luminance_value, and ccv_avg_luminance_value should correspond only to values within the visually relevant region.

- D.3.41	Omnidirectional projection indication SEI message semantics

The omnidirectional projection indication SEI message provides information to enable remapping of the colour samples of the output decoded pictures onto a spherical coordinate space in angular coordinates (ϕ, θ) for use in omnidirectional video applications for which the viewing perspective is from the origin looking outward toward the inside of the sphere. The spherical coordinates are defined so that ϕ is the azimuth (longitude, increasing eastward) and θ is the elevation (latitude, increasing northward) as depicted in Figure D.12.

When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer, an omnidirectional projection indication SEI message shall be present for the first picture of the CLVS.

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no omnidirectional projection indication SEI messages applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, and the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is not equal to 3, 4, or 5, or the value of quincunx_sampling_flag of the frame packing arrangement SEI message is not equal to 0, an omnidirectional projection indication SEI message with omni_projection_indication_cancel_flag equal to 0 shall not be present that applies to the picture. Decoders shall ignore omnidirectional projection indication SEI messages when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, and the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is not equal to 3, 4, or 5, or the value of quincunx_sampling_flag of the frame packing arrangement SEI message is not equal to 0.

When a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, an omnidirectional projection indication SEI message with omni_projection_indication_cancel_flag equal to 0 shall not be present that applies to the picture. Decoders shall ignore omnidirectional projection indication SEI messages when a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture.

omni_projection_information_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional projection indication SEI message in output order. omni_projection_information_cancel_flag equal to 0 indicates that omnidirectional projection indication information follows.

omni_projection_information_persistence_flag specifies the persistence of the omnidirectional projection indication SEI message for the current layer.

omni_projection_information_persistence_flag equal to 0 specifies that the omnidirectional projection indication SEI message applies to the current decoded picture only.

Let picA be the current picture. omni_projection_information_persistence_flag equal to 1 specifies that the omnidirectional projection indication SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a omnidirectional projection indication SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

Figure D.12 – Spherical surface coordinates ϕ, θ with yaw, pitch, and roll of the region of a sphere covered by the cropped output picture relative to the equator and 0 meridian

omni_projection_type indicates the particular mapping of the samples of the rectangular cropped output picture onto the spherical coordinate system.

When omni_projection_type is equal to 0, the indicated projection type is the equirectangular projection. The value of omni_projection_type shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for omni_projection_type are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore omnidirectional projection indication SEI messages with omni_projection_type greater than 0.

The value of omni_projection_type shall be the same for all omnidirectional projection indication SEI messages in the CLVS.

sub_geometry_flag indicates the presence of sub-geometry elements that describe the spherical region covered by the cropped output picture.

omni_projection_yaw_center specifies the value of yaw that corresponds to the center position of the cropped output picture, where the value is derived by performing a clockwise rotation around the up vector when viewed from the origin looking upwards (anticlockwise when looking toward the origin), in units of 2−16 degrees. The value of omni_projection_yaw_center shall be in the range of −180 * 216 (i.e., −11796480) to 180 * 216 − 1 (i.e., 11796479), inclusive. When not present, the value of omni_projection_yaw_center is inferred to be equal to 0.

omni_projection_pitch_center specifies the value of pitch that corresponds to the center position of the cropped output picture, where the value is derived by performing a clockwise rotation around the omni_projection_yaw_center when viewed from the origin looking outwards (anticlockwise when looking toward the origin), i.e., the right vector after yaw rotation, in units of 2−16 degrees. The value of omni_projection_pitch_center shall be in the range of −90 * 216 (i.e., −5898240) to 90 * 216 (i.e., 5898240), inclusive. When not present, the value of omni_projection_pitch_center is inferred to be equal to 0.

omni_projection_roll_center specifies the value of roll that corresponds to the center position of the cropped output picture, where the value is derived by performing a clockwise rotation around the pitch_center when viewed from the origin looking outwards (anticlockwise when looking toward the origin), i.e., the forward vector after yaw and pitch rotation, in units of 2−16 degrees. The value of omni_projection_roll_center shall be in the range of −180 * 216 (i.e., −11796480) to 180 * 216 − 1 (i.e., 11796479), inclusive. When not present, the value of omni_projection_roll_center is inferred to be equal to 0.

omni_projection_yaw_range specifies the range of values of yaw that are present in the spherical region covered by the cropped output picture, in units of 2−16 degrees. The value of omni_projection_yaw_range shall be in the range of 1 to 720 * 216 (i.e., 47185920), inclusive. When not present, the value of omni_projection_yaw_range is inferred to be equal to 360 * 216 (i.e., 23592960).

omni_projection_pitch_range specifies the range of values of pitch that are present in the spherical region covered by the cropped output picture, in units of 2−16 degrees. The value of omni_projection_pitch_range shall be in the range of 1 to 180 * 216 (i.e., 11796480), inclusive. When not present, the value of omni_projection_pitch_range is inferred to be equal to 180 * 216 (i.e., 11796480).

When omni_projection_type is equal to 0, equations are provided in this subclause for identifying angular coordinates in a spherical geometry for positions corresponding to the positions of the luma samples of the picture. For purposes of interpretation of chroma samples, the input to the indicated remapping process is the set of decoded sample values after applying an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format). This (unspecified) upsampling process should account for the relative positioning relationship between the luma and chroma samples as indicated by chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, when present.

The variables StereoWidthScale and StereoHeightScale are derived as follows:

–	If an omnidirectional projection indication SEI message with omni_projection_indication_cancel_flag equal to 0 applies to the current picture, and a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 also applies to the current picture, the variables StereoWidthScale and StereoHeightScale are determined according to Table D.22.

–	Otherwise, the values of StereoWidthScale and StereoHeightScale are both set equal to 1.

Table D.22 – Definition of StereoWidthScale and StereoHeightScale from frame_packing_arrangement_type

| frame_packing_arrangement_type | StereoWidthScale | StereoHeightScale |
| --- | --- | --- |
| 3 | 2 | 1 |
| 4 | 1 | 2 |
| 5 | 1 | 1 |

Luma dimensions of the cropped output picture or constituent frame, as shown in Figure D.13, are derived as follows:

viewWidth = ( pic_width_in_luma_samples −
	SubWidthC * ( conf_win_right_offset + conf_win_left_offset ) ) / StereoWidthScale	(D-52)

viewHeight = ( pic_height_in_luma_samples −
	SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset ) ) / StereoHeightScale	(D-53)

When omni_projection_type is equal to 0, positions are identified for one or two constituent frames as follows:

–	If StereoWidthScale is equal to 2, there are two constituent frames in the picture, for which both constituent frames have viewTopOffset equal to 0, the first constituent frame has viewLeftOffset equal to 0, and the second constituent frame has viewLeftOffset equal to viewWidth.

–	Otherwise, if StereoHeightScale is equal to 2, there are two constituent frames in the picture, for which both constituent frames have viewLeftOffset equal to 0, the first constituent frame has viewTopOffset equal to 0, and the second constituent frame has viewTopOffset equal to viewHeight.

–	Otherwise, there is only one constituent frame in the picture, for which viewLeftOffset is equal to 0 and viewTopOffset is equal to 0.

Figure D.13 – Relationship of the index variables i′ and j′ to the cropped output of decoded picture luma samples when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 and frame_packing_arrangement_type equal to 3 or 4 is not present that applies to the picture

When omni_projection_type is equal to 0, the cropped output picture samples at locations (i, j) are indicated as corresponding to angular coordinates in a spherical geometry for positions corresponding to the positions of the luma samples of the picture. For purposes of interpretation of chroma samples, the input to the indicated remapping process is the set of decoded sample values after applying an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format). This (unspecified) upsampling process should account for the relative positioning relationship between the luma and chroma samples as indicated by chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, when present.

When omni_projection_type is equal to 0, for i equal to conf_win_left_offset to conf_win_left_offset + viewWidth − 1, inclusive, and j equal to conf_win_top_offset to conf_win_top_offset + viewHeight − 1, inclusive, the corresponding angular coordinates (ϕ, θ) for the sample locations, in degrees, are given by the following equirectangular mapping equations:

ϕmin = ( omni_projection_yaw_center − omni_projection_yaw_range ÷ 2 ) * 2−16	(D-54)

ϕmax = ( omni_projection_yaw_center + omni_projection_yaw_range ÷ 2 ) * 2−16	(D-55)

θmin = ( omni_projection_pitch_center − omni_projection_pitch_range ÷ 2 ) * 2−16	(D-56)

θmax = ( omni_projection_pitch_center + omni_projection_pitch_range ÷ 2 ) * 2−16	(D-57)

ω = omni_projection_roll_center * 2−16		(D-58)

α = ϕmax − ( i′ + 0.5 ) * ( ϕmax − ϕmin ) ÷ viewWidth		(D-59)

β = θmax − ( j′ + 0.5 ) * ( θmax − θmin ) ÷ viewHeight		(D-60)

ϕ = Cos( ω′ ) * α − Sin( ω′ ) * β		(D-61)

θ = Sin( ω′ ) * α + Cos( ω′ ) * β		(D-62)

where

i′ = i − ( SubWidthC * conf_win_left_offset + viewLeftOffset )	(D-63)

j′ = j − ( SubWidthC * conf_win_top_offset + viewTopOffset )	(D-64)

ω′ = ω * π ÷ 180		(D-65)

and π is Archimedes' constant 3.141 592 653 589 793...

Figure D.14 illustrates the correspondence between the cropped output picture samples and angular coordinates for the case with omni_projection_type equal to 0 and sub_geometry_flag equal to 0.

The example shown in Figure D.12 illustrates a projection mapping with omni_projection_type equal to 0, sub_geometry_flag equal to 0, omni_projection_yaw_center equal to 30 * 216, omni_projection_pitch_center equal to 20 * 216, omni_projection_roll_center equal to 10 * 216, omni_projection_yaw_range equal to 90 * 216, and omni_projection_pitch_range equal to 45 * 216.

When the value of omni_projection_yaw_range exceeds 360 * 216 (i.e., 23592960), the cropped output picture contains sample regions that cover overlapping areas of the spherical geometry. The samples of the picture with i′ within the range of i′min to i′max, inclusive, are sufficient to provide complete angular coverage, although the overlapping region samples with i′ in the range of 0 to viewWidth − 1, inclusive, that are outside the range of i′min to i′max, inclusive, may be also used for display (e.g., using blending or cropping or some combination thereof), as determined by the application, where i′min and i′max are derived as follows:

i′min = Floor( ( ϕmax − 180 ) * viewWidth ÷ ( ϕmax − ϕmin ) − 0.5 )		(D-66)

i′max = Ceil( ( ϕmax + 180 ) * viewWidth ÷ ( ϕmax − ϕmin ) − 0.5 )		(D-67)

Figure D.14 – Equirectangular mapping between positions of luma samples and spherical coordinates when sub_geometry_flag is equal to 0, as viewed from the viewer’s perspective (i.e., viewed looking outward toward the inside of the sphere)

- D.3.42	Omnidirectional viewport SEI message semantics

The omnidirectional viewport SEI message specifies the coordinates of one or more regions of spherical-coordinate geometry, bounded by four great circles, corresponding to viewports recommended for display. The reference spherical coordinate system used for the omnidirectional viewport SEI message is the same as for the omnidirectional projection indication SEI message with omni_projection_type equal to 0.

omni_viewport_id contains an identifying number that may be used to identify the purpose of the one or more recommended viewport regions.

Values of omni_viewport_id from 0 to 511, inclusive, may be used as determined by the application. Values of omni_viewport_id from 512 to 1023 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of omni_viewport_id in the range of 512 to 1023, inclusive, shall ignore it.

omni_viewport_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional viewport SEI message in output order. omni_viewport_cancel_flag equal to 0 indicates that omnidirectional viewport information follows.

omni_viewport_persistence_flag specifies the persistence of the omnidirectional viewport SEI message for the current layer.

omni_viewport_persistence_flag equal to 0 specifies that the omnidirectional viewport SEI message applies to the current decoded picture only.

Let picA be the current picture. omni_viewport_persistence_flag equal to 1 specifies that the omnidirectional viewport SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an omnidirectional viewport SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an omnidirectional projection indication SEI message with omni_projection_information_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the omnidirectional viewport SEI message in decoding order, an omnidirectional viewport SEI message with omni_viewport_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore omnidirectional viewport SEI messages with omni_viewport_cancel_flag equal to 0 that do not follow, in decoding order, an omnidirectional projection indication SEI message with omni_projection_information_cancel_flag equal to 0 in the CLVS that applies to the current picture.

omni_viewport_cnt_minus1 specifies the number of recommended viewport regions that are indicated by the SEI message.

omni_viewport_yaw_center[ i ] indicates the center of the i-th recommended viewport region, around the up vector, in units of 2−16 degrees. The value of omni_viewport_yaw_center[ i ] shall be in the range of −180 * 216 (i.e., −11796480) to 180 * 216 − 1 (i.e., 11796479), inclusive.

omni_viewport_pitch_center[ i ] indicates the center of the i-th recommended viewport region, around the omni_viewport_yaw_center, i.e., the right vector after yaw rotation, in units of 2−16 degrees. The value of omni_viewport_pitch_center[ i ] shall be in the range of −90 * 216 (i.e., −5898240) to 90 * 216 (i.e., 5898240), inclusive.

omni_viewport_roll_center[ i ] indicates the center of the i-th recommended viewport region, around the omni_viewport_pitch_center, i.e., the forward vector after yaw and pitch rotation, in units of 2−16 degrees. The value of omni_viewport_roll_center[ i ] shall be in the range of −180 * 216 (i.e., −11796480) to  216 − 1 (i.e., 11796479), inclusive.

omni_viewport_yaw_range[ i ] indicates the size of the i-th recommended viewport region of the projection mapped decoded picture in range of values of yaw in units of 2−16 degrees. The value of omni_viewport_yaw_range[ i ] shall be in the range of 1 to 360 * 216 (i.e., 23592960), inclusive.

omni_viewport_pitch_range[ i ] indicates the size of the i-th recommended viewport region of the projection mapped decoded picture, in range of values of pitch in units of 2−16 degrees. The value of omni_viewport_pitch_range[ i ] shall be in the range of 1 to 180 * 216 (i.e., 11796480), inclusive.

- D.3.43	Regional nesting SEI message semantics

The regional nesting SEI message provides a mechanism to associate SEI messages with regions of the picture. The associated SEI messages are conveyed within the regional nesting SEI message.

A regional nesting SEI message contains one or more SEI messages. When an SEI message is contained in a regional nesting SEI message, the contained SEI message is referred to as a region-nested SEI message. When an SEI message is not contained in a regional nesting SEI message, the SEI message is referred to as a non-region-nested SEI message.

For each region-nested SEI message in a regional nesting SEI message, one or more regions are specified in the regional nesting SEI message, and the semantics of the region-nested SEI message are to be interpreted as applying to each of these regions.

The list listOfRegionNestableMessageTypes includes the following types of SEI messages:

–	Film grain characteristics SEI message,

–	Post filter hint SEI message,

–	Tone mapping information SEI message identified with a particular value of tone_map_id,

–	Chroma resampling filter hint SEI message,

–	Knee function information SEI message identified with a particular value of knee_function_id,

–	Colour remapping information SEI message identified with a particular value of colour_remap_id,

–	Content colour volume SEI message.

NOTE 1 – SEI messages of each of the following are considered different types of SEI messages: 1) tone mapping information SEI messages with different values of tone_map_id, 2) knee function information SEI messages with different values of knee_function_id, and 3) colour remapping information SEI messages with different values of colour_remap_id.

When an SEI message of a particular type in listOfRegionNestableMessageTypes has film_grain_characteristics_cancel_flag, tone_map_cancel_flag, knee_function_cancel_flag, or colour_remap_cancel_flag equal to 1, regardless of whether it is region-nested or non-region-nested, it cancels the persistence of all the region-nested SEI messages of that type, regardless of their associated regions. When an SEI message of a particular type having film_grain_characteristics_persistence_flag, tone_map_persistence_flag, knee_function_persistence_flag, or colour_remap_persistence_flag equal to 1 is region-nested, the persistence of the SEI message is determined by the semantics of the SEI message, irrespective of which region it applies to.

NOTE 2 – A region-nested SEI message has the same persistence scope as if the SEI message was non-region-nested.

NOTE 3 – A region-nested SEI message does not cancel the persistence of a non-region-nested SEI message of the same type.

The list listOfAllowedRegionalNestableMessageTypes includes all the entries in the list listOfRegionNestableMessageTypes and also the following additional types of SEI messages:

–	User data registered by Rec. ITU-T T.35 SEI message,

–	User data unregistered SEI message.

In bitstreams conforming to this version of this Specification, the regional nesting SEI message shall not contain any SEI message that is not in listOfAllowedRegionNestableMessageTypes. Decoders encountering a region-nested SEI message that does not belong to listOfAllowedRegionNestableMessageTypes shall ignore the region-nested SEI message.

When an access unit contains both region-nested SEI messages of a particular type in listOfRegionNestableMessageTypes and non-region-nested SEI messages of the same type, decoders shall ignore either all the region-nested SEI message of that type or all the non-region-nested SEI messages of that type. Unless indicated otherwise by some means not specified in this Specification, when an access unit contains both region-nested SEI messages of a particular type in listOfRegionNestableMessageTypes and non-region-nested SEI messages of the same type, the region-nested SEI messages should be preferred to be considered as applicable to the access unit.

A region-nested SEI messages should not be extracted and sent as a non-region-nested SEI message, as the values signalled in the region-nested SEI message may not be applicable outside the indicated regions.

regional_nesting_id contains an identifying number that may be used to identify the purpose of the one or more SEI messages that are region-nested in the regional nesting SEI message. The value of regional_nesting_id shall be in the range of 0 to 216 − 1, inclusive.

Values of regional_nesting_id from 0 to 255 and from 512 to 215 − 1 may be used as determined by the application. Values of regional_nesting_id from 256 to 511 and from 215 to 216 − 1 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of regional_nesting_id in the range of 256 to 511, inclusive, or in the range of 215 to 216 − 1, inclusive, shall ignore it.

regional_nesting_num_rect_regions specifies the number of rectangular regions specified in the regional nesting SEI message. The value of regional_nesting_num_rect_regions shall be in the range of 1 to 255, inclusive. The value of regional_nesting_num_rect_regions equal to 0 is reserved for future use by ITU-T | ISO/IEC and shall not be used in bitstreams conforming to this version of this Specification. Decoders shall ignore regional nesting SEI messages with regional_nesting_num_rect_regions equal to 0.

regional_nesting_rect_region_id[ i ] specifies the identifier for the i-th rectangular region specified in the regional nesting SEI message.

Unless indicated otherwise by some means not specified in this Specification, when a sample belongs to more than one region indicated as applying to more than one region-nested SEI message of a particular type in listOfRegionNestableMessageTypes, among these region-nested SEI messages, only those that are associated with the region that has the greatest value of regional_nesting_rect_region_id[ ] are considered as applying to the sample, while the rest of these region-nested SEI messages are considered as not applying to the sample.

NOTE 4 – When there are more than one of these region-nested SEI messages associated with the region that has the greatest value of regional_nesting_rect_region_id[ ], they are identical per other expressed constraints.

It is a requirement of bitstream conformance that the value of regional_nesting_rect_region_id[ i ] shall not be the same for any two different values of i in the range of 0 to regional_nesting_num_rect_regions − 1, inclusive, in the regional nesting SEI message.

When a region-nested SEI message of a particular type in listOfRegionNestableMessageTypes is indicated as applying to a list of regions listA in the current picture and another region-nested SEI message of the same type is indicated as applying to another list of regions listB in the current picture, it is a requirement of bitstream conformance that, for any pair of regions formed by choosing one from listA and the other from listB, the value of regional_nesting_rect_region_id[ ] of the two regions shall not be the same unless the two regions are identical (i.e., both position and size are the same) and the two region-nested SEI messages are identical.

regional_nesting_rect_‌left_offset[ i ], regional_nesting_rect_‌right_offset[ i ], regional_nesting_rect_‌top_offset[ i ], and regional_nesting_rect_‌bottom_offset[ i ] specify the coordinates of the i-th rectangular region specified in the SEI message. The offsets for the rectangular region are specified in units of luma samples. The i-th rectangular region contains the luma samples with horizontal picture coordinates from SubWidthC * regional_nesting_rect_‌left_offset[ i ] to pic_width_in_luma_samples − ( SubWidthC * regional_nesting_rect_‌right_offset[ i ] + 1 ), inclusive, and vertical picture coordinates from SubHeightC * regional_nesting_rect_‌top_offset[ i ] to pic_height_in_luma_samples − ( SubHeightC * regional_nesting_rect_‌bottom_offset[ i ] + 1 ), inclusive.

The value of SubWidthC * ( regional_nesting_rect_‌left_offset[ i ] + regional_nesting_rect_‌right_offset[ i ] ) shall be less than pic_width_in_luma_samples and the value of SubHeightC * ( regional_nesting_rect_‌top_offset[ i ] + regional_nesting_rect_‌bottom_offset[ i ] ) shall be less than pic_height_in_luma_samples.

num_sei_messages_in_regional_nesting_minus1 plus 1 specifies the number of region-nested SEI messages in the regional nesting SEI message. The value of num_sei_messages_in_regional_nesting_minus1 shall be in the range of 0 to 255, inclusive.

num_regions_for_sei_message[ i ] specifies the number of regions to which the i-th region-nested SEI message is associated. When regional_nesting_num_rect_regions is greater than 0, the value of num_regions_for_sei_message[ i ] shall be in the range of 0 to regional_nesting_num_rect_regions, inclusive. When num_regions_for_sei_message[ i ] is equal to 0, the i-th region-nested SEI message applies to all the regions specified in the regional nesting SEI message.

regional_nesting_sei_region_idx[ i ][ j ] specifies the index, into the list of regions specified in the regional nesting SEI message, of the j-th region to which the i-th region-nested SEI message in the regional nesting SEI message is associated. The value of regional_nesting_sei_region_idx[ i ][ j ] shall be in the range of 0 to regional_nesting_num_rect_region − 1, inclusive.

- D.3.44	Motion-constrained tile sets extraction information sets SEI message semantics

The motion-constrained tile sets extraction information sets SEI message provides supplemental information that can be used in the motion-constrained tile set (MCTS) sub-bitstream extraction as specified below to generate a conforming bitstream for an MCTS set. The information consists of a number of extraction information sets, each defining a number of MCTS sets and containing RBSP bytes of the replacement VPSs, SPSs, and PPSs to be used during the MCTS sub-bitstream extraction process. Each extraction information set can be shared by multiple MCTS sets, i.e., is used for extraction of any of these MCTS sets.

An MCTS extraction information sets SEI message shall not be present in an access unit unless there is a temporal MCTS SEI message present in the access unit. A temporal MCTS SEI message present in the same access unit as an MCTS extraction information sets SEI message is referred to as the associated temporal MCTS SEI message of the MCTS extraction information sets SEI message.

An MCTS extraction information sets SEI message applies to the same set of pictures as the associated temporal MCTS SEI message, i.e., the associatedPicSet of the MCTS extraction information sets SEI message is the same as the associatedPicSet of the associated temporal MCTS SEI message.

When more than one MCTS extraction information sets SEI message is present for the pictures of associatedPicSet, these MCTS extraction information sets SEI messages shall contain identical content.

NAL units that contain tiles belonging to any particular MCTS mctsA shall not contain tiles that do not belong to mctsA.

num_info_sets_minus1 plus 1 specifies the number of extraction information sets contained in the MCTS extraction information sets SEI message. The value of num_info_sets_minus1 shall be in the range of 0 to 2047, inclusive.

The i-th extraction information set is assigned an MCTS extraction information set identifier value equal to i.

num_mcts_sets_minus1[ i ] plus 1 specifies the number of MCTS sets that share the i-th extraction information set. The value of num_mcts_sets_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

num_mcts_in_set_minus1[ i ][ j ] plus 1 specifies the number of MCTSs in the j-th MCTS set that is associated with the i-th extraction information set. The value of num_mcts_in_set_minus1[ i ][ j ] shall be in the range of 0 to 511, inclusive.

idx_of_mcts_in_set[ i ][ j ][ k ] specifies the MCTS index of the k-th MCTS in the j-th MCTS set that is associated with the i-th extraction information set. The value of idx_of_mcts_in_set[ i ][ j ][ k ] shall be in the range of 0 to 511, inclusive.

num_vps_in_info_set_minus1[ i ] plus 1 specifies the number of replacement VPSs in the i-th extraction information set. The value of num_vps_in_info_set_minus1[ i ] shall be in the range of 0 to 15, inclusive.

vps_rbsp_data_length[ i ][ j ] specifies the number of RBSP data bytes of the j-th replacement VPS in the i-th extraction information set.

num_sps_in_info_set_minus1[ i ] plus 1 specifies the number of replacement SPSs in the i-th extraction information set. The value of num_sps_in_info_set_minus1[ i ] shall be in the range of 0 to 15, inclusive.

sps_rbsp_data_length[ i ][ j ] specifies the number of RBSP data bytes of the j-th replacement SPS in the i-th extraction information set.

num_pps_in_info_set_minus1[ i ] plus 1 specifies the number of replacement PPSs in the i-th extraction information set. The value of num_pps_in_info_set_minus1[ i ] shall be in the range of 0 to 63, inclusive.

pps_nuh_temporal_id_plus1[ i ][ j ] minus1 specifies the temporal identifier of the j-th replacement PPS in the i-th extraction information set.

pps_rbsp_data_length[ i ][ j ] specifies the number of RBSP data bytes of the j-th replacement PPS in the i-th extraction information set.

mcts_alignment_bit_equal_to_zero shall be equal to 0.

vps_rbsp_data_byte[ i ][ j ][ k ] contains the k-th byte of the RBSP data of the j-th replacement VPS in the i-th extraction information set.

sps_rbsp_data_byte[ i ][ j ][ k ] contains the k-th byte of the RBSP data of the j-th replacement SPS in the i-th extraction information set.

pps_rbsp_data_byte[ i ][ j ][ k ] contains the k-th byte of the RBSP data of the j-th replacement PPS in the i-th extraction information set.

The MCTS sub-bitstream extraction process is specified as follows:

- Let a bitstream inBitstream, a target MCTS set index mctsSetIdxTarget, a target MCTS extraction information set identifier mctsEisIdTarget, and a target highest TemporalId value mctsTidTarget be the inputs to the MCTS sub-bitstream extraction process.

- The output of the MCTS sub-bitstream extraction process is a sub-bitstream outBitstream derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	The lists ausWithVps, ausWithSps. and ausWithPps are set to consist of all access units within outBitstream containing VCL NAL units with nal_unit_type equal to VPS_NUT, SPS_NUT, or PPS_NUT.

–	Remove all SEI NAL units that contain non-MCTS-nested SEI messages.

NOTE– A "smart" bitstream extractor might include appropriate non-MCTS-nested SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as MCTS-nested SEI messages in the original bitstream.

–	Remove from outBitstream all of the following NAL units:

–	VCL NAL units that contain tiles not belonging to any of the MCTSs with MCTS index equal to idx_of_mcts_in_set[ mctsEisIdTarget ][ mctsSetIdxTarget ][ k ] for each value of k in the range of 0 to num_mcts_in_set_minus1[ mctsEisIdTarget ][ mctsSetIdxTarget ], inclusive.

–	Non-VCL NAL units with nal_unit_type equal to VPS_NUT, SPS_NUT, or PPS_NUT.

–	Insert into each access unit within the list ausWithVps in outBitstream num_vps_in_info_set_minus1[ mctsEisIdTarget ] plus 1 VPS NAL units generated from the RBSP data of the list of replacement VPSs in the mctsEisIdTarget-th MCTS extraction information set. For each VPS NAL unit that is generated the nuh_layer_id is set equal to 0 and nuh_temporal_id_plus1 is set equal to 1.

–	Insert into each access unit within the list ausWithSps in outBitstream num_sps_in_info_set_minus1[ mctsEisIdTarget ] plus 1 SPS NAL units generated from the RBSP data of the list of replacement SPSs in the mctsEisIdTarget-th MCTS extraction information set. For each SPS NAL unit that is generated the nuh_layer_id is set equal to 0 and nuh_temporal_id_plus1 is set equal to 1.

–	Insert into each access unit within the list ausWithPps in outBitstream PPS NAL units generated from the RBSP data of the replacement PPSs, in the mctsEisIdTarget-th MCTS extraction information set, for which pps_nuh_temporal_id_plus1[ mctsEisIdTarget ][ j ] is less than or equal to mctsTidTarget. For each PPS NAL unit that is generated the nuh_layer_id is set equal to 0, and for the PPS NAL unit that is generated from the RBSP data of the j-th replacement PPS in the mctsEisIdTarget-th MCTS extraction information set, nuh_temporal_id_plus1 is set equal to pps_nuh_temporal_id_plus1[ mctsEisIdTarget ][ j ].

–	Remove from outBitstream all NAL units with TemporalId greater than mctsTidTarget.

- The coding tree block raster and tile scanning conversion process as specified in clause 6.5.1 is invoked with the syntax element values of the replacement SPS and PPS as inputs. The output CtbAddrRsToTs[ ctbAddrRs ] is assigned to extCtbAddrRsToTs[ ctbAddrRs ] and CtbAddrTsToRs[ ctbAddrTs ] is assigned to extCtbAddrTsToRs[ ctbAddrTs ].

–	For each remaining VCL NAL units in outBitstream, adjust the slice segment header as follows:

–	For the first VCL NAL unit within each access unit, set the value of first_slice_segment_in_pic_flag equal to 1, and set the value of slice_segment_address to be equal to 0.

–	For each remaining VCL NAL units in outBitstream, let ctbAddrRs be the value of the raster scan address of the last CTB in the previous VCL NAL unit in bitstream order within a coded picture of outBitstream, set the value of first_slice_segment_in_pic_flag equal to 0, and set the value of slice_segment_address equal to extCtbAddrTsToRs[ extCtbAddrRsToTs[ ctbAddrRs ] + 1 ].

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the MCTS sub-bitstream extraction process specified in this clause shall be a conforming bitstream.

- D.3.45	Motion-constrained tile sets extraction information nesting SEI message semantics

The motion-constrained tile sets extraction information nesting SEI message, also referred to as the MCTS nesting SEI message, provides a mechanism to carry and associate the SEI messages with bitstream subsets corresponding to one or more MCTSs. An SEI message contained in an MCTS nesting SEI message is referred to as MCTS-nested or an MCTS-nested SEI message, and an SEI message that is not contained in an MCTS nesting SEI message is referred to as a non-MCTS-nested SEI message.

In the MCTS sub-bitstream extraction process as specified in the semantics of the MCTS extraction information sets SEI message, the MCTS-nested SEI messages applicable to the MCTSs in an MCTS set in an access unit can be included in the corresponding access unit of the extracted sub-bitstream as non-MCTS-nested SEI messages.

An MCTS-nesting SEI message shall not be present for a picture unless the picture belongs to the associatedPicSet of a temporal MCTS SEI message. This temporal MCTS SEI message is referred to as the associated temporal MCTS SEI message of the MCTS-nesting SEI message.

An SEI NAL unit containing an MCTS nesting SEI message shall not contain any other SEI message that is not MCTS-nested in the MCTS nesting SEI message.

all_mcts_flag equal to 0 specifies that the list nesting_mctsid is set to consist of the MCTS indices indicated by all instances of idx_of_associated_mcts[ i ] for i from 0 to num_associated_mcts_minus1, inclusive. all_mcts_flag equal to 1 specifies that the list nesting_mctsid consists of the MCTS indices of all the MCTSs indicated by the associated temporal MCTS SEI message.

The MCTS-nested SEI messages apply to all MCTSs for which the MCTS indices are included in the list nesting_mctsid.

num_associated_mcts_minus1 plus 1 specifies the number of the following MCTS indices. The value of num_associated_mcts_minus1 shall be in the range of 0 to 511, inclusive.

idx_of_associated_mcts[ i ] indicates the MCTS index of the i-th MCTS associated with the following MCTS-nested SEI messages. The value of idx_of_associated_mcts[ i ] shall be in the range of 0 to 511, inclusive.

num_sei_messages_in_mcts_extraction_nesting_minus1 plus 1 specifies the number of the following MCTS-nested SEI messages.

mcts_nesting_zero_bit shall be equal to 0.

- In E.3.1 (VUI parameters semantics), after the paragraphs specifying the semantics of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, add the following paragraph:

When chroma_format_idc is equal to 1 and the decoded video content is intended for interpretation according to Rec. ITU-R BT.2020 or Rec. ITU-R BT.2100, chroma_loc_info_present_flag should be equal to 1, and chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field should both be equal to 2.

- In F.14.3.1 (General SEI payload semantics), in Table F.4, replace the row for the bitstream partition nesting SEI message with the following:

| Bitstream partition nesting | - Depending on the SEI messages contained in the bitstream partition nesting SEI message: each of these SEI messages has the same persistence scope as if the SEI message was not contained in the bitstream partition nesting SEI message. |
| --- | --- |

- In F.14.3.1 (General SEI payload semantics), replace the following paragraphs:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 148, inclusive, 161, 165, 167 and 168.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 148, inclusive, and 160 to 168, inclusive.

- with the following:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 154, inclusive, 161, 165, 167, and 168.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 154, inclusive, and 160 to 168, inclusive.

- In F.14.3.2.7 (Scalable nesting SEI message semantics for multi-layer extensions), replace the following phrase

It is a requirement of bitstream conformance that the following restrictions apply on nesting of SEI messages:

- with the following:

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in a scalable nesting SEI message:

- In F.14.3.4 (Inter-layer constrained tile sets SEI message semantics), replace the following sentence:

Decoders encountering an indicated value of ilcts_id[ i ] in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore (remove from the bitstream and discard) it.

- with the following:

Decoders encountering an indicated value of ilcts_id[ i ] in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

- In G.14.3.1 (General SEI payload semantics), replace the following paragraphs:

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 148, inclusive, 161, 165, 167, 168, 177, 178 and 179.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 148, inclusive, 160 to 168, inclusive, and 176 to 180, inclusive.

- with the following:

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 154, inclusive, 161, 165, 167, 168, 177, 178, and 179.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 154, inclusive, 160 to 168, inclusive, and 176 to 180, inclusive.

- In I.14.3.1 (General SEI payload semantics), replace the following paragraphs:

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 148, inclusive, 161, 165, 167, 168, 177, 178 and 179.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 148, inclusive, 160 to 168, inclusive, and 176 to 181, inclusive.

- with the following:

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 154, inclusive, 161, 165, 167, 168, 177, 178, and 179.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 154, inclusive, 160 to 168, inclusive, and 176 to 181, inclusive.

____________________