| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Torino, IT, 15–21 July 2017 | Document: JCTVC-AB0024 |
| --- | --- |

| Title: | Mapping SDR content into HDR signal containers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laborites, Inc. |  |  |

_____________________________

# Abstract

A proposed common condition in JVET is to map BT.709 SDR content into the color volume of BT.2100 PQ by a direct translation of absolute linear light values. In practice however, the look of SDR content is usually altered when displayed on HDR monitors to appear brighter, to make the content appear more as if had originated from an HDR camera, or was re-graded by a colorist to have more of an "HDR look". Otherwise, SDR content mapped into HDR containers by direct linear light mapping will usually appear too dim to viewers, as reported in surveys. It is suggested in this proposal that a more appropriate convention for the testing of SDR content with the JVET toolkit under investigation is to code SDR content in its "native" BT.709 container, as is the intention of services to continue to deliver SDR content to end consumer devices, even if all equipment in the delivery chain is capable of HDR.  SDR BT.709 content directly mapped in a HDR BT.2100 PQ HDR container utilizes only 9 out of 10 bits of precision, and thus can significantly impact coding results. The practices of mapping SDR into HLG may also differ from SDR to PQ, though there is a strong desire to have common appearance at key maximum display light levels, such as 1000 cd/m2. Version 2 of this proposal adds PowerPoint slides to the .zip archive. This document is also registered as JVET-G0054.

# Background

The direct mapping of linear light in the JVET scheme involves first converting non-linear SDR to linear display light, then non-linear PQ signal. The BT.1886 EOTF (Electric to Optic Transfer Function) serves as the usual inverse OETF (Optical to Electric Transfer Function) applied to the non-linear R'G'B' domain SDR signal to convert to linear display light. In the particular JVET scheme referenced in this contribution, the CIE xyY coordinate for each color is maintained in the mapping of SDR to HDR, as is the brightness level.  A consumer electronics (CE)  HDR display controller will in practice typically boost the input SDR content brightness by 50 to 100% to appear more like HDR content in order to meet viewer expectations. For example, 100 nits (cd/m^2) reference white level of SDR is mapped to approximately 200 nits in the HDR panel. SDR content color may also be expanded to appear more vibrant, but such a step is less important usually to viewer expectations that also are more varied in the color aspect compared to HDR look expectations. However, the CE display only knows that the content is intended to be SDR when it originates in an SDR container, such as BT.709.  The display would not know how to tell true HDR content in an HDR container from SDR content transported through the HDR container by the direct mapping method.  There are no flags in VUI or SEI to indicate direct mapping transport. The HDR display would not know that the content is not intended to appear to be as dim as the literal code levels indicate. Therefore, SDR content directly mapped into an HDR container does not provide any useful coding results, other than perhaps to serve as a hypothetical performance comparison to that of content when contained in BT.709. The SDR signal directly mapped into HDR container also utilizes less code levels (essentially only 9-bits out of 10-bits), while the BT.709 container can use all 10-bits. This can impact the rate-distortion performance by several decibels.

# Proposal

Though the author of this contribution is unaware of all the motivations behind the JVET SDR into HDR container mapping, it is suggested that if it is desired to simulate the expanded look of SDR content on HDR displays, that steps analogous to the hypothetical inverse of the EETF in section 5.4.1. in [1] or the algorithm described in [2] be considered. Note that some commercial professional equipment follows similar workflow in order to provide a common "HDR/WCG look" in studio environments with a mix of older SDR and newer HDR cameras. This is analogous to the SD to HD transition practice in broadcast studios that took place from the late 1990's to late 2000's of up-converting standard-definition sources in HD production for both budget or otherwise unavoidable reasons.

A future companion to BT.2100 and BT.2390 [1] expected to be published in late 2017 will provide simple examples for the upconversion of SDR to HDR, such as this figure for BT.709 to BT.2100 HLG.

W3C is also planning to update timed text specification [3] with a similar workflow for the application of blending SDR graphics onto HDR content.  Conversion of SDR to HLG could also be rendered to have a similar look as SDR to PQ observing the BT.2390 HLG/PQ conversion routines, described in [4].

# References

[1] BT.2390: Technical Report companion for BT.2100.

[2] Simon Thompson, "Conversion of Convention Video for Display on High Dynamic Range Televisions", SMPTE Annual Technical Conference (ATC) 2016.  DOI: 10.5594/JMI.2017.2660718

[3] Timed Text Markup Language 2 (TTML2):  https://w3c.github.io/ttml2/spec/ttml2.html - hdr-compositing [draft]

[4] JCTVC-Y0022: “Transcoding between PQ and HLG”

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.