| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Torino, IT, 15–21 July 2017 | Document: JCTVC-AB0027r2 |
| --- | --- |

| Title: | VUI: clarifying chroma sample location type |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories, Inc. |  |  |

_____________________________

# Abstract

This proposal suggests changing or providing additional drawings to Figure E-1 (location of chroma samples) of the HEVC spec. that may provide cleaner, easier to understand illustrations for some readers, for each known chroma 4:2:0 location type. It is also suggested to provide a table that defines the chroma sub-sample spatial offset with respect to the luma sample so that explicit calculation can be directly provided for use in the design of resampling filters applied in 4:2:0 to 4:4:4 conversion, or image scaling for example, rather than be left to human interpretation.  Version 2 (revision 1) of this document corrects the figure in section 2. Version 3 (revision 2) updates the offset table and provides formulae for computing the position of the chroma sample center in the overall luma picture sample grid. 4:2:0 pass fail picture test pattern examples from Stacey Spears are included in the JCTVC-AB0027-v3.zip archive under the “/CA_Pass_Fail_E subdirectory.  It is suggested that the additional figures, tables, and formulae in this proposal also be incorporated into a future update of CICP.

# Introduction

The current E-1 figure is reproduced below:

Figure E.1 – Location of chroma samples for top and bottom fields for chroma_format_idc equal to 1 (4:2:0 chroma format) as a function of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field

Figure E-1 combines all six chroma sample location types into one illustration. While the figure provides useful information on the relationship between the different chroma location types, it has been observed that readers spend considerable energy trying to interpret the figure. It is suggested that a series of specific type figures, at least for progressive scan pictures, follow E-1 to complement E-1 and provide a path for readers to ease into the concepts conveyed in E-1, and provide cases that the reader may only be interested in.

# Proposal

A mismatch has developed between ITU-R BT.2020, BT.2100 and HDMI / CTA 861 specifications, where BT.2020 & BT.2100 have specified type 2, while HDMI still mandates type 0 from MPEG-2 and AVC practices. Type 1 is legacy from JPEG and MPEG-1. This discrepancy between ITU-R and HDMI developed despite ample time for alignment between the two standards bodies organizations. Ultra HD (4K) Blu-Ray Disc BD-ROM 3.1 also mandates type 2. HEVC and AVC specification in VUI already had language to make these types distinct from one another.

To help prevent such mismatches from happening again, it is suggested that cleaner figures be provided in future editions of video specifications such as BT.2100, while a complete set of types also be included AVC and HEVC, along with formulae and tables that provide quantitate spatial offsets between luma and chroma subsamples.  An example figure showing the spatial relationship between luma samples (shown in black) and chroma 4:2:0 samples (shown in red) and the centers (shown as dots) is suggested below for the three commonly practiced types (type 0 is the default HEVC, AVC, and MPEG-2 type, and is associated coded BT.709 signals;  type 1 from MPEG-1, H.261, and JPEG-1; type 2 for BT.2020 and BT.2100 signals) :

Figure 2: spatial relationship for the three common 4:2:0 location types

Tables and formula defining the exact spatial offsets between luma and chroma samples were attempted in the semantics for the mapping of chroma samples to spherical coordinates in the omnidirectional projection indication SEI message drafts [1], but were not included in the recent ballot draft (JCTVC-AA1005) in favor of a cleaner natural language instructions to convert 4:2:0 to 4:4:4 by the usual, obvious chroma resampling process. A table listing chroma sample spatial offsets in unit luma samples is provided below:

| chroma_format_idc | ChromaLocType | HorizontalOffsetC | VerticalOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0 | +1/2 |
| 1 (4:2:0) | 1 | +1/2 | +1/2 |
| 1 (4:2:0) | 2 | 0 | 0 |
| 1 (4:2:0) | 3 | +1/2 | 0 |
| 1 (4:2:0) | 4 | 0 | +1 |
| 1 (4:2:0) | 5 | +1/2 | +1 |
| 2 (4:2:2) | – | 0 | 0 |
| 3 (4:4:4) | – | 0 | 0 |

The relationship between HorizontalOffsetC and VerticalOffsetC is shown in Figure 3 below:

Figure 3: spatial offset variables relating chroma and luma sample center locations

The relevant location of the center of the chroma sample from the chroma sample grid at coordinate [xC,yC]  in the luma picture sample grid is computed as:

xL = 2*xC + HorizontalOffsetC

yL = 2*yC + VerticalOffsetC

In filter design, HorizontalOffsetC and VerticalOffsetC would typically serve as the phase offset for each of the respective horizontal and vertical (separable) filter passes in 4:2:0 / 4:4:4 chroma resampling. For example, in the calculation of the typical sinc function [2] in the chroma down-sampler or upsampler [3], the value for phase parameter p would be the respective HorizontalOffsetC and VerticalOffsetC values used to compute the filter tap g(i) in each pass:

g(i) = sin(x) / x

where:

x = π * ( i + p ) / D

D is the down-sampling factor (usually 2.0 in 4:4:4 to 4:2:0 conversion)

and i is a tap index between [ -taps/2 , taps/2 ]

# References

[1] JCTVC-Z1005, " HEVC Additional Supplemental Enhancement Information (Draft 1)"

[2] JVT-I018, "Color format down conversion for test sequence generation".

[3] JVT-I019, "Color format up conversion for video display".

I018, "C fort

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.