| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Torino, IT, 15–21 July 2017 | Document: JCTVC- AB0031 |
| --- | --- |

| Title: | Cubemap projection support in the Omnidirectional Projection indication SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin | Tel Email: | yago.sanchez@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Currently, HEVC DAM3[1], also present in JCTVC-AA1005[2], only specifies the equirectangular projection (ERP) in the omnidirectional projection indication SEI message. JCTVC-AA0034[3] proposed to add support for Cubemap Projection (CMP). In the previous meeting, it was pointed out that the proposed contribution does not specify the packing format and that the compact CMP should be considered. In addition, in order to be aligned with the ERP projection, frame packing arrangements as for ERP has been considered. Therefore, the current contribution provides an update with the mapping for the compact CMP projection.

# Introduction

Besides ERP, CMP is another widely used projection format in virtual reality (VR) and 360 video applications; the first one is certainly ERP. Recent Google and Facebook posts [4][5] show that they are both working on some variants of CMP. Many 360 video players offer support for CMP such as CodePlex VR Player [6]. There are also streaming solutions using CMP for viewport adaptation such as CastLabs 360 adapt [7]. Established rendering frameworks such as OpenGL support CMP through dedicated texture formats.

In addition, CMP has technical advantages compared to ERP. Firstly, it does not oversample for the poles as ERP. This not only reduces the number of samples for processing, but also provides a benefit for tiled streaming approaches, as further discussed below. Secondly, picture content in each cube face look more natural than in an ERP picture, a straight line looks more like a straight line, a translational motion looks more like a translational motion, which potentially benefit video coding using today's video codecs. Indeed, in terms of BD-rate it has been shown that the performance of CMP in comparison to ERP is slightly better [8].

Although tile streaming is also possible with ERP, CMP is by design better suitable for tiled streaming. The “square” nature of the CMP projection, i.e. its 6 cube surface planes or faces, allow to easily align the tiling with the projection structure. The apparent oversampling issue towards the top and bottom boundary in ERP is largely avoided in CMP that has approx. uniform sampling in all viewing directions. Tiled streaming approaches using CMP benefit from this characteristic as the number of tiles covering a fixed field of view (FOV) is independent of viewing direction, whereas for ERP, the number of tiles may vary significantly depending on viewing direction.

Reasonable evaluation criteria for adoption of a projection different than ERP have been defined in the OMAF projection CE [10]. In a joint contribution at the last MPEG Meeting [9], multiple proponents reported fulfilment of all defined evaluation criteria. The evaluation results are summarized in the following:

- Industry support

- Better fit for tiled streaming than ERP

- Software availability for CMP

- Better compression efficiency compared to ERP (although not main argument)

- Normative text provided for mapping (see section 2)

- Removes extreme warping of the poles – more “sensible”, spatially uniform coding format

- Memory bandwidth consumption reduction for rendering (see [11])

# Proposal

Based on the reasons mentioned above, this contribution proposes to include the cubemap projection (CMP) to HEVC DAM3 as the second projection, and propose the equations below for specifying the mapping from the sample locations to sphere coordinates.

The angular coordinates  and  are defined in the ranges of [-π, π) and [- π/2, π/2], respectively.

Figure 1. Compact CMP packing (CMP3x2).

The corresponding angular coordinates (, ) for the luma sample locations, in radians, are given by the following CMP mapping equations:

atan(x)[0, π)

atan2(x)[- π/2, π/2)

Lw = viewWidth  ÷ 3

Lh = viewHeight ÷ 2

L = viewWidth * StereoWidthScale  ÷ 3

where ViewWidth * StereoWidthScale  ÷ 3 shall be equal to ViewHeight ÷ 2 * StereoHeightScale

When omni_projection_type is equal to 1 (CMP), for i equal to conf_win_left_offset to conf_win_left_offset + viewWidth − 1, inclusive, and j equal to conf_win_top_offset to conf_win_top_offset + viewHeight − 1, inclusive, the corresponding angular coordinates (ϕ, θ) for the sample locations, in degrees, are given by the following CMP mapping equations:

For j′=0… Lh – 1 and i′= 0 … Lw – 1

i′′= StereoWidthScale * i′

j′′= StereoHeightScale * j′

(Left Face)

 = -(π ÷ 2) + atan2( (i′′ − ( L - 1 ) ÷ 2) ÷(  L ÷ 2) )		                                              (1.1)

 = atan2( − ( j′′ − ( L - 1) ÷ 2) ÷ sqrt(( i′′ − ( L -1 ) ÷ 2)2 + (L ÷ 2)2 ))		                    (1.2)

For j′=0… Lh – 1 and i′= Lw … 2Lw – 1

i′′= StereoWidthScale * i′ − L

j′′= StereoHeightScale * j′

(Front Face)

 = atan2((i′′ − ( L - 1 ) ÷ 2) ÷( L  ÷ 2 )	                                                                                      (1.3)

 = atan2( − ( j′′ − ( L - 1) ÷ 2) ÷ sqrt(( i′′ − ( L - 1 ) ÷ 2)2 + (L  ÷ 2)2 ))	                                  (1.4)

For j′=0… Lh – 1 and i′= 2Lw … 3Lw – 1

i′′= StereoWidthScale * i′ − 2L

j′′= StereoHeightScale * j′

(Right Face)

 = π ÷ 2 + atan2((i′′ − ( L - 1 ) ÷ 2) ÷(L ÷ 2) )		                                                             (1.9)

 = atan2( − ( j′′ − ( L - 1) ÷ 2) ÷ sqrt(( i′′ − ( L - 1 ) ÷ 2)2 + (L ÷ 2)2 ))	                                  (1.10)

For j′= Lh … 2 Lh – 1 and i′= 0 … Lw – 1

(Bottom Face)

j′′= L – 1 − StereoWidthScale * i′

i′′= StereoHeightScale * j′ − L

 = atan(abs( i′′ − ( L - 1 ) ÷ 2) ÷ ( − ( j′′ − ( L - 1) ÷ 2)))* sign( i′′ − ( L - 1 ) ÷ 2)                       (1.11)

 = − π ÷ 2 + ( atan( sqrt(( i′′ − ( L - 1 ) ÷ 2)2 + ( j′′ − ( L - 1) ÷ 2)2 ) ÷ (  L ÷ 2) )                         (1.12)

For j′= Lh … 2 Lh – 1 and i′= Lw … 2Lw – 1

j′′= 2L – 1 − StereoWidthScale * i′

i′′= StereoHeightScale * j′ − L

(Back Face)

′= -π if sign( i′′ − ( L - 1 ) ÷ 2)>0 and π else

 = atan2((i′′ − ( L - 1 ) ÷ 2) ÷(  L ÷ 2) ) + ′			                                                 (1.13)

 = atan2( − ( j′′ − ( L - 1) ÷ 2) ÷ sqrt((i′′ − ( L -1 ) ÷ 2)2 + (L ÷ 2)2 ))                                            (1.14)

For j′= Lh … 2 Lh – 1 and i′= 2Lw … 3Lw – 1

(Top Face)

j′′= 3L – 1 − StereoWidthScale * i′

i′′= StereoHeightScale * j′ − L

 = atan(abs( i′′ − ( L - 1 ) ÷ 2) ÷ (( j′′ − ( L - 1) ÷ 2)))* sign( i′′ − ( L - 1 ) ÷ 2) 	                      (1.15)

 = π ÷ 2 −  atan( sqrt(( i′′ − ( L - 1 ) ÷ 2)2 + ( j′′ − ( L - 1) ÷ 2)2 ) ÷ (  L ÷ 2) )	                      (1.16)

 = Cos(  ) *  − Sin(  ) * 

 = Sin(  ) *  + Cos(  ) * 

where

i′ = i − SubWidthC * conf_win_left_offset

j′ = j − SubHeightC * conf_win_top_offset

 = = omni_projection_roll_center * 2−16 * π ÷ 180

# References

[1] N16881, “ISO/IEC 23008-2:201x/DAM3 Additional supplemental enhancement information”.

[2] JCTVC-AA1005, “HEVC Additional Supplemental Enhancement Information (Draft 2)”, J. Boyce, A. K. Ramasubramonian, R. Skupin, G. J. Sullivan, A. M. Tourapis.

[3] JCTVC-AA0034,” The cubemap projection”, Y. Sanchez, R. Skupin, S. Oh, S. Lee, Y.-K. Wang.

[4] https://youtube-eng.googleblog.com/2017/03/improving-vr-videos.html?m=1

[5] https://code.facebook.com/posts/637561796428084/end-to-end-optimizations-for-dynamic-streaming/

[6] CodePlex VR Player, https://vrplayer.codeplex.com/

[7] Castlabs, 360adapt, http://castlabs.com/products/360adapt/

[8] m39782,” OMAF PROJ-VE: Segmented Sphere Projection (SSP) for 360 video”

[9] m40531,” OMAF: Cubemap Projection (CMP) support for 360 video”, Yago Sanchez, Robert Skupin, Sejin Oh, Sooyeon Lee, Ye-Kui Wang.

[10] N16639,” Descriptions of Comparison Experiments for Omnidirectional Media Application Format”

[11] http://wftp3.itu.int/av-arch/jctvc-site/2017_03_AA_Hobart/JCTVC-AA_Notes_d8.doc

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).