| J oint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Torino, IT, 15–21 July 2017 | Document: JCTVC-AB0035 |
| --- | --- |

| Title: | Unified adaptive search range setting in JEM and HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yukinobu Yasugi | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

This contribution is a follow up contribution of JVET-F0044 and JCTVC-AA0043. It proposes the common adaptive search range of 96-384 for RA aiming to unify the current JEM’s 256-256 search range and HM’s 64_256 adaptive search range.

In JEM 6.0 RA condition, the suggested range of 96_384 shows 0.05 % bdrate gain with 2 % encoding time reduction.

# Introduction

Currently HM and JEM uses different search ranges, which cause additional difficulties for comparison. At the last meeting, it was agreed that harmonization would be desirable but postponed due to some CfE configuration.

It is noted that number of experiments in JVET-F0044[1] and JCTVC-AA0043[2] have confirmed that the proposed 96-384 adaptive search range would be one of the best option for HM and JEM in terms of runtime-performance balance and impact for CTC.

# Experimental results

|  | Over HM-16.6-JEM-6.0 (sharp) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.09% | 0.01% | 96% | 101% |
| Class A2 | -0.16% | -0.25% | -0.37% | 98% | 100% |
| Class B | -0.01% | 0.00% | 0.04% | 99% | 101% |
| Class C | 0.00% | 0.03% | -0.07% | 98% | 102% |
| Class D | -0.02% | -0.16% | 0.03% | 99% | 103% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0.05% | -0.09% | -0.07% | 98% | 101% |
| Class F (optional) | 0.16% | 0.27% | 0.27% | 98% | 104% |

s

# Conclusion

To unify HM and JEM’s test configuration, adaptive search range of 96-384 is proposed.

# References

[1] Y. Yasugi, T. Ikai, “AHG5: Performance evaluation of Adaptive Search Range on JEM-5.0.1”, JVET-F0044, JVET 6th Meeting: Hobart, AU, 31 March – 7 April 2017

[2] T. Ikai, Y. Yasugi, “Unified adaptive search range setting in HM and JEM”, JCTVC-AA0043, JCT-VC 27th Meeting: Hobart, AU, 31 March – 7 April 2017

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).