| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Torino, IT, 15–21 July 2017 | Document: JCTVC-AB0044 |
| --- | --- |

| Title: | Proposed cubemap projection information SEI message draft |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin | Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes draft text for changes to the High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 | ISO/IEC 23008-2) to specify an additional supplemental enhancement information (SEI) message for cubemap projection information.

- Replace D.2.1 with the following:

- D.2.1	General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| picture_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 56 ) |  |
| green_metadata( payloadsize )/* specified in ISO/IEC 23001-11 */ |  |
| else if( payloadType = = 128 ) |  |
| structure_of_pictures_info( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| active_parameter_sets( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 131 ) |  |
| temporal_sub_layer_zero_index( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 134 ) |  |
| region_refresh_info( payloadSize ) |  |
| else if( payloadType = = 135 ) |  |
| no_display( payloadSize ) |  |
| else if( payloadType = = 136 ) |  |
| time_code( payloadSize ) |  |
| else if( payloadType = = 137 ) |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 138 ) |  |
| segmented_rect_frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 139 ) |  |
| temporal_motion_constrained_tile_sets( payloadSize ) |  |
| else if( payloadType = = 140 ) |  |
| chroma_resampling_filter_hint( payloadSize ) |  |
| else if( payloadType = = 141 ) |  |
| knee_function_info( payloadSize ) |  |
| else if( payloadType = = 142 ) |  |
| colour_remapping_info( payloadSize ) |  |
| else if( payloadType = = 143 ) |  |
| deinterlaced_field_identification( payloadSize ) |  |
| else if( payloadType = = 144 ) |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 146 ) |  |
| coded_region_completion( payloadSize ) |  |
| else if( payloadType = = 147 ) |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) |  |
| cmp( payloadSize ) |  |
| else if( payloadType = = 151 ) |  |
| cube_viewport( payloadSize ) |  |
| else if( payloadType = = 152 ) |  |
| regional_nesting( payloadSize ) |  |
| else if( payloadType = = 153 ) |  |
| mcts_extraction_info_sets( payloadSize ) |  |
| else if( payloadType = = 154 ) |  |
| mcts_extraction_info_nesting( payloadSize ) |  |
| else if( payloadType = = 155 ) |  |
| cubemap_projection_information( payloadSize ) [Ed. (RS): Maybe the projection related SEI messages should be assigned a continuous payloadType range?] |  |
| else if( payloadType = = 160 ) |  |
| layers_not_present( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 161 ) |  |
| inter_layer_constrained_tile_sets( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 162 ) |  |
| bsp_nesting( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 163 ) |  |
| bsp_initial_arrival_time( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 164 ) |  |
| sub_bitstream_property( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 165 ) |  |
| alpha_channel_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 166 ) |  |
| overlay_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 167 ) |  |
| temporal_mv_prediction_constraints( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 168 ) |  |
| frame_field_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 176 ) |  |
| three_dimensional_reference_displays_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 177 ) |  |
| depth_representation_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 178 ) |  |
| multiview_scene_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 179 ) |  |
| multiview_acquisition_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 180 ) |  |
| multiview_view_position( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 181 ) |  |
| alternative_depth_info( payloadSize ) /* specified in Annex I */ |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 146 ) |  |
| coded_region_completion( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

- Renumber clause D.2.46 (Reserved SEI message syntax) as D.2.47.

- Add clause D.2.46, as follows:

- D.2.46	Cubemap projection information SEI message syntax

| cubemap_projection( payloadSize ) { | Descriptor |
| --- | --- |
| cmp_cancel_flag |  |
| if( !cmp_cancel_flag ) { | u(1) |
| cmp_persistence_flag |  |
| cmp_rotation_flag | u(1) |
| cmp_padding_flag | u(1) |
| cmp_reserved_zero_4bits | u(1) |
| if( cmp_padding_flag = = 1 ) { | u(4) |
| cmp_padding_type |  |
| cmp_padding_sample_range_minus1 | u(8) |
| } | u(8) |
| if( cmp_rotation_flag = = 1 ) { |  |
| cmp_yaw_rotation |  |
| cmp_pitch_rotation | i(32) |
| cmp_roll_rotation | i(32) |
| } | i(32) |
| } |  |
| } |  |

- In D.3.1, replace the following paragraphs:

The list SingleLayerSeiList is set to consist of the payloadType values 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, and 134 to 154, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, and 134 to 154, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, and 135 to 154, inclusive.

- with the following:

The list SingleLayerSeiList is set to consist of the payloadType values 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, and 134 to 155, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, and 134 to 155, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, and 135 to 155, inclusive.

- In D.3.1, in Table D.1, append the following rows to the end of the table:

| Cubemap projection information | Specified by the syntax of the SEI message |
| --- | --- |

- Renumber clause D.3.46 (Reserved SEI message semantics) as D.3.47.

- Add subclause D.3.46, as follows:

- D.3.46	Cubemap projection information SEI message semantics

The cubemap projection information SEI message provides information to enable remapping of the colour samples of the output decoded pictures onto a cubic geometry in a three dimensional Cartesian coordinate system (X, Y, Z) and a spherical coordinate space in angular coordinates (ϕ, θ) for use in cubemap video applications for which the viewing perspective is from the origin looking outward.

When a cubemap projection information SEI message is present for any picture of a CLVS of a particular layer, a cubemap projection information SEI message shall be present for the first picture of the CLVS and no Equirectangular projection information SEI message shall be present for any picture of the CLVS. [Ed. (RS): How should we deal with potential future projection SEIs]

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no cubemap projection information SEI messages applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, and the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is not equal to 3, 4, or 5, or the value of quincunx_sampling_flag of the frame packing arrangement SEI message is not equal to 0, a cubemap projection information SEI message with cmp_cancel_flag equal to 0 shall not be present that applies to the picture. Decoders shall ignore cubemap projection information SEI messages when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, and the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is not equal to 3, 4, or 5, or the value of quincunx_sampling_flag of the frame packing arrangement SEI message is not equal to 0.

When a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, a cubemap projection information SEI message with cmp_information_cancel_flag equal to 0 shall not be present that applies to the picture. Decoders shall ignore cubemap projection information SEI messages when a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture.

cmp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous cubemap projection information SEI message in output order. cmp_cancel_flag equal to 0 indicates that cubemap projection information follows.

cmp_persistence_flag specifies the persistence of the cubemap projection information SEI message for the current layer.

cmp_persistence_flag equal to 0 specifies that the cubemap projection information SEI message applies to the current decoded picture only.

Let picA be the current picture. cmp_persistence_flag equal to 1 specifies that the cubemap projection information SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a cubemap projection information SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

cmp_rotation_flag equal to 1 indicates a rotation of the global XYZ coordinate system according to cmp_yaw_rotation, cmp_pitch_rotation and cmp_roll_rotation. cmp_rotation_flag equal to 0 indicates that no rotation is applied to the global XYZ coordinate system.

cmp_reserved_zero_4bits when present, shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for cmp_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of cmp_reserved_zero_4bits.

cmp_padding_type equal to 0 indicates that the sample values within the padding area are unspecified. cmp_padding_type equal to 1 indicates that the value of each sample inside the padding area is equal to the value of the spatially nearest sample outside the padding area in the adjacent padded face. cmp_padding_type equal to 2 indicates that the values of the samples inside the padding area are equal to the values of the samples that are projected to the face neighboring the padded face. cmp_padding_type equal to 3 indicates that the values of samples inside the padding area are derived through projection to the extended planar surface of the face adjacent to the padding area. The value of cmp_padding_type shall be in the range of 0 to 3. [Ed. (RS): Possibly more detail and alignment required. OMAF does not specify cmp_padding_type equal to 2. Current cmp_padding_type cannot indicate quality transitions such as OMAF region-wise packing syntax gb_type[i] equal to 2.]

cmp_padding_sample_range_minus1 plus 1 indicates the thickness of the padding areas in terms of samples. The value of cmp_padding_sample_range_minus1 shall be in the range of 0 to 255. [Ed. (RS): Enforced alignment with possible block boundaries to be considered?]

cmp_yaw_rotation specifies the value of the yaw rotation angle that is applied to the cropped output picture, where the value is derived by performing a clockwise rotation around the up vector (i.e. the Y axis) when viewed from the origin looking upwards (anticlockwise when looking toward the origin), in units of 2−16 degrees. The value of cmp_yaw_rotation shall be in the range of −180 * 216 (i.e., −11796480) to 180 * 216 − 1 (i.e., 11796479), inclusive. When not present, the value of cmp_yaw_rotation is inferred to be equal to 0.

cmp_pitch_rotation specifies the value of the pitch rotation angle that is applied to the cropped output picture, where the value is derived by performing a clockwise rotation around the cmp_yaw_rotation (i.e. the Z axis after applying cmp_yaw_rotation) when viewed from the origin looking outwards (anticlockwise when looking toward the origin), i.e., the right vector after yaw rotation, in units of 2−16 degrees. The value of cmp_pitch_rotation shall be in the range of −90 * 216 (i.e., −5898240) to 90 * 216 (i.e., 5898240), inclusive. When not present, the value of cmp_pitch_rotation is inferred to be equal to 0.

cmp_roll_rotation specifies the value of the roll rotation angle that is applied to the cropped output picture, where the value is derived by performing a clockwise rotation around the pitch_rotation (i.e. the X axis after applying cmp_pitch_rotation) when viewed from the origin looking outwards (anticlockwise when looking toward the origin), i.e., the forward vector after yaw and pitch rotation, in units of 2−16 degrees. The value of cmp_roll_rotation shall be in the range of −180 * 216 (i.e., −11796480) to 180 * 216 − 1 (i.e., 11796479), inclusive. When not present, the value of cmp_roll_rotation is inferred to be equal to 0.

When cmp_yaw_rotation, cmp_pitch_rotation or cmp_roll_rotation are not equal to 0, the Cartesian coordinate system XYZ used to describe the cubic geometry is rotated with respect to a global world coordinate system X′Y′Z′ as described in the following. Each sample point (X, Y, Z) is rotated to (X′, Y′, Z′) by application of the following matrix operations:

(D-1)

yaw = cmp_yaw_rotation * π ÷ 180		(D-2)

pitch = cmp_pitch_rotation * π ÷ 180		(D-3)

roll = cmp_roll_rotation* π ÷ 180		(D-4)

(D-5)

(D-6)

(D-7)

NOTE – Cubemap video content may be rotated with respect to the correct global spatial orientation before encoding in order to enhance coding efficiency or move projection geometry edges into content areas that are less susceptible to coding artefacts from discontinuities. In this case, this rotation must be compensated for when cubemap video content is rendered after decoding to maintain correct global spatial orientation.

[Ed. (RS): Above equations are from a ERP SEI message draft and need revision and alignment.]

For purposes of interpretation of chroma samples, the input to the indicated remapping process is the set of decoded sample values after applying an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format). This (unspecified) upsampling process should account for the relative positioning relationship between the luma and chroma samples as indicated by chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, when present.

The variables StereoWidthScale and StereoHeightScale are derived as follows:

–	If a cubemap projection information SEI message with cmp_information_cancel_flag equal to 0 applies to the current picture, and a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 also applies to the current picture, the variables StereoWidthScale and StereoHeightScale are determined according to Table D.X.

–	Otherwise, the values of StereoWidthScale and StereoHeightScale are both set equal to 1.

Table D.X – Definition of StereoWidthScale and StereoHeightScale from frame_packing_arrangement_type

| frame_packing_arrangement_type | StereoWidthScale | StereoHeightScale |
| --- | --- | --- |
| 3 | 2 | 1 |
| 4 | 1 | 2 |
| 5 | 1 | 1 |

Luma dimensions of the cropped output picture or constituent frame, as shown in Figure D.X are derived as follows:

viewWidth = ( pic_width_in_luma_samples –
	SubWidthC * ( conf_win_right_offset + conf_win_left_offset ) ) / StereoWidthScale	(D-8)

viewHeight = ( pic_height_in_luma_samples –
	SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset ) ) / StereoHeightScale	(D-9)

Unpadded luma dimensions of the cropped output picture or constituent frame are derived as follows:

unpaddedViewWidth = viewWidth – ( 2 * cmp_padding_sample_range_minus1 + 1 )	(D-10)

unpaddedViewHeight = viewHeight – ( 4 * ( cmp_padding_sample_range_minus1 + 1 )	(D-11)

Positions are identified for one or two constituent frames as follows:

–	If StereoWidthScale is equal to 2, there are two constituent frames in the picture, for which both constituent frames have viewTopOffset equal to 0, the first constituent frame has viewLeftOffset equal to 0, and the second constituent frame has viewLeftOffset equal to viewWidth.

–	Otherwise, if StereoHeightScale is equal to 2, there are two constituent frames in the picture, for which both constituent frames have viewLeftOffset equal to 0, the first constituent frame has viewTopOffset equal to 0, and the second constituent frame has viewTopOffset equal to viewHeight.

–	Otherwise, there is only one constituent frame in the picture, for which viewLeftOffset is equal to 0 and viewTopOffset is equal to 0.

Figure D.X – Cube face arrangement and relationship of the index variables i and j to the cropped output of decoded picture luma samples when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 and frame_packing_arrangement_type equal to 3 or 4 is not present that applies to the picture

The cubemap projection information SEI message assumes the face packing arrangement shown in Figure D.X, with face indices and labels as specified in Table D.Y. The cubemap faces are labelled with face index, PX, PY, PZ, NX, NY, NZ with “P” denoting positive axis direction and “N” denoting negative axis direction and face index. Sample dimensions of each unpadded face are quadratic and faces in the bottom row are rotated clockwise by 90° with respect to the faces in the top row. Position of label and a blue marker wihtin each face corresponds the position of labels and markers in Figure D.Z .

Table D.Y. Face index and label of cubemap.

| Face index | Face label | Description |
| --- | --- | --- |
| 0 | PX | Front cubemap face with positive X axis value |
| 1 | NX | Back cubemap face with negative X axis value |
| 2 | PY | Top cubemap face with positive Y axis value |
| 3 | NY | Bottom cubemap face with negative Y axis value |
| 4 | PZ | Right cubemap face with positive Z axis value |
| 5 | NZ | Left cubemap face with negative Z axis value |

The Cartesian coordinate system is defined so that the origin is in the center of the cube. The X axis points towards the the front cubemap face plane , the Z axis points towards the right cubemap face plane and the Y axis points towards the top cubemap face plane as illustrated in Figure D.Z. The spherical coordinates are defined so that ϕ is the azimuth (longitude, increasing eastward) and θ is the elevation (latitude, increasing northward) as depicted in Figure D.Z.

Figure D.Z. Coordinate definitions for the cubemap projection.

The following equations are used for derivation of the cropped decoded output picture sample positions in a Cartesian and spherical coordinate system.

Lw = unpaddedViewWidth ÷ 3		(D-12)

Lh = unpaddedViewHeight ÷ 2 		(D-13)

L = max(Lw * StereoWidthScale, Lh* StereoHeightScale) 	(D-14)

Cw=L/Lw			(D-15)

Ch=L/Lh			(D-16)

i′ = i −( SubWidthC * conf_win_left_offset + viewLeftOffset + cube_padding_sample_range ) 	(D-17)

j′ = j −( SubWidthC * conf_win_top_offset + viewTopOffset + cube_padding_sample_range )	(D-18)

where

Lw = Lh			(D-19)

This following provides equations for identifying Cartesian coordinates (X, Y, Z) on a cubic geometry surface in three-dimensional space for positions corresponding to the positions (i, j) of the luma samples of the cropped decoded output picture.

i′′ = −( 2 * ( i′ % Lw ) ÷ Lw ) + 1		(D-20)

j′′ = −( 2 * ( j′ % Lh ) ÷ Lh ) + 1		(D-21)

The coordinates (X, Y, Z) are derived using Table D.Z given the position (i, j) on a given face with face index f and given the equations

Table D.Z. (X, Y, Z) derivation given (i, j) and face index

| Face index | X | Y | Z |
| --- | --- | --- | --- |
| 0 | 1.0 | j′′ | i′′ |
| 1 | −1.0 | −i′′ | j′′ |
| 2 | i′′ | 1.0 | j′′ |
| 3 | i′′ | −1.0 | j′′ |
| 4 | −i′′ | j′′ | 1.0 |
| 5 | i′′ | j′′ | −1.0 |

This following provides equations for identifying angular coordinates (, ) in radians in a spherical geometry for positions corresponding to the positions (i, j) of the luma samples of the unpadded, cropped output picture, as given by the following equations:

Atan(x)[ 0, π ) 		(D-22)

Atan2(x)[ −π/2, π/2 )		(D-23)

For the cubemap faces in the top row, the following applies:

For i equal to conf_win_left_offset + cmp_padding_sample_range to conf_win_left_offset + unpaddedViewWidth − 1, inclusive, and j equal to conf_win_top_offset + cmp_padding_sample_range to conf_win_top_offset + cmp_padding_sample_range + ( unpaddedViewHeight ÷ 2 ) − 1, inclusive, the corresponding angular coordinates ( ϕ, θ ) for the sample locations, in degrees, are given below.

For the samples depicting the right cubemap face with face index 4, i.e. for j′ equal to 0 to Lh – 1 and i′ equal to 0 to Lw – 1, inclusive, the following applies:

i′′= StereoWidthScale * Cw * i′		(D-24)

j′′= StereoHeightScale * Ch * j′		(D-25)

 = π ÷ 2 + Atan2(−(i′′ −( L − 1 ) ÷ 2) ÷(L ÷ 2 ))	(D-26)

 = Atan2( −( j′′ −( L − 1) ÷ 2) ÷ Sqrt(( i′′ −( L − 1 ) ÷ 2)2 + (L ÷ 2)2 ))	(D-27)

For the samples depicting the front cubemap face with face index 0, i.e. for j′ equal to 0 to Lh – 1 and i′ equal to Lw to 2 * Lw – 1, inclusive, the following applies:

i′′= StereoWidthScale * Cw * i′ − L		(D-28)

j′′= StereoHeightScale * Ch * j′		(D-29)

 = Atan2(−(i′′ −( L − 1 ) ÷ 2 ) ÷( L ÷ 2 )		(D-30)

 = Atan2( −( j′′ −( L − 1) ÷ 2 ) ÷ Sqrt(( i′′ −( L − 1 ) ÷ 2)2 + (L ÷ 2 )2 ))	(D-31)

For the samples depicting the left cubemap face with face index 5, i.e. for j′ equal to 0 to Lh – 1 and i′ equal to 2 * Lw to 3 * Lw – 1, inclusive, the following applies:

i′′= StereoWidthScale * Cw * i′ − 2 * L		(D-32)

j′′= StereoHeightScale * Ch * j′		(D-33)

 = Atan2( −(i′′ −( L − 1 ) ÷ 2) ÷( L ÷ 2 )) −( π ÷ 2 )	(D-34)

 = Atan2( −( j′′ −( L − 1 ) ÷ 2 ) ÷ Sqrt(( i′′ −( L −1 ) ÷ 2)2 + ( L ÷ 2 )2 ))	(D-35)

For the cubemap faces in the bottom row, the following applies:

For i equal to conf_win_left_offset + cmp_padding_sample_range to conf_win_left_offset + cmp_padding_sample_range + unpaddedViewWidth − 1, inclusive, and j equal to conf_win_top_offset + ( 3 * cmp_padding_sample_range ) + ( unpaddedViewHeight ÷ 2 ) to conf_win_top_offset + ( 3 * cmp_padding_sample_range + unpaddedViewHeight − 1, inclusive, the corresponding angular coordinates ( ϕ, θ ) for the sample locations, in degrees, are given below.

For the samples depicting the bottom cubemap face with face index 3, i.e. for j′ equal to Lh to 2 * Lh – 1 and i′ equal to 0 to Lw – 1, inclusive, the following applies:

j′′= L − 1 − StereoWidthScale * Cw * i′		(D-36)

i′′= StereoHeightScale * Ch * j′ − L		(D-37)

 = −Sign( i′′ −( L − 1 ) ÷ 2 ) * Atan( Abs( i′′ −( L − 1 ) ÷ 2 ) ÷ ( −( j′′ −( L − 1) ÷ 2 )))	(D-38)

 = − π ÷ 2 + ( Atan( Sqrt(( i′′ −( L − 1 ) ÷ 2 )2 + ( j′′ −( L − 1) ÷ 2 )2 ) ÷ ( L ÷ 2 ))	(D-39)

For the samples depicting the back cubemap face with face index 1, i.e. for j′ equal to Lh to 2 * Lh – 1 and i′ equal to Lw to 2 * Lw – 1, inclusive, the following applies:

j′′= 2L – 1 − StereoWidthScale * Cw * i′		(D-40)

i′′= StereoHeightScale * Ch * j′ − L		(D-41)

′= Sign( i′′ −( L − 1 ) ÷ 2 )>0 ? π : −π		(D-42)

 = Atan2( −(i′′ −( L − 1 ) ÷ 2 ) ÷( L ÷ 2 ) ) + ′	(D-43)

 = Atan2( −( j′′ −( L − 1) ÷ 2 ) ÷ Sqrt(( i′′ −( L −1 ) ÷ 2 )2 + (L ÷ 2 )2 ))	(D-44)

For the samples depicting the top cubemap face with face index 2, i.e. for j′ equal to Lh to 2 * Lh – 1 and i′ equal to 2 * Lw to 3 * Lw – 1, inclusive, the following applies:

j′′= 3 * L – 1 − StereoWidthScale * Cw * i′		(D-45)

i′′= StereoHeightScale * Ch * j′ − L		(D-46)

 = −Sign( i′′ −( L − 1 ) ÷ 2 ) * Atan( Abs( i′′ −( L − 1 ) ÷ 2 ) ÷ (( j′′ −( L − 1) ÷ 2 )))	(D-47)

 = π ÷ 2 − Atan( Sqrt(( i′′ −( L − 1 ) ÷ 2 )2 + ( j′′ −( L − 1) ÷ 2 )2 ) ÷ ( L ÷ 2 ) )	(D-48)

and π is Archimedes' constant 3.141 592 653 589 793...

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).