| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–24 Oct. 2017 | Document: JCTVC-AC0027 |
| --- | --- |

| Title: | Centralized Texture Depth Packing SEI Message for HEVC and AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC – Revision of Output Document JCTVC-AA1008 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang (NCKU, Taiwan) Guan-Cheng Chen We-Jong Yang | Tel: Email: | +886-917727713 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University, Taiwan |  |  |

_____________________________

# Abstract

This document is a revision of Output Document JCTVC-AA1008 [1]. The purpose of the proposed “Centralized Texture Depth Packing SEI message for HEVC and AVC” is to deliver one texture view plus one depth map packed into the samples of a monoscopic (texture only) stream for HEVC and AVC. In this document, “large_depth_type” flag is removed for simplicity and “color_packing_type” flag is added to support some existed broadcasting systems that YCbCr components are not accessible. Accordingly, a new adjusted RGB method for color depth packing is proposed and evaluated. Figures and texts are revised for better presentations.

- Centralized texture depth packing SEI message syntax

- Replace specification of in clause 7.4.4 with the following:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages nor centralized texture depth packing SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages present in the CVS.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages.

In subclause D.2.1 (General SEI message syntax), add the following:

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture_depth_packing( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers of the respective specification.

Add a new subclause D.2.X “centralized_texture_depth_packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text of the respective standard.

D.2.X	Centralized texture depth packing (CTDP) SEI message syntax

| centralized_texture_depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture_depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture_depth_packing_cancel_flag ) { |  |
| ctdp_packing_type | u(3) |
| color_packing_type | u(1) |
| baseline_dist_flag | u(1) |
| focal_length_flag | u(1) |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( baseline_dist_flag ) |  |
| view_syn_rep_info_element( BdistanceSign, BdistanceExp, BdistanceMantissa, BdistanceManLen ) |  |
| if( focal_length_flag ) |  |
| view_syn_rep_info_element( FlengthSign, FlengthExp, FlengthMantissa, FlengthManLen ) |  |
| if( z_near_flag ) |  |
| view_syn_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| view_syn_rep_info_element ( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| view_syn_rep_info_element ( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| view_syn_rep_info_element ( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| centralized_texture_depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |
| } |  |

In Table F.4, add the persistence scope for the centralized_texture_depth_packing SEI message as follows:

| - SEI message | - Persistence scope |
| --- | --- |
| centralized texture depth packing | - Specified by the syntax of the SEI message |

Add a new subclause D.3.X “centralized texture depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following subclause D.3.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.3.X Centralized texture depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains a specific packing arrangement of texture and depth samples associated with one view. This information can be used by a process applied after decoding to appropriately interpret the samples and process them appropriately for display of 3D video or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture_depth_packing_cancel_flag equal to 1 indicates that the centralized texture_depth packing (CTDP) arrangement SEI message cancels the persistence of any previous centralized texture depth packing SEI message in output order. centralized_texture_depth_packing_cancel_flag equal to 0 indicates that centralized texture depth packing is applicable.

ctdp_packing_type indicates the intended interpretation of the constituent frames as specified in Table D-X1.

Table D-X1 – Definition of ctdp_packing_type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the frame of the CTDP-TB type with an arrangement of top colored depth (CDT), texture (T), and bottom colored depth (CDB) regions from top to bottom, as illustrated in Figure D-X1. |
| 1 | Indicates the frame of the CTDP-LR type with an arrangement of left colored depth (CDL), texture (T), and right colored depth (CDR) regions from left to right, as illustrated in Figure D-X2. |

Values of ctdp_packing_type that do not appear in Table D-X1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture depth packing SEI messages that contain reserved values of ctdph_packing type. Subsequently, .

color_packing_type equal to 1 indicates the depth map is packed into the colored depth map in YCbCr color components. color_packing_type equal to 0 indicates the depth map is packed into the colored depth map in RGB color components.

When ctdp_packing_type is not equal to 0 or 1, color_packing_type is ignored and reserved for future use by ITU-T | ISO/IEC.

The interpretations of ctdp_packing_type and color_packing_type how the CTDP frame to retrieve colored depth regions and the texture region and how to recover them to the original full-resolution depth map and texture frame are described as follows.

It is assumed that the original full-resolution texture frame and depth map are both of size W x H, where W and H are the width and height expressing pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

For ctdp_packing_type equal to 0, the packed frame from top to bottom is horizontally divided into top colored depth (CDT), texture (T), and bottom colored depth (CDB) regions. With the width of W in units of frame luma samples, the height of top colored depth (CDT) and bottom colored depth (CDB) regions, called the half depth height, in units of frame luma samples is expressed by

HHD = (H/36)  2,                 	 			(D-X1)

Hence, with the width of W, the height of the texture (T) region in units of frame luma samples becomes:

HT = H  2HHD.		     (D-X2)

As illustrated in Figure D-X1, the CTDP-TB packed frame, by referring HHD defined in (D-X1) and HT defined in (D-X2), is divided into CDT, T and CDB regions from top to bottom. The height of CDT and CDB regions is HHD and the height of T region is HT in units of frame luma samples while the width of CDT, T and CDB regions is W in units of frame luma samples.

If H is not divisible to 36, the actually packing depth region will be smaller such that the number of active horizontal lines of packing depth region becomes (H/36)36. To achieve best packing performance, the removed depth lines are used to carry the reserved texture lines. In other words, the top and bottom of the depth region are removed while the corresponding top and bottom of the texture regions are reserved for direct packing. The half number of the reserved horizontal lines for non-divisible H with respect to 36, is given as

HHR = (H–(H/36)×36) / 2.                                 (D-X3)

If H is not divisible to 36, hence, the texture (T) region is further interpreted to three portions: top reserved texture (TRT), texture (T), and bottom reserved texture (TRB) from top to bottom as shown in Figure D-X1, which the texture (T) and depth (D) regions still keep the same correspondence as the condition that H is divisible to 36.

Figure D-X1 Arrangement of the CTDP-TB frame and its depacking steps (ctdp_packing type = 0)

For ctdp_packing_type equal to 1, the packed frame from left to right is vertically divided into left colored depth (CDL), texture (T), and right colored depth (CDR) regions. With the height of H in units of frame luma samples, the width of left and right colored depth (CDL and CDR) regions, called the half depth width, in units of frame luma samples is expressed by

WHD = (W/48)  2,                 	 			(D-X4)

Hence, with the height of H, the width of the texture (T) region in units of frame luma samples becomes:

WT = W  2WHD.	   	  (D-X5)

As illustrated in Figure D-X2, the CTDP-LR packed frame referring to WHD defined in (D-X4) and WT defined in (D-X5) is divided into CDL, T, and CDR regions from left to right. The width of CDL and CDR regions is WHD and the width of T region is WT in units of frame luma samples while the height of CDL, T and CDR regions is H in units of frame luma samples.

If W is not divisible to 48, the actually packing depth region will be smaller such that the number of active vertical lines of packing depth regions becomes (W/48)48. To achieve best packing performance, the removed depth lines are used to carry the reserved texture lines. In other words, the top and bottom of the depth region are removed while the corresponding top and bottom of the texture regions are reserved for direct packing. The half number of the reserved horizontal lines for non-divisible H with respect to 48, is given as

WHR = (W–(W/48)×48) / 2.                                (D-X6)

If W is not divisible to 48, hence, the texture (T) region is further interpreted to three portions: left reserved texture (TRL), texture (T), and right reserved texture (TRR) from left to right as shown in Figure D-X2, which the texture (T) and depth (D) regions still keep the same correspondence as the condition that H is the divisible to 48.

Figure D-X2 Arrangement of the CTDP-LR frame and its depacking steps (ctdp_packing_type = 1)

After splitting the colored depth and texture regions, the paired top and bottom or left and right colored depth regions should be first merged together to become one color depth (CD) region as follows.

–	If ctdp_packing_type is equal to 0, as shown in Figures D-X1, the paired top and bottom colored depth regions, which are denoted as CDT and CDB in either YCbCr or RGB representation, are vertically flipped and merged to obtain one colored depth (CD) region. If the paired top and bottom colored depth regions are respectively denoted as CDT and CDB, the colored depth after flipping and merging processes is expressed by:

for( x = 0; x < W; x++ )

for( y = 0; y < HHD; y++ ) {

CD[x][y] = CDT[x][HHD y 1]

CD[x][HHD + y] = CDB[x][HHD  y 1]

}

where HHD is the height of top and bottom colored depth regions in units of frame luma samples. After vertically flipping and merging processes as shown in Figure D-X1, the height of the colored depth (CD) regions, becomes 2HHD, while the width of the colored depth is W in units of frame luma samples.

–	If ctdp_packing_type is equal to 1, as shown in Figures D-X2, the paired left and right colored depth regions, which are CDL and CDR in either YCbCr or RGB representation are horizontally flipped and merged to obtain one colored depth (CD) regions. If the paired left and right colored depth regions are respectively denoted as CDL and CDR, the colored depth after flipping and merging processes is expressed by:

for( x = 0; x < WHD; x++ )

for( y = 0; y < H; y++ ) {

CD[x][y] =CDL[WHD x 1][y]

CD[ x + WHD ][ y ] = CDR[WHDx 1][ y ]

}

where WHD is the width of left and right colored depth regions in units of frame luma samples. After horizontal flipping and merging processes as shown in Figure D-X2, the width of the colored depth (CD) region becomes 2WHD, while the height of the depth is H in units of frame luma samples.

After flipping and merging, the colored depth, CD region is depacked to depth (D) region. The locations of 4 color subpixels of colored depth (CD) and their associated YCbCr or RGB subpixels with respect to the positions of 12 depth pixels are specified as shown in Figure D-X3. If ctdp_packing_type is equal to 0, the height of depth regions, after horizontal placement of subpixels, is tripled to become 6HHD, as shown in the left side of Figure X-D3. while the width of the depth is W in units of frame luma samples. If ctdp_packing_type is equal to 1, the width of depth regions, after horizontal placement of subpixels, is tripled to become 6WHD, as shown in the right side of Figure X-D3. while the height of the depth is H in units of frame luma samples.

Figure D-X3 Positions of 4 pixels of colored depth, 12 YCbCr components, 12 RGB subpixels with respect

to the positions of 12 depacked depth pixels for ctdp_packing_type=0 and tdp_packing_type = 1

If the video coding system adopts YCbCr 4:4:4 chroma format, the subpixel values of the colored depth (CD) region to the pixel values of depth (D) region becomes obviously simple and straightforward. However, for YCbCr 4:2:2 and YCbCr 4:2:0 chroma formats, the restorations of depth values should be further specified as follows.

For YCbCr 4:2:0 chroma format, in every 4 color pixels of color depth, there are actually 6 YCbCr components with 4 Y’s, 1 Cb’s and 1 Cr’s components involved where {Cbi+1,j，Cri+1,j}, {Cbi,j+1, Cri,j+1} and {Cbi+1,j+1, Cri+1,j+1} marked in black color are all replaced by {Cbi,j，Cri,j}. Accordingly, there are 6 depth pixels selected to be fully depacked from 6 YCbCr components. To achieve better packing performances, these 6 fully-depacked depth pixels are arranged in quincunx positions as Figure D-X4. It is noted that the depacking RGB subpixel values without any transformations or arrangements will be changed to R’G’B’ subpixel values, which might not well match with the original RGB values.

Figure D-X4 Colored depth with YCbCr 4:2:0 chroma format and 12 R’G’B’ subpixels with respect to

the positions of 6 fully-depacked depth pixels (white color) and 6 unknown depth pixels (black color)

If color_packing_type is equal to 1, the colored depth packing and depacking processes can be directly operated with YCbCr components, which are used to carry 6 fully depacked depth pixels marked in white color with proper mapping functions. The incorrect R’G’B’ subpixel values due to incorrect chroma components can be ignored.

If color_packing_type is equal to 0, the colored depth packing and depacking can only be operated with RGB components, which are needed to be adjusted in the packing side such that 6 fully-depacked depth pixels marked in white color can be correctly recovered in the depacking side.

As to the unknown 6 depth pixels can be interpolated from the full-depacked depth pixels with proper interpolations. The detailed mapping functions for YCbCr to depth values (color_packing_type = 1) and adjustments for RGB components (color_packing_type = 0) are described as follows.

If color_packing_type is equal to 1, it indicates that both CTDP depth packing and depth depacking computations can directly access YCbCr values. The depth values will be depacked from Y, Cb, and Cr components in the following steps. The depth value in range of {0, 255} depacked from Y’s component in range of {16, 235} through a linear mapping function is expressed by,

D = (19078* (Y − 16) + 213) / 214,	(D-X7)

while the depth value depacked from the chroma component Cx, which could be Cr or Cb in a piecewise linear DPCM fashion is given as,

f(Cx, D1, D2)=    (D-X8)

The chroma component, Cx in the range of {16. 240} is first transformed to the DPCM residual in the range of {255, 255} by the piecewise linear mapping function defined in (D-X8) while the DPCM residual adds the DPCM prediction, which is the average of D1 and D2, to recover the depth value. In (D-X8), it consists of three linear DPCM decoding functions. The first linear function with the slope of 2.3364 maps Cx in the range of {16, 123} to the DPCM residual in the range of {5, 255}. The second linear function with the slope of 1.000 maps Cx in the range of {123. 133} to the DPCM residual in the range of {5, 5}. The third linear function with the slope of 2.3364 maps Cx in the range of {133, 240} to the residual in range of {5, 255}.

- If ctdp_packing_type is equal to 0, the left side of Figure D-X4 shows the horizontally-placed positions of each 4 sets of Y, Cb and Cr components, which only retain {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1, Cbi,j, Cri,j} in YCbCr 4:2:0 chroma format for i = 0, 2, 4,…, (2HHD2), j = 0, 2, 4….，(W2) to represent depth information. After this vertically-placed arrangement of YCbCr 4:2:0 components, the height of the depth, D region is expanded 3 times of the colored depth (CD) region to become 6HHD, while W, the width of the depth region is unchanged. To recover the depth values, as shown in the left side of Figure D-X4, four luma components {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1} in range of {16, 235} are first respectively mapped to 4 selected depth samples {D3i,j, D3i1,j+1, D3i+4, j, D3i+5, j+1}, in the range of {0, 255} expressed as:

D3i, j = (19078* ((Yi, j − 16) + 213) / 214,	(D-X9)

D3i1, j+1 = (19078 * (Yi, j+1 − 16) + 213) / 214,	(D-X10)

D3i4, j = (19078 * (Yi+1, j − 16) + 213) / 214,	(D-X11)

D3i+5, j+1 = (19078 * (Yi+1, j+1 − 16) + 213) / 214.	(D-X12)

By using 4 depth values depacked from Y components as predictions, two chroma components, {Cbi,j,Cri,j} are respectively transformed to two depth values, {D3i+2,j, D3i+3, j+1} by the piecewise linear DPCM function defined in (D-X8) as:

D3i+2, j = f(Cbi, j, D3i, j, D3i+4,j), 	(D-X13)

D3i+3, j+1 = f(Cri, j, D3i+1,j+1, D3i+5, j+1) 	(D-X14)

for i = 0, 2, 4…, (2HHD2), j = 0, 2, 4…, (W2). The above 6 equations stated from (D-X9) to (D-X14) can recover six depth values in quincunx positions while the reminding six unknown depth values (black color), D3i+1,j, D3i+3,j, D3i+5,j, D3i,j+1, D3i+2,j+1 and D3i+4,j+1, shown in the left side of Figure D-X4, can be interpolated from those known depth values.

- If ctdp_packing_type is equal to 1, the right side of Figure D-X4 shows the vertically-placed positions of each 4 sets of Y, Cb and Cr components, which only retain {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1, Cbi,j, Cri,j} in YCbCr 4:2:0 chroma format for i = 0, 2, 4,…, (H2), j = 0, 2, 4….，(2WHD2) to represent depth information. After this horizontally-placed arrangement of YCbCr 4:2:0 components, the width of the depth (D) region is expanded 3 times of the colored depth (CD) region to become 6WHD, while H, the height of the depth region is unchanged.To recover the depth values, as shown in the right side of Figure D-X4, four luma components {Yi,j, Yi+1,j, Yi+1,j, Yi+1,j+1} in range of {16, 235} are first respectively mapped to four selected depth samples {Di,3j, Di1,3j+1, Di,3 j+4, Di+1, 3j+5}, in the range of {0, 255} expressed as:

Di,3j = (19078* ((Yi,j − 16) + 213) / 214,	(D-X15)

Di1,3j+1 = (19078 * (Yi+1, j − 16) + 213) / 214,	(D-X16)

Di, 3j4 = (19078 * (Yi, j+1 − 16) + 213) / 214,	(D-X17)

Di+1, 3j+5 = (19078 * (Yi+1, j+1 − 16) + 213) / 214,	(D-X18)

By using 4 depth values depacked from Y components as predictions, two chroma components, {Cbi,j, Cri,j} are respectively transformed to two depth values, {Di,3j+2, Di+1,3j+3} by the piecewise linear DPCM  function defined in (D-X8) as:

Di, 3j+2 = f(Cbi, j, Di, 3j, Di, 3j+4), 	(D-X20)

Di+1, 3j+3 = f(Cri, j, Di+1, 3j+1, Di+1, 3j+5), 	(D-X21)

for i = 0, 2, 4, …, (H2), j = 0, 2, 4, …, (2WHD2). The above 6 equations stated from (D-X15) to (D-X21) can recover six depth values in quincunx positions while the reminding six unknown depth values (black color), Di,3j+1, Di,3j+3, Di, 3j+5, Di+1, 3j, Di+1, 3j+2 and Di+1,3j+4 shown in the right side of Figure D-X4, can be interpolated from those known depth values.

If color_packing_type is equal to 0, it indicates that both CTDP depth packing and depth depacking processes can involve RGB values. It is noted that the RGB components after packing process will be transformed to YCbCr components in the encoder side while the RGB components in the decoder side are transformed from YCbCr components. To keep 6 depth values correct, the discussion of RGB adjustments are described in the follows.

- If ctdp_packing_type is equal to 0, in the packing side, the depth values without any adjustments will be vertically packed into RGB components to become colored depth as:

Ri,j=D3i,j, Gi,j=D3i+1,j, and Bi,j=D3i+2,j 	(D-X22)

for i = 0, 1, 2, …, 2HHD, j = 0, 1, 2,…, W. As shown in the left side of Figure D-X3, only the first set of YCbCr components with {Yi,j, Cbi,j, Cri,j} can correctly recover the first set of RGB components {Ri,j, Gi,j, Bi,j}. Without any adjustments, the reminding 3 sets of YCbCr components with {Yi,j+1, Cbi,j, Cri,j}, {Yi+1,j, Cbi,j, Cri,j}, and {Yi+1,j+1, Cbi,j, Cri,j} will not be possible to recover correct all RGB components since they use different chroma components. In order to correctly depacked 6 depth values in quincunx positions in the depacking side, the four sets of RGB components should be adjusted in the packing side as follows.

In the packing side, the first set, {Ri,j,Gi,j,Bi,j} are adjusted as

Ri,j'=Ri,j,      {Gi,j'=Ri+1,j+1,    and     {Bi,j'=Bi,j,                          (D-X23)

and further transformed into YCbCr color space to become {Yi,j',Cbi,j',Cri,j'}, which can be fully recovered to {D3i,j,D3i+3,j+1,D3i+2,j} in the depacking side. By using RGB to YCbCr transform, the chroma components of the first set, Cbi,j' and Cri,j' are respectively expressed by

Cbi,j'= 0.1482Ri,j – 0.2910Ri+1,j+1 + 0.4392Bi,j+128,                (D-X24)

and

Cri,j' = 0.4392Ri,j – 0.3678Ri+1,j+1 – 0.0714Bi,j+128.                 (D-X25)

For the second set YCbCr components, {Yi,j+1,Cbi,j',Cri,j'}, the recovered RGB can be expressed by

Ri,j+1'Gi.j+1'Bi,j+1'=1.16440.00011.59601.1644−0.3917−0.81301.16442.01730.0001+Yi,j+1−16Cbi,j'−128Cri,j'−128,                 (D-X26)

If Yi,j+1 is computed from {Ri,j+1, Gi,j+1, Gi,j+1} by the regular RGB to YCbCr transform, the all recovered RGB components stated in (D-X26) will be different from the original RGB values. Instead of the original Yi,j+1, we suggest to adjust Yi,j+1' to make one of three subpixels, for example, the green subpixel to be correct as Gi,j+1'=Gi,j+1. Thus, from (D-X26), the Y-adjustment equality is given as

Gi,j+1=1.1644×(Yi,j+1'16)0.3917×(Cbi,j'1280.8130×(Cri,j'128),               (D-X27)

By substituting (D-X24) and (D-X25) into (D-27), Yi,j+1'can be solved easily. For the givenYi,j+1', there are many ways to adjust {Ri,j+1',Gi,j+1',Bi,j+1'}. For data smoothing point of view, we suggest to make the RGB components the same as:

Ri,j+1'=Gi,j+1'=Bi,j+1'=Gi,j+1+0.2990Ri,j0.4130Ri+1,j+1+0.1140Bi,j.              (D-X28)

Similarly, for the third set, {Yi+1,j, Cbi,j, Cri,j}, we also choose to keep green subpixel correct asGi+1,j'=Gi+1,j. The corresponding RGB components should are adjusted to:

Ri+1,j'=Gi+1,j'=Bi+1,j'=Gi+1,j+0.2990Ri,j0.4130Ri+1,j+1+0.1140Bi,j.              (D-X29)

As to the fourth set, {Yi+1,j+1, Cbi,j, Cri,j}, we choose to keep blue subpixel correct asBi+1,j+1'=Bi+1,j+1, the RGB components should are adjusted to:

Ri+1,j+1'=Gi+1,j+1'=Bi+1,j+1'=Bi+1,j+1 + 0.2990Ri,j+0.5870Ri+1,j+10.8860Bi,j.       (D-X30)

It is noted that the above RGB adjustments are processed in the packing side.

With the above RGB adjustments in the packing side, the procedures of the colored depth to depth transformation become straightforward. After retrieving all RGB components from the video decoder, six depth values in vertical placement of quincunx positions as shown in the left side of Figure D-X4 are depacked as

D3i,j=Ri,j';                                                   	(D-X31)

D3i+2,j=Bi,j';                                                   	(D-X32)

D3i+3,j+1=Gi,j';                                                	(D-X33)

D3i+4,j=Gi+1,j';                                                 	(D-X34)

D3i+1,j+1=Gi,j+1';                                                	(D-X35)

D3i+5,j+1=Bi+1,j+1';                                               	(D-X36)

for i=0, 2, 4…, (2HHD2), j=0, 2, 4…, (W2). The above 6 color depacking equations, stated from (D-X24) to (D-X29), recover six depth values while the reminding six unknown depth values (black color) as shown in the left side of Figure D-X4, D3i+1,j, D3i+3,j, D3i+5,j, D3i,j+1, D3i+2,j+1, D3i+4,j+1 can be interpolated from those known depth values.

- If ctdp_packing_type is equal to 1, in the packing side, the depth values without any adjustments will be vertically packed into RGB components to become the colored depth as:

Ri,j=Di,3j, Gi,j=Di,j+1, and Bi,j=Di,3j+2 	(D-X37)

for i = 0, 1, 2, …, H, j = 0, 1, 2,…, 2WHD. For YCbCr 4:2:0 chroma format in 4 sets of color pixels, as shown in right side of Figure D-X3, only the first set of YCbCr components with {Yi,j, Cbi,j, Cri,j} can correctly recover the first set of RGB components {Ri,j, Gi,j, Bi,j}. Without any adjustments, the reminding 3 sets of YCbCr components with {Yi,j+1, Cbi,j, Cri,j}, {Yi+1,j, Cbi,j, Cri,j}, and {Yi+1,j+1, Cbi,j, Cri,j} will not be possible to recover all RGB components since they use different chroma components. In order to correctly depacked 6 depth values in quincunx positions in the depacking side, the four sets of RGB components should be adjusted in the packing side as follows.

In the packing side, the first set, {Ri,j,Gi,j,Bi,j} are adjusted as (D-X23) and further transformed into YCbCr color space to become {Yi,j',Cbi,j',Cri,j'}, which can be fully recovered to {Di,3j,Di+1,3j+3,Di,3j+2} in the depacking side. By using RGB to YCbCr transform, the chroma components of the first set, Cbi,j' and Cri,j' are respectively expressed by (D-X24) and (D-X25). Similarly, for the second set, to keep the green subpixel, Gi,j+1 be recovered in the depacking side, the RGB components should be adjusted to {Ri,j+1',Gi,j+1',Bi,j+1'} as stated in (D-X28) in the packing side. For the third set, to keep the green subpixel, Gi+1,j be recovered in the depacking side, the RGB components should be adjusted to {Ri+1,j',Gi+1,j',Bi+1,j'} as shown in (D-X29) in the packing side. For the four set, to keep the blue subpixel, Bi+1,j+1 be covered in the depacking side, the RGB components should be adjusted to {Ri+1,j',Gi+1,j',Bi+1,j'} as defined in (D-X30) in the packing side. With the above RGB adjustments in the packing side, the procedure of the colored depth to depth transformation becomes straightforward. After retrieving all RGB components from the video decoder, the 6 depth values in the horizontal placement of quincunx positions as shown in the right side of Figure D-X4 are depacked as

Di,3j=Ri,j';                                                 	(D-X38)

Di,j3+2=Bi,j';                                                  	(D-X39)

Di+1,3j+3=Gi,j';                                               	(D-X40)

Di,3j+4=Gi,j+1';                                               	(D-X41)

Di+1,3j+1=Gi+1,j';                                              	(D-X42)

Di+1,3j+5=Bi+1,j+1';                                             	(D-X43)

for i=0, 2, 4…, (H2), j=0, 2, 4…, (2WHD2). The above 6 color depacking equations, stated from (D-X38) to (D-X43), recover six depth values {Di,3j, Di,3j+2, Di,3j+4, Di+1,3j+1, Di+1,3j+3, Di+1,3j+5} while the reminding six unknown depth values (black color) as shown in the right side of Figure D-X4, {Di,3j+1, Di,3j+3, Di,3j+5, Di+1,3j, Di+1,3j+2, Di+1,3j+4} can be interpolated from those known depth values.

For YCbCr 4:2:2 chroma format, in every 4 color pixels of color depth, there are with 4 Y’s, 2 Cb’s and 2 Cr’s components. By applying the similar color packing concept for YCbCr 4:2:0, there are 8 depth pixels can be fully depacked. To achieve better packing performances, these 8 fully-depacked depth pixels are arranged in quincunx positions with two extra corner positions as Figure D-X5, where {Cbi,j+1，Cri,j+1} and {Cbi+1,j+1, Cri+1,j+1} marked in black color are respectively replaced by {Cbi,j，Cri,j} and {Cbi+1,j, Cri+1,j}. The depacking RGB subpixel values without any transformations or arrangements will be changed to R’G’B’ subpixel values, which might not perfectly match with the original RGB values.

Figure D-X5 Colored depth with YCbCr 4:2:2 chroma format and 12 R’G’B’ subpixels with respect to

the positions of 8 fully-depacked depth pixels (white color) and 4 unknown depth pixels (black color)

If color_packing_type is equal to 1, the colored depth packing and depacking process can be directly operated with YCbCr components, which are used to carry 8 fully depacked depth pixels marked in white color in proper mapping functions. If color_packing_type is equal to 0, the colored depth packing and depacking can only be operated with RGB components, which are needed to be adjusted such that the 8 fully-depacked depth pixels marked in white color can be correctly recovered. The unknown 4 depth pixels can be interpolated from 8 (or nearby sets of 8) full-depacked depth pixels with proper interpolations. The detailed mapping functions for YCbCr to depth values (color_packing_type = 1) and adjustments for RGB components (color_packing_type = 0) for additional two depth values are similar to the above processes.

If color_packing_type is equal to 1, the colored depth depacking process can be directly transforms 8 YCbCr components, {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1, Cbi,j, Cri,j, Cbi+1,j, Cri+1,j } of each 4 sets of Y, Cb and Cr components to 8 depth pixels with proper mapping functions.

If ctdp_packing_type is equal to 0, the vertical positions of 8 YCbCr components to recover 8 depth values, as shown in the left side of Figure D-X5, similar to YCbCr 4:2:0 chroma component, four luma components {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1} in range of {16, 235} are first mapped to four selected depth samples {D3i,j, D3i1,j+1, D3i+4, j, D3i+5, j+1}, in the range of {0, 255} stated as (D-X9), (D-X10), (D-X11), (D-X12), respectively.

By using similar concept as YCbCr 4:2:0 chroma components, four chroma components, {Cbi,j, Cri,j, Cbi+1,j, Cri+1,j } are respectively transformed to two depth values, {D3i,j+1, D3i+2,j, D3i+3, j+1 D3i+5,j} by the piecewise linear DPCM function defined in (D-X8) as:

D3i, j+1 = f(Cbi, j, D3i, j, D3i+1,j+1), 	(D-X44)

D3i+2, j = f(Cri, j, D3i,j, D3i+4, j) 	(D-X45)

D3i+3, j+1 = f(Cbi+1, j, D3i+1, j, D3i+5,j+1), 	(D-X46)

D3i+5, j = f(Cri+1, j, D3i+1,j+1, D3i+5, j+1) 	(D-X47)

for i = 0, 2, 4…, (2HHD2), j = 0, 2, 4…, (W2). Thus, we can recover 8 depth values in quincunx positions while the reminding 4 unknown depth values (black color), D3i+1,j, D3i+3,j, D3i+2,j+1, D3i+4,j+1, shown in the left side of Figure D-X5, can be interpolated from those known depth values.

- If ctdp_packing_type is equal to 1, the horizontal positions of 8 YCbCr components to recover 8 depth values, as shown in the right side of Figure D-X5, similar to YCbCr 4:2:0 chroma component, four luma components {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1} in range of {16, 235} are first mapped to four selected depth samples {Di,3j, Di1,3j+1, Di,3 j+4, Di+1, 3j+5}, in the range of {0, 255} stated as (D-X15), (D-X16), (D-X17), (D-X18), respectively.

By using similar concept as YCbCr 4:2:0 chroma components, four chroma components, {Cbi,j, Cri,j, Cbi+1,j, Cri+1,j } are respectively transformed to two depth values, {Di+1,3j, Di,3j+2, Di+1,3j, Di+1,3j+3} by the piecewise linear DPCM function defined in (D-X8) as:

Di+1, 3j = f(Cbi, j, Di,3j, Di+1,3j+1),                                            (D-X44)

Di, 3j+2 = f(Cri, j, Di,3j, Di, 3j+4), 	(D-X45)

Di+1, 3j = f(Cbi+1, j, Di, 3j, Di+1,3j+1), 	(D-X46)

Di+1, 3j+3 = f(Cri+1, j, Di+1,3j+1, Di+1, 3j+5) 	(D-X47)

for i = 0, 2, 4, …, (H2), j = 0, 2, 4, …, (2WHD2). Thus, we can recover 8 depth values in quincunx positions while the reminding 4 unknown depth values (black color), Di,3j+1, Di,3j+3, Di+1,3j+3, Di+1,3j+4, shown in the right side of Figure D-X5, can be interpolated from those known depth values.

If color_packing_type is equal to 0, it indicates that both CTDP depth packing and depth depacking computations will involve RGB values. It is noted that the RGB components after packing process will be transformed to YCbCr components in the encoder side while the RGB components in the decoder side are transformed from YCbCr components. The discussion of RGB adjustments are described in the follows.

- If ctdp_packing_type is equal to 0, the depth values, similar to YCbCr 4:2:0, the vertically-packed RGB components are adjusted in the packing side. In depacking side, eight depth values in vertical placement of quincunx positions as shown in the left side of Figure D-X5 are depacked as

D3i,j=Ri,j';                                                   	(D-X48)

D3i+2,j=Bi,j';                                                   	(D-X49)

D3i+1,j+1=Gi,j';                                                	(D-X50)

D3i,j+1=Ri,j+1';                                                	(D-X51)

D3i+3,j+1=Ri+1,j';                                                	(D-X52)

D3i+4,j=Gi+1,j';                                              	(D-X53)

D3i+5,j=Bi+1,j';                                                  	(D-X54)

D3i+5,j=Bi+1,j+1';                                                  	(D-X55)

for i=0, 2, 4…, (2HHD2), j=0, 2, 4…, (W2). The above 8 color depacking equations, stated from (D-X48) to (D-X55), recover 8 depth values while the reminding 4 unknown depth values (black color) as shown in the left side of Figure D-X5, D3i+1,j, D3i+3,j, D3i+2,j+1, D3i+4,j+1 can be interpolated from those known depth values.

- If ctdp_packing_type is equal to 1, the depth values, similar to YCbCr 4:2:0, the horizontally-packed RGB components are adjusted in the packing side. In depacking side, eight depth values in vertical placement of quincunx positions as shown in the right side of Figure D-X5 are depacked as

Di,3j=Ri,j';                                                 	(D-X56)

Di,j3+2=Bi,j';                                                  	(D-X57)

Di,3j+4=Gi,j';                                                	(D-X58)

Di+1,3j=Ri+1,j';                                              	(D-X59)

Di+1,3j+1=Gi+1,j';                                              	(D-X60)

Di+1,3j+2=Bi+1,j';                                             	(D-X61)

Di,3j+5=Bi,j+1';                                               	(D-X62)

Di+1,3j+5=Bi+1,j+1';                                               	(D-X63)

for i=0, 2, 4…, (H2), j=0, 2, 4…, (2WHD2). The above 8 color depacking equations, stated from (D-X38) to (D-X43), recover 8 depth values while the reminding 4 unknown depth values (black color) as shown in the right side of Figure D-X5, {Di,3j+1, Di,3j+3, Di+1,3j+2, Di+1,3j+4} can be interpolated from those known depth values.

After the color depacking procedures, the colored depth (CD) region has been converted to gray depth (D) region. The colored depth (CD) region with the height of 2HHD.is vertically magnified to 3 times to become the gray depth (D) region with the height of 6HHD for ctdp_packing_type equal to 0. The colored depth (CD) region with the width of 2WHD is horizontally magnified to 3 times to become the depth (D) region with the width of 6WHD for ctdp_packing_type equal to 1. After the depacking procedures, depth region and texture region should be further upsampled and processed to construct the recovered texture and the recovered depth regions, which are both with the original resolution of H × W in units of frame luma samples, as follows.

Figure D-X6 The line-up pixels of depacked texture and depacked depth regions for ctdp_packing_type = 0

- For ctdp_packing_type equal to 0, after the above depacking procedures, the texture (T) and depth (D) regions are with sizes of W×HT and W×6HHD, respectively. The relationship of line-up pixels of depth (D) region and texture (T) region is shown in Figure D-X6. The constructions of the recovered texture and the recovered depth regions are described in the follows.

- If H is divisible to 36, it gives HHR = 0 as stated in (D-X3). In Figure D-X1, the texture reserved regions (marked with yellow color) become vanished. It is easy to verify that 6HHD= H/3 and HT = 8H/9. To recover the original resolution, the depth (D) region is vertically upsampled by a factor of 3:1 and the texture is vertically upsampled by a factor of 9: 8.

As shown in Figure D-X6, every 8 horizontal lines of texture (T) region are repetitively lined up with every 3 horizontal lines of depth (D) region. The texture pixels, Ti,j (blue color) after 9:8 vertical upsampling obtain the recovered texture pixels,Ti,j(yellow color). The depth pixels, Di,j (white color) are fully-depacked pixels obtained from color components while those in gray color represent the interpolated values during the color depacking process. Finally,Di,jmarked in dotted gray color are the vertically-interpolated depth values after 3:1 upsampling process.

- If H is not divisible to 36, it gives HHR0 as stated in (D-X3). In this case, the texture (T) region accompanies with two additional top and bottom reserved texture regions with the size of HHR x W. Thus, as shown in Figure D-X7, the texture (T) region from top to bottom is further interpreted to three portions, top reserved texture (TRT), texture (T), and bottom reserved texture (TRB) regions, which are with the heights of HHR, 16HHD and HHR, respectively. The horizontal line-up phases of texture (T) and depth (D) regions are shown in Figure D-X6.

To recover the original resolution of texture region, as shown in D-X7, the recovered texture region consists of top reserved texture (TRT), recovered texture (T) and bottom reserved texture (TRB) regions from top to bottom. To recover the original resolution of depth region, the recovered depth region consists of top extrapolated depth (DET), recovered depth (D) and bottom extrapolated depth (DEB) regions from top to bottom, where extrapolated depth should be computed from the recovered depth region by extrapolation methods.

Figure D-X7 Recovery of texture and depth regions by upsampling with direct copies of reserved texture regions and extrapolation of removed depth regions to the original resolution for ctdp_packing_type = 0  if H is not divisible to 36

- For ctdp_packing_type equal to 1, after the above depacking procedures, the texture (T) and depth (D) region are with the sizes of WT×H and 6WHD×H, respectively. The relationship of line-up pixels of depth (D) region and texture (T) region is shown in Figure D-X8. The constructions of the recovered texture and the recovered depth regions are described in the follows.

- If W is divisible to 48, it gives WHR = 0 as stated in (D-X6). In Figure D-X2, the texture reserved regions (marked with yellow color) become vanished. It is easy to verify that 6WHD= W/4 and WT = 11W/12. To recover the original resolution, the depth (D) region is horizontally upsampled by a factor of 4:1 and the texture is horizontally upsampled by a factor of 12:11.

As shown in Figure D-X8, every 11 vertical lines of texture (T) region are repetitively lined up with every 4 vertical lines of depth (D) region, The texture pixels, Ti,j (blue color) after 12:11 horizontal upsampling obtain the recovered texture pixels,Ti,j(yellow color). The depth pixels, Di,j (white color) are the fully-depacked pixel obtained from color components while those in gray color represent the interpolated values during the color depacking process. Finally,Di,jmarked in dotted gray color are the horizontally-interpolated values after 4:1 upsampling process.

Figure D-X8 The line-up pixels of depacked texture and depacked depth regions for ctdp_packing_type = 1

- If W is not divisible to 48, it gives WHR0 as stated in (D-X6). In this case, the texture (T) region accompanies with two additional left and right reserved texture regions with the size of H x WHR. Thus, as shown in Figure D-X9, the texture (T) region from left to right is further interpreted to three portions, left reserved texture (TRL), texture (T), and right reserved texture (TRR) regions, which are with the widths of WHR , 22WHD and WHR, respectively. The vertical line-up phases of texture (T) and depth (D) regions are shown in Figure D-X8.

To recover the original resolution of texture region, as shown in D-X9, the recovered texture region consists of left reserved texture (TRL), recovered texture (T) and right reserved texture (TRR) regions from left to right. To recover the original resolution of depth region, the recovered depth region consists of left extrapolated depth (DEL), recovered depth (D) and right extrapolated depth (DER) regions from left to right, where extrapolated depth should be computed from the recovered depth region by extrapolation methods.

Figure D-X9 Recovery of texture and depth regions by upsampling with direct copies of reserved texture regions and extrapolation of removed depth regions to the original resolution

for ctdp_packing_type = 1 if W is not divisible to 48.

baseline_dist_flag equal to 0 specifies that the syntax elements specifying the baseline distance are not present in the syntax structure. baseline_dist_flag equal to 1 specifies that the syntax elements specifying the baseline distance value are present in the syntax structure.

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure. focal_length _flag equal to 1 specifies that the syntax elements specifying the focal length value are present in the syntax structure.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table D-X2. In Table D-X2, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to (1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table D-X2 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[i]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

The variables in the x column of Table D-X3 are derived from the respective variables in the s, e, n and v columns of Table D-X3 as follows:

- If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s* 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table D-X3 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| Bdistance | BdistanceSign | BdistanceExp | BdistanceMantissa | BdistanceManLen |
| Flength | FlengthSign | FlengthExp | FlengthMantissa | FlengthManLen |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture.

The units for the ZNear, ZFar and Bdistance when present, are identical but unspecified.

When depth_representation_type is equal to 0 or 2, the disparity D can further be obtained from the value of Z, Bdistance, and Flength.

D = Bdistance * Flength ÷ Z								(D-X19)

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_‌representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first(0, 0) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {

pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev1 = depth_nonlinear_representation_model[ k ]

pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev2 = depth_nonlinear_representation_model[ k + 1 ] (G-10)

x1 = pos1 − dev1

y1 = pos1 + dev1

x2 = pos2 − dev2

y2 = pos2 + dev2

for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )

DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )

}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture depth packing arrangement SEI message.

centralized_texture_depth_packing_persistence_flag equal to 0 specifies that the centralized texture depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture_depth_packing_persistence_flag equal to 1 specifies that the centralized texture depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins;

–	The bitstream ends;

–	A frame in an access unit containing a centralized texture depth packing SEI message with the same.

- D.4.X	View synthesis representation information element syntax

| view_syn_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the view synthesis representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

# Performance evaluations of YCbCr and RGB color packing methods

Comparing to [1], in this document, the color packing in RGB components with some adjustements is proposed for the cases that the packing/depacking processes cannot access YCbCr components directly. The performance comparisions of YCbCr and RGB components should be evaluated by simulations,

Without any video coding systems, the performances of the CTDP formats with one-view and one-depth, where the colored depth is packed with YCbCr color components and RGB subpixels in YCbCr4:2:0 chroma format. In the simulations, we adopt Shark and UndoDancer sequences for performance evaluations of uncoded CTDP videos for CTDP-TB and CTDB-LR formats. Table 2.1 and Table 2.1 show the PSNR and SSIM performances of depacked texture, depth, and their synthesized virtual views generated by VSRS rendering system for CTDP-TB and CTDP-LR formats, respectively.

Table 2.1. Objective performances of YCbCr and RGB color packing for uncoded CTDP-TB format (depth:1/9, texture:8/9)

| Performances | Depacked Texture and Depth |  |  |  |  |  | Synthesized Virtual View Generated by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Depacked Texture |  |  | Depacked Depth |  |  |  |  |  |
|  | Original YCbCr | Direct RGB | Adjusted RGB | Original YCbCr | Direct RGB | Adjusted RGB | Original YCbCr | Direct RGB | Adjusted RGB |
| PSNR | 46.1485 | 47.0919 |  | 44.1021 | 40.8486 | 44.6246 | 38.8875 | 37.1904 | 38.0283 |
| SSIM | 0.9938 | 0.9938 |  | 0.9918 | 0.9890 | 0.9943 | 0.9867 | 0.9737 | 0.9754 |

Table 2.2. Objective performances of YCbCr and RGB color packing for uncoded CTDP-LR format ( depth:1/12, texture:11/12)

| Performances | Depacked Texture and Depth |  |  |  |  |  | Synthesized Virtual View Generated by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Depacked Texture |  |  | Depacked Depth |  |  |  |  |  |
|  | Emulated YCbCr | Direct RGB | Adjusted RGB | Emulated YCbCr | Direct RGB | Adjusted RGB | Emulated YCbCr | Direct RGB | Adjusted RGB |
| PSNR | 49.2137 | 49.2443 |  | 39.8881 | 35.9820 | 40.4315 | 38.2884 | 36.0823 | 37.4933 |
| SSIM | 0.9958 | 0.9955 |  | 0.9873 | 0.9802 | 0.9905 | 0.9882 | 0.9737 | 0.9768 |

From the above simulations, the packing performances of depth and texture by using the colored depth with RGB adjustment method is higher than those with the YCbCr mapping method. We found that the colored depth in YCbCr still performs better than the other color packing methods in virtual view syntheses. For some existing TV broadcasting systems, if the access of YCbCr components becomes impossible, we could pack the depth to the colored depth in RGB subpixels, which are adjusted in the packing side. Without RGB adjustments, after color transformation and YCbCr 4:2:0, the direct RGB color depth packing method performs the worst. The newly-proposed RGB adjustment method can greatly improve the performances in colored depth packing while the colored depth cannot be performed in the YCbCr components.

For the coding comparisons, the CTDP-TB and CTDP-LR formats are encoded by HEVC HM13.0 and compared to 3D-HEVC HTM8.0. The QP setting for both HM13.0 and HTM8.0 are 27, 32, 37, and 42. Two CG 3D video test sequences, Shark and UndoDancer sequences are used. With respect to 3D-HEVC, Tables 2.3 and 2.4 respectively show the BDBR and BDPSNR performances of texture and depth for CTDP-TB format while Tables 2.5 and 2.6 respectively exhibit the BDBR and BDPSNR performances of texture and depth for CTDP-LR format. The results show that the adjusted RGB method performs the best coding performances with HEVC codec. As to the synthesized virtual view generated by VSRS, the color packing with emulated YCbCr method shows the best performance.

Table 2.3. Average BDBR (%) results for CTDP-TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  | Depacked Depth |  |  | Virtual View by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB |
| ra | 13.6254 | 9.6406 | 11.8079 | -30.6426 | -40.8299 | -41.9352 | 25.7829 | 27.0899 | 28.5642 |
| ldp | 8.2266 | 4.6375 | 7.0231 | -53.8872 | -62.9424 | -61.2222 | 13.9024 | 16.1361 | 16.9534 |
| ai | 6.8931 | 3.9400 | 5.4887 | -19.7578 | -20.9339 | -30.0731 | 29.2954 | 35.9331 | 34.6573 |
| ave | 9.5817 | 6.0727 | 8.1065 | -34.7625 | -41.5687 | -44.4102 | 22.9936 | 26.3864 | 26.7250 |

Table 2,4. Average BDPSNR (dB) results for CTDP-TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  | Depacked Depth |  |  | Virtual View by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB |
| ra | -0.4247 | -0.3044 | -0.3737 | 1.3941 | 1.7552 | 2.1227 | -0.6789 | -0.7336 | -0.7511 |
| ldp | -0.2707 | -0.1585 | -0.2344 | 3.6932 | 5.0084 | 4.4448 | -0.4051 | -0.4917 | -0.4940 |
| ai | -0.2557 | -0.1489 | -0.2088 | 0.8799 | 0.5620 | 1.5138 | -0.8694 | -1.0608 | -1.0012 |
| ave | -0.3170 | -0.2039 | -0.2723 | 1.9902 | 2.4419 | 2.6937 | -0.6511 | -0.7620 | -0.7487 |

Table 2.5 Average BDBR (%) results for CTDP-LR format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  | Depacked Depth |  |  | Virtual View by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB |
| ra | 11.7019 | 7.9095 | 10.3264 | 26.5047 | 16.5991 | 6.3530 | 67.8342 | 61.9436 | 64.9970 |
| ldp | 6.9614 | 3.4178 | 5.9485 | -30.2637 | -36.2061 | -40.3581 | 42.2946 | 39.2266 | 40.5865 |
| ai | 3.7328 | 0.9848 | 2.8620 | 27.8123 | 30.3497 | 11.1768 | 69.7477 | 73.1888 | 68.4575 |
| ave | 7.4653 | 4.1040 | 6.3790 | 8.0178 | 3.5809 | -7.6094 | 59.9588 | 58.1196 | 58.0137 |

Table 2.6. Average BDPSNR (dB) results for CTDP-LR format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  | Depacked Depth |  |  | Virtual View by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB | Emulated YUV | Direct RGB | Adjusted RGB |
| ra | -0.3560 | -0.2415 | -0.3184 | -1.0712 | -1.2349 | -0.3254 | -1.3992 | -1.3615 | -1.3630 |
| ldp | -0.2231 | -0.1122 | -0.1935 | 1.3046 | 0.9894 | 2.0469 | -1.0077 | -0.9979 | -0.9781 |
| ai | -0.1370 | -0.0344 | -0.1072 | -1.6007 | -2.4781 | -0.8882 | -1.6165 | -1.7321 | -1.6039 |
| ave | -0.2387 | -0.1293 | -0.2064 | -0.4558 | -0.9079 | 0.2778 | -1.3411 | -1.3638 | -1.3150 |

# Patent rights declaration(s)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

[1] J.-F. Yang and G. J. Sullivan, “Conceptual study of potential centralized texture depth packing SEI message for HEVC and AVC”, Joint Collaborative Team on Video Coding of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, the 27th Meeting: Hobart, AU, Output Document:. JCTVC-AA1006, April 2017.