| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–25 Oct. 2017 | Document: JCTVC-AC0031 |
| --- | --- |

| Title: | On static regions for region-wise packing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin Y. Sanchez | Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports asserted issues for the current region-wise packing SEI message design in JCTVC-AB1005 which affect viewport-dependent streaming schemes. It is asserted that the current design prohibits usage of more effective prediction structures when streams in a stream-switching scenario share part of the packed picture.

It is proposed to changes to the existing region-wise packing SEI message insofar as to defining two regions types within the region-wise packing SEI message: namely static (remaining static for whole CLVS) and dynamic (that can change at any picture).

# Introduction

The region-wise packing SEI message was introduced during the last JCT-VC meeting in JCTVC-AB1005 in order to achieve alignment with OMAF and allow for different use-cases related to omnidirectional video. This contribution proposes changes to the existing region-wise packing SEI message. Same changes are proposed to Omnidirectional MediA Format (OMAF) in order to achieve alignment between the region-wise packing SEI message and region-wise packing box defined in OMAF.

The region-wise packing SEI message allows providing viewport-dependent streaming services in which the currently user viewport is represented within the video bitstream at a higher resolution than the remaining areas within the omnidirectional video, as illustrated in Figure 1.

Figure 1: (left) Full 360 content at two resolutions, (right) mixture of high-res viewport VP1 and low-res region covering the full content.

Such solutions can be realized using the HEVC viewport-dependent baseline profile of OMAF, for instance. One alternative is to provide several (per-viewport) bitstreams, as illustrated for 3 viewports in Figure 2.

Figure 2: Multiple viewport-dependent streams for VP1 - VP3 sharing the same low-res region covering the full content.

In order to be able to switch from one bitstream to another, frequent cross-bitstream aligned RAPs (around every few hundred of milliseconds) need to be provided. However, this configuration leads to an undesirable bitrate overhead for the light blue shaded region in Figure 2. Since the low-resolution content is present in all bitstreams, an appropriate encoding scheme could lessen the RAP requirement for the low-resolution picture region and hence reduce overall streaming bitrate. The idea is to provide only a partial RAP, i.e. I-slice for with a temporal prediction break only for VPx related picture areas following in decoding order.

However, with the current syntax of the region-wise packing SEI message, it is not possible to signal at systems level (e.g. using SDP in RTP streaming or MPD in DASH streaming) that the low-res content region within different bitstreams is represented with the same static packing configuration.

In this contribution, we propose defining static (constant for the whole CLVS) and dynamic (can change at any picture) regions, as described in Section 2, that intent to allow specifying system-level constraints that are necessary for such a streaming service. More concretely, in a given profile signaled in an MPD, or e.g. in a specific configuration of the OMAF HEVC viewport-dependent baseline profile (as described above), it would be a requirement that the low-resolution region is described as static region and that all offered bitstreams (Representations in the DASH nomenclature) contain the same description of the low-res static regions. Thus, a conformance point for the concatenation of bitstreams that results from bitstream switching can be established, fulfilling that the static regions specified in the region-wise packing SEI message are constant for the whole CLVS.

# Proposal

| regionwise_packing( payloadSize ) { | Descriptor |
| --- | --- |
| rwp_cancel_flag | u(1) |
| if( !rwp_cancel_flag ) { |  |
| rwp_persistence_flag | u(1) |
| rwp_regions_type_idc | u(2) |
| rwp_reserved_zero_64bits | u(64) |
| if(rwp_regions_type_idc = = 0 ¦¦ rwp_regions_type_idc = = 2) |  |
| static_num_packed_regions | u(8) |
| if(rwp_regions_type_idc = = 1 ¦¦ rwp_regions_type_idc = = 2) |  |
| dynamic_num_packed_regions | u(8) |
| proj_picture_width | u(16) |
| proj_picture_height | u(16) |
| for( i = 0; i < (static_num_packed_regions + dynamic_num_packed_regions); i++ ) { |  |
| rwp_reserved_zero_4bits[ i ] | u(4) |
| packing_type[ i ] | u(4) |
| if(packing_type[ i ] = = 0 ) { |  |
| proj_region_width[ i ] | u(16) |
| proj_region_height[ i ] | u(16) |
| proj_region_top[ i ] | u(16) |
| proj_region_left[ i ] | u(16) |
| transform_type[ i ] | u(3) |
| rwp_reserved_zero_5bits[ i ] | u(5) |
| packed_region_width[ i ] | u(16) |
| packed_region_height[ i ] | u(16) |
| packed_region_top[ i ] | u(16) |
| packed_region_left[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |

- Update D.3.41.4 as follow:

The region-wise packing SEI message provides information to enable remapping of the colour samples of the cropped output pictures onto projected pictures. It defines corresponding regions in the packed picture and projected picture. These regions can be dynamic regions that can change within the CLVS or static regions that cannot change within the CLVS.

Let picA be the current picture. rwp_persistence_flag equal to 1 specifies that the region-wise packing SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a region-wise packing SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

Static regions (if any) specified by the region-wise packing SEI message shall be the same for each decoded picture within the CLVS.

[...]

rwp_regions_type_idc specifies the type of the regions in the SEI message. rwp_regions_type_idc equal to 0 indicates that only static regions are defined for the CLVS. rwp_regions_type_idc equal to 1 indicates that only dynamic regions are defined for the CLVS. rwp_regions_type_idc equal to 2 indicates that both static regions and dynamic regions are defined for the CLVS.

static_num_packed_regions specifies the number of static packed regions. The value of static_num_packed_regions shall be less than 255. If static_num_packed_regions is not preset, it is inferred to be equal to 0. The value of static_num_packed_regions shall be greater than 0.

dynamic_num_packed_regions specifies the number of dynamic packed regions. The value of dynamic_num_packed_regions shall be less than 255. If dynamic_num_packed_regions is not preset, it is inferred to be equal to 0.

The sum of static_num_packed_regions and dynamic_num_packed_regions shall be greater than 0 and less than 255.

The first static_num_packed_regions describe regions for which the value of syntax elements proj_region_width[ i ],  proj_region_height[ i ],  proj_region_top[ i ],  proj_region_left[ i ], transform_type[ i ], packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], packed_region_left[ i ] are the same in all region-wise packing SEI messages in the bitstream that apply to the pictures within the current layer of the CLVS.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).