| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–25 Oct. 2017 | Document: JCTVC-AC0032 |
| --- | --- |

| Title: | On coverage signalling for omnidirectional video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin Y. Sanchez | Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports asserted issues in the coverage signalling of JCTVC-AB1005 and proposes an addition to a projection format independent coverage signalling in fashion similar to OMAF as proposed in JCTVC-AC0024.

It is proposed to add a sphere_exact_match_flag to indicate the presents of samples outside the indicated sphere coverage within the decoded picture.

Revision 1 of this document adds a presentation slide deck.

# Introduction

The current Specification text in JCTVC-AB1005 contains explicit coverage signalling support for equirectangular projected video in the equirectangular projection SEI message (azimuth range above 360 degree), while the cubemap projection SEI message does not support such signalling. Instead, for cube map projected video, coverage information can only be derived from the region-wise packing SEI message.

This design is asserted to be undesirable since a content author would need to parse multiple syntax structures to understand the coverage. Instead, it would be desirable to have high-level instance for coverage signaling that can be parsed irrespective of the usage of a specific projection format or region-wise packing as in OMAF.

However, for CMP coverage signalling with four great circles, e.g. as in the OMAF Specification or JCTVC-AC0024, it is not possible to exactly signal the sphere coverage of a rectangular subsection of the projected picture that crosses cube faces. Especially, when the top and/or bottom faces are included, as illustrated in Figure 1, where the decoded picture contains samples of the whole back face and half of the bottom face.

Figure 1: Cubemap projection with restricted coverage.

This contribution proposes to add signalling that indicates whether the signalled sphere coverage region (e.g. using great and/or small circles) corresponds to a perfect match of the samples in the decoded picture.

Such a signalling provides a more accurate information on whether additional samples outside of the signalled sphere region are present in the decoded picture. Such information could help a client to understand that when downloading several bitstreams covering different sphere regions, the union of the signalled sphere coverage is accurate or not, i.e. whether decoded pictures of the multiple bitstreams can be trivially combined to a continuous spherical image without overlap or gaps.

# Proposal

The proposal in this document is to signal whether the defined sphere region correspond to an exact match of the samples present in the decoded picture. I.e., if all samples and only the samples of the defined sphere region are present in the decoded picture, or if the decoded picture contains all samples of the defined sphere region but some additional samples might be present.

| - sphere_coverage( payloadSize ) { | Descriptor |
| --- | --- |
| - [...] |  |
| - sphere_coverage_reserved_zero_65bits | u(65) |
| - sphere_coverage_exact_match_flag | u(1) |
| - [...] |  |
| - } |  |

- sphere_coverage_exact_match_flag equal to 1 specifies that all decoded picture samples that lie within the indicated sphere coverage are present and that no decoded picture samples lie outside the indicated sphere coverage. sphere_coverage_exact_match_flag equal to 0 specifies that all decoded picture samples that lie within the indicated sphere coverage are present but further samples may also be present.

- NOTE 1 – When sphere_coverage_exact_match_flag is equal to 1 in the sphere coverage SEI messages of multiple bitstreams that cover disjunct but spherically adjacent subsections of an omnidirectional video, the decoded pictures of the multiple streams can be combined to a continuous spherical image without overlap or gaps.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).