| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–25 Oct. 2017 | Document: JCTVC-AC0036 |
| --- | --- |

| Title: | Input Document to AHG6: Industry In-Use Combinations of Video Property Description Points |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Input Document for AHG6 – Discussion of Technical Report |  |  |
| Author(s) or Contact(s): | Yasser Syed Chris Seeger Chad Fogg Lars Borg | Tel: Email: | 303-246-8413 yasser_syed@comcast.com chris.seeger@nbcuni.com chad.fogg@gmail.com borg@adobe.com |
| Source: | Comcast, NBCU, Movielabs, Adobe |  |  |

_____________________________

# Abstract

This document contains a draft of a technical report on prominent video property description points combinations applied in industry. This input document is to be used for a discussion on a planned Technical Report (23091-4) which is a non-normative document of industry best practices on describing existing combinations of video properties, some of which are coding points, and their representations in different carriage systems. The combinations of properties and the permutations of all possible values being considered can reach hundreds of choices to describe a stream and some of these combinations are not expected to be used in practice (such as using the ITU-R BT.2100 perceptual quantization (PQ) transfer characteristics with ITU-R BT.601 colour primaries). Only a small subset of these combinations (such as BT.2100 colour primaries with BT.2100 PQ and hybrid log-gamma transfer characteristics) is used in practice. This document aims to avoid mistaken assumptions on video property combinations made by vendors of various content processing tools that may be part of a production equipment chain. With the increased usage of high-dynamic range and the increased use of look-up tables in television systems, these content processing mistakes could increasingly become magnified. Lastly, this Technical Report aims to help its readers, especially toolset developers, avoiding unnecessary, complicated tailoring of their content processing tools to specific areas of the workflow, and thus improve the repurposing of their tools to different parts of the workflow.

------- Input Document is a draft of what may go into a PDTR-----------------------------------------------------

# Scope

This planned Technical Report is a non-normative document of industry best practices on describing existing combinations of video properties and their representations in different carriage systems.

# References

- W16694 Geneva Jan 2017-  23091-2 Information technology – Coding-independent code points – Part 2: Video.

- SMPTE 2067-21:2016 Interoperable Master Format- Application #2E.

- Apple QuickTime™ Video Color Management in AV Foundation.

- Apple ProRes™ SMPTE RDD Draft for MXF.

- SMPTE ST 2086:2014: Mastering Display Color Volume Metadata Supporting High Luminance and Wide Color Gamut Images.

- SMPTE 377-1:2011 Am1:2012 Am2:2012: Material Exchange Format (MXF)- File Format Specification.

- ISO/IEC 14496-10/ ITU-T H.264, Information technology – Coding of audio-visual objects Part 10: Advanced Video Coding.

- ISO/IEC 23008-2/ ITU-T H.265, Information Technology – High efficiency coding and media delivery in heterogeneous environments Part 2: High efficiency video coding.

- ITU-R BT.2100-1:2017 Image parameter values for high dynamic range television for use in production and international programme exchange.

# Definitions

This document defines the following terms. The definitions used in the AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) and HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2) standards also apply.

| - a | electro-optical transfer function (EOTF): The function used in the post-decoding process to convert from a non-linear representation to a linear representation. |
| --- | --- |
|  | full range: A range in a fixed-point (integer) representation that spans the full range of values that could be expressed with that bit depth, such that, for 10-bit signals, black corresponds to code value 0 and peak white corresponds to code value 1023 for Y, as per the full range definition from Rec. ITU-R BT.2100. |
|  | inverse electro-optical transfer function (inverse EOTF): A function used in the pre-encoding process to convert from a linear representation to a non-linear representation, computed as the inverse of the EOTF. NOTE to entry – In this document the pre-encoding process is assumed to operate on HDR/WCG video content that has been prepared for a hypothetical reference viewing environment as shown in Error: Reference source not found. The content preparation step may contain processing such as applying an opto-optical transfer function (OOTF), in which the HDR/WCG video is converted from one linear representation (corresponding to the scene) to another linear representation (corresponding to the display). The OOTF has the role of applying a “rendering intent”. In systems where no such OOTF is applied in the content preparation step, the process of converting from a linear representation (corresponding to the scene) to a non-linear representation is typically called the opto-electrical transfer function (OETF). |
|  | narrow range: A range in a fixed-point (integer) representation that does not span the full range of values that could be expressed with that bit depth such that, for 10 bit representations, the range from 64 (black) to 940 (peak white) is used for Y and the range from 64 to 960 is used for Cb and Cr, as per the narrow range definition from Rec. ITU-R BT.2100. NOTE to entry – Narrow range is, in some applications, called by synonyms such as: “limited range”, “video range”, “legal range”, “SMPTE range” or “standard range”. |
|  | opto-electrical transfer function (OETF): The function that converts linear scene light into the video signal, typically within a camera. |
|  | opto-optical transfer function (OOTF): A function that maps relative scene linear light (typically the camera output signal) to display linear light (typically, the signal driving a mastering monitor). |
|  | random access point access unit (RAPAU): An access unit in the bitstream containing an intra coded picture with the property that all pictures following the intra coded picture in output order can be correctly decoded without using any information preceding the Random Access Point Access Unit in the bitstream. |
|  | transfer function: In this document, a transfer function refers to any of the following; EOTF, inverse EOTF, OETF, inverse OETF, OOTF, or inverse OOTF. |

# Abbreviations and acronyms

This document uses the following abbreviations and acronyms:

AVC		Advanced Video Coding (Rec. ITU-T H.264 | ISO/IEC 14496-10)

CL		Constant Luminance

EOTF		Electro-Optical Transfer Function

HD		High Definition

HDR		High Dynamic Range

HEVC		High Efficiency Video Coding (Rec. ITU-T H.265 | ISO/IEC 23008-2)

HVS		Human Visual System

LUT		Look-up Table

NCL		Non-Constant Luminance

PQ		Perceptual Quantizer (as defined in SMPTE ST 2084 and Rec. ITU-R BT.2100)

QP		Quantization Parameter

RAPAU		Random Access Point Access Unit

RGB		Colour System using Red, Green, and Blue components

SDR		Standard Dynamic Range

SEI		Supplemental Enhancement Information

OETF		Opto-Electrical Transfer Function

OOTF		Opto-Optical Transfer Function

VUI		Video Usability Information

WCG		Wide Colour Gamut

XYZ		The CIE 1931 colour space. Y corresponds to the luminance signal.

YCbCr		Colour space representation commonly used for video/image distribution as a way of encoding RGB information, also commonly expressed as YCbCr, YCBCR, or YCBCR. The relationship between YCbCr and RGB is dictated by certain signal parameters, such as colour primaries, transfer characteristics, and matrix coefficients. Unlike the (constant luminance) Y component in the XYZ representation, Y in this representation might not be representing the same quantity. Y is commonly referred to as “luma”. Cb and Cr are commonly referred to as “chroma”.

# Overview

The Coding-Independent Code Points (CICP) Video standard (ISO/IEC 23091-2) defines code points and fields that identify video signal types that are independent of the coded video layer stream. The combinations of properties and the permutations of all possible values being considered can reach hundreds of choices to describe a stream and some of these combinations are not expected to be used in practice (such as using the ITU-R BT.2100 perceptual quantization (PQ) transfer characteristics with ITU-R BT.601 colour primaries). Only a small subset of these combinations (such as BT.2100 colour primaries with BT.2100 PQ and hybrid log-gamma transfer characteristics) is used in industry. These properties, usually expressed in "metadata", can exist across the production and distribution workflows and knowledge of these properties and combinations has value as content gets processed in the E2E [End-to-End] production to distribution workflow chain.

This planned Technical Report is a non-normative document of industry best practices on describing existing combinations of video properties and their representations in different carriage systems. This document aims to avoid mistaken assumptions on video property combinations made by vendors of various content processing tools that may be part of a production equipment chain. With the increased usage of high-dynamic range and the increased use of look-up tables in television systems, these content processing mistakes could increasingly become magnified.  Lastly, this Technical Report aims to help its readers, especially toolset developers, avoiding unnecessary, complicated tailoring of their content processing tools to specific areas of the workflow, and thus improve the repurposing of their tools to different parts of the workflow.

This Technical Report will first talk about the types of common standard color volumes and the properties associated to define them. Then for each common standard color volume what different parameters of the associated properties are in use in industry. Lastly depending on domain (production, distribution) how each of these associated properties are carried.

# Domains- Capture, Production, Distribution

The following diagram describes the different domains that video content may exist and can be edited or converted in a workflow.  Similar conversions may be done in different domains and tools need to understand

Figure 1 Video Workflows Through Different Carriage Domains

# Common Combinatorial Descriptions

Separately defining video properties with each possible enumerated values can fully describe all cases of the video property in isolation. Describing video content requires looking collectively at the groups of video properties. Some combination of values of the video properties may collectively not make sense (e.g. xxx) or are not being used by the industry. This section defines commonly associated video properties and those combinations of values in use for the content industry today. Additionally, it defines for each Domain (Capture, Production, Distribution) what methods of carriage are commonly used to convey these video properties.

## Common Colour Volumes Descriptions

Common Colour Volumes describes combinations of video properties that are needed to do conversions on colour volumes. These may be conversions to other colour volumes, changes in bit depth, changes in colour sampling, non-linear optimizations and may also include transformations based on carriage and bit rate restrictions. Tools performing these conversions need to know the values of these video properties to decode the video and consistently perform its operations.

### Defined Properties

- Color Primary [ref- coding points]

- Electro-Optical Transfer Function [ref- coding points]

- Color Representation [ref- coding points]

- Color Sampling [ref-]

- Minimum Bit Depth [ref]

- Full/Narrow Scale [ref]

### Narrow Dynamic Range- Narrow Colour Gamut

This colour volume describes SDR video which describe the majority of the production and distribution workflows used in the industry today. There are several combinations of values of these video properties that are used for this colour volume. Table 1 describes these combinations. Note there are several one-way operations that can be performed for this colour Volume including bit depth reductions, color sampling reductions, and full to narrow scale operations.

Table 1 SDR Common Colour Volume Descriptions

### Narrow Dynamic Range- Wide Colour Gamut

This colour volume describes SDR with Wide Color Gamut video which is keyed off the color primary video property. Please note that the same color is described with two different values depending on the color primary container used. It is important for tools to understand what color volume it is operating in to make sure the conversion is handled consistently.

Table 2 SDR w/ WCG Common Colour Volume Descriptions

### High Dynamic Range- Wide Colour Gamut

This colour volume describes what is known as HDR video which is used in next generation video technologies. Tools including static and dynamic LUTs, tone mapping, and bit rate compression need to understand these combination values to produce consistent conversions.

Table 3 HDR Common Colour Volume Descriptions

### Domain Carriage Formats

Carriage formats in each of the domains (Capture, Production, Distribution) can differ. In the Capture and Production Domains, Color Volume Information can be carried in metadata carried in a MXF wrapper or in an MOV Wrapper that accompanies Video Formats such as RAW, DPX, OpenEXR or as Out of Band captured in the content workflow. Colour Volume Information in the distribution domain can be carried within the video stream as syntax information in the selected video format such as MPEG- AVC, MPEG-HEVC, MPEG-2 through Mechanisms, such as VUI, Profile/ Level, or SEI information. The combinations describing the color volume can mainly be identified by Color Primary, EOTF, and Color Representation parameters in each of the carriage domains. See Table 4, Table 5, Table 6.

Table 4 Carriage of SDR Colour Volume

Table 5 Carriage of SDR w/ WCG Colour Volume

Table 6 Carriage of HDR Colour Volume

## Common Mastering Display Colour Volume Descriptions

The Mastering Display Information describes the colour volume of a display that was used for viewing while authoring the video content. It is the display where creative work is done during the mastering process and can be represented by a combination of video display properties. When shown on other displays, this type of video display information can be used to more closely reproduce the creative intent originally achieved in the mastering suite. Again, only certain combination of video display properties are used in the mastering display environment. Tools performing conversions of video content would be able to preserve more of colour reproduction intent with knowledge of these combinations of in-use values for the video display property parameters.

### Defined Properties

- Mastering Display Primaries [ref- ST 2086]

- Mastering Display White Point Chromaticity [ref- ST 2086]

- Mastering Display Maximum Luminance [ref- ST 2086]

- Mastering Display Minimum Luminance [ref- ST 2086]

### 709-Gamma_2.4, D65, 1000nit

This represents display colour volume environments dealing with mastering of SDR/HDR content on LED/LCD displays capable of 1000 nits of peak brightness but restricted to BT.709 Colorspace.

Table 7 MDCV – BT.709- Gamma_2.4, D65, 1000 Nit

### 709-Gamma_2.4, D65, 4000nit

This represents display colour volume environments dealing with mastering of SDR/HDR content on LED/LCD displays capable of 4000 nits of peak brightness but restricted to BT.709 colorspaces.

Table 8 MDCV – BT.709- Gamma_2.4, D65, 4000 Nit

### BT.2020, D65, 1000nit

This represents display colour volume environments dealing with mastering of HDR content on OLED displays capable of 1000 nits of peak brightness and wider BT.2020 Colorspaces.

Table 9 MDCV – BT.2020, D65, 1000 Nit

### P3, D65, 1000nit

This represents display colour volume environments dealing with mastering of HDR content on OLED displays capable of 1000 nits of peak brightness and P3 defined colorspaces.

Table 10 MDCV –P3, D65, 1000 Nit

### Domain Carriage Formats

Carriage formats for mastering display colour volume in each of the domains (Capture, Display, Distribution) contain the same payload but in different wrappers. In the capture and production and domains, display colour volume information can be carried as metadata in an MOV wrapper in the ‘mdvc’ atom that accompanies video formats such as RAW, DPX, OpenEXR. In the MXF wrapper, it is carried in the Generic Picture Essence descriptor as described by Annex C of ST 2067-21. In the distribution domain, the mastering display colour volume information is carried the mastering display colour volume SEI message (payload 137) that needs to be repeated at least every random access point access unit (RAPAU).

Table 11 Carriage of MDCV parameters

# Patent rights declaration(s)

Comcast/NBCU, Adobe may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Movielabs does not have any current or pending patent rights relating to the technology described in this contribution.