| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–24 October 2017 | Document: JCTVC-AC0037 |
| --- | --- |

| Title: | On Region-Wise Packing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd., Camas, WA 98607, USA | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document modifications are proposed to region-wise packing (RWP) SEI Message and sample location mapping. The proposed modifications provide alignment between the region-wise packing SEI messages and the corresponding region-wise packing signaling in OMAF SoDIS.

Following is proposed:

- Signaling of a constituent pictures matching regions flag in RWP SEI

- Updates to constraints related to RWP SEI syntax elements

- Signaling of packed picture width and height in the RWP SEI

- Updates to mapping of luma sample locations within a decoded picture to sphere coordinates relative to the global coordinate axes

# Introduction

JCTVC-AB1005 [1] Draft3 describes HEVC additional supplemental enhancement information. This includes a region-wise packing SEI message. The region-wise packing SEI message provides information to enable remapping of the colour samples of the cropped output pictures onto projected pictures.

W16950 –OMAF SoDIS text [2] defines the omnidirectional media format for coding, storage, delivery, and rendering of omnidirectional images and video and the associated audio. It includes region-wise packing signaling structure.

There are some discrepancies between region-wise packing (RWP) signaling in OMAF SoDIS [2] and in HEVC  Draft 3 [1]. It is proposed to align the signaling of region-wise packing information in [1] and [2]. For this following is proposed.

- Signaling of a constituent pictures matching regions flag in RWP SEI

- Updates to constraints related to RWP SEI syntax elements

- Signaling of packed picture width and height in the RWP SEI

- Updates to mapping of luma sample locations within a decoded picture to sphere coordinates relative to the global coordinate axes

# Proposal

The specification text for the above proposals are shown highlighted compared to JCTVC-AB1005 [1].

D.2.41.3 Region-wise packing SEI message syntax

| regionwise_packing( payloadSize ) { | Descriptor |
| --- | --- |
| rwp_cancel_flag | u(1) |
| if( !rwp_cancel_flag ) { |  |
| rwp_persistence_flag | u(1) |
| constitutent_picture_matching_regions | u(1) |
| rwp_reserved_zero_65bits | u(65) |
| num_packed_regions | u(8) |
| proj_picture_width | u(16) |
| proj_picture_height | u(16) |
| packed_picture_width | u(16) |
| packed_picture_height | u(16) |
| for( i = 0; i < num_packed_regions; i++ ) { |  |
| rwp_reserved_zero_4bits[ i ] | u(4) |
| packing_type[ i ] | u(4) |
| if( packing_type[ i ] = = 0 ) { |  |
| proj_region_width[ i ] | u(16) |
| proj_region_height[ i ] | u(16) |
| proj_region_top[ i ] | u(16) |
| proj_region_left[ i ] | u(16) |
| transform_type[ i ] | u(3) |
| rwp_reserved_zero_5bits[ i ] | u(5) |
| packed_region_width[ i ] | u(16) |
| packed_region_height[ i ] | u(16) |
| packed_region_top[ i ] | u(16) |
| packed_region_left[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |

D.3.41.3 Region-wise packing SEI message semantics

The region-wise packing SEI message provides information to enable remapping of the colour samples of the cropped output pictures onto projected pictures.

If a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is not present, StereoFlag, TopBottomFlag, and SideBySideFlag are all set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 1.

Otherwise, the following applies:

–	StereoFlag is set equal to 1.

–	If the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 3, TopBottomFlag is set equal to 0, SideBySideFlag is set equal to 1, HorDiv1 is set equal to 2 and VerDiv1 is set equal to 1.

–	Otherwise, if the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 4, TopBottomFlag is set equal to 1, SideBySideFlag is set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 2.

–	Otherwise, TopBottomFlag is set equal to 0, SideBySideFlag is set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 1.

rwp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous region-wise packing SEI message in output order. rwp_cancel_flag equal to 0 indicates that region-wise packing information follows.

rwp_persistence_flag specifies the persistence of the region-wise packing SEI message for the current layer.

rwp_persistence_flag equal to 0 specifies that the region-wise packing SEI message applies to the current decoded picture only.

Let picA be the current picture. rwp_persistence_flag equal to 1 specifies that the region-wise packing SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a region-wise packing SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the region-wise packing SEI message in decoding order, a region-wise packing SEI message with rwp_persistence_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore region-wise packing SEI messages with rwp_persistence_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 in the CLVS that applies to the current picture.

constituent_picture_matching_reg equal to 1 specifies that the projected region information, packed region information, in this SEI message applies individually to each constituent picture and that the packed picture and the projected picture have the same stereoscopic frame packing format indicated by the frame packing arrangement SEI message that applies to the picture. constituent_picture_matching_reg equal to 0 specifies that the projected region information, packed region information, in this SEI message apply to the projected picture. When StereoFlag is equal to 0 or StereoFlag is equal to 1 and frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 5, constituent_picture_matching_reg shall be equal to 0.

rwp_reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_6bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_6bits[ i ].

num_packed_regions specifies the number of packed regions for which the projected region information, and packed region information, is indicated in this SEI message. The value of num_packed_regions shall be greater than 0. When constituent_picture_matching_reg is equal to 1, the total number of packed regions is equal to 2 * num_packed_regions and the information in the projected and packed region information in the SEI message applies to each constituent picture of the projected picture and the packed picture.

proj_picture_width and proj_picture_height specify the width and height, respectively, of the projected picture. The value of proj_picture_width and proj_picture_height shall be both greater than 0.

packed_picture_width and packed_picture_height specify the width and height, respectively, of the packed picture, in relative packed picture sample units. packed_picture_width and packed_picture_height shall both be greater than 0.

rwp_reserved_zero_4bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_4bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_4bits[ i ].

packing_type[ i ] specifies the type of region-wise packing. packing_type[ i ] equal to 0 indicates rectangular region-wise packing. Other values are reserved. The value of packing_type[ i ] shall be equal to 0 in this version of this Specification. Decoders shall allow values of packing_type[ i ] greater than 0 and shall ignore all region-wise packing SEI messages with packing_type[ i ] greater than 0 for any value of i.

proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ], and proj_region_left[ i ] are indicated in units of luma samples in a projected picture with width and height equal to proj_picture_width and proj_picture_height, respectively.

proj_region_width[ i ] specifies the width of the i-th projected region. proj_region_width[ i ] shall be greater than 0.

proj_region_height[ i ] specifies the height of the i-th projected region. proj_region_height[ i ] shall be greater than 0.

proj_region_top[ i ] and proj_region_left[ i ] specify the top luma sample row and the left-most luma sample column, respectively, of the i-th projected region in the projected picture when constituent_picture_matching_reg is equal to 0, or in the constituent picture of the projected picture when constituent_picture_matching_reg is equal to 1. The values of proj_region_top[ i ] and proj_region_left[ i ], shall be in the range from 0, inclusive, indicating the top-left corner of the projected picture, to proj_picture_height − 1, inclusive, and proj_picture_width − 1, inclusive, respectively.

The values of proj_region_width[i], proj_region_height[i], proj_region_top[i], and proj_region_left[i] are constrained as follows:

- If StereoFlag is equal to 0 or TopBottomFlag is equal to 1, proj_region_width[i] shall be in the range of 1 to proj_picture_width, inclusive.

- Otherwise (SideBySideFlag is equal to 1), proj_region_width[i] shall be in the range of 1 to proj_picture_width / 2, inclusive.

- proj_region_height[i] shall be greater than 0.

- When constituent_picture_matching_reg is equal to 0, the values of proj_region_top[i] and proj_region_left[i] shall be in the range from 0, inclusive, indicating the top-left corner of the projected picture, to proj_picture_height − 1, inclusive, and proj_picture_width − 1, inclusive, respectively. When constituent_picture_matching_reg is equal to 1, the values of proj_region_top[i] and proj_region_left[i] shall be in the range from 0, inclusive, to proj_picture_height / VerDiv1 − 1, inclusive, and proj_picture_width / HorDiv1 − 1, inclusive, respectively.

- When StereoFlag is equal to 0 or constituent_picture_matching_reg is equal to 0, proj_region_height[i] and proj_region_top[i] shall be constrained such that proj_region_height[i] + proj_region_top[i] − 1 is less than proj_picture_height.

- When StereoFlag is equal to 1 and the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 3 or 4, the following applies:

- proj_region_width[i] shall be less than or equal to proj_picture_width / HorDiv1.

- proj_region_height[i] shall be less than or equal to proj_picture_height / VerDiv1.

- If proj_region_left[i] is less than proj_picture_width / HorDiv1, proj_region_width[i] and proj_region_left[i] shall be constrained such that proj_region_width[i] + proj_region_left[i] − 1 is less than proj_picture_width / HorDiv1.

- Otherwise (proj_region_left[i] is greater than or equal to proj_picture_width / HorDiv1), proj_region_width[i] and proj_region_left[i] shall be constrained such that proj_region_width[i] + proj_region_left[i] − proj_picture_width / HorDiv1 − 1 is less than proj_picture_width / HorDiv1.

- If proj_region_top[i] is less than proj_picture_height / VerDiv1 then proj_region_height[i] and proj_region_top[i] shall be constrained such that proj_region_height[i] + proj_region_top[i] − 1 is less than proj_picture_height / VerDiv1.

- Otherwise (i.e. when proj_region_top[i] is greater than or equal to proj_picture_height / VerDiv1) then proj_region_height[i] and proj_region_top[i] shall be constrained such that proj_region_height[i] + proj_region_top[i] − proj_picture_height / VerDiv1 − 1 is less than proj_picture_height / VerDiv1.

- When constituent_picture_matching_reg is equal to 1, the following applies:

- proj_region_width[i] and proj_region_left[i] shall be constrained such that proj_region_width[i] + proj_region_left[i] − 1 is less than proj_picture_width / HorDiv1.

- proj_region_height[i] and proj_region_top[i] shall be constrained such that proj_region_height[i] + proj_region_top[i] − 1 is less than proj_picture_height / VerDiv1.

The sum of proj_region_width[ i ] and proj_region_left[ i ] shall be less than proj_picture_width. The sum of proj_region_height[ i ] and proj_region_top[ i ] shall be less than proj_picture_height.

The values of proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ] and proj_region_left[ i ] shall be such that the projected region identified by these fields is within a single constituent picture of the projected picture.

NOTE – Two projected regions may partially or entirely overlap with each other.

transform_type[ i ] specifies the rotation and mirroring to be applied to the i-th packed region to remap to the i-th projected region. When transform_type[ i ] specifies both rotation and mirroring, rotation applies before mirroring. The values of transform_type[ i ] are specified in Table D. X:

Table D.X – transform_type[ i ] values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (counter-clockwise) |
| 3 | rotation by 180 degrees (counter-clockwise) after mirroring horizontally |
| 4 | rotation by 90 degrees (counter-clockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (counter-clockwise) |
| 6 | rotation by 270 degrees (counter-clockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (counter-clockwise) |

rwp_reserved_zero_5bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_5bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_5bits[ i ].

packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] specify the width, height, the top luma sample row, and the left-most luma sample column, respectively, of the packed region in the region-wise packed picture.

Let packedPicWidth and packedPicHeight be the width and height of the conformance cropping window of the region-wise packed picture. The values of packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] are constrained as follows:

- packed_region_width[ i ] and packed_region_height[ i ] shall both be greater than 0.

- The values of packed_region_top[ i ] and packed_region_left[ i ] shall be in the range from 0, inclusive, indicating the top-left corner luma sample of the region-wise packed picture, to packedPicHeightpacked_picture_height − 1, inclusive, and packedPicWidthpacked_picture_width − 1, inclusive, respectively.

- If constituent_picture_matching_region is equal to 0, then the following applies:

- The sum of packed_region_width[ i ] and packed_region_left[ i ] shall be less than packedPicWidth.

- The sum of packed_region_height[ i ] and packed_region_top[ i ] shall be less than packedPicHeight.

- Otherwise (constituent_picture_matching_reg is equal to 1), the following applies:

- The sum of packed_reg_width[i] and packed_reg_left[i] minus 1 shall be less than packed_picture_width / HorDiv1.

- The sum of packed_reg_height[i] and packed_reg_top[i] minus 1 shall be less than packed_picture_height / VerDiv1.

- The rectangle specified by packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] shall be non-overlapping with the rectangle specified by packed_region_width[ j ], packed_region_height[ j ], packed_region_top[ j ], and packed_region_left[ j ] for any value of j in the range of 0 to i – 1, inclusive.

- D.3.41.5.5	Mapping of luma sample locations within a region-wise packed picture to sphere coordinates relative to the global coordinate axes

This clause specifies the semantics of luma sample locations within a region-wise packed picture to sphere coordinates relative to the global coordinate axes.

This clause uses variables HorDiv1, VerDiv1, RotationFlag, StereoFlag, TopBottomFlag, SideBySideFlag, ConstituentPicWidth, ConstituentPicHeight, and RegionWisePackingFlag that are derived specific to the type of the decoded picture for which the corresponding region-wise packed picture this clause is applied to.

offsetX is set equal to 0.5 and offsetY is set equal to 0.5.

The values of the variables NumRegions, PackedRegLeft[n], PackedRegTop[n], PackedRegWidth[n], PackedRegHeight[n], ProjRegLeft[n], ProjRegTop[n], ProjPictureWidth[n], ProjPictureHeight[n], TrasnformType[n], PackingType[n] are set as follows:

- If constituent_picture_matching_regions is equal to 0: NumRegions is set equal to num_packed_regions and for n in the range of 0 to NumRegions-1 : PackedRegLeft[n] is set equal to packed_region_left[n] , PackedRegTop[n] is set equal to packed_region_top[n], PackedRegWidth[n] is set equal to packed_region_width[n], PackedRegHeight[n] is set equal to packed_region_height[n], ProjRegLeft[n] is set equal to proj_region_left[n], ProjRegTop[n] is set equal to proj_region_top[n], ProjRegWidth[n] is set equal to proj_region_width[n], ProjRegHeight[n] is set equal to proj_region_height[n], TransformType[n] is set equal to transform_type[n], PackingType[n] is set equal to packing_type[n]

- If constituent_picture_matching_regions is equal to 1:  NumRegions is set equal to 2*num_packed_regions.

- When TopBottomFlag is set equal to 1 following variables are set: ProjLeftOffset and PackedLeftOffset are both set equal to 0, ProjTopOffset is set equal to proj_picture_height / 2 and PackedTopOffset is set equal to packed_picture_height / 2

- When SideBySideFlag is set equal to 1 following variables are set: ProjLeftOffset is set equal to proj_picture_width / 2, PackedLeftOffset is set equal to packed_picture_width / 2, ProjTopOffset and PackedTopOffset are both set equal to 0

-For n in the range of 0 to NumRegions/2-1 : PackedRegLeft[n] is set equal to packed_region_left[n], PackedRegTop[n] is set equal to packed_region_top[n], PackedRegWidth[n] is set equal to packed_region_width[n], PackedRegHeight[n] is set equal to packed_region_height[n], ProjRegLeft[n] is set equal to proj_region_left[n], ProjRegTop[n] is set equal to proj_region_top[n], ProjRegWidth[n] is set equal to proj_region_width[n], ProjRegHeight[n] is set equal to proj_region_height[n], TransformType[n] is set equal to transform_type[n], PackingType[n] is set equal to packing_type[n]

- For n in the range of NumRegion/2 to NumRegions-1 : NIdx is set equal to (n - NumRegion/2), PackedRegLeft[n] is set equal to packed_region_left[NIdx] + PackedLeftOffset, PackedRegTop[n] is set equal to packed_region_top[NIdx] + PackedTopOffset, PackedRegWidth[n] is set equal to packed_region_width[NIdx], PackedRegHeight[n] is set equal to packed_region_height[NIdx], ProjRegLeft[n] is set equal to proj_region_left[NIdx] + ProjLeftOffset, ProjRegTop[n] is set equal to proj_region_top[NIdx] + ProjTopOffset, ProjRegWidth[n] is set equal to proj_region_width[NIdx], ProjRegHeight[n] is set equal to proj_region_height[NIdx], TransformType[n] is set equal to transform_type[NIdx], PackingType[n] is set equal to packing_type[NIdx]

If RegionWisePackingFlag is equal to 1, the following applies for each packed region n in the range of 0 to NumRegionsnum_regions − 1, inclusive:

–	For each sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region with packing_type[ n ]PackingType[ n ] equal to 0 (i.e., with rectangular region-wise packing), the following applies:

–	The corresponding sample location (xProjPicture, yProjPicture) of the projected picture is derived as follows:

–	x is set equal to xPackedPicture – packed_region_left[ n ]PackedRegLeft [ n ].

–	y is set equal to yPackedPicture – packed_region_top[ n ]PackedRegTop [ n ].

–	Clause D.3.41.5.4 is invoked with x, y, packed_region_width[ n ], packed_region_height[ n ], proj_region_width[ n ], proj_region_height[ n ], transform_type[ n ],PackedRegWidth[ n ], PackedRegHeight[ n ], ProjRegWidth[ n ], ProjRegHeight[ n ], TransformType[ n ], offsetX and offsetY as inputs, and the output is assigned to sample location (i, j).

–	xProjPicture is set equal to proj_region_left[ n ]ProjRegLeft [ n ] + i.

–	When StereoFlag is equal to 0 or TopBottomFlag is equal to 1, and when xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width.

–	When SideBySideFlag is equal to 1, the following applies:

–	When proj_region_left[ n ]ProjRegLeft [ n ] is less than proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width / 2, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

–	When proj_region_left[n]ProjRegLeft [ n ] is greater than or equal to proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

–	yProjPicture is set equal to proj_region_top[ n ]ProjRegTop[ n ] + j.

–	Clause D.3.41.5.6 is invoked with xProjPicture, yProjPicture, ConstituentPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the luma sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region in the decoded picture.

Otherwise, the following applies for each sample location (x, y) within the region-wise packed picture:

–	xProjPicture is set equal to x + offsetX.

–	yProjPicture is set equal to y + offsetY.

–	Clause D.3.4.15.6 is invoked with xProjPicture, yProjPicture, ConstituentPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the sample location (x, y) within the region-wise packed picture.

# Conclusion

We propose to adopt all the proposals in this document into a revision of HEVC Draft 3.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

# References

1. JCTVC-AB1005, “HEVC Additional Supplemental Enhancement Information (Draft 3),” July 2017, Torino

2. W16950, “Study of ISO/IEC 23000-20 Omnidirectional Media Format,” July 2017, Torino