| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting: Macao, CN, 19–24 Oct. 2017 | Document: JCTVC-AC0040 |
| --- | --- |

| Title: | SCC software and text discrepancy for rounding with bipred restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jill Boyce Iole Moccagatta Lidong Xu | Email: | jill.boyce@intel.com iole.moccagatta@intel.com lidong.xu@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

A discrepancy has been found between the SCC draft text and the SCM software, related to rounding and bipred restriction. We believe that the draft text is correct, and that the software should be changed to correct the discrepancy. A software ticket has been submitted, #1479.

# Description

In the HEVC specification, using H.265 (V4) 12/2016 as a reference specification, the rounding in (8-98) is performed before checking the condition for bi-pred restriction.  But in the SCM code, the implementation is the other way around.  As such, HM 16.15_SCM8.5 mismatches the specification. This impacts HEVC SCC conformance stream expected for bitstream MVRESIDC_C_MS_1.

We believe that the specification is correct, and that the software should be changed to correct the discrepancy.

## Relevant specification text

Section 8.5.3 Decoding process for prediction units in inter prediction mode

8.5.3.2 Derivation process for motion vector components and reference indices

8.5.3.2.1 General

uLX[ 0 ] = ( ( ( ( mvpLX[ 0 ] >> 2 ) + mvdLX[ 0 ] ) << 2 ) + 216 ) % 216 				 (8-98)

When all of the following conditions are true, refIdxL1 is set equal to −1 and predFlagL1 is set equal to 0.

– predFlagL0 is equal to 1.

– predFlagL1 is equal to 1.

– nPbSw is equal to 8.

– nPbSh is equal to 8.

– TwoVersionsOfCurrDecPicFlag is equal to 1.

– noIntegerMvFlag is equal to 1.

– identicalMvs is equal to 0.

…

8.5.3.2.2 General

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

… 1 – 8 …

- The following assignments are made with N being the candidate at position merge_idx[ xOrigP ][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-1)

predFlagLX = predFlagLXN		(8-2)

    - When use_integer_mv_flag is equal to 0 and the reference picture is not the current picture, the following applies:

mvLX[ 0 ] = mvLXN[ 0 ]		(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]		(8-4)

    - Otherwise (use_integer_mv_flag is equal to 1 or the reference picture is the current picture), the following applies:

mvLX[ 0 ] = ( mvLXN[ 0 ]  >>  2 )  <<  2	(8-5)

mvLX[ 1 ] = ( mvLXN[ 1 ]  >>  2 )  <<  2	(8-6)

- When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies:

refIdxL1 = −1		(8-7)

predFlagL1 = 0		(8-8)

# Patent rights declaration(s)

Intel does not have any current or pending patent rights relating to the technology described in this contribution.