| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 30th Meeting: Gwangju, KR, 20–26 Jan. 2018 | Document: JCTVC-AD0002-v3 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Chris Rosewarne (Canon) Matteo Naccari (BBC) Jens-Rainer Ohm (RWTH Aachen) Karl Sharman (Sony) Gary J. Sullivan (Microsoft) Ye-Kui Wang (Qualcomm) | Email: | benjamin.bross@hhi.fraunhofer.de Chris.Rosewarne@cisra.canon.com.au Matteo.Naccari@bbc.co.uk ohm@ient.rwth-aachen.de Karl.Sharman@eu.sony.com garysull@microsoft.com yekuiw@qti.qualcomm.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 29th meeting in Macao, CN (October 2017) and the 30th meeting in Gwangju, KR (January 2018).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Collect reports of errata for the HEVC specification and the HDR technical reports. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |

# Status of AHG2 Work

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The open issues were studied and resolved in accordance with the below table. In some cases, editorial changes to the HEVC specification were noted for HEVC v5 (see items marked “Fix” below) and the issue tracker has been updated to reflect these.

| Ticket ID | Ticket headline | Comments |
| --- | --- | --- |
| #1315 | Inconsistent variable naming: CpbCnt | Agreed by the authors of this AHG report. CpbCnt is also used several times to specify an inclusive range (in addition to the <= to < suggested change). Fixes: The variable CpbCnt is set equal to cpb_cnt_minus1[ subLayerId ] + 1 instead of cpb_cnt_minus1[ subLayerId ], and replace the "<=" in the syntax loops with "<", and adjust other places containing CpbCnt accrodingly. In F.14.3.2.2, change the two instances of "in the range of 0 to CpbCnt." to include the word "inclusive" and adapt the range accordingly as well. |
| #1316 | C.2.2: minor editing issue | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1375 | minor fix for the spec | Agreed by the authors of this AHG report. Checked the 2016/12 release of the HEVC specification and ‘intraPredModeC’ after ‘Table 8-3’ should be uppercase. Fix: In the caption of Table 8-3, change "intraPredModeC" to "IntraPredModeC". |
| #1384 | orphaned symbol in the spec | Two parts: - Incorporate the definition of ‘n’ as suggested in one comment. Fix: In E.3.1, the semantics of log2_max_mv_length_horizontal and log2_max_mv_length_vertical, change the following sentence: A value of n asserts that no value of a motion vector component is outside the range of −2n to 2n − 1, inclusive, in units of quarter luma sample displacement. to A value of n asserts that no value of a motion vector component is outside the range of −2n to 2n − 1, inclusive, in units of quarter luma sample displacement, where n refers to the value of log2_max_mv_length_horizontal and log2_max_mv_length_vertical for the horizontal and vertical component of the MV, respectively. - Set the range constraint on log2_max_mv_length_horizontal to 0..15, also a suggested in another comment. CLOSED: The suggested semantics is present in the "Last Call" text for ITU-T HEVC text version 5. |
| #1386 | typo in spec | INVALID: For coding of the last significant coefficient position, the prefix specifies the length of the suffix, and the two are then used to specify the actual last significant position (for X and Y). |
| #1395 | collocated_ref_idx inference seems missing | CLOSED: The suggested semantic is present in the 2016/12 release of the HEVC specification. |
| #1398 | Typos in the Text | The 2016/12 release of the HEVC specification was checked and: - The typo ‘resample_phase_set_prsent_flag’ is not present. - This aspect is agreed by the authors of this AHG report. The redundant ‘(‘ in subclause F.7.3.2.3.6 is still present. Fix: In F.7.3.2.3.6, change idxShiftY = idxY + ( ( i << ( cm_octant_depth − inpDepth ) ) to idxShiftY = idxY + ( i << ( cm_octant_depth − inpDepth ) ) - The issue regarding xPb/yPb is not present (note: now affected clause is H.8.1.4.3 and the variables were renamed to xP/yP). - xP/yP are now the correct names to use (so no action needed). |
| #1404 | Input samples to INTRA_PLANAR (JCTVC_R1013_v6) | INVALID: The intra ‘reference samples’ are always defined in this way (2 x nTbS) even if a particular intra prediction mode does not use all of them (e.g., see also INTRA_DC) – that way the text for ‘sample array p’ in 8.4.4.2.1 is simpler. |
| #1415 | tsShift calculation with extended_precision_processing_flag | Agreed by the authors of this AHG report. Fix: Replace Equation 8-29 with tsShift = 5 + Log2( nTbS ) |
| #1436 | non-used variable "controlParaAct" | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification (subclause 8.4.4.1 uses controlParaAct). |
| #1439 | IRAP constraint for Main 4:4:4 Intra profile | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1448 | offset_len_minus1 range | WONTFIX: It was agreed that 1-byte slice segments don’t make sense due to CABAC initialization. However, for those cases where it is not easy to figure out the "real" least value, it is sufficient to give a less restrictive value range. Therefore, this ticket can be closed without an action. |
| #1451 | not clear spelling | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification (an explicit list of the PCM-related syntax is provided). |
| #1454 | ambiguous formula | The initially reported issue (missing parentheses) is not present in the 2016/12 release of the HEVC specification. Update on the second comment, reproduced below: “It seems it is not possible (but me had not found any explicit constraint in the spec) to refer from POC=15 to POC=16 in a long-term sense, coz (8-5) will compute POC=0 as a reference, not 16.” The second comment is not clear, and anyway it seems asking for a clarification of the spec not reporting a bug. Asked the reporter to explain in more detail of the comment. |
| #1455 | Typo on transformSkip & rotateCoeffs | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1403 | un-used index in the loop in eq. (F-53) (JCTVC-R1013_v6) | Agreed by the authors of this AHG report (checked 2016/12 release of the HEVC specification). Fix: In F.7.4.1, Equation F-54, replace for( i = 0, j = 0; i < NumActiveRefLayerPics; i++ ) with for( i = 0; i < NumActiveRefLayerPics; i++ ) |
| #1390 | Duplicated lines in 8.4.4.2.2 | Agreed by the authors of this AHG report (checked 2016/12 release of the HEVC specification). Fix: In 8.4.4.2.2, remove the following bullet item from the list of inputs to the process: - a variable cIdx specifying the colour component of the current block, |
| #1306 | ScalingFactor for chroma 32x32 mismatch with reference code | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification (the 32x32 chroma scaling list for 4:4:4 is set directly from ScalingList, and the DC coefficient is set using scaling_list_dc_coef_minus8. |
| #1305 | Decoding process for chroma residual blocks, inconsistent for 444 | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1319 | Superfluous condition on residualDpcm in intra blocks | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1363 | Log2MaxTrafoSize is not defined | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1424 | nCS1L is not defined but used | Agreed by the authors of this AHG report (checked 2016/12 release of the HEVC specification). Fix: In 8.5.2, change the luma location ( xBl, yBl ) set equal to ( nCS1L + ( nCbSL >> 2 ), 0 ) to the luma location ( xBl, yBl ) set equal to ( ( nCbSL >> 1 ) + ( nCbSL >> 2 ), 0 ) |
| #1425 | refIdxL10zeroCandm and mvL10zeroCandm has typo | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1426 | split_cu_flag specifies whether a coding quadtree is split into coding quadtrees. | WONTFIX: The coding quadtree is the whole tree structure that splits the CTU into CUs. Hence saying that a quadtree is split into coding quadtrees is ambiguous as well and would not improve the current wording. Splitting a CU into four CUs would turn that CU into a coding quadtree node while not splitting would let the CU become a leaf and remain a CU. Although the wording might not be ideal, we cannot think of a better wording so we suggest to keep the current one and close the ticket without an action. |
| #1427 | (8-155) and (8-157) do not seem to be used | Agreed by the authors of this AHG report (checked 2016/12 release of the HEVC specification). In fact, (8-155) and (8-157) could be removed as refIdxB set in 3. Used in 5. is set again in 5. because availableFlagLXB is re-initialized to 0 in 5. Up to the editors discretion to remove the un-used equations. |
| #1428 | there seems to be a typo about anticlockwise_rotation | The reported issue (0x8000000 should be 0x8000) is present in the NOTE in the 2016/12 release of the HEVC specification. Agreed by the authors of this AHG report. Fix: In D.3.17, the NOTE, change both instances of "0x8000000" to "0x8000". |
| #1302 | WP unnecessary condition for log2Wd < 1 | Agreed by the authors of this AHG report: The condition is redundant. Fix: In Equations 8-277 and 8-278, remove the condition checks “if( log2Wd >= 1 )” and the associated “else” clauses. |
| #1200 | Problems in 9.3.3.2 Truncated Rice (TR) binarization process | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1348 | Inconsistency in HEVC spec and between the spec and HM | CLOSED: As discussed as part of the ticket, there is nothing wrong in the specification text. The third aspect of the comment regarding the inconsistency between HM encoder and HEVC specification was checked against current HM and the mismatch is fixed. |
| #659 | "byte_alignment" presence condition in the slice_data syntax is not correct for tiles_enabled_flag=1 and entropy_coding_sync_enabled_flag=1 | CLOSED: - The byte_alignment aspect is not present in the 2016/12 release of the HEVC specification. - The “current quantization group” aspect (ticket 740 is duplicate of this aspect). - The initialization and memorization condition in 9.3 includes support for simultaneous use of tiles and wavefront (e.g., as is possible in the High Throughput profiles). |
| #826 | Monochrome support | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #843 | Luma and chroma motion vector cleanup | WONTFIX: The separated luma and chroma motion vectors have been used to handle different chroma formats, thus the present HEVC specification text is does not need modification. Suggest to close the ticket without an action. |
| #958 | POC difference 0 occurs in text of JCTVC-L0030-v3 | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #967 | 9.2.3.2.2 : redundant condition for codIOffset and codIRange | WONTFIX: The bitstream constraint may be redundant (considering the constraint present in subclause 9.3.2.6 initialization process for the arithmetic decoding engine of the 2016/12 release of the HEVC specification), however the presence of this constraint did not appear to introduce any issue or ambiguity into the text. Suggest to close the ticket without an action. |
| #1046 | Possible errors in semantics for scaling_list data | CLOSED: The range for loop variable issue and the default for DC value issue are not present in the 2016/12 release of the HEVC specification. |
| #1055 | Typo in Section 9.3.3.9 of v34 | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1067 | mismatch between HEVC text and HM reference software | WONTFIX: Although the derivation process for collocated motion vectors is specified at the block level, performing aspects of this process at the picture level is also possible without altering the output of this process. Suggest to close the ticket without an action. |
| #1075 | collocated_from_l0 vs collocated_from_l0_flag | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1077 | Log2MinIpcmCbSizeY range issue when MinCbLog2SizeY = 6 | INVALID: As commented, the range constraint on Log2MaxIpcmCbSizeY prevents occurance of the invalid range. Suggest to close the ticket without an action. |
| #1086 | Sum of the NumBytesInNalUnit equation for access unit n need correction | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1091 | Description of Truncated Rice binarization | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1092 | Typo in 9.3.3.9 Binarization process for coeff_abs_level_remaining | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1110 | Clarification needed in SPS ordering information semantics | CLOSED: The suggested clarification is present in the 2016/12 release of the HEVC specification. |
| #1114 | Question about log2_max_mv_length_horizontal | Agreed: The looser restriction (16 not 15) is still present in the VUI parameters semantics. Moreover, there seems a decision to set to 15 (see comment on ticket 1384). CLOSED: The suggested semantic is present in the "Last Call" text for ITU-T HEVC text version 5. |
| #1131 | A typo in vui_num_ticks_poc_diff_one_minus1 semantics | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1151 | A typo in max_bits_per_min_cu_denom semantics | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1158 | Text bug in semantics for tone mapping information SEI | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1163 | Undefined variable in Derivation process for collocated motion vectors | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1170 | reserved value in Table E-4 | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification (including with respect to later additions to the table). |
| #1174 | Improvement of HEVC ver.1 defect report | CLOSED: This was addressed long ago. |
| #1228 | minor correction to Table 9-37 | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1268 | Typographical errors (various sections) | Two instances of ‘curent’ were found in the 2016/12 release of the HEVC specification. Fix: Search and replace all instances of "curent" with "current". All other reported errors in the ticket have been addressed already. |
| #1309 | Potential for reference pictures to be reactivated across a CVS boundary | INVALID: The behaviour is clarified in comment history. |
| #1048 | Typo in | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1057 | num_long_term_sps | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1081 | Redundant condition in availability calculation | CLOSED: The redundant condition is harmless. |
| #1164 | A close bracket in Equation (C-11) | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1189 | wrong cross-reference | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |
| #1013 | missing read_bits(1) when pcm_flag is decoded as 1 | INVALID: For end_of_slice_segment_flag the ‘one bit’ corresponds with rbsp_stop_one_bit – part of subclause 7.3.2.11 rbsp_trailing_bits(), part of subclause 7.3.2.10 rbsp_slice_segment_trailing_bits(), which follows slice_segment_data, which terminates with a end_of_slice_segment_flag equal to one, so again a redundant ‘one bit’ is avoided. For end_of_subset_one_bit the ‘one bit’ corresponds with alignment_bit_equal_to_one – part of subclause 7.3.2.12 byte_alignment(), which follows end_of_subset_one_bit so avoids a redundant ‘one bit’. In both cases, a one-valued flag decoded at a lower level syntax table is effectively reused as a flag required to be one at a higher level syntax table(where CABAC decoding is terminated). When terminating CABAC due to pcm_flag equal to one, zero or more pcm_alignment_zero_bit follow pcm_flag but the coding unit syntax table continues after the PCM block. As such, there is no redundant ‘one bit’ introduced by a higher level syntax table that can be avoided by also re-interpreting the pcm_flag (as is done in the other two cases). |
| #1312 | H.265 official spec, 8.5.3.2.8 "Derivation process for collocated motion vectors", currPb instead of colPb. | CLOSED: This issue is not present in the 2016/12 release of the HEVC specification. |

Additionally, it was also noted an inconsistency for the space character when a clause in the standard was referred: in some cases the normal space character ‘ ’ is used whilst in others the CTRL+space character is inserted instead.

Another typo in the 2016/12 release of the HEVC specification was reported to be present in section G.14.3.3 3D reference displays information SEI message semantics. In the semantics of num_sample_shift_plus512 change the following:

- Otherwise, ( ( num_sample_shift_plus512[ i ] − 512 ) is greater than 0 ), it is recommended that the left view in the stereo pair corresponding to the i-th reference baseline and the i-th reference display should be shifted in the right direction by ( 512 − num_sample_shift_plus512[ i ] ) samples with respect to the right view of the stereo pair.

to

- Otherwise, ( ( num_sample_shift_plus512[ i ] − 512 ) is greater than 0 ), it is recommended that the left view in the stereo pair corresponding to the i-th reference baseline and the i-th reference display should be shifted in the right direction by ( num_sample_shift_plus512[ i ] − 512 ) samples with respect to the right view of the stereo pair.

# Recommendations

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Resolve open issues according to the issue tracker status for v5 of HEVC.