| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 31st Meeting: San Diego, US, 13–20 Apr. 2018 | Document: JCTVC-AE0002 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Chris Rosewarne (Canon) Jens-Rainer Ohm (RWTH Aachen) Karl Sharman (Sony) Gary J. Sullivan (Microsoft) Ye-Kui Wang (Huawei) | Email: | benjamin.bross@hhi.fraunhofer.de chris.rosewarne@cisra.canon.com.au ohm@ient.rwth-aachen.de karl.sharman@eu.sony.com garysull@microsoft.com yekui.wang@huawei.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 30th meeting in Gwangju, KR (January 2018) and the 31st meeting in San Diego, US (April 2018).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Develop proposed improvements to the JCTVC-AB1002 HEVC Test Model 16 (HM 16) Update 9 of Encoder Description - Collect reports of errata for the HEVC specification and the HDR technical reports. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |

# Status of AHG2 Work

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

There remain 13 open issues in the above-mentioned issue tracker. These are understood to be resolved, in both the ITU-T HEVC version 5 that is currently being published and the ISO/IEC committee draft of HEVC 4th edition that is in the MPEG output document N17205.

The following input documents were noted that contain some items that may result in changes and corrections that should be included in a JCT-VC output document for HEVC errata:

- JCTVC-AE0021 – “Proposed correction to the indication of the high throughput screen content coding extensions profiles”. This document asserts that that an error has been identified in the definition and signalling of the high throughput screen content coding (SCC) profiles.

- JCTVC-AE0026 – “Refinements to Mastering Display Color Volume SEI message semantics that align to the year 2018 edition of SMPTE ST 2086”. This document suggests changing the semantics of this SEI message to reflect changes to the corresponding SMPTE standard, anticipated to be released in 2018.

- MPEG input document m42317 – “Summary of Voting on ISO/IEC CD 23008-2”.

Additionally, the following errata items were reported:

- In clause 7.4.9.14, change

cu_qp_delta_abs specifies the absolute value of the difference CuQpDeltaVal between the luma quantization parameter of the current coding unit and its prediction.

cu_qp_delta_sign_flag specifies the sign of CuQpDeltaVal as follows:

- If cu_qp_delta_sign_flag is equal to 0, the corresponding CuQpDeltaVal has a positive value.

- Otherwise (cu_qp_delta_sign_flag is equal to 1), the corresponding CuQpDeltaVal has a negative value.

When cu_qp_delta_sign_flag is not present, it is inferred to be equal to 0.

When cu_qp_delta_abs is present, the variables IsCuQpDeltaCoded and CuQpDeltaVal are derived as follows:

IsCuQpDeltaCoded = 1		(7-87)

CuQpDeltaVal = cu_qp_delta_abs * ( 1 − 2 * cu_qp_delta_sign_flag )	(7-88)

The value of CuQpDeltaVal shall be in the range of −( 26 + QpBdOffsetY / 2 ) to +( 25 + QpBdOffsetY / 2 ), inclusive.

to

cu_qp_delta_abs specifies the absolute value of the difference CuQpDeltaVal between the luma quantization parameter of the current coding unit and its prediction.

When cu_qp_delta_abs is present, the variable IsCuQpDeltaCoded is set equal to 1.

cu_qp_delta_sign_flag, when present, specifies the sign of CuQpDeltaVal as follows. When cu_qp_delta_sign_flag is not present, it is inferred to be equal to 0.

The variable CuQpDeltaVal is set as follows:

CuQpDeltaVal = cu_qp_delta_abs * ( 1 − 2 * cu_qp_delta_sign_flag )	(7-87)

The value of CuQpDeltaVal shall be in the range of −( 26 + QpBdOffsetY / 2 ) to +( 25 + QpBdOffsetY / 2 ), inclusive.

- In the title of clause D.3.41.1, change "semanitcs" to "semantics".

- In clause D.3.41.6.3, change Equation D-58 from

ϕ = ϕd * π ÷ 180
θ = θd * π ÷ 180
α = αd * π ÷ 180
β = βd * π ÷ 180
γ = γd * π ÷ 180
x1 = Cos( ϕ ) * Cos( θ )
y1 = Sin( ϕ ) * Cos( θ )
z1 = Sin( θ )
x2 = Cos( β ) * Cos ( γ ) * x1 − Cos( β ) * Sin( γ ) * y1 + Sin( β ) * z1	(D-58)
y2 = ( Cos( α ) * Sin( γ ) + Sin( α ) * Sin( β ) * Cos( γ ) ) * x1 +
	( Cos( α ) * Cos( γ ) − Sin( α ) * Sin( β ) * Sin( γ ) ) * y1 −
	Sin( α ) * Cos( β ) * z1
z2 = ( Sin( α ) * Sin( γ ) − Cos( α ) * Sin( β ) * Cos( γ ) ) * x1 +
	( Sin( α ) * Cos( γ ) + Cos( α ) * Sin( β ) * Sin( γ ) ) * y1 +
	Cos( α ) * Cos( β ) * z1
ϕ′ = Atan2( y2, x2 ) * 180 ÷ π
θ′ = Asin( z2 ) * 180 ÷ π

to (just swapping of α and γ)

ϕ = ϕd * π ÷ 180
θ = θd * π ÷ 180
α = αd * π ÷ 180
β = βd * π ÷ 180
γ = γd * π ÷ 180
x1 = Cos( ϕ ) * Cos( θ )
y1 = Sin( ϕ ) * Cos( θ )
z1 = Sin( θ )
x2 = Cos( β ) * Cos ( α ) * x1 − Cos( β ) * Sin( α ) * y1 + Sin( β ) * z1	(D-58)
y2 = ( Cos( γ ) * Sin( α ) + Sin( γ ) * Sin( β ) * Cos( α ) ) * x1 +
	( Cos( γ ) * Cos( α ) − Sin( γ ) * Sin( β ) * Sin( α ) ) * y1 −
	Sin( γ ) * Cos( β ) * z1
z2 = ( Sin( γ ) * Sin( α ) − Cos( γ ) * Sin( β ) * Cos( α ) ) * x1 +
	( Sin( γ ) * Cos( α ) + Cos( γ ) * Sin( β ) * Sin( α ) ) * y1 +
	Cos( γ ) * Cos( β ) * z1
ϕ′ = Atan2( y2, x2 ) * 180 ÷ π
θ′ = Asin( z2 ) * 180 ÷ π

# Recommendations

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Review the three input documents listed above and the three topics listed above.

- Confirm resolution of the 13 open issues in the issue tracker and close them.