| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 31st Meeting: San Diego, US, 13–20 Apr. 2018 | Document: JCTVC-AE0022r2 |
| --- | --- |

| Title: | Centralized Texture Depth Packing SEI Message for HEVC and AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC – Revision of JCTVC-AD0022 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang (NCKU, Taiwan) Guan-Cheng Chen Kuan-Ting Lee | Tel: Email: | +886-916727713 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University, Taiwan |  |  |

_____________________________

# Abstract

This document is a revision of JCTVC-AD0022 [1]. The purpose of the proposed “Centralized Texture Depth Packing SEI message for HEVC and AVC” is to deliver one texture view plus one depth map packed into the samples of a monoscopic (texture only) stream for HEVC and AVC. In this document, “color_packing_space” is removed and added “chroma usage”. Thus, the color packing space is unified without the need of DPCM coding of packing chroma components. To fit the above revisions, the order of color depacking and flip-and-merge procedures is exchanged.

- Centralized texture depth packing SEI message syntax

- Replace specification of in clause 7.4.4 with the following:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages nor centralized texture depth packing SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages present in the CVS.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages.

In subclause D.2.1 (General SEI message syntax), add the following:

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture_depth_packing( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers of the respective specification.

Add a new subclause D.2.X “centralized_texture_depth_packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text of the respective standard.

D.2.X	Centralized texture depth packing (CTDP) SEI message syntax

| centralized_texture_depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture_depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture_depth_packing_cancel_flag ) { |  |
| ctdp_packing_type | u(3) |
| chroma_subsampling_type | u(1) |
| chroma_usage | u(2) |
| baseline_dist_flag | u(1) |
| focal_length_flag | u(1) |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( baseline_dist_flag ) |  |
| view_syn_rep_info_element( BdistanceSign, BdistanceExp, BdistanceMantissa, BdistanceManLen ) |  |
| if( focal_length_flag ) |  |
| view_syn_rep_info_element( FlengthSign, FlengthExp, FlengthMantissa, FlengthManLen ) |  |
| if( z_near_flag ) |  |
| view_syn_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| view_syn_rep_info_element ( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| view_syn_rep_info_element ( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| view_syn_rep_info_element ( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| centralized_texture_depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |
| } |  |

In Table F.4, add the persistence scope for the centralized_texture_depth_packing SEI message as follows:

| - SEI message | - Persistence scope |
| --- | --- |
| centralized texture depth packing | - Specified by the syntax of the SEI message |

Add a new subclause D.3.X “centralized texture depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following subclause D.3.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.3.X Centralized texture depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains a specific packing arrangement of texture and depth samples associated with one view. This information can be used by a process applied after decoding to appropriately interpret the samples and process them appropriately for display of 3D video or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture_depth_packing_cancel_flag equal to 1 indicates that the centralized texture_depth packing (CTDP) arrangement SEI message cancels the persistence of any previous centralized texture depth packing SEI message in output order. centralized_texture_depth_packing_cancel_flag equal to 0 indicates that centralized texture depth packing is applicable.

ctdp_packing_type indicates the intended interpretation of the constituent frames as specified in Table D-X1.

Table D-X1 – Definition of ctdp_packing_type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the frame of the CTDP-TB type with an arrangement of top colored depth (CDT), texture (TS), and bottom colored depth (CDB) regions from top to bottom, as illustrated in Figure D-X1. |
| 1 | Indicates the frame of the CTDP-LR type with an arrangement of left colored depth (CDL), texture (TS), and right colored depth (CDR) regions from left to right, as illustrated in Figure D-X2. |

Values of ctdp_packing_type that do not appear in Table D-X1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture depth packing SEI messages that contain reserved values of ctdp_packing_type.

chroma_subsampling_type equal to 0 indicates the video coder using direct subsampling to achieve chroma components downsizing. chroma_subsampling_type equal to 1 indicates the video coder using average subsampling to achieve chroma components downsizing for YCbCr 4:2:0 and YCbCr 4:2:2 chroma formats.

chroma_usage indicates the intended interpretation of the usage of chroma components in the colored depth regions as specified in Table D-X2, where the interpretation is based on 4 colored depth pixels, which are equivalent to 12 RGB subpixels or 4 luminance components and 8 chrominance components.

Table D-X2 – Definition of chroma_usage

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the 4 colored depth pixels using 4 luminance components only to carry 4 depth pixels, which could be adopted by the video coders with YCbCr 4:2:0 and YCbCr 4:2:2 chroma formats |
| 1 | Indicates the 4 colored depth pixels using 4 luminance components and 2 chroma components to carry 6 depth pixels, which could be adopted by the video coders with YCbCr 4:2:0 chroma format. |
| 2 | Indicates the 4 colored depth pixels using 4 luminance components and 4 chroma components to carry 8 depth pixels, which could be adopted by the video coders with YCbCr 4:2:2 chroma format. |
| 3 | Indicates 4 colored depth pixels using 4 luminance components and 8 chroma components to carry 12 depth pixels, which could be adopted by the video coders with YCbCr 4:4:4 chroma format. |

The bit depth of the luminance components is BitDepthY = 8 + bit_depth_luma_minus8 and the bit depth of the chorma components is BitDepthC = 8 + bit_depth_chorma_minus8.

The interpretations of ctdp_packing_type and chroma_usage how the CTDP frame to retrieve colored depth regions and the texture region and how to recover them to the original full-resolution depth map and texture frame are described as follows.

It is assumed that the original full-resolution texture frame and depth map are both of size WxH, where W and H are the width and height expressing pic_width_in_luma_samples and pic_height_in_luma_samples, respectively. For chroma_usage, the packing and depacking processes will adopt the same computation of color transform and chroma subsampling in the HEVC/AVC video encoder and the same computation of the inverse color transform and chroma upsamping as the HEVC/AVC video decoder.

For ctdp_packing_type equal to 0, the packed frame from top to bottom is horizontally divided into top colored depth (CDT), texture (TS), and bottom colored depth (CDB) regions. With the width of W in units of frame luma samples, the height of top colored depth (CDT) and bottom colored depth (CDB) regions, called the half depth height, in units of frame luma samples is expressed by

HHD = (H/36)  2.                 	 			(D-X1)

For simplifying CTDP packing and depacking processes, H is assumed to be divisible by 36. Hence, with the width of W, the height of the texture (TS) region in units of frame luma samples becomes:

HT = H  2HHD.		     (D-X2)

As illustrated in Figure D-X1, the CTDP-TB packed frame, by referring HHD defined in (D-X1) and HT defined in (D-X2), is divided into CDT, TS and CDB regions from top to bottom. The height of CDT and CDB regions is HHD and the height of TS region is HT in units of frame luma samples while the width of CDT, TS and CDB regions is W in units of frame luma samples.

Figure D-X1 Arrangement of the CTDP-TB frame and its depacking steps (ctdp_packing type = 0)

For ctdp_packing_type equal to 1, the packed frame from left to right is vertically divided into left colored depth (CDL), texture (TS), and right colored depth (CDR) regions. With the height of H in units of frame luma samples, the width of left and right colored depth (CDL and CDR) regions, called the half depth width, in units of frame luma samples is expressed by

WHD = (W/48)  2.                 	 			(D-X3)

For simplifying CTDP packing and depacking processes, W is assumed to be divisible by 48. Hence, with the height of H, the width of the texture (TS) region in units of frame luma samples becomes:

WT = W  2WHD.	   	  (D-X4)

As illustrated in Figure D-X2, the CTDP-LR packed frame referring to WHD defined in (D-X3) and WT defined in (D-X4) is divided into CDL, TS, and CDR regions from left to right. The width of CDL and CDR regions is WHD and the width of TS region is WT in units of frame luma samples while the height of CDL, TS and CDR regions is H in units of frame luma samples.

Figure D-X2 Arrangement of the CTDP-LR frame and its depacking steps (ctdp_packing_type = 1)

After splitting depth and texture regions, the top and bottom colored depth, i.e., CDT and CDB regions for ctdp_packing type = 0 or left and right colored depth, i.e., CDL and CDR regions for ctdp_packing type = 1 should be respectively depacked to the top and bottom depth, i.e., DT and DB regions for ctdp_packing type = 0 or left and right depth, i.e., DL and DR regions for ctdp_packing type = 1.

For simplicity of figures and equations, C with color depth pixels denotes the color depth region to simpily represent CDT, CDB,, CDL or CDR region while D with single depth pixels represents the depth region, which could be DT, DB, DL or DR regions, One color depth pixel, Cij with its Rij, Gij, and Bij subpixels in C region will be conceptually used to represent parts of three depth pixels in D region.

Figure D-X3 Positions of 4 colored depth pixels and their 12 RGB subpixels with respect to the positions of 12 depacked depth pixels for ctdp_packing_type=0 and tdp_packing_type = 1

To further consider HEVC/AVC video coders, the locations of a set of 4 colored depth pixels (Ci,j, Ci,j+1, Ci+1,j and Cj+1,j+1), which are associated with 12 RGB subpixels mapping to the positions of 12 depth pixels are specified as shown in Figure D-X3. If ctdp_packing_type is equal to 0, the height of depth regions, after horizontal placement of subpixels, is tripled from 2 to become 6, as shown in the left side of Figure X-D3. while the width of the depth is still 2. If ctdp_packing_type is equal to 1, the width of depth regions, after horizontal placement of subpixels, is tripled from 2 to become 6, as shown in the right side of Figure X-D3. while the height of the depth is still 2. For YCbCr4:4:4 chroma format, we can directly depack all 12 RGB subpixels to 12 depth valuse with the same positions. The assignment of 12 depth values and 12 RGB subpixels are straightforward.

For video coders with YCbCr 4:2:0 chroma format, in every 4 colored depth pixels, there are with 4 Y components and Cb and Cr components, which could carry 6 depth pixels being correctly packed. Therefore, the one-to-one assignments of 12 depth values and 12 RGB subpixel values as shown in Figure D-X3 cannot be directly used. The RGB assignment could be conducted as the following:

-  If chroma_usage equals to 0, only four Y components are used to carry four selected depth pixels as shown in Figure D-X4. In the packing side, in every 4 color depth pixels with 12 RGB subpixel values, only 4 depth values are selected for packing and depacking processes.

For ctdp_packing_type=0, in the depacking process, four selected depth values marked in white color in the left side of Figure D-X4 are depacked as:

D3i,j = Ri,j,                                            (D-X5)

D3i+4,j = Gi+1,j,                                         (D-X6)

D3i+1,j+1 = Gi,j+1,                                        (D-X7)

D3i+5,j+1 = Bi+1,j+1.                                      (D-X8)

The reminding 8 depth values in dark gray color need to be interpolated by referring 4 selected depth values and associated texture information. Finally, the height of the top and bottom colored depth, CDT and CDB regions, HHD after color depecting is tripled to 3HHD as the height of the top and bottom depth, DT and DB regions while the width of all regions is W in units of frame luma samples.

Accordingly, in the packing process, we assign 12 RGB subpixels with 4 selected depth values as shown in the left side of Figure D-X4 to become::

Ri,j  = Gi,j =  Bi,j  =  D3i,j,                                (D-X9)

Ri+1,j = Gi+1,j = Bi+1,j = D3i+4,j,                              (D-X10)

Ri.j+1 = Gi,j+1 = Bi,j+1 = D3i+1,j+1,                            (D-X11)

Ri+1,j+1 = Gi+1,j+1 = Bi+1,j+1 = D3i+5,j+1.                        (D-X12)

Figure D-X4 Assignments of 12 RGB subpixels related to 4 selected depth pixels for chroma_usage = 0

For ctdp_packing_type=1, in the color depacking process, four selected depth values marked in white color in the right side of Figure D-X4 are depacked as:

Di,3j = Ri,j,                                            (D-X13)

Di,3j+4 = Gi,j+1,                                          (D-X14)

Di+1,3j+1 = Gi+1,j,                                         (D-X15)

Di+1,3j+5 = Bi+1,j+1.                                        (D-X16)

The reminding 8 depth values in dark gray color need to be interpolated by referring 4 selected depth values and associated texture information. Finally, the width of the left and right colored depth, CDL and CDR regions, WHD after color depecting is tripled to 3WHD as the width of the left and right depth, DL and DR regions while the height of all regions is H in units of frame luma samples.

Accordingly, in the color packing process, we assign 12 RGB subpixels as shown in the right side of Figure D-X4 to become:

Ri,j  =  Gi,j =  Bi,j  = Di,3j,                               (D-X17)

Ri.j+1 = Gi,j+1 = Bi,j+1 = Di,3j+4,                             (D-X18)

Ri+1,j = Gi+1,j = Bi+1,j = Di+1,3j+1,                            (D-X19)

Ri+1,j+1 = Gi+1,j+1 = Bi+1,j+1 = Di+1,3j+5.                        (D-X20)

For the above assignments stated in (D-X9) - (D-X12) and (D-X17) - (D-20) in the packing process, we choose R. G and B components to be the same, which will force all chroma components to be the mid-gray level as C0 = (1 << BitDepthC) If video coding system performs chroma components subsampling and upsampling, the results will be all equal to C0. Thus, we can recover the four selected depth values directly from the values of RGB subpixels in their associated positions without any adjustments.

-  If chroma_usage equals to 1, four Y components are used to carry four selected depth pixels and two chroma components are further used to convey two additional depth pixels as shown in Figure D-X5. In the packing side, in every 4 color depth pixels with 12 RGB subpixel values, only 6 depth values in quincunx positions marked in white color, are selected in packing and depacking processes.

Figure D-X5 Assignments of 12 RGB subpixels related to 6 selected depth pixels for chroma_usage = 1

For ctdp_packing_type=0, all 12 the RGB subpixel values could be modified since their associated chroma components are modified by YCbCr 4:2:0 color transform. In general, we assume that the color packing and color depacking processes are conducted in the RGB domain. Here, the color packing process is described first. As shown in the left side of Figure D-X5, in the packing process, we assign the RGB subpixels for the first colored depth pixel (Ci,j) as:

Ri,j = D3i,j,                                       (D-X21)

Gi,j = D3i+3,j+1,                                    (D-X22)

Bi,j = D3i+2,j,                                      (D-X23)

With the above assignments, the chroma components are not C0 for the first colored depth pixel in general. For the reminding three sets of RGB subpixels, the assignments of RGB pixel values are still the same as those stated in (D-X10), (D-X11), and (D-X12). The chroma components for these three colored depth pixels are still all 128. Thus, we have four sets of YCbCr components as {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0 }, {Yi+1,j, C0, C0 }and {Yi+1,j+1, C0, C0 } in the packing side before the video encoding process, where C0 = (1 << BitDepthC).

Figure D-X6 Adjustment in YCbCr color space for compensation of chroma 4:2:0 subsampling and upsampling

Figure D-X6 shows the adjustments in YCbCr color space for the compensation of chroma 4:2:0 subsampling and upsampling after video coding system. Normally, after the video coding and decoding processes, the four sets of YCbCr components become {Yi,j, Cb’i,j, Cr’i,j}, {Yi,j+1, Cb’i,j, Cr’i,j}, {Yi+1,j, Cb’i,j, Cr’i,j} and {Yi+1,j+1, Cb’i,j, Cr’i,j} after the chroma subsampling in the video encoder and the upsampling in video decoder if we ignore the coding loss. If chroma_subsampling_type equals to 0, the subsampled chroma components in the encoder are Cb’i,j = Cbi,j and Cr’i,j = Cri,j for direct subsampling of 4 chroma components. If chroma_subsampling_type equals to 1, the subsamples in the encoder are Cb’i,j= (Cbi,j+ 3*C0 )/4, Cr’i,j= (Cri,j+ 3*C0 )/4 for average subsampling of 4 chroma components. If we can access YCbCr components in the depacking process, with 4 fixed Y components, we can easily adjust the two chroma components as Cbi,j = Cb’i,j and Cri,j = Cr’i,j for direct subsampling or Cbi,j = 4Cb’i,j 3*C0  and Cri,j = 4Cr’i,j 3*C0 for average subsampling of chorma components. In the color depacking process, if YCbCr components can be accessed, as shown in Figure D-X6, we can fully recover the four sets of YCbCr components, {Yi,j, Cb’i,j, Cr’i,j}, {Yi,j+1, Cb’i,j, Cr’i,j}, {Yi+1,j, Cb’i,j, Cr’i,j} and {Yi+1,j+1, Cb’i,j, Cr’i,j} to obtain {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, C0, C0} and {Yi+1,j+1, C0, C0}.

In the color depacking process, if the access of YCbCr components is not possible, we only have 4 sets of deviated RGB subpixels as {R’i,j, G’i,j, B’i,j}, {R’i,j+1, G’i,j+1, B’i,j+1}, {R’i+1,j, G’i+1,j, B’i+1,j} and {R’i+1,j+1, G’i+1,j+1, B’i+1,j+1}. As shown in Figure D-X7, we only need to additionally perform the color transform for all color pixels as to obtain {Yi,j, Cb’i,j, Cr’i,j}, {Yi,j+1, Cb’i,j, Cr’i,j}, {Yi+1,j, Cb’i,j, Cr’i,j} and {Yi+1,j+1, Cb’i,j, Cr’i,j} first and then perform the same YCbCr adjustments to obtain {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, C0, C0}and {Yi+1,j+1, C0, C0}. Finally, the adjusted YCbCr components after inverse color transform can fully recover all assigned RGB subpixels as {Ri,j, Gi,j, Bi,j}, {Ri,j+1, Gi,j+1, Bi,j+1}, {Ri+1,j, Gi+1,j, Bi+1,j } and {Ri+1,j+1, Gi+1,j+1, Bi+1,j+1}. Thus, the 6 selected depth values are finally depacked as

D3i,j = Ri,j,                                            (D-X24)

D3i+2,j = Bi,j,                                           (D-X25)

D3i+4,j = Gi+1,j,                                         (D-X26)

D3i+1,j+1 = Gi,j+1,                                        (D-X27)

D3i+3,j+1 = Gi,j,                                         (D-X28)

D3i+5,j+1 = Bi+1,j+1.                                      (D-X29)

Figure D-X7 Adjustment in RGB color space for compensation of chroma 4:2:0 subsampling and upsampling

The reminding 6 depth values in dark gray color need to be interpolated by referring 6 selected depth values and associated texture information. Finally, the height of the top and bottom colored depth, CDT and CDB regions, HHD after color depecting is tripled to 3HHD as the height of the top and bottom depth, DT and DB regions while the width of all regions is W in units of frame luma samples.

For ctdp_packing_type=1, all 12 the RGB subpixel values could be modified since their associated chroma components are modified by YCbCr 4:2:0 color transform. For clear explanations, the color packing process is described first. As shown in the right side of Figure D-X5, in the packing process, we assign RGB subpixels for the first color pixel (Ci,j) as:

Ri,j = Di,3j,                                       (D-X30)

Gi,j = Di+1,3j+3,                                    (D-X31)

Bi,j =  Di,3j+2,                                     (D-X32)

With the above assignments, the chroma components are not 128 for the first color pixel in general. For the reminding three sets of RGB subpixels, the assignments of RGB pixel values are stated in (D-X18), (D-X19), and (D-X20), which are still with the same assignments for chroma_usage = 0. The 6 selected depth pixels are in quincunx positions. The chroma components for these three color pixels are still all C0. Thus, we have four sets of YCbCr components as {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, C0, C0}and {Yi+1,j+1, C0, C0} originally, , where C0 = (1 << BitDepthC).

Similarly, Figure D-X6 shows the adjustments in YCbCr color space while Figure D-X7 exhibits the adjustments in RGB color space to obtain {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, C0, C0}and {Yi+1,j+1, C0, C0}and  to recover all the adjusted RGB subpixels as {Ri,j, Gi,j, Bi,j}, {Ri,j+1, Gi,j+1, Bi,j+1}, {Ri+1,j, Gi+1,j, Bi+1,j } and {Ri+1,j+1, Gi+1,j+1, Bi+1,j+1}. The 6 selected depth values are finally depacked as

Di,3j = Ri,j,                                            (D-X33)

Di,3j+2 = Bi,j,                                           (D-X34)

Di,3j+4 = Gi,j+1,                                          (D-X35)

Di+1,3j+1 = Gi+1,j,                                         (D-X36)

Di+1,3j+3 = Gi,j.                                          (D-X37)

Di+1,3j+5 = Bi+1,j+1.                                        (D-X38)

The reminding depth values in dark gray color need to be interpolated by referring 6 selected depth values and associated texture information. Finally, the width of the left and right colored depth, CDL and CDR regions, WHD after color depecting is tripled to 3WHD as the width of the left and right depth, DL and DR regions while the height of all regions is H in units of frame luma samples.

- If chroma_usage equals to 2, four Y components are used to carry four selected depth pixels and four chroma components i.e., 2 Cb and 2 Cr, are used to convey four additional depth pixels as shown in Figure D-X8. In the packing side, in every 4 color depth pixels with 12 RGB subpixel values, only 8 depth values marked in white color are selected in packing and depacking processes. In this case, the video coding system is with YCbCr 4:2:2 chroma format.

Figure D-X8 Assignments of 12 RGB subpixels related to 8 selected depth pixels for chroma_usage = 2

For ctdp_packing_type=0, all 12 the RGB subpixel values could be modified since their associated chroma components are modified by YCbCr 4:2:2 color transform. In general, we assume that the color packing and color depacking processes are conducted in RGB domain. Here, the color packing process is described first. As shown in the left side of Figure D-X8, in the packing process, we assign RGB subpixels for the first colored depth pixel (Ci,j) and the third colored depth pixel (Ci+1,j) as:

Ri,j = D3i,j,                                       (D-X39)

Gi,j = D3i+1,j+1,                                    (D-X40)

Bi,j = D3i+2,j,                                      (D-X41)

Ri+1,j = D3i+3,j+1,                                   (D-X42)

Gi+1,j = D3i+4,j,                                    (D-X43)

Bi+1,j = D3i+5,j,                                     (D-X44)

With the above assignments, the chroma components are not C0 for the first and third colored depth pixels in generals. For the second (Ci,j+1) and fourth (Ci+1,j+1) colored depth pixels, their subpixels are still assigned as

Ri.j+1 = Gi,j+1 = Bi,j+1 = D3i,j+1,                             (D-X45)

Ri+1,j+1 = Gi+1,j+1 = Bi+1,j+1 = D3i+5, j+1.                       (D-X46)

Since these two sets of RGB subpixel values are the same, the chroma components for these two set colored depth pixels are still all 128. Thus, we have four sets of YCbCr components as {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, Cbi+1,j, Cri+1+1,j} and {Yi+1,j+1, C0, C0} before the video encoding process originally.

Similar as Figure D-X6, the adjustments in YCbCr color space is still needed for the compensation of chroma 4:2:2 subsampling and upsampling after video coding system. Normally, four sets of YCbCr components become {Yi,j, Cb’i,j, Cr’i,j}, {Yi,j+1, Cb’i,j, Cr’i,j}, {Yi+1,j, Cb’i+1,j, Cr’i+1,j} and {Yi+1,j+1, Cb’i+1,j, Cr’i+1,j} after the chroma subsampling in the video encoder and the upsampling in video decoder if we ignore the coding loss. If chroma_subsampling_type equals to 0, the subsampled chroma components in the encoder are Cb’i,j=Cbi,j, Cb’i+1,j = Cbi+1,j and Cr’i,j = Cri,j, Cr’i+1,j = Cri+1,j for direct subsampling of chroma components. If chroma_subsampling_type equals to 1, the subsampled chroma components are Cb’i,j= (Cbi,j+C0)/2, Cb’i+1,j = (Cbi+1,j+C0)/2, Cr’i,j = (Cri,j+C0)/2 and Cb’i+1,j = (Cbi+1,j+C0)/2 for average subsampling of two chroma components. If we can access YCbCr components in the depacking process, with 4 fixed Y components, we can easily restore the two chroma components easily by Cbi,j = Cb’i,j, Cbi+1,j = Cb’i+1,j and Cri,j = Cr’i,j Cri+1,j = Cr’i+1,j for direct subsampling or Cbi,j = 2Cb’i,jC0, Cbi+1,j = 2Cb’i+1,jC0 and Cri,j = 2Cr’i,jC0, Cbi+1,j = 2Cb’i+1,jC0 for average subsampling of chroma components. By using chroma_subsampling_type, it is noted that the color depacker will know if the video encoder adopts direct or average subsampling in downsizing the chroma components such that it can properly recover the first and third RGB subpixels, {Ri,j, Gi,j, Bi,j } and {Ri+1,j, Gi+1,j, Bi+1,j }. As to the second and fourth set of RGB subpixels, we can set their chroma components to C0. Thus, we can also recover {Ri,j+1, Gi,j+1, Bi,j+1 } and {Ri+1,j+1, Gi+1,j+1, Bi+1,j+1 }

In the color depacking process, if the access of YCbCr components is not possible, we only have 4 sets of deviated RGB subpixels as {R’i,j, G’i,j, B’i,j}, {R’i,j+1, G’i,j+1, B’i,j+1}, {R’i+1,j, G’i+1,j, B’i+1,j} and {R’i+1,j+1, G’i+1,j+1, B’i+1,j+1}. Similar to Figure D-X7, we perform the same color transform for all color pixels as the video coder to obtain {Yi,j, Cb’i,j, Cr’i,j}, {Yi,j+1, Cb’i,j, Cr’i,j}, {Yi+1,j, Cb’i+1,j, Cr’i+1,j} and {Yi+1,j+1, Cbi+1,j, Cri+1,j} first and then perform the adjustments to obtain {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, Cbi+1,j, Cri+1,j}and {Yi+1,j+1, C0, C0}. Finally, the adjusted YCbCr components are inverse color transform to obtain the adjusted RGB subpixels as {Ri,j, Gi,j, Bi,j}, {Ri,j+1, Gi,j+1, Bi,j+1}, {Ri+1,j, Gi+1,j, Bi+1,j } and {Ri+1,j+1, Gi+1,j+1, Bi+1,j+1}. The 6 selected depth values are finally depacked as

D3i,j = Ri,j,                                            (D-X47)

D3i+2,j = Bi,j,                                           (D-X48)

D3i+4,j = Gi+1,j,                                         (D-X49)

D3i+5,j = Bi+1,j.                                         (D-X50)

D3i,j+1 = Ri,j+1,                                         (D-X51)

D3i+1,j+1 = Gi,j,                                          (D-X52)

D3i+3,j+1 = Ri+1,j,                                        (D-X53)

D3i+5,j+1 = Bi+1,j+1.                                      (D-X54)

For ctdp_packing_type=1, all 12 the RGB subpixel values could be modified since their associated chroma components are modified by YCbCr 4:2:2 color transform. For interlace video with YCbCr 4:2:2, the even and odd horizontal lines, which could be coded in separate fields, will be handled with the same color packing and depacking. To fit the possible interlace video, it is noted that the depth pixels for packing are not selected in quincunx fashions. In general, we assume that the color packing and color depacking processes are conducted in RGB domain. Here, the color packing process is described first. As shown in the right side of Figure D-X8, in the packing process, we assign RGB subpixels for the first colored depth pixel (Ci,j) and the third colored depth pixel (Ci+1,j) as:

Ri,j = Di,3j,                                        (D-X55)

Gi,j = Di,3j+1,                                       (D-X56)

Bi,j = Di,3j+3,                                       (D-X57)

Ri+1,j = Di+1,3j,                                     (D-X58)

Gi+1,j = Di+1,3j+1,                                    (D-X59)

Bi+1,j = Di+1,3j+3,                                    (D-X60)

With the above assignments, the chroma components are not C0 for the first and third color pixel in generals. For the second (Ci,j+1) and fourth (Ci+1,j+1) color pixels, the subpixels are assigned as

Ri.j+1 = Gi,j+1 = Bi,j+1 = Di,3j+5,                             (D-X61)

Ri+1,j+1 = Gi+1,j+1 = Bi+1,j+1 = Di+1,3j+5.                        (D-X62)

Since these two sets of RGB subpixel values are the same, the chroma components for these two sets of colored depth pixels are still all 128. Thus, we have four sets of YCbCr components as {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, Cbi+1,j, Cri+1+1,j} and {Yi+1,j+1, C0, C0} before the video encoding process originally.

Similar as Figure D-X6, the adjustments in YCbCr color space is still needed for the compensation of chroma 4:2:2 subsampling and upsampling after video coding system. Normally, four sets of YCbCr components become {Yi,j, Cb’i,j, Cr’i,j}, {Yi,j+1, Cb’i,j, Cr’i,j}, {Yi+1,j, Cb’i+1,j, Cr’i+1,j} and {Yi+1,j+1, Cb’i+1,j, Cr’i+1,j} after the chroma subsampling in the video encoder and the upsampling in video decoder if we ignore the coding loss. The subsamples in the encoder are Cb’i,j=Cbi,j, Cb’i+1,j=Cbi+1,j and Cr’i,j= Cri,j, Cr’i+1,j=Cri+1,j for direct subsampling of chroma components or Cb’i,j= (Cbi,j+ C0)/2, Cb’i+1,j= (Cbi+1,j+ C0)/2, Cr’i,j= (Cri,j+ C0)/2 and Cb’i+1,j= (Cbi+1,j+ C0)/2 for average subsampling of chroma components. If we can access YCbCr components in the depacking process, with 4 fixed Y components, we can easily restore the two chroma components easily by Cbi,j = Cb’i,j, Cbi+1,j = Cb’i+1,j and Cri,j = Cr’i,j Cri+1,j = Cr’i+1,j for direct subsampling or Cbi,j = 2Cb’i,jC0, Cbi+1,j = 2Cb’i+1,jC0 and Cri,j = 2Cr’i,jC0, Cbi+1,j = 2Cb’i+1,jC0 for average subsampling of chroma components. It is noted that the color depacker should know if the video encoder adopts direct or average subsampling in downsizing the chroma components such that it can properly recover the first and third RGB subpixels, {Ri,j, Gi,j, Bi,j } and {Ri+1,j, Gi+1,j, Bi+1,j }. As to the second and fourth set of RGB subpixels, we can set their chroma components to 128. Thus, we can also recover {Ri,j+1, Gi,j+1, Bi,j+1 } and {Ri+1,j+1, Gi+1,j+1, Bi+1,j+1 }

In the color depacking process, if the access of YCbCr components is not possible, we only have 4 sets of deviated RGB subpixels as {R’i,j, G’i,j, B’i,j}, {R’i,j+1, G’i,j+1, B’i,j+1}, {R’i+1,j, G’i+1,j, B’i+1,j} and {R’i+1,j+1, G’i+1,j+1, B’i+1,j+1}. Similar to Figure D-X7, we perform the same color transform for all colored depth pixels as the video coder to obtain {Yi,j, Cb’i,j, Cr’i,j}, {Yi,j+1, Cb’i,j, Cr’i,j}, {Yi+1,j, Cb’i+1,j, Cr’i+1,j} and {Yi+1,j+1, Cbi+1,j, Cri+1,j} first and then perform the adjustments to obtain {Yi,j, Cbi,j, Cri,j}, {Yi,j+1, C0, C0}, {Yi+1,j, Cbi+1,j, Cri+1,j}and {Yi+1,j+1, C0, C0}. Finally, the adjusted YCbCr components are inverse color transform to obtain the adjusted RGB subpixels as {Ri,j, Gi,j, Bi,j}, {Ri,j+1, Gi,j+1, Bi,j+1}, {Ri+1,j, Gi+1,j, Bi+1,j } and {Ri+1,j+1, Gi+1,j+1, Bi+1,j+1}. The 6 selected depth values are finally depacked as

Di,3j = Ri,j,                                            (D-X63)

Di,3j+1 = Gi,j,                                           (D-X64)

Di,3j+3 = Bi,j,                                           (D-X65)

Di,3j+5 = Bi,j+1.                                         (D-X66)

Di+1,3j = Ri+1,j,                                         (D-X67)

Di+1,3j+1 =Gi+1,j,                                         (D-X68)

Di+1,3j+3 = Bi+1,j,                                        (D-X69)

Di+1,3j+5 = Bi+1,j+1.                                      (D-X70)

- If chroma_usage equals to 3, the chroma components will not have subsampling and upsampling processes involved for for YCbCr 4:4:4 format. As shown in Figure D-X3, 12 depth pixels are directly mapped to 12 RGB subpixel values.

For ctdp_packing_type=0, the corresponding subpixel and depth pairs as shown in the left size of Figure D-X3 are (Ri,j, D3i,j), (Gi,j, D3i+1,j), (Bi,j, D3i+2,j), (Ri,j+1, D3i,j+1), (Gi,j+1, D3i+1,j+1), (Bi,j+1, D3i+2,j+1), (Ri+1,j, D3i+3,j), (Gi+1,j, D3i+4,j), (Bi+1,j, D3i+5,j), (Ri+1,j+1, D3i+3,j+1), (Gi+1,j+1, D3i+4,j+1), and (Bi+1,j+1, D3i+5,j+1) in color packing and depacking processes.

For ctdp_packing_type=1, the corresponding subpixel and depth pairs as shown in the right size of Figure D-X3 are (Ri,j, Di,3j), (Gi,j, Di,3j+1), (Bi,j, Di,3j+2), (Ri,j+1, Di,3j+3), (Gi,j+1, Di,3j+4), (Bi,j+1, Di,3j+5), (Ri+1,j, Di+1,3j), (Gi+1,j, Di+1,3j+1), (Bi+1,j, Di+1,3j+2), (Ri+1,j+1, Di+1,3j+3), (Gi+1,j+1, Di+1,3j+4), and (Bi+1,j+1, Di+1,3j+5) in color packing and depacking processes

After colored depth to depth conversion, the paired top and bottom depth DT and DB or paired left and right depth, DL and DR regions should be merged together to become one depth (DS) region as follows.

–	If ctdp_packing_type is equal to 0, as shown in Figure D-X1, the paired top and bottom depth regions, which are denoted as DT and DB in RGB representation, are vertically flipped and merged to obtain one depth (DS) region. For the paired top and bottom depth regions, DT and DB, the depth region (DS) after flipping and merging processes is expressed by:

for( x = 0; x < W; x++ )

for( y = 0; y < 3HHD; y++ ) {

DS[x][y] = DT[x][3HHD y 1]

DS[x][3HHD + y] = DB[x][3HHD  y 1]

}

where 3HHD is the height of top and bottom depth regions in units of frame luma samples. After vertically flipping and merging processes as shown in Figure D-X1, the height of the depth (DS) regions, becomes 6HHD, while the width of the depth is W in units of frame luma samples.

–	If ctdp_packing_type is equal to 1, as shown in Figure D-X2, the paired left and right depth regions, which are denoted as DL and DR in RGB representation are horizontally flipped and merged to obtain one depth (DS) region. If the paired left and right depth regions are respectively denoted as DL and DR, the region depth (DS) after flipping and merging processes is expressed by:

for( x = 0; x < 3WHD; x++ )

for( y = 0; y < H; y++ ) {

DS[x][y] =DL[3WHD x 1][y]

DS[ x + 3WHD ][ y ] = DR[3WHDx 1][ y ]

}

where 3WHD is the width of left and right depth regions in units of frame luma samples. After horizontal flipping and merging processes as shown in Figure D-X2, the width of the depth (DS) region becomes 6WHD, while the height of the depth is H in units of frame luma samples.

After the flipping and merging procedures, the depth (DS) region is with the height of 6HHD and the width of W while its corresponding texture (TS) region is with the height of HT and the width of W as shown in Figure D-X1 for ctdp_packing_type equal to 0. The depth (DS) region is with the width of 6WHD and the height of H while its corresponding texture (TS) region is with the width of WT and the height of H as shown in Figure D-X2 for ctdp_packing_type equal to 1.

To complete the final depacking process, depth (DS) region and texture (TS) region should be further upsampled to construct the recovered depth () and the recovered texture () regions, which are both with the original resolution of H×W in units of frame luma samples, as follows.

Figure D-X8 Line-up pixel positions of depacked texture and depth regions for ctdp_packing_type = 0

- For ctdp_packing_type equal to 0, after the above depacking procedures, the texture (TS) and depth (DS) regions are with sizes of W×HT and W×6HHD, respectively. The relationship of line-up pixels of depth (DS) region and texture (TS) region is shown in Figure D-X6. The constructions of the recovered texture and the recovered depth regions are described in the follows.

From (D-X1) and (D-X2), it is easy to verify that 6HHD= H/3 and HT = 8H/9. To recover the original resolution, the depth (DS) region is vertically upsampled by a factor of 3:1 and the texture is vertically upsampled by a factor of 9: 8. It is noted that the texture upcampling could be performed first while the depth upsamping could refer the upsampled texture samples to achieve better results.

As shown in Figure D-X6, every 8 horizontal lines of texture (TS) region are repetitively lined up with every 3 horizontal lines of depth (DS) region. The texture pixels, Ti,j (blue color) after 9:8 vertical upsampling obtain the recovered texture pixels,Ti,j(yellow color). The depth pixels, Di,j (white color) are fully depacked to the recovered depth pixels, Di,j where the white depth pixels are the depth value obtained from the color depacking process and the dotted-gray depth pixels are the vertically-interpolated depth values after 3:1 upsampling process.

- For ctdp_packing_type equal to 1, after the above depacking procedures, the texture (T) and depth (D) region are with the sizes of WT×H and 6WHD×H, respectively. The relationship of line-up pixels of depth (DS) region and texture (TS) region is shown in Figure D-X8. The constructions of the recovered texture and the recovered depth regions are described in the follows.

From (D-X3) and (D-X4). it is easy to verify that 6WHD= W/4 and WT = 11W/12. To recover the original resolution, the depth (DS) region is horizontally upsampled by a factor of 4:1 and the texture region (TS) is horizontally upsampled by a factor of 12:11. It is noted that the texture upcampling could be performed first while the depth upsamping could refer the upsampled texture samples to achieve better results.

As shown in Figure D-X9, every 11 vertical lines of texture (TS) region are repetitively lined up with every 4 vertical lines of depth (DS) region, The texture pixels, Ti,j (blue color) after 12:11 horizontal upsampling obtain the recovered texture pixels,Ti,j(yellow color). The depth pixels, Di,j (white color) are fully depacked to the recovered depth pixels,Di,j where the white depth pixels are the depth values obtained from the color depacking process and the dotted-gray depth pixels are the horizontally-interpolated values after 4:1 upsampling process.

Figure D-X7 Line-up pixel positions of depacked texture and depth regions for ctdp_packing_type = 1

baseline_dist_flag equal to 0 specifies that the syntax elements specifying the baseline distance are not present in the syntax structure. baseline_dist_flag equal to 1 specifies that the syntax elements specifying the baseline distance value are present in the syntax structure.

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure. focal_length_flag equal to 1 specifies that the syntax elements specifying the focal length value are present in the syntax structure.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of the recovered depth value Di,j as specified in Table D-X3. In Table D-X3, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to (1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table D-X3 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each recovered depth value represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the recovered depth value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the recovered depth value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each recovered depth value represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the recovered depth value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the recovered depth value equal to maxVal represents DMax (specified below). |
| 2 | Each recovered depth value represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the recovered depth value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the recovered depth value equal to maxVal represents ZNear (specified below). |
| 3 | Each recovered depth value represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[i]. When d_min_flag is equal to 1, the recovered depth value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the recovered depth value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

The variables in the x column of Table D-X4 are derived from the respective variables in the s, e, n and v columns of Table D-X3 as follows:

- If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s* 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table D-X4 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| Bdistance | BdistanceSign | BdistanceExp | BdistanceMantissa | BdistanceManLen |
| Flength | FlengthSign | FlengthExp | FlengthMantissa | FlengthManLen |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of the recovered depth value width of the coded picture. The units for Zi,j, ZNear, ZFar and Bdistance when present, are identical but unspecified.

When depth_representation_type is equal to 1, the disparity di,j can be obtained for values of maxVal, DMax, and DMin as

di,j = (Di,j/ maxVal) * (DMax – DMin) + DMin.                 (D-X71)

When depth_representation_type is equal to 0, the Zi,j value value can be obtained for values of maxVal, ZFar and DZnear as

Zi,j = 1 / {(Di,j / maxVal) * (1/ZNear – 1/ZFar) – 1/ZFar}.         (D-X72)

When depth_representation_type is equal to 2, the Zi,j value can be obtained for values of maxVal, ZFar and DZnear as

Zi,j = (Di,j / maxVal) * (ZFar – ZNear)– ZNear.                (D-X73)

When depth_representation_type is equal to 0 or 2, the disparity di,j can further be obtained from the value of Zi,j, Bdistance, and Flength as

di,j = Bdistance * Flength ÷ Zi,j					(D-X74)

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of the recovered depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of the recovered depth values to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_‌representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, the recovered depth map contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform the recovered depth values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {

pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev1 = depth_nonlinear_representation_model[ k ]

pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev2 = depth_nonlinear_representation_model[ k + 1 ] (G-10)

x1 = pos1 − dev1

y1 = pos1 + dev1

x2 = pos2 − dev2

y2 = pos2 + dev2

for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )

DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )

}

When depth_representation_type is equal to 3, DepthLUT[ Di,j ] for the recovered depth value Di,j in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture depth packing arrangement SEI message.

centralized_texture_depth_packing_persistence_flag equal to 0 specifies that the centralized texture depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture_depth_packing_persistence_flag equal to 1 specifies that the centralized texture depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins;

–	The bitstream ends;

–	A frame in an access unit containing a centralized texture depth packing SEI message with the same.

- D.4.X	View synthesis representation information element syntax

| view_syn_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the view synthesis representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

# Performance evaluations of color packing methods

Comparing to [1], in this document, the new colored depth packing in RGB subpixel components by selected depth assignment, which can be depacked both in YCbCr and RGB domains, is proposed. The packing performance comparisons of DPCM YCbCr [1] and adjusted-RGB [1], and assigned-RGB in Y only as well as assigned-RGB in YCbCr packing methods are evaluated by simulations,

Without any video coding systems, the performances of the CTDP formats with one-view and one-depth, where the colored depth is packed with DPCM YCbCr [1] and adjusted-RGB [1], and assigned-RGB in Y only as well as assigned-RGB in YCbCr packing methods, while YCbCr4:2:0 chroma format with direct subsampling is performed. In the simulations, we adopt Shark and UndoDancer sequences for performance evaluations of uncoded CTDP videos for CTDP-TB and CTDB-LR formats. Table 2.1 and Table 2.1 show the PSNR and SSIM performances of depacked texture, depth, and their synthesized virtual views generated by VSRS rendering system for CTDP-TB and CTDP-LR formats, respectively.

Table 2.1. Objective performances of DPCM-based YCbCr, adjusted RGB, assigned RGB packing methods

for uncoded CTDP-TB format (depth:1/9, texture:8/9)

| Performances | Depacked Texture |  |  | Depacked Depth |  |  |  | Synthesized Virtual View Generated by VSRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DPCM-based YUV | Adjust RGB | Assign RGB | DPCM-based YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) | DPCM-based YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) |
| PSNR | 46.149 | 47.092 |  | 44.102 | 44.625 | 41.200 | 44.163 | 38.888 | 38.028 | 37.500 | 37.973 |
| SSIM | 0.9938 | 0.9938 |  | 0.9918 | 0.9943 | 0.9880 | 0.9910 | 0.9867 | 0.9754 | 0.9760 | 0.9770 |

Table 2.2. Objective performances of DPCM-based YCbCr, adjusted RGB, new RGB packing methods

for uncoded CTDP-LR format ( depth:1/12, texture:11/12)

| Performances | Depacked Texture |  |  | Depacked Depth |  |  |  | Synthesized Virtual View Generated by VSRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DPCM-based YUV | Adjust RGB | Assign RGB | DPCM-based YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) | DPCM-based YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) |
| PSNR | 49.214 | 49.244 |  | 39.888 | 40.432 | 37.102 | 40.039 | 38.288 | 37.493 | 36.200 | 37.419 |
| SSIM | 0.9958 | 0.9955 |  | 0.9873 | 0.9905 | 0.9820 | 0.9860 | 0.9882 | 0.9768 | 0.9750 | 0.9770 |

From the above simulations, the packing performances of depth and texture by using the colored depth with RGB adjustment method is higher than those with  DPCM-based YCbCr mapping method. We found that the colored depth in YCbCr still performs better than the other color packing methods in virtual view syntheses. For some existing TV broadcasting systems, if the access of YCbCr components becomes impossible, we could pack the depth to the colored depth in RGB subpixels, which are adjusted in the packing side. Without RGB adjustments, after color transformation and YCbCr 4:2:0, the assigned RGB color depth packing method performs slightly worst. However, the newly-proposed assigned-RGB method can greatly be conducted in both RGB and YCbCr spaces.

For the coding comparisons, the CTDP-TB and CTDP-LR formats are encoded by HEVC HM13.0 and compared to 3D-HEVC HTM8.0. The QP setting for both HM13.0 and HTM8.0 are 27, 32, 37, and 42. Two CG 3D video test sequences, Shark and UndoDancer sequences are used. With respect to 3D-HEVC, Tables 2.3 and 2.4 respectively show the BDBR and BDPSNR performances of texture and depth for CTDP-TB format while Tables 2.5 and 2.6 respectively exhibit the BDBR and BDPSNR performances of texture and depth for CTDP-LR format. The results show that the assigned-RGB method performs the best depth coding performances with HEVC codec. As to the synthesized virtual view generated by VSRS, the color packing with DPCM-based YCbCr method shows the best performance. The newly-proposed assigned RGB method has the advantages of simple realization, independent of color transform, RGB/YCbCr operatability.

Table 2.3. Average BDBR (%) results for CTDP-TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  |  |  | Depacked Depth |  |  |  | Virtual View by VSRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DPCM YUV | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV |  | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) |
| ra | 13.6254 | 11.8079 | 10.2413 | 11.3696 | -30.6426 |  | -41.9352 | -37.3007 | -43.3599 | 25.7829 | 28.5642 | 27.3187 | 27.1948 |
| ldp | 8.2266 | 7.0231 | 5.4145 | 6.5359 | -53.8872 |  | -61.2222 | -59.7979 | -62.1773 | 13.9024 | 16.9534 | 15.2695 | 15.8299 |
| ai | 6.8931 | 5.4887 | 4.0781 | 5.1018 | -19.7578 |  | -30.0731 | -15.7421 | -31.9427 | 29.2954 | 34.6573 | 35.4913 | 32.4152 |
| ave | 9.5817 | 8.1065 | 6.5779 | 7.6691 | -34.7625 |  | -44.4102 | -37.6136 | -45.8266 | 22.9936 | 26.7250 | 26.0265 | 25.1466 |

Table 2,4. Average BDPSNR (dB) results for CTDP-TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  |  |  | Depacked Depth |  |  |  | Virtual View by VSRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DPCM YUV | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV |  | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) |
| ra | -0.4247 | -0.3737 | -0.3233 | -0.3598 | 1.3941 |  | 2.1227 | 1.6104 | 2.2124 | -0.6789 | -0.7511 | -0.7264 | -0.7207 |
| ldp | -0.2707 | -0.2344 | -0.1829 | -0.2190 | 3.6932 |  | 4.4448 | 4.5038 | 5.2205 | -0.4051 | -0.4940 | -0.4581 | -0.4683 |
| ai | -0.2557 | -0.2088 | -0.1541 | -0.1940 | 0.8799 |  | 1.5138 | 0.3691 | 1.6067 | -0.8694 | -1.0012 | -1.0307 | -0.9518 |
| ave | -0.3170 | -0.2723 | -0.2201 | -0.2576 | 1.9902 |  | 2.6937 | 2.1611 | 3.0132 | -0.6511 | -0.7487 | -0.7384 | -0.7136 |

Table 2.5 Average BDBR (%) results for CTDP-LR format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  |  |  | Depacked Depth |  |  |  | Virtual View by VSRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DPCM YUV | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV |  | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) |
| ra | 11.7019 | 10.3264 | 9.8741 | 9.7372 | -1.0712 |  | 26.5047 | 14.3735 | 5.1253 | 67.8342 | 64.9970 | 73.0169 | 65.3218 |
| ldp | 6.9614 | 5.9485 | 4.9930 | 5.4378 | 1.3046 |  | -30.2637 | -39.8038 | -41.0947 | 42.2946 | 40.5865 | 46.4557 | 40.1147 |
| ai | 3.7328 | 2.8620 | 2.5671 | 2.3678 | -1.6007 |  | 27.8123 | 26.9585 | 10.5722 | 69.7477 | 68.4575 | 84.1826 | 68.8946 |
| ave | 7.4653 | 6.3790 | 5.8114 | 5.8476 | -0.4558 |  | 8.0178 | 0.5094 | -8.4657 | 59.9588 | 58.0137 | 67.8850 | 58.1103 |

Table 2.6. Average BDPSNR (dB) results for CTDP-LR format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  |  |  | Depacked Depth |  |  |  | Virtual View by VSRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DPCM YUV | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV |  | Adjust RGB | Assign RGB(Y) | Assign RGB (Y+C) | DPCM YUV | Adjust RGB | Assign RGB (Y) | Assign RGB (Y+C) |
| ra | -0.3560 | -0.3184 | -0.2981 | -0.3005 | -1.0712 |  | -0.3254 | -0.8546 | -0.2790 | -1.3992 | -1.3630 | -1.5043 | -1.3707 |
| ldp | -0.2231 | -0.1935 | -0.1614 | -0.1773 | 1.3046 |  | 2.0469 | 1.4883 | 2.0698 | -1.0077 | -0.9781 | -1.1181 | -0.9755 |
| ai | -0.1370 | -0.1072 | -0.0925 | -0.0883 | -1.6007 |  | -0.8882 | -1.9684 | -0.8756 | -1.6165 | -1.6039 | -1.8745 | -1.6130 |
| ave | -0.2387 | -0.2064 | -0.1840 | -0.1887 | -0.4558 |  | 0.2778 | -0.4449 | 0.3051 | -1.3411 | -1.3150 | -1.4990 | -1.3197 |

# Patent rights declaration(s)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

[1] J.-F. Yang, G.-C. Chen and W.-J. Yang, “Centralized texture depth packing SEI message for HEVC and AVC”, Joint Collaborative Team onVideo Coding of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, the 30th Meeting: Gwangju, KR, Document: JCTVC-AD0022, Oct. 2017.