| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 31st Meeting: San Diego, US, 13–20 Apr. 2018 | Document: JCTVC-AE0031 |
| --- | --- |

| Title: | Update of the 3D-HTM software and modifications for experimental studies |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report/Proposal |  |  |
| Author(s) or Contact(s): | Gerhard Tech Karsten Sühring Detlev Marpe | Tel: Email: | gerhard.tech@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Discussions at the 30th JCT-VC meeting (Gwangju, Korea, January 2018) determined several functionalities that are not supported by the HEVC reference software (since not required for conformance), but desired for experimental purposes [1]. This document presents a modified 3D-HTM version, which supports some of these functionalities and provides a basis for further extensions. Moreover, the modified 3D-HTM version is aligned with the latest HM version and comprises several clean-ups.

# Modifications

The following modifications have been made based on the latest HTM version, (i.e. HTM-16.2).

## Maximum number of layers / parameter set sharing

The HTM-16.2 encoder sends one SPS/PPS per layer. Because HEVC limits the number of SPS/PPS Ids to 16, this results in a limit of 16 layers.

To resolve this issue, the modified software's encoder uses parameter set sharing. More specifically, it first assigns the layers to different sets such that the following is true for each particular layer: The particular layer's set comprises all dependent or reference layers of the particular layer that have the same representation format as the particular layer. Then, it sends one SPS and PPS per determined set using the lowest layer Id in the respective set. (The representation format is considered as e.g. a depth layer with 4:0:0 chroma format can not refer to the SPS of the texture base layer with chorma format 4:2:0 due to the SPS requirement that "when present, the value of chroma_format_idc, shall be less than or equal to chroma_format_vps_idc of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ]." ).

With parameter set sharing the maximum number of layers for the modified HTM version can be larger than 16 (up to 63).  For compatibility with legacy encoder configurations, parameter set sharing is disabled by default, but can be enabled by a new cfg-file parameter (ShareParameterSets). The following example configurations are provided together with the software:

- MV-HEVC with texture of 63 views 
(\cfg\MV-HEVC\NonCTC\fullCfg_63view.cfg)

- 3D-HEVC with texture and depth of 31 views
(cfg\3D-HEVC\NonCTC\misc\fullCfg_31view.cfg)

## Representation formats

As required by the HEVC specification, HTM-16.2 supports for Multiview Main profile conforming layers and 3D Main profile conforming texture layers only the chroma format 4:2:0 and only 8 bit. For 3D Main profile conforming depth layers it only supports the chorma format 4:0:0 and only 8 bit. A further encoder restriction in HTM-16.2 is that all layers have the same picture size (which is, when layers are independent, actually not required by the Multiview or 3D Main profile).

To overcome these limitations the modified encoder allows configuring different representation formats. In particular, encoder parameters that can now be specified per representation format are related to the following:

- picture size (SourceWidth, SourceHeight, HorizontalPadding, VerticalPadding)

- bit depth (InputBitDepth, OutputBitDepth, MSBExtendedBitDepth, InternalBitDepth)

- chroma format (InputChromaFormat, ChromaFormatIDC)

- conformance window (ConfWinLeft, ConfWinRight, ConfWinTop, ConfWinBottom)

Individual layers can then be mapped to the representation formats with a new encoder parameter (LayerIdxInVpsToRepFormatIdx).

It should be noted that by default, the encoder issues an error for non-conforming configurations. To enable them the define NH_MV_ALLOW_NON_CONFORMING must be set to 1 in TypeDef.h and the profiles multiview-main_NONCONFORMING and 3d-main_NONCONFORMING must be used in the encoder cfg-file instead of multiview-main, and 3d-main, respectively.

Example cfg-files are provided together with the software for the following configurations:

- MV-HEVC, 8 bit texture, 10 bit depth
(cfg\MV-HEVC\NonConforming\fullCfg_aux_3view_aux_10bit.cfg)

- MV-HEVC, 6 independent layers with different representation formats
(cfg\MV-HEVC\NonConforming\fullCfg_aux_3view_simu_different_rep_formats.cfg)

It should be furthermore noted that for 3D-HEVC bit depths different from 8 bit and chroma formats different from 4:2:0 (for texture) and 4:0:0 (for depth) have not been tested and might lead to encoder crashes or undefined behaviour. Reason for this is that some low-level coding tools have been hard-coded to use the default values (e.g., DLT requires 8 bit and VSO requires 4:2:0 texture).

## Mapping of layers to GOP configurations

In HTM-16.2 the GOP layout needs to be configured per layer with one parameter line per picture in the GOP. Considering that usually many layers share the same GOP structure, this leads to redundant entries. To resolve this, an encoder parameter mapping layers to GOP structure definitions has been introduced (LayerIdxInVpsToGopDefIdx). The parameter specifies for each layer the index of the GOP structure.

This way, only a small number of GOP structures must be specified. Consequently, the size of the encoder cfg-file and the effort for creating it is significantly reduced (in particular in the case of many layers).

## Update to HM-16.18 and Cleanups

HTM-16.2 is based on HM-16.9. The modified HTM software has been updated to HM-16.18. Moreover, a large number of macros guarding individual 3D-HEVC tools have been removed. 3D-HEVC tools are now uniquely guarded by the NH_3D define (apart from encoder tools, which use still individual macro defines).

# RD-performance

The modifications have no effect on the RD-performance compared to HTM-16.2. Nevertheless, it should be noted that bit rates for single layers/pictures are now correctly reported in the encoder log and thus different from those reported in the encoder log of HTM-16.2 (which deviate minor from the correct bit rates.)

# Software availability

The modified software is available in the svn-repository:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-16.2-dev

# Conclusion and proposal

The modified HTM-software presented in this proposal updates the HTM-16.2 software to HM-16.18. Furthermore, it extents the encoder by 1) parameter set sharing which is required to support up to 63 layers; 2) the possibility to configure and signal different representation formats for different layer sets; and 3) a simplified configuration of GOP structures.

The encoder extensions allow to generate conforming bitstreams with properties not yet supported by the encoder (i.e. more than 16 layers or independent layers with different sizes). Moreover, they provide a basis for the implementation of further non-conforming functionalities required for experimental studies as reported in [1], e.g. signalling more than 63 layers, and 3D-HEVC with 10 bit depth maps.

It is proposed to release the modified software as new HTM version 16.3.

# References

[1] Issues affecting the usage of HEVC reference software for experimental studies (JCTVC-AD1011).