| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 31st Meeting: San Diego, US, 13–20 Apr. 2018 | Document: JCTVC-AE1100 |
| --- | --- |

| Title: | Common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Karl Sharman Karsten Suehring | Email: | karl.sharman@sony.com karsten.suehring@hhi.fraunhofer.de |
| Source: | JCT-VC |  |  |

_____________________________

# Abstract

This document defines common test conditions recommended for use in technical contributions to the 32nd and following JCT-VC meetings.

It includes the test conditions for 4:2:0 (version 1) and the Range Extensions (part of version 2).

# Introduction

Common test conditions (CTC) are defined for conducting experiments in an environment where the outcome of the experiments can be compared.

This document defines configurations for the HEVC model (HM) when studying 4:2:0 coding (HEVC version 1) and when studying Range Extensions coding (RExt; part of version 2) as outlined in Sections 2 and 3, respectively. It is an update of the previous CTC document, namely JCTVC-AC1100 and includes some updated test sequences of classes A1, A2 and B, corresponding to changes for the CTC used by JVET.

Excel templates are provided for reporting the results of experiments.

HM-16.19 software or newer is expected to be used for experiments. The common software package can be found at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.19; other options for obtaining the software can be found at http://hevc.info/.

For all test conditions, a combination of the following are used to control the encoder:

- an encoder configuration,

- the base QP,

- a sequence configuration,

- and in some cases, additional configurations are required. These additional configurations are highlighted in the text with this style.

The encoder produces a summary table, from which the PSNRs and bit rates for all frames is extracted and placed in one of the standard templates, alongside the output encoder and decoder run time; the decoder must be capable of decoding the stream.

# HM 4:2:0 common test conditions

The HM 4:2:0 CTC define eight test conditions, reflecting combinations of 8 and 10 bit coding, and of intra-only, random-access, and low-delay settings. They are listed in Table 1, alongside their associated configuration file that is provided in the cfg/ folder of the common software package.

Table 1. HM 4:2:0 configuration descriptions and associated configuration files

| Configurations | - All intra, main (AI) - All intra, main 10 (AI10) - Random access, main (RA) - Random access, main 10 (RA10) - Low-delay B, main (LB) - Low-delay B, main 10 (LB10) - Low-delay P, main (LP) - Low-delay P, main 10 (LP10) | - encoder_intra_main.cfg - encoder_intra_main10.cfg - encoder_randomaccess_main.cfg - encoder_randomaccess_main10.cfg - encoder_lowdelay_main.cfg - encoder_lowdelay_main10.cfg - encoder_lowdelay_P_main.cfg - encoder_lowdelay_P_main10.cfg |
| --- | --- | --- |

When the random access configurations are used, the period for intra-predicted frames for each test sequence needs to be specified as follows:

- IntraPeriod: specifies the intra refresh period in the random access configuration. The intra refresh period is dependent on the frame rate of the test sequence using the relationship shown in Table 2.

To ensure decoded frame checking, the following setting can be applied to the encoder and the decoder for all configurations:

- SEIDecodedPictureHash: can be set to 1.

Table 2. Intra period value for random access configurations.

| Frame rate (frames per second) | Intra period |
| --- | --- |
| 20 | 16 |
| 24 | 32 |
| 30 | 32 |
| 50 | 48 |
| 60 | 64 |
| 100 | 96 |

Note: In this version of the CTC the intra period has been changed for a frame rate of 24 fps from 24 to 32 to adapt to the GOP size now being 16.

## Encoding formats

The video sequences are coded using the 4:2:0 chroma format and the supplied configuration files do not require any additional encoding format configuration settings.

## Quantization parameter values

For each video sequence four quantization parameter values are to be used: 22, 27, 32 and 37. These values define the Quantization Parameter (QP) values used for the I-frames in a sequence (configuration files further define QP values used for other frames).

Table 3. HM 4:2:0 CTC QP ranges.

| QP range | QP values |
| --- | --- |
| Main tier (MT) | 22, 27, 32, 37 |

The following setting is applied to the encoder:

- QP: Specifies the quantization parameter to be applied (using values as defined in Table 3).

## Test sequence configuration

All test sequences are available to qualified members of JCT-VC/JVET only. Qualified members of JCT-VC/JVET can contact the JCT-VC chairs or their parent standardization body contact for login information.

Test sequences of classes A1 and A2 and MarketPlace and RitualDance of Class B are available on ftp://jvet@ftp.ient.rwth-aachen.de and ftp://jvet@ftp.hhi.fraunhofer.de, currently within the jvet-cfp/SDR folder.

Test sequences of classes B to F from ftp://ftp.tnt.uni-hannover.de/testsequences/.

Annex A includes MD5 hashes for the test sequences in the common test conditions.

### Test sequences

Table 4 defines the set of test sequences to be used for intra, random-access, and low-delay configurations. All frames (as defined by frame count in the table) shall be encoded for all sequences. All combinations of sequence, QP and configuration shall be used, except:

- Sequences of classes A1 and A2 are not tested in combination with any of the low delay configurations,

- class E sequences are not tested in combination with the random access configurations.

Configuration files for each test sequence with each encoding format are provided in the cfg/per-sequence/ folder.  These configuration files specify the parameters listed in Section 2.3.

Results are to be reported using the following template: JCTVC-AE1100_HM420.xlsm

Note: When the encoder operates in 8-bit mode (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range. Similarly when the encoder operates in 10-bit mode (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

Table 4. HM 4:2:0 test sequences.

| Class Size | Input bit-depth | Frame rate | Encoding format | Test sequence filename Test sequence configuration filename | Encoded frame count |
| --- | --- | --- | --- | --- | --- |
| Class A1 4096x2160/ 3840x2160 | 10-bit | 60 | YCbCr 4:2:0 | Tango2_3840x2160_60fps_10bit_420.yuv Tango.cfg | 294 |
|  | 10-bit | 60 | YCbCr 4:2:0 | FoodMarket4_3840x2160_60fps_10bit_420.yuv FoodMarket4.cfg | 300 |
|  | 10-bit | 30 | YCbCr 4:2:0 | Campfire_3840x2160_30fps_10bit_420_bt709_videoRange.yuv Campfire.cfg | 300 |
| Class A2 4096x2160/ 3840x2160 | 10-bit | 60 | YCbCr 4:2:0 | CatRobot1_3840x2160p_60_10_709_420.yuv CatRobot1.cfg | 300 |
|  | 10-bit | 60 | YCbCr 4:2:0 | DaylightRoad2_3840x2160_60fps_10bit_420.yuv DaylightRoad2.cfg | 300 |
|  | 10-bit | 50 | YCbCr 4:2:0 | ParkRunning3_3840x2160_50fps_10bit_420.yuv ParkRunning3.cfg | 300 |
| Class B 1920x1080 | 10-bit | 60 | YCbCr 4:2:0 | MarketPlace_1920x1080_60fps_10bit_420.yuv MarketPlace.cfg | 600 |
|  | 10-bit | 60 | YCbCr 4:2:0 | RitualDance_1920x1080_60fps_10bit_420.yuv RitualDance.cfg | 600 |
|  | 8-bit | 50 | YCbCr 4:2:0 | Cactus_1920x1080_50.yuv Cactus.cfg | 500 |
|  | 8-bit | 50 | YCbCr 4:2:0 | BasketballDrive_1920x1080_50.yuv BasketballDrive.cfg | 500 |
|  | 8-bit | 60 | YCbCr 4:2:0 | BQTerrace_1920x1080_60.yuv BQTerrace.cfg | 600 |
| Class C 832x480 | 8-bit | 50 | YCbCr 4:2:0 | BasketballDrill_832x480_50.yuv BasketballDrill.cfg | 500 |
|  | 8-bit | 60 | YCbCr 4:2:0 | BQMall_832x480_60.yuv BQMall.cfg | 600 |
|  | 8-bit | 50 | YCbCr 4:2:0 | PartyScene_832x480_50.yuv PartyScene.cfg | 500 |
|  | 8-bit | 30 | YCbCr 4:2:0 | RaceHorses_832x480_30.yuv RaceHorsesC.cfg | 300 |
| Class D 416x240 | 8-bit | 50 | YCbCr 4:2:0 | BasketballPass_416x240_50.yuv BasketballPass.cfg | 500 |
|  | 8-bit | 60 | YCbCr 4:2:0 | BQSquare_416x240_60.yuv BQSquare.cfg | 600 |
|  | 8-bit | 50 | YCbCr 4:2:0 | BlowingBubbles_416x240_50.yuv BlowingBubbles.cfg | 500 |
|  | 8-bit | 30 | YCbCr 4:2:0 | RaceHorses_416x240_30.yuv RaceHorses.cfg | 300 |
| Class E 1280x720 | 8-bit | 60 | YCbCr 4:2:0 | FourPeople_1280x720_60.yuv FourPeople.cfg | 600 |
|  | 8-bit | 60 | YCbCr 4:2:0 | Johnny_1280x720_60.yuv Johnny.cfg | 600 |
|  | 8-bit | 60 | YCbCr 4:2:0 | KristenAndSara_1280x720_60.yuv KristenAndSara.cfg | 600 |
| Class F Mixed size | 8-bit | 50 | YCbCr 4:2:0 | BasketballDrillText_832x480_50.yuv BasketballDrillText.cfg | 500 |
|  | 8-bit | 30 | YCbCr 4:2:0 | ChinaSpeed_1024x768_30.yuv ChinaSpeed.cfg | 500 |
|  | 8-bit | 30 | YCbCr 4:2:0 | SlideEditing_1280x720_30.yuv SlideEditing.cfg | 300 |
|  | 8-bit | 20 | YCbCr 4:2:0 | SlideShow_1280x720_20.yuv SlideShow.cfg | 500 |

# HM RExt common test conditions

RExt development defined three different test condition scenarios, as set out in the following three subsections. The remaining subsections define additional aspects of the test conditions.

## Natural content coding conditions (RExt-CTC)

The test sequences listed in section 3.4.1 are to be used with their associated configuration files, along with the indicated additional parameters.

The configurations as listed in Table 6 and any associated additional parameters and the QP ranges as described in section 3.3.2 are to be used. All combinations of test sequence, configuration and QP are to be processed, except the super-high-tier is only used for intra simulations.

Results are to be reported using the following template: JCTVC-AE1100_RExt.xlsm

## High bit-depth coding conditions (RExt-HBD)

There are two different test conditions for the RExt high bit depth CTCs, as set out in the following two sections. The sequences used for testing are detailed in Section 3.4.2.

### Lossy high bit-depth coding conditions

The test sequences listed in section 3.4.2 are to be used, along with the indicated additional parameters.

The intra and low delay configurations as listed in Table 6 are to be used.

The QP ranges as described in this section are to be used and the following setting is applied:

- InternalBitDepth: Set equal to the internal bit-depth for a given test case (see Table 5).

Results are to be reported using the following template: JCTVC-AE1100_RExtHBD_lossy.xlsm

Table 5 defines two QP ranges and two internal bit-depths for testing.  Each QP range includes four QP values.  For each configuration, each test sequence is encoded using each QP value for a given QP range.  The QP values are used for encoding intra-predicted frames and the configuration files further define QP offsets to be applied for inter-predicted frames.

All combinations of test sequence, configuration and QP are to be processed, except where the indicated frame count in section 3.4.2 is not defined.

Table 5. QP ranges for high-bit depth testing.

| Internal bit-depth | QP range | QP values |
| --- | --- | --- |
| 12 | Medium tier (MT) | 8, 5, 2, -1 |
| 12 | High tier (HT) | -4, -7, -10, -13 |
| 16 | Medium tier (MT) | -16, -19, -22, -25 |
| 16 | High tier (HT) | -28, -31, -34, -37 |

Extended precision processing shall be enabled, and the high precision forward transform shall be enabled in the encoder. The corresponding software settings are:

- Software compiled with #define RExt__HIGH_BIT_DEPTH_SUPPORT  1

- ExtendedPrecision: Must be set to 1.

### Lossless high bit-depth coding conditions

The test sequences listed in section 3.4.2 are to be used, along with the indicated additional parameters.

The intra and low delay configurations as listed in Table 6 are to be used.

The following parameters are to be applied to configure the encoder for lossless coding:

- QP: Must be set to 0.

- TransquantBypassEnableFlag: Must be set to 1.

- CUTransquantBypassFlagForce: Must be set to 1.

- IntraReferenceSmoothing: Must be set to 0.

- CostMode: Must be set to ‘lossless’.

Simulations are to be run with an internal bit depth of 12 and 16 by applying the following setting:

- InternalBitDepth: Set equal to the internal bit-depth.

All combinations of test sequence and configuration and bit depth are to be processed, except where the indicated frame count in section 3.4.2 is not defined.

Results are to be reported using the following template: JCTVC-AE1100_RExtHBD_lossless.xlsm

## General HM RExt configurations

The general HM RExt CTC defines three configurations, as shown in Table 6. The listed configuration files are provided in the cfg/ folder of the common software package.

Table 6. RExt configurations and their associated configuration files.

| Configurations: | - All intra (AI), - Random access (RA), - Low-delay B (LD) | - encoder_intra_main_rext.cfg - encoder_randomaccess_main_rext.cfg - encoder_lowdelay_main_rext.cfg |
| --- | --- | --- |

When the random access configuration is used, the period for intra-predicted frames for each test sequence needs to be specified, as follows:

- IntraPeriod: specifies the intra refresh period in the random access configuration. The intra refresh period is dependent on the frame rate of the test sequence using the relationship shown in Table 2.

To ensure decoded frame checking, the following setting can be applied to the encoder and the decoder for all configurations:

- SEIDecodedPictureHash: can be set to 1.

Note: Experiments investigating R’G’B’ coding tools should also provide results using Y’CbCr 4:4:4.

Note: In this version of the CTC the intra period is changed for a frame rate of 24 fps from 24 to 32 to adapt to the GOP size now being 16.

### Encoding formats

The per-sequence configuration files define the chroma format and channel order (GBR) as required.

For test sequences using the RGB colour space, the decoder can be configured with:

- OutputColourSpaceConvert: Should be set to ‘GBRtoRGB’.

Notes:

The setting of OutputInternalColourSpace is optional since it will not affect the coding performance.

### Quantization parameter values

Table 7 defines three QP ranges used in some of the RExt CTCs.  Each QP range includes four QP values.  For each configuration (AI, RD and LD), each test sequence is encoded using each QP value for a given QP range.  The QP values are used for encoding intra-predicted frames and the configuration files further define QP offsets to be applied for inter-predicted frames.

The super high tier (SHT) is used for AI configurations only.

Table 7. RExt QP ranges.

| QP range | QP values |
| --- | --- |
| Main tier (MT) | 22, 27, 32, 37 |
| High tier (HT) | 17, 22, 27, 32 |
| Super high tier (SHT) | 12, 17, 22, 27 |

The following setting is applied to the encoder:

- QP: Specifies the quantization parameter to be applied (using values as defined in Table 7).

## Test sequence configuration

Test sequences are available to qualified members of JCT-VC from the following locations:

- ftp://ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/upload (please contact the JCT-VC chairs for login information).

- ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/1080p50_CgrLevels_SINC_FILTER_SVTdec05_/

Note that test sequences from the EBU (i.e. with the string ‘EBU’ included in the test sequence name) are password protected.  The password can be obtained by contacting the EBU using the form located at:

- https://tech.ebu.ch/EBU_SVT_Public_Test_Sequences

A password is required to unzip the archives containing test sequences in the “16 bit HDR 4:0:0 Medical” and “IBDI 4:0:0” classes (see section 3.4.2).  Contact the authors of contribution JCTVC-O0172 to obtain this password.

SGI test sequences from the ‘SVT Multiformat’ set must be converted to YUV (RGB) format before use by the HM software.  A conversion utility is included in this contribution for this purpose.

Annex A includes MD5 hashes for the test sequences in the common test conditions.

Note: the encoded frame count may be less than the total frame count for a given test sequence.  In particular, the encoded frame count may also be less than the length of 10 seconds that is recommended for subjective viewing according to ITU-R Recommendation BT.500.  Reduction of the encoded frame count has only been performed where the statistics of the frame data are stable throughout the test sequence.

### Natural content coding conditions test sequences

Table 8 defines the set of test sequences to be used for natural content coding (RExt-CTC).  The encoded frame count, the input bit-depth and the encoding format are specified.  The encoded frame count may be less than the frame count of the test sequence, and for those FramesToBeEncoded must be used.

For each test sequence, the internal bit-depth (InternalBitDepth) is configured to be the same as the input bit-depth, except for VenueVu in RGB 4:4:4, which uses an internal bit-depth of 10-bits (even though the input bit-depth is 8-bits).

Configuration files for each test sequence are provided in the cfg/per-sequence/ folder.

Table 8. RExt CTC test sequences.

| Class / Encoding format | Input bit-depth | Frame rate | Test sequence filename | Encoded frame count |
| --- | --- | --- | --- | --- |
| RGB 4:4:4 | 12 | 30 | Traffic_2560x1600_30_12bit_444_crop.rgb Traffic_RGB_12bit.cfg | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_444.rgb Kimono_RGB_10bit.cfg | 240 |
|  | 10 | 50 | EBULupoCandlelight_1920x1080_50_10bit_444.rgb EBULupoCandlelight_RGB_10bit.cfg | 250 |
|  | 10 | 50 | EBURainFruits_1920x1080_50_10bit_444.rgb EBURainFruits_RGB_10bit.cfg | 250 |
|  | 8 | 30 | VenueVu_1920x1080_30_8bit_444.rgb VenueVu_RGB.cfg | 150 |
|  | 10 | 30 | DucksAndLegs_1920x1080_30_10bit_444.rgb DucksAndLegs_RGB_10bit.cfg | 150 |
|  | 10 | 50 | OldTownCross_1920x1080_50_10bit_444.rgb OldTownCross_RGB_10bit.cfg | 250 |
|  | 10 | 24 | ParkScene_1920x1080_24_10bit_444.rgb ParkScene_RGB_10bit.cfg | 240 |
| YCbCr 4:4:4 | 10 | 30 | Traffic_2560x1600_30_10bit_444_crop.yuv Traffic_444_10bit.cfg | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_444.yuv Kimono_444_10bit.cfg | 240 |
|  | 10 | 50 | EBULupoCandlelight_1920x1080_50_10bit_444.yuv EBULupoCandlelight_444_10bit.cfg | 250 |
|  | 10 | 50 | EBURainFruits_1920x1080_50_10bit_444.yuv EBURainFruits_444_10bit.cfg | 250 |
|  | 10 | 30 | VenueVu_1920x1080_30_10bit_444.yuv VenueVu_444_10bit.cfg | 150 |
|  | 10 | 60 | BirdsInCage_1920x1080_60_10bit_444.yuv BirdsInCage_444_10bit.cfg | 300 |
|  | 10 | 50 † | CrowdRun_1920x1080_50_10bit_444.yuv CrowdRun_444_10bit.cfg | 250 |
| YCbCr 4:2:2 | 10 | 30 | Traffic_2560x1600_30_10bit_422_crop.yuv Traffic_422_10bit.cfg | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_422.yuv Kimono_422_10bit.cfg | 240 |
|  | 10 | 50 | EBUHorse_1920x1080_50_10bit_422.yuv EBUHorse_422_10bit.cfg | 250 |
|  | 10 | 50 | EBUGraphics_1920x1080_50_10bit_422.yuv EBUGraphics_422_10bit.cfg | 250 |
|  | 10 | 50 | EBUWaterRocksClose_1920x1080_50_10bit_422.yuv EBUWaterRocksClose_422_10bit.cfg | 250 |
|  | 10 | 50 | EBUKidsSoccer_1920x1080_50_10bit_422.yuv EBUKidsSoccer_422_10bit.cfg | 250 |
|  | 10 | 50 | Seeking_1920x1080_50_10bit_422.yuv Seeking_422_10bit.cfg | 250 |

† Note that this was erroneously defined as 24 in JCTVC-P1006.

### High bit-depth coding test sequences

The encoded frame count may be less than the frame count of the test sequence, and for those FramesToBeEncoded must be used.

Table 9 lists the test sequences to be used for both lossless and lossy coding conditions for high bit-depth coding.

Table 9. High bit-depth coding test sequences.

| Class | Test sequence filename | Encoded frame count |  | Input bit-depth | MSB extended bit-depth |
| --- | --- | --- | --- | --- | --- |
|  |  | AI | LD |  |  |
| SVT 16 bit from 10 bit source (intermediate processing) | OldTownCross_1920x1080_50_16bit_444.rgb OldTownCross_RGB_16bit.cfg | 50 | 50 | 16 | 0 |
|  | CrowdRun_1920x1080_50_16bit_444.rgb CrowdRun_RGB_16bit.cfg | 50 | 50 | 16 | 0 |
|  | DucksTakeOff_1920x1080_50_16bit_444.rgb DucksTakeOff_RGB_16bit.cfg | 50 | - | 16 | 0 |
|  | InToTree_1920x1080_50_16bit_444.rgb InToTree_RGB_16bit.cfg | 50 | - | 16 | 0 |
|  | ParkJoy_1920x1080_50_16bit_444.rgb ParkJoy_RGB_16bit.cfg | 50 | - | 16 | 0 |
| IBDI 4:4:4 | Traffic_2560x1600_30_12bit_444_crop.rgb Traffic_RGB_12bit.cfg | 60 | 60 | 12 | 0 |
| 16 bit HDR | FruitStall_1920x1080_24_16bit_444.rgb FruitStall_RGB_16bit.cfg | 120 | 120 | 16 | 0 |
| 16 bit from 16 bit HDR source | Bubbles_4096x2160_24_16bit_444.rgb Bubbles_RGB_16bit.cfg | 60 | - | 16 | 0 |
| 16 bit HDR 4:0:0 Medical | AX_Head_1240x960_16bit_400.yuv Head_400_16bit.cfg | 250 | 250 | 16 | 0 |
| IBDI 4:0:0 | CT_ Cardiac_512x512_12bit_400.yuv Cardiac_400_12bit.cfg | 250 | 250 | 12 | 0 |
|  | CT_ LongrunShort_512x512_12bit_400.yuv LongRunShort_400_12bit.cfg | 500 | 500 | 12 | 0 |
| Synthesised RGB HDR | EBURainFruits_1920x1080_50_10bit_444.rgb EBURainFruits_RGB_10bit+2MSB.cfg EBURainFruits_RGB_10bit+4MSB.cfg EBURainFruits_RGB_10bit+6MSB.cfg | 50 | 50 | 10 | 12,14,16 |
|  | Kimono1_1920x1080_24_10bit_444.rgb Kimono_RGB_10bit+2MSB.cfg Kimono_RGB_10bit+4MSB.cfg Kimono_RGB_10bit+6MSB.cfg | 60 | - | 10 | 12,14,16 |

For test sequences in the “16 bit HDR 4:0:0 Medical” and “IBDI 4:0:0” classes, the frame rate is set to 50, even though the test sequences do not have an inherent frame rate.

# Annex A. Test sequence md5sums

For each test sequence included in the common test conditions, the ‘md5sum’ value is as follows:

## HM420 test sequences

0471a59c423b7059c5c6c8b395e864a9  Tango2_3840x2160_60fps_10bit_420.yuv

a378b34190f54f688d048a9a8b46a8ac  FoodMarket4_3840x2160_60fps_10bit_420.yuv

63d3d9f9e4e8b5c344e89840e84e6428  Campfire_3840x2160_30fps_10bit_420_bt709_videoRange.yuv

03a89792693fd9ecfd72ef2590025e97  CatRobot1_3840x2160p_60_10_709_420.yuv

bf1d22643afb41b1521749d2749fb5f0  DaylightRoad2_3840x2160_60fps_10bit_420.yuv

e7a1d1ebff269767ec4bffd2998d1154  ParkRunning3_3840x2160_50fps_10bit_420.yuv

dc668e7f28541e4370bdbdd078e61bba  MarketPlace_1920x1080_60fps_10bit_420.yuv

a3cb399a7b92eb9c5ee0db340abc43e4  RitualDance_1920x1080_60fps_10bit_420.yuv

3fddb71486f209f1eb8020a0880ddf82  Cactus_1920x1080_50.yuv

d38951ad478b34cf988d55f9f1bf60ee  BasketballDrive_1920x1080_50.yuv

efde9ce4197dd0b3e777ad32b24959cc  BQTerrace_1920x1080_60.yuv

bd215136fed04067d82c10b2e49b2c7c  BasketballDrill_832x480_50.yuv

f889efea02b0c9a7d174b0f7a99cb51b  BQMall_832x480_60.yuv

4766c455665b6d228a6390e3d3ff2647  PartyScene_832x480_50.yuv

0a351df99f22d837bc528bd4901c6968  RaceHorses_832x480_30.yuv

bfd9abbdc677790130dc4023b4e409f0  BasketballPass_416x240_50.yuv

713ef64958345859b9bae986c3a3f763  BQSquare_416x240_60.yuv

50a520722f0e906b7884b6b9fea48699  BlowingBubbles_416x240_50.yuv

290a63e86213abc4459fce1dbd39edbe  RaceHorses_416x240_30.yuv

4ce5d72311b32acce62614f63225fba5  FourPeople_1280x720_60.yuv

be83259b3ccdada2213fbd8dea20bf6e  Johnny_1280x720_60.yuv

aa3931974e34deba15a1018ba3bf5e0c  KristenAndSara_1280x720_60.yuv

480881343081e3f4adf85075a1ed0f08  BasketballDrillText_832x480_50.yuv

9dfcbb87d095987891cf90cc593c5ba9  ChinaSpeed_1024x768_30.yuv

dbf1d0e765ba9f775c5208adb4dc6c0c  SlideEditing_1280x720_30.yuv

d6b28f127b9ddf088eb9db9fc5e611bd  SlideShow_1280x720_20.yuv

## RExt Natural content test sequences

2ef18e36431be68c7434b40c4c1047bb  Traffic_2560x1600_30_12bit_444_crop.rgb

437db0d18f414b74e4c62e0485da0716  Kimono1_1920x1080_24_10bit_444.rgb

3835ce632e0a45b8d529cb20dcca8b04  EBULupoCandlelight_1920x1080_50_10bit_444.rgb

20c876a78951a985bf604e050e75ff62  EBURainFruits_1920x1080_50_10bit_444.rgb

8f98fbad7261bcd78f86d8ba8fd4a19b  VenueVu_1920x1080_30_8bit_444.rgb

a6e0c402fd1cc116a72fb2c0c2ffc07a  DucksAndLegs_1920x1080_30_10bit_444.rgb

ffaa261159ba236bb1f4671246201f7f  OldTownCross_1920x1080_50_10bit_444.rgb

612640a857ff95cc7bd3859a4e2b8a4f  ParkScene_1920x1080_24_10bit_444.rgb

5f6f594778d01c8150e9df877ed760f0  Traffic_2560x1600_30_10bit_444_crop.yuv

afd06b6a7823d854b89277ccee1e2aa8  Kimono1_1920x1080_24_10bit_444.yuv

0943294a422600371c988193e36cc3dc  EBULupoCandlelight_1920x1080_50_10bit_444.yuv

14e011ee5ceb1c8d7a046e2cc0401774  EBURainFruits_1920x1080_50_10bit_444.yuv

6f5d8ac82ff84043603025edd3e9a8eb  VenueVu_1920x1080_30_10bit_444.yuv

f018bed8cdfab52505d9169fe946d9bf  BirdsInCage_1920x1080_60_10bit_444.yuv

e9b443a3adc7538c4d4104f141cce0fb  CrowdRun_1920x1080_50_10bit_444.yuv

423e0e2d2c3ead29c267b349b988f083  Traffic_2560x1600_30_10bit_422_crop.yuv

d8bd4994c1b68392f296a297be3114fa  Kimono1_1920x1080_24_10bit_422.yuv

3bef26469ee0d1a3ae5fc972497f17b9  EBUHorse_1920x1080_50_10bit_422.yuv

1c200a39b4ec8216f2a63c403df4e015  EBUGraphics_1920x1080_50_10bit_422.yuv

b493aa9a920cf8e1490280279432285c  EBUWaterRocksClose_1920x1080_50_10bit_422.yuv

d2f9262feea06b7159ae2a2beb83701b  EBUKidsSoccer_1920x1080_50_10bit_422.yuv

d595003e45041e17f021a3db388381e9  Seeking_1920x1080_50_10bit_422.yuv

## RExt High bit-depth test sequences

6004ff26cafcf2b066faa00c935a7621  CrowdRun_1920x1080_50_16bit_444.rgb

2ab943d7b1c6e3b26a2c6da416b69828  ParkJoy_1920x1080_50_16bit_444.rgb

d882efad39ae3d360896ec09d9213424  DucksTakeOff_1920x1080_50_16bit_444.rgb

6f7d413472f856741e95543217f9795d  InToTree_1920x1080_50_16bit_444.rgb

ae5bab60c3dfb16399971b9afff120da  OldTownCross_1920x1080_50_16bit_444.rgb

2ef18e36431be68c7434b40c4c1047bb  Traffic_2560x1600_30_12bit_444_crop.rgb

87520165ee9151842da0cb1e0daaa22d  FruitStall_1920x1080_24_16bit_444.rgb

4e39ba06b7aacce48c831c794f7fbe9d  Bubbles_4096x2160_24_16bit_444.rgb

297cce085b06bc645ef156c2579e8b98  AX_Head_1240x960_16bit_400.yuv

877ffcfcd7917d614c22409ef7b75bfb  CT_Cardiac_512x512_12bit_400.yuv

94e02eb70901228aa72a0481f3f9019a  CT_LongrunShort_512x512_12bit_400.yuv

20c876a78951a985bf604e050e75ff62  EBURainFruits_1920x1080_50_10bit_444.rgb

437db0d18f414b74e4c62e0485da0716  Kimono1_1920x1080_24_10bit_444.rgb