| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 33rd Meeting: Macao, CN, 6–12 Oct. 2018 | Document: JCTVC-AG0002-v2 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Chris Rosewarne (Canon) Jens-Rainer Ohm (RWTH Aachen) Karl Sharman (Sony) Gary J. Sullivan (Microsoft) Ye-Kui Wang (Huawei) | Email: | benjamin.bross@hhi.fraunhofer.de chris.rosewarne@cisra.canon.com.au ohm@ient.rwth-aachen.de karl.sharman@eu.sony.com garysull@microsoft.com yekui.wang@huawei.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 32nd meeting in Ljubljana, SI (July 2018) and the 33rd meeting in Macao, CN (October 2018).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Develop proposed improvements to the JCTVC-AB1002 HEVC Test Model 16 (HM 16) Update 9 of Encoder Description - Collect reports of errata for the HEVC specification and the HDR technical reports. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |

# Status of AHG2 work

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

Ticket#1499 reports an error in Equation 8-47. The error was confirmed. And the change should be as follows:

In 8.4.4.2.5 (Specification of intra prediction mode INTRA_DC), change Equation 8-47 from

dcVal = x'= 0nTbS − 1p[ x' ][ −1 ]+y'= 0nTbS − 1p[ −1 ][ y' ]+nTbS>>( k−1 )	(8-47)

to

dcVal = x'= 0nTbS − 1p[ x' ][ −1 ]+y'= 0nTbS − 1p[ −1 ][ y' ]+nTbS>>( k+1 )	(8-47)

Where "( k − 1 )" is changed to "( k + 1 )", and there is no other change.

Furthermore, there is an error in the semantics of the mastering display colour volume SEI message. This should be corrected as follows:

In D.3.28 (Mastering display colour volume SEI message semantics), replace the following paragraph

display_primaries_y[ c ], when in the range of 5 to 42 000, inclusive, specifies the normalized y chromaticity coordinate of the colour primary component c of the mastering display, according to the CIE 1931 definition of y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in increments of 0.00002. When display_primaries_y[ c ] is not in the range of 5 to 37 000, inclusive, the normalized y chromaticity coordinate of the colour primary component c of the mastering display is unknown or unspecified or specified by other means not specified in this Specification.

with the following:

display_primaries_y[ c ], when in the range of 5 to 42 000, inclusive, specifies the normalized y chromaticity coordinate of the colour primary component c of the mastering display, according to the CIE 1931 definition of y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15), in increments of 0.00002. When display_primaries_y[ c ] is not in the range of 5 to 42 000, inclusive, the normalized y chromaticity coordinate of the colour primary component c of the mastering display is unknown or unspecified or specified by other means not specified in this Specification.

Input documents relevant to HEVC errata to this meeting include JCTVC-AG0021 and JCTVC-AG0021:

| JCTVC-AG0021 | Some errata items for HEVC and AVC | Y.-K. Wang (Huawei), A. M. Tourapis (Apple), G. J. Sullivan (Microsoft) |
| --- | --- | --- |
| JCTVC-AG0022 | Some errata items for HEVC | Y.-K. Wang (Huawei) |

# Recommendations

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Confirm the resolution of the ticket#1499 and the other fix in Section 1 of JCTVC-AG0002, and address other open tickets, if any, in the issue tracker and close them.

- Review suggested fixes in JCTVC-AG0021 and JCTVC-AG0022.

- Review JCTVC-AG0021 and JCTVC-AG0022.

- Remember to incorporate the corrections made to the draft integrated text for the next ISO/IEC HEVC edition decided at the previous JCT-VC meeting (i.e., the 32nd JCT-VC meeting in Ljubljana) into the next edition of the ITU-T text.

- Change the name of this AHG to be something like "HEVC and AVC specifications maintenance" and include collect errata reports for the AVC specification into a mandate, or create another AHG for this.