| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 33rd Meeting: Macao, CN, 6–12 Oct. 2018 | Document: JCTVC-AG0023 |
| --- | --- |

| Title: | Centralized Texture Depth Packing SEI Message for HEVC and AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC – Revision of Output Document JCTVC-AA1008 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang (NCKU, Taiwan) Guan-Cheng Chen Wei-Jong Yang | Tel: Email: | +886-917727713 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University, Taiwan |  |  |

_____________________________

# Abstract

This document is a revision of Output Document JCTVC-AA1008 [1] by keeping YCbCr colour packing method and removing irrelevant parts. The revision covers the cases of general depth values with 8 bits or more. The purpose of the proposed “Centralized Texture Depth Packing SEI message for HEVC and AVC” is to deliver one texture view plus one depth map packed into the samples of a monoscopic (texture only) stream for HEVC and AVC. In simulation part, we add the performances of YCbCr.colour packing with and without texture-based depth upsampling method.

- Centralized texture depth packing SEI message syntax

- Replace specification of in clause 7.4.4 with the following:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages nor centralized texture depth packing SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages present in the CVS.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages.

In subclause D.2.1 (General SEI message syntax), add the following:

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture_depth_packing( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers of the respective specification.

Add a new subclause D.2.X “centralized_texture_depth_packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text of the respective standard.

D.2.X	Centralized texture depth packing (CTDP) SEI message syntax

| centralized_texture_depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture_depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture_depth_packing_cancel_flag ) { |  |
| ctdp_packing_type | u(3) |
| baseline_dist_flag | u(1) |
| focal_length_flag | u(1) |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( baseline_dist_flag ) |  |
| view_syn_rep_info_element( BdistanceSign, BdistanceExp, BdistanceMantissa, BdistanceManLen ) |  |
| if( focal_length_flag ) |  |
| view_syn_rep_info_element( FlengthSign, FlengthExp, FlengthMantissa, FlengthManLen ) |  |
| if( z_near_flag ) |  |
| view_syn_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| view_syn_rep_info_element ( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| view_syn_rep_info_element ( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| view_syn_rep_info_element ( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| centralized_texture_depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |
| } |  |

In Table F.4, add the persistence scope for the centralized_texture_depth_packing SEI message as follows:

| - SEI message | - Persistence scope |
| --- | --- |
| centralized texture depth packing | - Specified by the syntax of the SEI message |

Add a new subclause D.3.X “centralized texture depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following subclause D.3.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.3.X Centralized texture depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains a specific packing arrangement of texture and depth samples associated with one view. This information can be used by a process applied after decoding to appropriately interpret the samples and process them appropriately for display of 3D video or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture_depth_packing_cancel_flag equal to 1 indicates that the centralized texture_depth packing (CTDP) arrangement SEI message cancels the persistence of any previous centralized texture depth packing SEI message in output order. centralized_texture_depth_packing_cancel_flag equal to 0 indicates that centralized texture depth packing is applicable.

ctdp_packing_type indicates the intended interpretation of the constituent frames as specified in Table D-X1.

Table D-X1 – Definition of ctdp_packing_type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the frame of the CTDP-TB type with an arrangement of top coloured depth (CDT), texture (T), and bottom coloured depth (CDB) regions from top to bottom, as illustrated in Figure D-X1. |
| 1 | Indicates the frame of the CTDP-LR type with an arrangement of left coloured depth (CDL), texture (T), and right coloured depth (CDR) regions from left to right, as illustrated in Figure D-X2. |

Values of ctdp_packing_type that do not appear in Table D-X1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture depth packing SEI messages that contain reserved values of ctdp_packing_type. Subsequently,.

The interpretations of ctdp_packing_type how the CTDP frame to retrieve coloured depth regions and the texture region and how to recover them to the original full-resolution depth map and texture frame are described as follows.

It is assumed that the original full-resolution texture frame and depth map are both of size WxH, where W and H are the width and height expressing pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

For ctdp_packing_type equal to 0, the packed frame from top to bottom is horizontally divided into top coloured depth (CDT), texture (T), and bottom coloured depth (CDB) regions. With the width of W in units of frame luma samples, the height of top coloured depth (CDT) and bottom coloured depth (CDB) regions in units of frame luma samples is expressed by

HD = (H/36) * 2.                 	 			(D-X1)

For simplifying CTDP packing and depacking processes, H is divisible by 36 by removing top and bottom horizontal lines or adding dummy top and bottom horizontal lines evenly. Hence, with the width of W, the height of the texture (T) region in units of frame luma samples becomes:

HT = H  2*HD.		     (D-X2)

As illustrated in Figure D-X1, the CTDP-TB packed frame, by referring HD defined in (D-X1) and HT defined in (D-X2), is divided into CDT, T and CDB regions from top to bottom. The height of CDT and CDB regions is HD and the height of T region is HT in units of frame luma samples while the width of CDT, T and CDB regions is W in units of frame luma samples.

Figure D-X1 Arrangement of the CTDP-TB frame and its depacking steps (ctdp_packing type = 0)

For ctdp_packing_type equal to 1, the packed frame from left to right is vertically divided into left coloured depth (CDL), texture (T), and right coloured depth (CDR) regions. With the height of H in units of frame luma samples, the width of left and right coloured depth (CDL and CDR) regions in units of frame luma samples is expressed by

WD = (W/48) * 2.                 	 			(D-X3)

For simplifying CTDP packing and depacking processes, W should be divisible by 48 by removing right and left vertical lines or adding dummy right and left vertical lines evenly. Hence, with the height of H, the width of the texture (T) region in units of frame luma samples becomes:

WT = W  2 * WD.	   	  (D-X4)

As illustrated in Figure D-X2, the CTDP-LR packed frame referring to WD defined in (D-X3) and WT defined in (D-X4) is divided into CDL, T, and CDR regions from left to right. The width of CDL and CDR regions is WD and the width of T region is WT in units of frame luma samples while the height of CDL, T and CDR regions is H in units of frame luma samples.

Figure D-X2 Arrangement of the CTDP-LR frame and its depacking steps (ctdp_packing_type = 1)

After splitting the coloured depth and texture regions, the paired top and bottom or left and right coloured depth regions should be first merged together to become one colour depth (CD) region as follows.

–	If ctdp_packing_type is equal to 0, as shown in Figures D-X1, the paired top and bottom coloured depth regions, which are denoted as CDT and CDB in YCbCr representation, are vertically flipped and merged to obtain one coloured depth (CD) region. If the paired top and bottom coloured depth regions are respectively denoted as CDT and CDB, the coloured depth after flipping and merging processes is expressed by:

for( x = 0; x < W; x++ )

for( y = 0; y < HD; y++ ) {

CD[x][y] = CDT[x][HD y 1]

CD[x][HD + y] = CDB[x][HD  y 1]

}

where HD is the height of top and bottom coloured depth regions in units of frame luma samples. After vertically flipping and merging processes as shown in Figure D-X1, the height of the coloured depth (CD) regions, becomes 2*HD, while the width of the coloured depth is W in units of frame luma samples.

–	If ctdp_packing_type is equal to 1, as shown in Figures D-X2, the paired left and right coloured depth regions, which are CDL and CDR in YCbCr are horizontally flipped and merged to obtain one coloured depth (CD) regions. If the paired left and right coloured depth regions are respectively denoted as CDL and CDR, the coloured depth after flipping and merging processes is expressed by:

for( x = 0; x < WD; x++ )

for( y = 0; y < H; y++ ) {

CD[x][y] =CDL[WD x 1][y]

CD[ x + WD ][ y ] = CDR[WDx 1][ y ]

}

where WD is the width of left and right coloured depth regions in units of frame luma samples. After horizontal flipping and merging processes as shown in Figure D-X2, the width of the coloured depth (CD) region becomes 2*WD, while the height of the depth is H in units of frame luma samples.

After flipping and merging, the coloured depth, CD region is depacked to depth (D) region as follows. The locations of 4 colour subpixels of coloured depth (CD) and their associated YCbCr subpixels with respect to the positions of 12 depth pixels are specified as shown in Figure D-X3. If ctdp_packing_type is equal to 0, the height of depth regions, after horizontal placement of subpixels, is tripled to become 6*HD, as shown in the left side of Figure D-X3. while the width of the depth is W in units of frame luma samples. If ctdp_packing_type is equal to 1, the width of depth regions, after horizontal placement of subpixels, is tripled to become 6*WD, as shown in the right side of Figure D-X3. while the height of the depth is H in units of frame luma samples.

Figure D-X3 Positions of 4 pixels of coloured depth, 12 YCbCr components, 12 RGB subpixels with respect

to the positions of 12 depacked depth pixels for ctdp_packing_type=0 and tdp_packing_type = 1

As shown in Figure D-X3, the YCbCr color depth packing every 4 coloured depth pixels in the CD region will map to 12 YCbCr components, which are interpreted as 12 depth pixel values of depth (D) region. However, the video coding could adopt YCbCr 4:4:4, YCbCr 4:2:2 and YCbCr 4:2:0 chroma formats, the restorations of depth values should be further specified as follows.

For YCbCr 4:2:0 chroma format, in every 4 colour pixels of colour depth, there are 6 YCbCr components, {Yi,j, Yi+1,j, Yi,j+1, Yi+1,j+1, Cbi,j, Cri,j} encoded. The chroma components, {Cbi+1,j, Cri+1,j}, {Cbi,j+1, Cri,j+1} and {Cbi+1,j+1, Cri+1,j+1} marked in black colour are not existed. Accordingly, there are 6 depth pixels selected to be fully depacked from 6 YCbCr components. To achieve better packing performances, these 6 depacked depth pixels are arranged in quincunx positions as Figure D-X4.

Figure D-X4 Coloured depth with YCbCr 4:2:0 chroma format with respect to

the positions of 6 fully-depacked depth pixels (grat colour) and 6 unknown depth pixels (black colour)

As shown in Figure D-X4, these 6 YCbCr components, {Yi,j, Yi+1,j, Yi,j+1, Yi+1,j+1, Cbi,j, Cri,j}, which are used to carry 6 depacked depth pixels marked in gray colour with proper mapping functions. As to the unknown 6 depth pixels can be interpolated from the 6 depacked depth pixels with proper interpolations. The detailed mapping functions for YCbCr to depth values are described as follows.

The depth values will be depacked from Y, Cb, and Cr components in the following steps. As shown in Figure D-X5, the depth value in range of {0, maxVal} is depacked from Y’s component in range of {16, maxVal 20} through a linear mapping function is expressed by,

D =g(Y) = (214* (Y − 16)*maxValue / (maxValue36) + 213) / 214, 	(D-X5)

where the variable maxVal is set equal to (1<<(8+bit_depth_luma_minus8))−1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Figure D-X5 Linear mapping function D=g(Y) for depth value depacked from Y component

The differential depth value in range of {maxVal, maxVal} depacked from the chroma component C in the range of {16, maxC 15}, which could be Cr or Cb in a piecewise linear DPCM approach is given as,

(D-X6)

where the variable maxC is set equal to (1<<(8+bit_depth_chroma_minus8))−1, where bit_depth_chroma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId. Accordingly, the variable max2 is set equal to (1 << ( 7 + bit_depth_chroma_minus8 ) ) and m1 = max2 + m5, m0 = max2 – m5, where m5 is set equal to (max2 *5)/128 and slope = (maxValue–m5) / (m016). In (D-X6), the chroma component, C is first transformed to the DPCM residual in the range of {maxVal, maxVal} by the piecewise linear mapping function shown in Figure D-X6, while the DPCM residual adds the DPCM prediction, which is the average of D1 and D2, to recover the depth value. The values, D1 and D2 are the nearest depth values depacked from Y components. In (D-X6), it consists of three linear DPCM decoding functions. The first linear function maps C in the range of {16, m0} to the DPCM residual in the range of {m5 maxValue} with slope = (maxValue – m5) / (m0  16). The second linear function one-to-one maps C in the range of {m0. m1} to the DPCM residual in the range of {m5, m5}. The third linear function C in the range of {m1, maxC-15} to the residual in range of {m5, maxValue} with the same slope as the first linear function.

Figure D-X6 Piecewise linear mapping function D=f(C, 0, 0) with zero prediction for residual depth value depacked from C chroma component (C= Cr or Cb)

- If ctdp_packing_type is equal to 0, the left side of Figure D-X4 shows the verticaly-placed positions of 4 sets of YCbCr components, which use {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1, Cbi,j, Cri,j} in YCbCr 4:2:0 chroma format for i = 0, 2, 4,…, (2*HD2), j = 0, 2, 4….，(W2) to represent 6 depth values. After this vertically-placed arrangement of YCbCr 4:2:0 components, the height of the depth, D region is expanded 3 times of the coloured depth (CD) region to become 6*HD, while the width of the depth region W is unchanged.

To recover the depth values, as shown in the left side of Figure D-X4, four luma components {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1} in range of {16, maxVal 20} are linearly mapped to 4 selected depth values {D3i,j, D3i1,j+1, D3i+4, j, D3i+5, j+1}, in the range of {0, maxVal} respectively expressed as:

D3i, j = g(Yi, j),	(D-X7)

D3i1, j+1 = g(Yi, j+1),	(D-X8)

D3i4, j = g(Yi+1, j),	(D-X9)

D3i+5, j+1 = g(Yi+1, j+1).	(D-X10)

By using 4 depth values depacked from Y components as predictions, two chroma components, {Cbi,j, Cri,j} are respectively transformed to two depth values, {D3i+2,j, D3i+3, j+1} by the piecewise linear DPCM function defined in (D-X6) as:

D3i+2, j = f(Cbi, j, D3i, j, D3i+4,j), 	(D-X11)

D3i+3, j+1 = f(Cri, j, D3i+1,j+1, D3i+5, j+1), 	(D-X12)

for i = 0, 2, 4…, (2*HD2), j = 0, 2, 4…, (W2). The above 6 equations stated from (D-X7) to (D-X12) can recover 6 depth values in quincunx positions while the reminding 6 unknown depth values (black colour), D3i+1,j, D3i+3,j, D3i+5,j, D3i,j+1, D3i+2,j+1 and D3i+4,j+1, shown in the left side of Figure D-X4, can be interpolated from those known depth values.

- If ctdp_packing_type is equal to 1, the right side of Figure D-X4 shows the horizontally-placed positions of 4 sets of YCbCr components, which use {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1, Cbi,j, Cri,j} in YCbCr 4:2:0 chroma format for i = 0, 2, 4,…, (H2), j = 0, 2, 4….，(2*WD2) to represent 6 depth values. After this horizontally-placed arrangement of YCbCr 4:2:0 components, the width of the depth (D) region is expanded 3 times of the coloured depth (CD) region to become 6WD, while H, the height of the depth region is unchanged.

To recover the depth values, as shown in the right side of Figure D-X4, four luma components {Yi,j, Yi+1,j, Yi+1,j, Yi+1,j+1} in range of {16, maxVal 20} are linearly mapped to 4 selected depth values {Di,3j, Di1,3j+1, Di,3 j+4, Di+1, 3j+5}, in the range of {0, maxVal} respectively expressed as:

Di,3j = g(Yi,j),	(D-X13)

Di1,3j+1 = g(Yi+1, j),	(D-X14)

Di, 3j4 = g(Yi, j+1),	(D-X15)

Di+1, 3j+5 = g(Yi+1, j+1),	(D-X16)

By using 4 depth values depacked from Y components as predictions, two chroma components, {Cbi,j, Cri,j} are respectively transformed to two depth values, {Di,3j+2, Di+1,3j+3} by the piecewise linear DPCM  function defined in (D-X6) as:

Di, 3j+2 = f(Cbi, j, Di, 3j, Di, 3j+4), 	(D-X17)

Di+1, 3j+3 = f(Cri, j, Di+1, 3j+1, Di+1, 3j+5), 	(D-X18)

for i = 0, 2, 4, …, (H2), j = 0, 2, 4, …, (2*WD2). The above 6 equations stated from (D-X13) to (D-X18) can recover 6 depth values in quincunx positions while the reminding 6 unknown depth values (black colour), Di,3j+1, Di,3j+3, Di, 3j+5, Di+1, 3j, Di+1, 3j+2 and Di+1,3j+4 shown in the right side of Figure D-X4, can be interpolated from those known depth values.

Figure D-X7 Coloured depths in YCbCr components in YCbCr 4:2:2 chroma format with respect to

the positions of 8 fully-depacked depth pixels (gray colour) and 4 unknown depth pixels (black colour)

For YCbCr 4:2:2 chroma format, in every 4 colour pixels of colour depth, there are 8 YCbCr components, {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1, Cbi,j, Cri,j, Cbi+1,j, Cri+1,j} encoded. The chroma components, {Cbi,j+1, Cri,j+1} and {Cbi+1,j+1, Cri+1,j+1} marked in black colour are not existed as shown in the left side of Figure D-X7. By applying the similar colour packing concept for YCbCr 4:2:0, there are 8 depth pixels can be fully depacked. To achieve better packing performances, these 8 depacked depth pixels are arranged in quincunx positions with two extra corner positions as Figure D-X7.

The 8 YCbCr components are used to carry 8 fully depacked depth pixels marked in white colour in proper mapping functions. The unknown 4 depth pixels can be interpolated from fully depacked depth pixels with proper interpolations. Similar to YCbCr 4:2:0, the 8 YCbCr components{Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1, Cbi,j, Cri,j, Cbi+1,j, Cri+1,j}  can be depacked to 8 depth values are as follows.

If ctdp_packing_type is equal to 0, the vertical positions of 8 YCbCr components to recover 8 depth values, as shown in the left side of Figure D-X7, similar to YCbCr 4:2:0 chroma component, four luma components {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1} in range of {16, maxVal 20} are first mapped to four selected depth samples {D3i,j, D3i1,j+1, D3i+4, j, D3i+5, j+1}, in the range of {0, maxVal} stated as (D-X7), (D-X8), (D-X9), (D-X10), respectively.

Similar to YCbCr 4:2:0 chroma components, four chroma components, {Cbi,j, Cri,j, Cbi+1,j, Cri+1,j} are respectively transformed to four depth values, {D3i,j+1, D3i+2,j, D3i+3, j+1 D3i+5,j} by the piecewise linear DPCM function defined in (D-X6) as:

D3i, j+1 = f(Cbi, j, D3i, j, D3i+1,j+1), 	(D-X19)

D3i+2, j = f(Cri, j, D3i,j, D3i+4, j) 	(D-X20)

D3i+3, j+1 = f(Cbi+1, j, D3i+1, j, D3i+5,j+1), 	(D-X21)

D3i+5, j = f(Cri+1, j, D3i+1,j+1, D3i+5, j+1) 	(D-X22)

for i = 0, 2, 4…, (2*HD2), j = 0, 2, 4…, (W2). Thus, we can recover 8 depth values in quincunx positions while the reminding 4 unknown depth values (black colour), D3i+1,j, D3i+3,j, D3i+2,j+1, D3i+4,j+1, shown in the left side of Figure D-X7, can be interpolated from those known depth values.

- If ctdp_packing_type is equal to 1, the horizontal positions of 8 YCbCr components to recover 8 depth values, as shown in the right side of Figure D-X7, similar to YCbCr 4:2:0 chroma component, four luma components {Yi,j, Yi,j+1, Yi+1,j, Yi+1,j+1} in range of {16, maxVal 20} are first mapped to four selected depth samples {Di,3j, Di1,3j+1, Di,3 j+4, Di+1, 3j+5}, in the range of {0, maxVal} stated as (D-X13), (D-X14), (D-X15), (D-X16), respectively.

By using similar concept as YCbCr 4:2:0 chroma components, four chroma components, {Cbi,j, Cri,j, Cbi+1,j, Cri+1,j } are respectively transformed to two depth values, {Di+1,3j, Di,3j+2, Di+1,3j, Di+1,3j+3} by the piecewise linear DPCM function defined in (D-X8) as:

Di+1, 3j = f(Cbi, j, Di,3j, Di+1,3j+1),                                            (D-X23)

Di, 3j+2 = f(Cri, j, Di,3j, Di, 3j+4), 	(D-X24)

Di+1, 3j = f(Cbi+1, j, Di, 3j, Di+1,3j+1), 	(D-X25)

Di+1, 3j+3 = f(Cri+1, j, Di+1,3j+1, Di+1, 3j+5) 	(D-X26)

for i = 0, 2, 4, …, (H2), j = 0, 2, 4, …, (2*WD2). Thus, we can recover 8 depth values in quincunx positions while the reminding 4 unknown depth values (black colour), Di,3j+1, Di,3j+3, Di+1,3j+3, Di+1,3j+4, shown in the right side of Figure D-X5, can be interpolated from those known depth values.

If the video coding system adopts YCbCr 4:4:4 chroma format, as shown in Figure D-X3, every 4 coloured depth pixels in the CD region with 12 YCbCr components can be interpreted as 12 depth pixel values of depth (D) region. The reminding {Cbi,j+1, Cri,j+1, Cbi+1, j+1, Cri+1, j+1} can carry the reminding depth pixels by piecewise linear PCM functions.

After the colour depacking procedures, the coloured depth (CD) region has been converted to gray depth (D) region. The coloured depth (CD) region with the height of 2*HD.is vertically magnified to 3 times to become the gray depth (D) region with the height of 6*HD for ctdp_packing_type equal to 0. The coloured depth (CD) region with the width of 2WD is horizontally magnified to 3 times to become the depth (D) region with the width of 6*WD for ctdp_packing_type equal to 1. After the depacking procedures, depth region and texture region should be further upsampled and processed to construct the recovered texture and the recovered depth regions, which are both with the original resolution of H×W in units of frame luma samples, as follows.

- For ctdp_packing_type equal to 0, after the above depacking procedures, the texture (T) and depth (D) regions are with sizes of W×HT and W×6*HD, respectively. The relationship of line-up pixels of depth (D) region and texture (T) region is shown in Figure D-X6. The constructions of the recovered texture and the recovered depth regions are described in the follows.

If H is divisible to 36, it easy to verify that 6*HD= H/3 and HT = 8*H/9. To recover the original resolution, the depth (D) region is vertically upsampled by a factor of 3:1 and the texture is vertically upsampled by a factor of 9: 8. As shown in Figure D-X8, every 8 horizontal lines of texture (T) region are repetitively lined up with every 3 horizontal lines of depth (D) region. The texture pixels, Ti,j (blue colour) after 9:8 vertical upsampling obtain the recovered texture pixels,Ti,j(yellow colour). The depth pixels, Di,j (white colour) are fully-depacked pixels obtained from colour components while those in gray colour represent the interpolated values during the colour depacking process. Finally,Di,jmarked in dotted gray colour are the vertically-interpolated depth values after 3:1 upsampling process.

Figure D-X8 The line-up pixels of depacked texture and depacked depth regions for ctdp_packing_type = 0

- For ctdp_packing_type equal to 1, after the above depacking procedures, the texture (T) and depth (D) region are with the sizes of WT × H and 6*WD × H, respectively. The relationship of line-up pixels of depth (D) region and texture (T) region is shown in Figure D-X8. The constructions of the recovered texture and the recovered depth regions are described in the follows.

If W is divisible to 48, it is easy to verify that 6*WD= W/4 and WT = 11*W/12. To recover the original resolution, the depth (D) region is horizontally upsampled by a factor of 4:1 and the texture is horizontally upsampled by a factor of 12:11. As shown in Figure D-X9, every 11 vertical lines of texture (T) region are repetitively lined up with every 4 vertical lines of depth (D) region, The texture pixels, Ti,j (blue colour) after 12:11 horizontal upsampling obtain the recovered texture pixels,Ti,j(yellow colour). The depth pixels, Di,j (white colour) are the fully-depacked pixel obtained from colour components while those in gray colour represent the interpolated values during the colour depacking process. Finally,Di,jmarked in dotted gray colour are the horizontally-interpolated values after 4:1 upsampling process.

Figure D-X9 The line-up pixels of depacked texture and depacked depth regions for ctdp_packing_type = 1

baseline_dist_flag equal to 0 specifies that the syntax elements specifying the baseline distance are not present in the syntax structure. baseline_dist_flag equal to 1 specifies that the syntax elements specifying the baseline distance value are present in the syntax structure.

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure. focal_length _flag equal to 1 specifies that the syntax elements specifying the focal length value are present in the syntax structure.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of the recovered depth value Di,j as specified in Table D-X2. In Table D-X2, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to (1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table D-X2 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each recovered depth value represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the recovered depth value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the recovered depth value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each recovered depth value represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the recovered depth value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the recovered depth value equal to maxVal represents DMax (specified below). |
| 2 | Each recovered depth value represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the recovered depth value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the recovered depth value equal to maxVal represents ZNear (specified below). |
| 3 | Each recovered depth value represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[i]. When d_min_flag is equal to 1, the recovered depth value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the recovered depth value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

The variables in the x column of Table D-X3 are derived from the respective variables in the s, e, n and v columns of Table D-X3 as follows:

- If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s* 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table D-X3 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| Bdistance | BdistanceSign | BdistanceExp | BdistanceMantissa | BdistanceManLen |
| Flength | FlengthSign | FlengthExp | FlengthMantissa | FlengthManLen |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of the recovered depth value width of the coded picture. The units for Zi,j, ZNear, ZFar and Bdistance when present, are identical but unspecified.

When depth_representation_type is equal to 0, the Zi,j value can be obtained for values of maxVal, ZFar and DZnear as

Zi,j = 1 / {(Di,j / maxVal) * (1/ZNear – 1/ZFar) – 1/ZFar}.         (D-X27)

When depth_representation_type is equal to 1, the disparity di,j can be obtained for values of maxVal, DMax, and DMin as

di,j = (Di,j/ maxVal) * (DMax – DMin) + DMin.                 (D-X28)

When depth_representation_type is equal to 2, the Zi,j value can be obtained for values of maxVal, ZFar and DZnear as

Zi,j = (Di,j / maxVal) * (ZFar – ZNear)– ZNear.                (D-X29)

When depth_representation_type is equal to 0 or 2, the disparity di,j can further be obtained from the value of Zi,j, Bdistance, and Flength as

di,j = (Bdistance * Flength ) ÷ Zi,j					(D-X30)

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of the recovered depth values to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_‌representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, the recovered depth map contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform the recovered depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first(0, 0) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {

pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev1 = depth_nonlinear_representation_model[ k ]

pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev2 = depth_nonlinear_representation_model[ k + 1 ] (G-10)

x1 = pos1 − dev1

y1 = pos1 + dev1

x2 = pos2 − dev2

y2 = pos2 + dev2

for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )

DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )

}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture depth packing arrangement SEI message.

centralized_texture_depth_packing_persistence_flag equal to 0 specifies that the centralized texture depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture_depth_packing_persistence_flag equal to 1 specifies that the centralized texture depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins;

–	The bitstream ends;

–	A frame in an access unit containing a centralized texture depth packing SEI message with the same.

- D.4.X	View synthesis representation information element syntax

| view_syn_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the view synthesis representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

# Performance evaluations of YCbCr colour packing method with Texture-based Depth Upsampling

Comparing to [1], in this document, we further proposed a texture-based depth upsampling method in the final step of depacking processes. The performance comparisons of YCbCr with and without the texture-based depth upsampling are evaluated by simulations as follows.

Texture-based depth upsampling (TDU) method is used to restore original resolution depth image by referring the upsampled texture frame. The TDU method can preserve the upsampling depth boundaries by an evolution weighted median filter, which is divided into three parts: 1) reliable image, 2) hierarchical upsampling and 3) evolution weighted upsampling. The edge boundary regions in the initial high-resolution depth map, which is upsampled temporarily by the bicubic filter, are detected as unreliable regions. Then, the pixels in unreliable regions is re-calculated by hierarchical upsampling. This hierarchical upsampling filter proceeds for every unreliable pixel by progressive scanning from top to bottom and left to right to interpolate the unreliable pixels by the evolution weighted median of its reliable neighboring pixels under the filtering mask. Iteratively, the above scanning window will be performed again to interpolate the unreliable pixels, which are at least two reliable neighboring pixels. Once all the unreliable pixels are re-calculated, the corresponding binary flags in the reliable image are set to 1 and the unreliable pixels in the initial temporary high resolution depth image are re-calculated guided by the similarity of adjacent pixels in texture image. The above texture-based depth upsampling method requires good quality texture frames. The CTDP-LR format with 11/12-texture, which can depack much better texture quality than the CTDP-TB with 8/9-texture..

Without any video coding systems, the performances of the CTDP formats with one-view and one-depth, where the depth is packed into YCbCr colour components in YCbCr4:2:0 chroma format. In the simulations, we adopt Shark and UndoDancer sequences for performance evaluations of uncoded CTDP videos for CTDB-LR format. Table 2.1 shows the PSNR and SSIM performances of depacked texture, depth, and their synthesized virtual views generated by VSRS rendering system. From these simulations, the texture-based depth upsampling method can improve the packing performances of depth and texture by using the YCbCr packing method.

Table 2.1. Objective performances of YCbCr colour packing with and without texture-based depth upsampling (TDU) method for uncoded CTDP-LR format (depth:1/12, texture:11/12)

| Performances | Depacked Texture and Depth |  |  |  |  |  | Synthesized Virtual View Generated by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Depacked Texture |  |  | Depacked Depth |  |  |  |  |  |
|  | Direct RGB | Direct YCbCr | Direct YCbCr+ | Direct RGB | Direct YCbCr | Direct YCbCr+ | Direct RGB | Direct YCbCr | Direct YCbCr+ |
| PSNR | 49.2443 | 49.2137 | 50.2522 | 35.9820 | 39.8881 | 40.5137 | 36.0823 | 38.2884 | 39.1031 |
| SSIM | 0.9955 | 0.9958 | 0.9963 | 0.9802 | 0.9873 | 0.9886 | 0.9737 | 0.9882 | 0.9879 |

* Direct YCbCr: Direct YCbCr colour depth packing method; Direct YCbCr+: Direct YCbCr colour depth packing method with TDU method.

For the coding comparisons, the videos in CTDP-LR format are encoded by HEVC HM13.0 and compared to 3D-HEVC HTM8.0. The QP setting for both HM13.0 and HTM8.0 are 27, 32, 37, and 42. Two CG 3D video test sequences, Shark and UndoDancer sequences are used. With respect to 3D-HEVC, Table 2.2 and Table 2.3 show the BDBR and BDPSNR performances of texture and depth for CTDP-LR format. The results show that the texture-based depth upsampling method can improve performances of the YCbCr colour packing with 0.0869 dB and 0.1311 dB with HEVC codec for the depacked depth and the synthesized virtual view generated by VSRS, respectively.

Table 2.2 Average BDBR (%) results for CTDP-LR format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  | Depacked Depth |  |  | Virtual View by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Direct RGB | Direct YCbCr | Direct YCbCr + | Direct RGB | Direct YCbCr | Direct YCbCr + | Direct RGB | Direct YCbCr | Direct YCbCr + |
| ra | 7.9095 | 11.7019 | 10.0607 | 16.5991 | 26.5047 | 22.3624 | 61.9436 | 67.8342 | 58.9028 |
| ldp | 3.4178 | 6.9614 | 5.8712 | -36.2061 | -30.2637 | -30.9079 | 39.226 | 42.2946 | 35.7900 |
| ai | 0.9848 | 3.7328 | 2.5841 | 30.3497 | 27.8123 | 27.3386 | 73.1888 | 69.7477 | 60.3423 |
| ave | 4.1040 | 7.4653 | 6.1720 | 3.5809 | 8.0178 | 6.2644 | 58.119 | 59.9588 | 51.6783 |

* Direct YCbCr: Direct YCbCr colour depth packing method; Direct YCbCr+: Direct YCbCr colour depth packing method with TDU method.

Table 2.3. Average BDPSNR (dB) results for CTDP-LR format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  |  | Depacked Depth |  |  | Virtual View by VSRS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Direct RGB | Direct YCbCr | Direct YCbCr + | Direct RGB | Direct YCbCr | Direct YCbCr + | Direct RGB | Direct YCbCr | Direct YCbCr + |
| ra | -0.2415 | -0.3560 | -0.3090 | -1.2349 | -1.0712 | -0.9416 | -1.3615 | -1.3992 | -1.2734 |
| ldp | -0.1122 | -0.2231 | -0.1901 | 0.9894 | 1.3046 | 1.3853 | -0.9979 | -1.0077 | -0.8752 |
| ai | -0.0344 | -0.1370 | -0.0954 | -2.4781 | -1.6007 | -1.5509 | -1.732 | -1.6165 | -1.4785 |
| ave | -0.1293 | -0.2387 | -0.1981 | -0.9079 | -0.4558 | -0.3691 | -1.3638 | -1.3411 | -1.2090 |

* Direct YCbCr: Direct YCbCr colour depth packing method; Direct YCbCr+: Direct YCbCr colour depth packing method with TDU method.

# Patent rights declaration(s)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

[1] J.-F. Yang and G. J. Sullivan, “Conceptual study of potential centralized texture depth packing SEI message for HEVC and AVC”, Joint Collaborative Team on Video Coding of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, the 27th Meeting: Hobart, AU, Output Document:. JCTVC-AA1008, April 2017.