| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 35th Meeting: Geneva, CH, 22–27 March 2019 | Document: JCTVC-AI0003 |
| --- | --- |

| Title: | JCT-VC AHG report: Software development and software technical evaluation (AHG3) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Ad-hoc group report |  |  |
| Author(s) or Contact(s): | Karsten Suehring (chair) Bin Li Vadim Seregin Karl Sharman Gerhard Tech Alexis Tourapis | Email: | karsten.suehring@hhi.fraunhofer.de libin@microsoft.com vseregin@qti.qualcomm.com karl.sharman@sony.com gerhard.tech@hhi.fraunhofer.de atourapis@apple.com |
| Source: | AHG chairs |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG on HEVC HM, SCM, SHM, HTM and HDRTools software development and software technical evaluation that have taken place between the 34th and 35th JCT-VC meetings.

# Introduction

The mandates given to the AhG are as follows:

•	Coordinate development of the HM, SCM, SHM, HTM, JM, JSVM, JMVM, and HDRTools software and its distribution.

•	Enable software support for recently standardized additional SEI messages.

•	Produce documentation of software usage for distribution with the software.

•	Prepare and deliver results, reporting templates and anchor test results according to JCT-VC common conditions.

•	Suggest configuration files for additional testing of tools.

•	Investigate how to minimize the number of separate codebases maintained for group reference software.

•	Coordinate with AHG2 on HEVC and AVC test model editing and errata reporting to identify any mismatches between software and text.

The current HEVC related software model versions are:

- HM 16.20 (Sep. 2018)

- HM 16.20 + SCM 8.8 (Mar. 2018)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- HDRTools 0.18 (Sep. 2018)

# HM related activities

## HM software

There have not been any further releases of the HM reference software.

## Further work

### List of adoptions outstanding

There are several modifications still to be included:

- The adopted changes in JCTVC-Y0038 that include changes in the GOP settings, which require coordination with JVET for JEM development.

- Fisheye SEI: A software patch had previously been provided to the software coordinators. It was reviewed just before the 35th JCTVC meeting, and was significantly modified. The original author has been asked to verify the latest revision.

- Annotated regions SEI: A second revision of the patch has been reviewed; an update is expected.

- JCTVC-AG00026 (Random Access encoding with HM for video-based point cloud codec): Software was provided and reviewed in several rounds. There were concerns from the software coordinators regarding structure and interaction with interlace coding, which have not been resolved yet.

- A change to the default value of FULL_NBIT to ‘1’ to align test conditions (see following section).

### Impact of future FULL_NBIT change

The simulation has been run to show the impact of setting the default value of FULL_NBIT to ‘1’ on the next HM revision.

This change only impacts non-8-bit coding (since with the macro disabled, all errors are converted into 8-bit video processing equivalent errors). The change also does not affect the high bit depth CTCs, as this flag was already set to 1 when the software is compiled in “high bit depth” mode.

|  | All Intra Main 10 |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.1% | -0.3% | -0.4% | -0.1% | -0.3% | -0.5% |
| Class A2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | -0.1% | -0.2% | -0.1% |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class F | -0.1% | -0.3% | -0.4% | -0.1% | -0.1% | -0.5% |
|  | Low delay B Main 10 |  |  | Low delay B Main 10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.2% |
| Class E | -0.1% | 0.2% | -1.0% | 0.0% | 0.2% | -0.8% |
| Overall | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | -0.3% |
|  | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | -0.3% |
| Class F | -0.1% | 0.1% | 0.0% | -0.2% | 0.0% | 0.5% |

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% |
| YCbCr 4:4:4 | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | -0.3% | -0.4% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.3% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |

### SEI Support

The following SEI messages currently do not have any support:

- Fisheye video info (JCTVC-AH1012) (payloadType == 152)

- Motion-constrained tile sets extraction info nesting (159)

- SEI manifest (JCTVC-AG1005) (200)

- SEI prefix indication (JCTVC-AG1005) (201)

- Annotated regions (JCTVC-AH1012) (202)

The following SEI messages currently do not have control mechanisms to configure the encoder to form them (although there is code to put the messages in the bit-stream):

- Pan scan rectangle (payloadType == 2)

- Filler payload (3)

- User data registered ITU T T35 (4)

- User data unregistered (5)

- Scene info (9)

- Picture snapshot (15)

- Progressive refinement segments (16, 17)

- Film grain characteristics (19)

- Post filter hint (22)

- Deinterlace field identification (143)

- Content light level information (144)

- Dependent RAP indication (145)

- Coded region completion (146)

- Ambient viewing environment (148)

### Persistent bug reports

The following are persistent bug reports where study is encouraged:

- High level picture types: IRAP, RASL, RADL, STSA:

Tickets #1096, #1101, #1333, #1334, #1346.

- Rate-control and QP selection – numerous problems with multiple slices:
		Tickets #1314, #1338, #1339.

- Field-coding:
		Tickets #1145, #1153.

- Decoder picture buffer:

Tickets #1277, #1286, #1287, #1304.

- NoOutputOfPriorPicture processing:

Tickets #1335, #1336, #1393.

- Additional decoder checks:

Tickets #1367, #1383.

# SCM related activities

There were 3 further releases of SCM on 22nd March, tagging the previously prepared merges. The tags created were for HM-16.18+SCM-8.8, HM-16.19+SCM-8.8 and HM-16.20+SCM-8.8.

# SHM related activities

There have not been any further developments to SHM’s SHVC during this meeting cycle.

# HTM related activities

There have not been any updates to the HTM of MV-HEVC and 3D-HEVC.

# HDRTools related activities

There have not been any releases to the HDRTools software. However, the current trunk includes SHVC down and up sampling filters.

# JM, JSVM, JMVM related activities

There have not been any updates to the JM, JSVM and JMVM software.

# Recommendations

- Continue to develop reference software based on HM 16.20, HM 16.20 + SCM 8.8, SHM 12.4, HTM 16.3 and HDRTools 0.18 and improve their quality.

- Test reference software more extensively outside of common test conditions.

- Add more conformance checks to the decoder to more easily identify non-conforming bit-streams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bit-streams that trigger bugs in the HM. Such bit-streams may also be useful for the conformance specification.

- Encourage people to submit configuration files that trigger bugs in HDRTools.

- Continue to investigate the merging of branches.

- Keep common test conditions aligned with JVET.