| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 36th Meeting: Gothenburg, SE, 6–12 July 2019 | Document: JCTVC-AJ0030-v1 |
| --- | --- |

| Title: | DPB size analysis of HM16.20 using RA configuration in current CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiang Ma Haitao Yang | Tel: Email: | maxiang6@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution provides an analysis of the decoded picture buffer (DPB) size of HM16.20 using RA configuration in current CTC. It is found that the DPB size can achieve 7 in HM, which is different from the value of high level syntax sps_max_dec_pic_buffering_minus1+1. It is suggested to modify the GOP structure to constrain the DPB size to be consistent with the syntax value.

# GOP16 prediction structure

In current CTC, GOP16 prediction structure is used. The highest temporal ID is 4 (from 0).

# Max DPB size derived from syntax

In the sps, there are three syntaxes which are relative to DPB size, sps_max_dec_pic_buffering_minus1[ i ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ]. The three syntax are defined for each sub layer.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… | u(4) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| …… |  |
| } | u(1) |

The meaning of sps_max_dec_pic_buffering_minus1[ i ] is defined as below:

- sps_max_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be less than or equal to vps_max_dec_pic_buffering_minus1[ i ] for each value of i. When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ].

Hence, the maxDecPicBufferingHighestTid = (sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1) could be considerd as the MaxDpbSize when decoding the sequence.

In current CTC, the HighestTid is 4, the sps_max_dec_pic_buffering_minus1[ 4] is 5, and maxDecPicBufferingHighestTid or MaxDpbSize is 6.

# DPB status analysis

Below is an analysis of the DPB status of HM16.20 [1] when using RA configuration in current CTC, including the POC of Picture in DPB, and the POC of last output picture. This status come from the decoding log file of first random access segment (RAS) of sequences RaceHorses (Intra Period is 32).

It can be seen that when decoding the picture with POC 7, the dpbFullness can achieve 7, which is larger than MaxDpbSize (which is 6). This case also occur when decoding picture with POC 11, 14, 13, 15.

# Suggestions

For currently GOP16 structure, the values of sps_max_dec_pic_buffering_minus1[i] (i=0~4) are 4,4,5,6,6.

Two suggestions are given like below:

## S1: Modify the GOP structure

Modify the GOP16 structure to make sure that the maximum needed DPB size is 6, which is consistent with currently syntax value.

Take Intra Period 48 for example:

For POC7： L0[6 4] L1[8 16] ==》 L0[6 0] L1[8 16]

For POC32：L0[16 0] L1[16 0] ==》L0[16] L1[16]

For POC23： L0[22 20] L1[24 32] ==》 L0[22 16] L1[24 32]

…

Below is the analysis of the DPB status of HM16.20, when using modified GOP16 structure, IntraPeriod = 48.

## S2: Modify the setting value of high level syntax

It is suggested to modify the values of sps_max_dec_pic_buffering_minus1[i] (i=0~4) as 4,4,5,6,7.

# Experimental results

Below is the test results of S1 (modify the GOP16 structure). Here note that the test sequences from JVET CTC[2] is used.

Anchor: HM16.20 using Original GOP16

Test: HM16.20 using Proposed GOP16

# Conclusion

It is found that the DPB size can achieve 7 of HM16.20, when using RA configuration in current CTC, which is larger than MaxDpbSize derived from the syntax sps_max_dec_pic_buffering_minus1 [i]. It is suggested to modify the GOP structure to constrain the DPB size to be consistent with the syntax value.

# References

[1] https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-16.20

[2] JVET common test conditions and software reference configurations for SDR video, JVET-N1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.