| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 37th Meeting: Geneva, CH, 4–10 October 2019 | Document: JCTVC-AK0029 |
| --- | --- |

| Title: | Perceptual redistribution of chroma information for improved HDR color representation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Maryam Azimi, Mahsa T. Pourazad, and Panos Nasiopoulos 2366 Main Mall St. | Tel: Email: | +1 778 926 7036 maryama@ece.ubc.ca |
| Source: | University of British Columbia (UBC) and TELUS Communications Inc. |  |  |

_____________________________

# Abstract

In this work, we propose a chroma processing method that adds to the existing NCL YCbCr 4:2:0 color encoding to improve its color accuracy and hence reduce its associated visible color errors for HDR color pixels. We investigate redistribution of code-words for the Cb and Cb chroma channels based on CIE DE2000 metric. To that end, we identify the colors that are represented with visible color shifts and the ones that are represented without visible errors using 10-bit YCbCr 4:2:0. We propose a non-linear function to redistribute code-words assigned to chroma values such that more code-words are assigned to those colors that bear larger color differences due to bit-depth quantization. We then quantize the new chroma channels (we refer to them as Cb* and Cr* to distinguish from the original Cb and Cr) with 10 bits, which is the current standard for HDR video delivery. The quantized YCb*Cr* signal is then compressed to evaluate the compression efficiency of the proposed method.

# Proposed Method

In this document we propose a chroma processing for YCbCr color encoding that exploits this characteristic of the HVS to assign more code-words during the bit-depth quantization to those colors our visual system is most sensitive to. To achieve that, we first identify how different colors are distorted due to bit-depth quantization and whether these distortions are visible to our eyes. We use the CIE DE2000 perception-based color difference metric to calculate the color differences between the original RGB color pixels and the ones represented by 10-bit YCbCr.

To construct the original RGB colors, we consider the colors in BT.2020 starting with the CIE 1976 Lu’v’ color space due to its perceptual uniformity. While L is constant, the u’ and v’ values are increased from 0 to 0.62 with step size of 0.001. According to [1], chromaticity changes lower than 0.001 are imperceptible to the human eye. Since constructing and analyzing all the colors of the BT.2020 gamut (230 of them if we consider 10 bits) is impractical, we sample these colors at different luminance (L) levels. The samples we use for L values are 0.05, 0.5, 5, 50, 100, and 400 nits, resulting in 6 color patches. We stopped at 400 nits, since the range of colors represented at maximum luminance of 1000 nits (maximum brightness supported by most of the current consumer HDR displays) is significantly reduced. Each of these color patches are then converted to YCbCr through the workflow shown in Fig. 1.

To understand how chroma quantization affects color perception, the chroma channels of Cb and Cr are then quantized to 8, 9 and 10 bits while the luma channel Y is always quantized to 10 bits. To obtain 8 and 9-bit chroma channels, we scale them by 1/4 and 1/2, respectively, and then represent them in 10-bit range. The resulting quantized Y10CbCr10, Y10CbCr9 and Y10CbCr8 (numbers representing the bit-depth for each channel) are then converted back to linear RGB, as shown in Fig. 1, by inverting the steps formerly taken to construct the YCbCr signals.

Using an error bar, Figs. 2 (a), (b) and (c) show the color differences between quantized YCbCr and the original linear RGB colors in terms of DE2000 at selected sampled luminance levels and bit-depths. It is observed that the color differences due to quantization are mainly visible around the white point as the chroma bit-depths are reduced, while saturated colors at the border of the gamut are still somewhat intact at 8 bits per color channel in terms of DE2000 (See Fig. 2 (c)). This implies that for an accurate color representation, colors around white point require more code-words than the ones further away. In contrast, saturated colors can be quantized with fewer bits and still be visually similar to the original color. This suggests a non-linear behavior in chroma channels in terms of required code-words.

Fig. 1. Workflow for calculating color errors in terms of DE2000 due to YCbCr quantization on BT.2020 sampled colors

Since saturated colors are the ones with Cb and Cr at the beginning and end of the supported [-0.5 to +0.5] range, the non-linear behavior of code-words required to encode chroma channels can be modeled with a simple sigmoid function:



The offset of 0.5 is added to maintain the chroma range at 
[-0.5 to +0.5]. Cr is also transferred by the same function as in (1). The transferred Cb* and Cr* channels can then be quantized using limited 10-bit code-words. Using the function in (1), most of the available code-words are allocated to colors around the white point with Cb and Cr values between [-0.2 and 0.2]. Fig. 2 (d) shows the color differences between YCb*Cr*, with chroma channels processed based on (1) and then quantized to 10 bits, and the original linear RGB colors in terms of DE2000.

Another observation from Figs. 2 (a), (b) and (c) is that the color errors are more concentrated along the vertical axis, from color yellow to blue. This range of colors corresponds to what the Cb channel represents. This observation implies a different behavior for Cr and Cb, indicating that a different code-words redistribution function may be needed for representing the two. However, in this work we focus on using a common transfer function for both and leave the study of deriving a specific function for each chroma channel for our future research.

# Results

Table I reports the reduction in number of pixels (in %) with visible color errors, as well as the average error over the entire frame comparing the proposed method to original 10-bit YCbCr, in 4:4:4 and 4:2:0 formats. As it is shown, the proposed method reduces the percentage of pixels with visible errors substantially in both 4:4:4 and 4:2:0 format, while also reducing the average error.

To evaluate how the proposed method affects the compression efficiency, we compressed the proposed 10-bit YCb*Cr* 4:2:0 signals. Videos were quantized to 10 bits using the BT.1361 limited range prior to the compression. The proposed 10-bit YCb*Cr* signals were compressed using HM 16.20 Main10 profile. The Chroma QP Offset was set to 0. QP values of 22, 27, 32, and 37 were used for all sequences. The IntraPeriod parameter was set to 32 for BalloonFestival and FireEater2, and 48 for Market3. Once the bitstreams are decoded, we convert them back to the linear RGB. HDRTools is used for objective evaluations.

Fig. 2. Color errors generated due to bit-depth quantization for (a) 10-bit YCbCr, (b) Y10CbCr9, (c) Y10CbCr8, and (d) the proposed Y10Cb*Cr*10 over the BT.2020 gamut using the error bars on the right. Numbers indicate the used bit-depth.

Table II provides the average bitrate saving/loss of the proposed 10-bit YCb*Cr* in terms of objective metrics using BD-rate, with 10-bit YCbCr as the anchor. As observed the proposed method outperforms the original YCbCr by an average of 50.2% in terms of DE100 over the three tested videos. Note that the luma adjustment was not applied on the anchor or the test videos. We expect that this method will further improve the compression efficiency of our method. In summary, our chroma processing scheme preserves colors through the quantization and subsampling processes much more accurately than the existing YCbCr approach.

# References

- G. W. Larson, “Logluv encoding for full-gamut high-dynamic range images”, Journal of Graphics Tools, vo. 3, no.1, pp. 15-31, 1998.

Table 1 Average DE2000 and percentage of pixels with DE2000 values greater than one (visible error) represented with the original YCbCr and the proposed YCb*Cr* with 4:4:4 and 4:2:0 chroma, compared to the originals (first frame)

| Sequence | Method | % of pixels with DE > 1 |  | Average DE of frame |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4:4:4 | 4:2:0 | 4:4:4 | 4:2:0 |
| SunRise | YCbCr10 | 62.38 | 88.94 | 1.80 | 5.26 |
|  | Proposed YCb*Cr*10 | 29.67 | 85.91 | 0.8128 | 4.86 |
| Market | YCbCr10 | 42.35 | 86.52 | 1.2062 | 6.07 |
|  | Proposed YCb*Cr*10 | 10.98 | 84.81 | 0.4792 | 5.8067 |
| BalloonFestival | YCbCr10 | 3.79 | 33.51 | 0.425 | 2.7661 |
|  | Proposed YCb*Cr*10 | 0.18 | 29.60 | 0.1819 | 2.6294 |

Table 2 Average Objective Results of Proposed 10-bit YCb*Cr* Compared to the Anchor of 10-bit YCbCr (HM 16.20)

| Sequence | Metrics |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPSNRY | wPSNRU | wPSNRV |
| SunRise | -75.99% | 22.88% | 23.68% | -37.81% | -22.05% |
| Market | -52.50% | 14.38% | 14.30% | -80.45% | 0.00% |
| BalloonFestival | -22.12% | 24.95% | 25.46% | -38.03% | -59.59% |
| Overall | -50.20% | 20.74% | 21.14% | -52.09% | -27.21% |

# Patent rights declaration(s)

University of British Columbia and TELUS Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TABLE II. Average Objective Results of Proposed 10-bit YCb*Cr* Compared to the Anchor of 10-bit YCbCr
(Negative Values Indicate an Improvement Over the Anchor, While Positive Values Indicate a Loss)

| Sequence | Metrics |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR XYZ |
| FireEater2 | -1.3% | 10.6% | -21.9% | -3.6% | -19.7% |
| Market3 | 10.4% | 13.5% | -6.1% | 4.9% | 0.0% |
| BalloonFestival | 6.3% | 18.6% | -23.5% | -3.0% | -20.3% |
| Overall | 5.2% | 14.2% | -17.1% | -0.6% | -13.3% |