| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B026 |
| --- | --- |

| Title: | Decoder-Side Motion Estimation with Modified Reference List |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sven Klomp Marco Munderloh Jörn Ostermann Institut für Informationsverarbeitung Appelstraße 9a 30167 Hannover Germany | Tel: Email: | +49-511-762-19590 klomp@tnt.uni-hannover.de |
| Source: | Leibniz Universität Hannover |  |  |

_____________________________

# Abstract

Recent research revealed that the data rate can be reduced by performing additional motion estimation at the decoder. As only already decoded data is used, no additional data has to be transmitted.

This document gives an update of the results presented in [1] on the performance of decoder-side motion estimation (DSME) with new test sequences. A dense motion vector field is estimated which reduces the rate by 6.9% in average compared to the H.264/AVC anchors at the same quality.

# Content

Abstract	1

Content	1

1	Introduction	2

2	DSME Architecture	2

2.1	Bidirectional true motion estimation	2

3	Performance Evaluation	3

3.1	Complexity	4

4	Conclusions	4

5	References	5

6	Patent rights declaration(s)	5

# Introduction

The goal of decoder-side motion estimation (DSME) is to overcome the drawbacks of conventional motion estimation, where large block sizes increase the prediction error and smaller block sizes increase the data rate for motion vectors, respectively.

# DSME Architecture

In DSME as proposed in [2], the current frame is predicted by performing motion estimation on adjacent frames at the decoder. This so called DSME frame is then fed into the reference picture buffer of a conventional coder to be used as an additional reference frame (Figure 1). The coder is now able to use the DSME frame as a reference for each macroblock. The conventional tools of the coder are now able to use the frame for prediction and coding in addition to the other reference frames. Since the interpolated frame is already motion compensated, the motion vector of the conventional motion estimation block is always zero, if the DSME frame is used for predicting a block.

Figure 1: DSME architecture

Due to this simple and flexible architecture, the motion estimation algorithm can be easily exchanged. In the following section, an improved motion estimation algorithm is proposed, which allows better prediction of the current frame.

# Performance Evaluation

The performance evaluations are based on class A and B of the test set proposed in the Call for Proposals [3]. The decoder-side motion estimation algorithms were implemented in the JM software version 16.2 [4] and the same configuration parameters as for the alpha anchors were used. Bjontegaard Delta [5] is used to evaluate the performance.

## Class A

^

| Sequence | BD-PSNR [dB] | BD-Rate [%] |
| --- | --- | --- |
| PeopleOnStreet | 0.59 | -10.03 |
| Traffic | 0.34 | -8.56 |

## Class B

| Sequence | BD-PSNR [dB] | BD-Rate [%] |
| --- | --- | --- |
| BasketballDrive | 0.19 | -5,25 |
| BQTerrace | 0.08 | -4.13 |
| Cactus | 0.21 | -6.41 |
| Kimono | 0.26 | -6.57 |
| ParkScene | 0.29 | -7.48 |

## Complexity

The proposed architecture introduces additional computational efforts at the encoder and also at the decoder. Until now, almost no particular efforts were made to optimise the algorithms complexity and the current decoder won’t work in real-time. However, similar motion estimation algorithms are used for temporal upscaling in nowadays flat screen TVs with 100 and 200 Hz techniques which may be combined with DSME.

# Conclusions

The advantages of decoder-side motion estimation are investigated and compared to the common H.264/AVC coding, in which the encoder performs all motion estimation tasks solely. For the Class A sequences, the rate was reduced by up to 10%. The gain for Class B with 7% is slightly lower.

# References

[1]	S. Klomp, J. Ostermann, “Decoder-side Motion Estimation for Improved Prediction,” ISO/IEC JTC1/SC29/WG11, MPEG Meeting, London, UK, June 2009, Doc. M16570.

[2]	S. Klomp, M. Munderloh, Y. Vatis, and J. Ostermann, “Decoder-Side Block Motion Estimation for H.264 / MPEG-4 AVC Based Video Coding”, IEEE International Symposium on Circuits and Systems, pp. 1641-1644, Taipei, Taiwan, May 2009.

[3]	ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11, “Joint Call for Proposals on Video Compression Technology”, N11113, January 2010, Kyoto, JP.

[4]	H.264/AVC Reference Software, http://iphome.hhi.de/suehring/tml/download/ version 16.2, 12th November 2009.

[5]	G. Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, 16th - 18th July, 2008, Doc.VCEG-AI11.

# Patent rights declaration(s)

Leibniz Universität Hannover may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).