| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B032 |
| --- | --- |

| Title: | TE1: Refinement Motion Compensation using Decoder-side Motion Estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Motoharu Ueda JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan Shigeru Fukushima JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: Tel: Email: | +81-46-836-9945 ueda.motoharu@jk-holdings.com +81-46-836-6597 fukushima.shigeru@jk-holdings.com |
| Source: | Victor Company of Japan, Limited (JVC) |  |  |

_____________________________

# Abstract

In TE1 activity, Decoder-side Motion Vector Derivation (DMVD) tool has been evaluated for improving coding efficiency for HEVC.

This proposal focuses on the tool “Refinement Motion Compensation using Decoder-side Motion Estimation (RMC)”. RMC was proposed in the previous proposal JCTVC-A108. The RMC tool aims to improve the quality of MC block in motion compensation process using DMVD process between reference pictures.

The RMC tool has been ported into TE1 codebase based on KTA2.6r1 because the first implementation was based on JM16.2 software. By the porting, the RMC tool has been available in TE1 activity.

The simulation results show that the proposed technique provides the BD-bitrate savings of average 1.965% up to 3.143% for CS1 and 1.757% (2.078%, without class A case) up to 11.503% for CS2 under TE1 common condition.

# Introduction

Motion compensation process can improve coding efficiency using temporal correlation between a target picture and a decoded reference pictures, but the process is more susceptible to coding distortion of a reference picture. Coding distortion of reference pictures such as infecting unneeded component and the missing textures brings degradation of prediction efficiency. For HEVC standardization, new approach is needed to improve these situations.

This technique named RMC (Refinement Motion Compensation using decoder-side motion estimation) aims to improve the quality of a MC block of motion compensation using decoder-side motion estimation between reference pictures.

The advantage in RMC is that the additional coding bits can be reduced because multiple MC blocks can be obtained by transmitting only one motion vector. Moreover, in the decoder side, since full-pixel motion vector is used for the template block of inter-reference motion estimation, the inter-reference motion estimation can be executed before generating the first MC block for first reference picture on sub-pixel accuracy.

In this document, the algorithm description of RMC is explained in section 2. The simulation results are shown in section 3. The conclusions of this work are mentioned in section 4.

# Algorithm Description

## Overview

Figure 1 Conceptual diagram of RMC

Figure 2 RMC in the case of P Slice

The conceptual diagram of RMC is shown in Figure 1.

In this technique, a first MC block is obtained from reference picture 1. A second MC block is obtained by executing inter-reference motion estimation. Finally, a combined MC block is generated from the first MC block and the second MC block.

RMC can smooth the distortion and supplement the partially missing textures to the reference picture like bi-predictive prediction. RMC needs only one motion vector for transmitting, therefore RMC can reduce additional information compared with bi-predictive prediction.

RMC is applicable to not only B slice but also P Slice. In the case of B Slice, reference MC block can lead other past and future reference pictures as shown Figure 1. In the case of P Slice, past reference MC block leads other past reference pictures as shown Figure 2.

Uni-predictive mode is replaced by RMC mode per prediction block adaptive. The information of the RMC applicable mode is sent in slice header. The flag of enabling RMC is sent per prediction block.

In decode-side motion vector derivation approach, the feature of RMC is as follows.

- Send one motion vector, and DMVD gets other motion vector for using bi-predictive prediction.

- Use full-pixel accuracy reference block as template of DMVD.

## Procedure

### Decoding procedure

The decoding steps of RMC are shown in Figure 3.

Figure 3  Decoding procedure of RMC

At first, template block of DMVD is set using the transmitted motion vector rounded to integer for reference picture 1.

Then, reference picture 1 is indicated by ref_idx transmitted in the bitstream.

In the case of P slice, reference picture 2 is indicated by ref_idx that is set to zero

In the case of B slice, reference picture 2 is indicated by ref_idx that is set to zero in another list.

Next, inter-reference motion estimation between reference picture 1 and reference picture 2 is performed using a template block obtained by the motion vector rounded to integer (See the upper part in Figure 4), where any motion estimation algorithm can be applied for inter-reference motion estimation as long as the same technique is used in both of encoder side and decoder side.

In this proposal, the motion vector rounded to integer for reference picture 1 is scaled based on relative distance between reference picture 1 from current picture and reference picture 2 from current picture like temporal direct mode in AVC as shown in Figure 4.

Figure 4  Derivation process of the combined MC block

Centered on the position indicated by the scaled motion vector, block based full search motion estimation is executed on reference picture 2 by full-pixel and sub-pixel accuracy hierarchically.

The inter reference motion vector is obtained by the result of the inter reference motion estimation.

A first MC block is obtained from reference picture 1 by executing motion compensation using the transmitted motion vector as the conventional motion compensation technique.

On the other hand, a second MC block is obtained from reference picture 2 by executing motion compensation using the motion vector calculated by adding inter-reference motion vector to the transmitted motion.

Finally, a combined MC block is generated by the synthesis of the first MC block and the second MC block as shown in Figure 4.

Following, there is explanation of decoding procedure about relation between DMVD process of RMC and final MC block.

The derivation of a template block for DMVD process in RMC is shown in Figure 5.

The transmitted motion vector for reference picture 1 is rounded to integer accuracy, and template block is extracted using motion vector rounded to integer accuracy from reference picture 1. In this process, sub-pixel filtering is not needed. Full-pixel accuracy template block represent the feature of area shown by the transmitted motion vector.

Figure 5  Derivation of a template block and a template area

Next, inter-reference motion estimation between template block and reference picture 2 is performed with full-pixel accuracy as shown in Figure 6, and get full-pixel inter-reference motion vector.

Figure 6  Example of inter-reference motion estimation (full-pixel accuracy)

Next, inter-reference motion estimation with sub-pixel motion accuracy between template block and reference picture 2 is performed around the position of full-pixel inter-reference motion vector as shown in Figure 7.

Inter-reference motion vector means relative position between reference pictures.

It is possible to execute inter-reference motion estimation independently of motion compensation by transmitted motion vector.

Figure 7  Example of inter-reference motion estimation (sub-pixel accuracy)

In the motion compensation stage, a final MC block is generated as shown in Figure 8.

A first MC block is obtained from reference picture 1 by executing motion compensation using the transmitted motion vector.

Since difference between transmitted motion vector and rounded transmitted motion vector means phase shift vector between template block and MC block, a second MC block is obtained from reference picture 2 by adding the phase shift vector to inter-reference motion vector.

A final MC block is generated by the combination of the first MC block and the second MC block like bi-predictive prediction.

Figure 8  Derivation of a final MC block by using phase shift MV

On the other aspect, RMC tries to match the position between reference picture 1 and reference picture 2 using DMVD approach and synthesizes them to make new MC block.

### Encoding procedure

The procedures on encoder side are as follows.

At first, motion estimation on full-pixel accuracy is executed.

Although any motion estimation algorithm can be applied, EPZS algorithm implemented into KTA is used in order to match the condition to the anchor.

Next, using the MC block obtained by motion estimation on full-pixel accuracy, inter-reference motion estimation is executed in the same procedures as the decoder side.

Finally, motion estimation on sub-pixel accuracy is executed within the range in which the full-pixel motion vector for inter-reference motion estimation does not change when the decided motion vector is rounded.

The combined MC block generated by combining the first MC block and the second MC block is used to evaluate the final motion vector on sub-pixel accuracy.

This final motion vector for reference picture 1 is only transmitted to the decoder.

Inter-reference motion vector is not needed to transmit since it can be derived at the decoder side.

## Implementation

### Syntax structure

The information of the RMC applicable mode is sent in slice header.

The flag of enabling RMC is sent per prediction block just after reference index except in the case that reference index is set to zero in P-picture or implicit setting of reference picture 2 in the case of RMC is same as reference picture 1. Implicit setting of reference picture 2 is described later.

### Prediction Mode

The prediction mode that can use RMC in this software is shown in Table 1.

Table 1  Specification of the prediction mode in RMC

### Implicit setting of reference picture 2

In the case of RMC, reference picture 2 is indicated implicitly as follows. (See Figure 9)

P Slice's case        reference picture 1 (base)    reference picture 2

Idx0                 -> Not used

Other                -> Idx0

B Slice's case        reference picture 1 (base)    reference picture 2

LIST0                -> LIST1 Idx0

LIST1                -> LIST0 Idx0

Figure 9  Relation of reference picture 1 and reference picture 2 on RMC

### Specification of decoder-side motion estimation

Search range for full-pixel ME:			±3 pixel

Search range for sub-pixel ME:			±3/4 pixel

Sub-pixel ME accuracy:				1/4 pixel

Sub-pixel filtering:					6-tap / 2-tap filter (same as AVC)

Matching technique:					SAD (luminance only)

Decoder-side motion estimation block size:		Same as MC block size

# Simulation Results

## BD-RD results

The results of experiment for the implemented RMC tool are as follows. The anchor settings are the common anchor conditions under the TE1 activity. (The maximum MB size is 32x32)

Table 2  BD-PSNR and BD-bitrate: CS1 under the TE1 common conditions

Table 3  BD-PSNR and BD-bitrate: CS2 under the TE1 common conditions

## Complexity results

The results of complexity assessment for the implemented RMC tool are as follows.

The results are the ratio between the RMC’s execution time and anchor’s execution time in the following simulation environments.

- CPU:		Core i7 860 2.8GHz

- Memory:	DDR3 8GB

- OS:			Windows 7 Professional 64bit

- Compiler:	Microsoft Visual C++ 2008 Express edition sp1

- Executable :	Win32

Table 4  Complexity of encoding and decoding time ratio: CS1 under the TE1 common conditions

Table 5  Complexity of encoding and decoding time ratio: CS2 under the TE1 common conditions

# Conclusions

This proposal shows that RMC can improve coding efficiency under TE1 common condition although RMC is adapted only for uni-directional prediction (simple prediction mode).

It will be improved by supporting other prediction mode such as direct, skip, bi-predictive prediction, and also prediction structure on HEVC test model.

We would try to show the effectiveness of our DMVD tools through the TE1 activity for HEVC standardization.

# References

[1] “Description of video coding technology proposal by JVC”,

JCTVC-A108, Dresden, April 2010.

[2] “Summary Report for TE1 on DMVD”,

JCTVC-B029, Geneva, July 2010.

[3] KTA Reference Software, http://iphome.hhi.de/suehring/tml/download/KTA/.

[4] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).