| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B038 |
| --- | --- |

| Title: | Performance Improved Directional Transform Techniques for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Wang, Jin Song, Haitao Yang, Mingyuan Yang, Haoping Yu, Huawei Technologies Co., Ltd. Huawei Building, 3 Xinxi Road, Haidian District, Beijing, China. Xin Zhao, Siwei Ma Institute of Digital Media, Peking University Room 2641, No. 2 Science Building, No. 5 Yiheyuan Road, Haidian District, Beijing, China | Tel: Email: | +86 10 82882517 wangdong1210@huawei.com haopingyu@huawei.com zhaoforever@gmail.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the two updated techniques on directional transform since the last meeting. The first one is the directional multiple transform (DMT) for inter residual blocks. Based on the direction information of the residual signal, multiple transforms are used, which are trained offline, in order to match the texture feature of the residues. With improved temporal predictive coding and syntax embedding implemented, the performance gain is further improved. The other technique is the rate distortion optimized transform (RDOT) for intra residual blocks, as described in detail in [2]. The transforms and the coefficient scanning order are further optimized and the performance gain is increased, under the discussed Transform AhG test condition. Moreover, since the selection of the transform used on one block is decided at encoder side, it does not introduce extra computation complexity at decoder side.

# Directional Multiple Transform

## Review of the Technique

The DMT was included in the proposal for CfP in the Dresden meeting [3]. Here it is reviewed briefly.

It is observed that there exists structured texture information in the residue blocks. And the DCT transform, which is used in the current H.264/AVC standard, cannot deal perfectly with these blocks containing various textures. Based on this consideration, DMT introduces multiple transforms based on the directional texture of the residual block. The residual blocks are categorized according to the direction of the textures, and the K-L transforms are then trained for each category. These K-L transforms and the DCT transform are evaluated together through RDO process for each block with residue available, as shown in figure 1. Currently 8 transforms are used for 8 directions accordingly. The following section 1.2 and 1.3 describe the updated techniques since the last meeting.

Figure 1. Encoding flow with DMT

## Temporal predictive coding

It is observed that the selected direction through RDO process is closely correlated with the texture information of the prediction block in the reference picture, calculated according to the motion vector.

The texture direction of the prediction block is derived based on the block gradient along all directions. The strongest gradient among them represents the texture direction of the prediction block.

After the RDO process, the side information indicating the transform selection is transmitted in the bitstream, before writing residual coefficients. Two types of side information are coded in the bitstream as described below. In the table 1, the related syntax in the bitstream is described.

- One bit flag “directional_transform_flag” to indicate the use of directional transform or DCT transform. It should be noted that this syntax is not transmitted through bitstream and will be embedded into the transform coefficients, as described in the next section.

- If direction directional transform is used, transform index to indicate which directional transform is used. To represent this information, the straightforward coding is expensive. For example, in case of using 8 KLT transforms, 3 bits are needed as the overhead. Due to the correlation described as above, the predictive coding is applied. The prediction is the direction calculated according to the predictor block and the surrounding blocks, and the actual syntax encoded is the difference between the current selection and this prediction. Through the experiments, it is observed that most of the syntax values encoded are 0 or 1, resulting much lower overhead with CABC entropy coding.

Table 1. Syntax elements for transform index.

……

if( CodedBlockPatternOfCurrentBlock){

if(directional_transform_flag)

transform_index

}

……

## Data Embedding

The syntax KLT_transform_flag is embedded in the luma residual coefficients in the implementation, in order to further reduce the overhead. The decoder obtains the transform selection information between inverse DCT/KLT by judging the parity of the sum of the values of the transform coefficient in the current block. Even parity means the inverse DCT is applied. The odd parity is for the inverse KLT.

At the encoder side, if the transform coefficients do not match the requirement, one coefficient will be modified (plus or minus 1) in order to make it correct.

## Experimental Results

The proposed technique is implemented into the KTA software with version 2.6r1. Currently it is only applied on the 8x8 blocks, and implementation on other block sizes will be done soon later. The anchor of the test condition is based on the VCEG-AJ10r1. Since the new coding tools in the KTA software are subject to the evaluation in the TE or through other means, they are not switched on in the experiment.

In terms of the large MB, it is switched off because the non-AVC transforms such as 16x16 transform are used. Such AVC-beyond transform techniques as well as the usage of these transforms in various coding mode are the study subjects of the transform AHG. And at this moment, only AVC transform techniques exist in the anchor configuration, with UseExtMB=0.

Two sets of test conditions are used: CS1 and CS2, according to the test conditions defined for CfP, in the document N11113 in Jan 2010 meeting. The QPs used for I slices (QP_I) are 22, 27, 32, 37, including wide bitrate range. QPs of P slices are QP_I+1. QPs of the first B layer are QP_I+2. QPs of the second B layer are QP_I+3. QPs of the third B layer are QP_I+4.

# Rate-Distortion Optimized Transform

## Algorithm description

The proposed rate-distortion optimized transform (RDOT) is an improved technique on the basis of the mode dependant directional transform (MDDT) technique [6]. Because it is observed that the residual usually present different statistical characteristics even for the same intra prediction (IP) mode, RDOT was invented to deal with these diversified residual blocks. RDOT uses several different transforms to the residual obtained by each IP mode, and select the best transform via rate-distortion optimization.

RDOT is applied to all IP modes in AVC standard. A flag is put in the MB header to indicate if RDOT is used. In case RDOT is used, additional bits are used to specify the index of selected transform matrix for both column and row transform. For more details, please refer to [2][3].

## Experimental results

# Conclusion

# References

- T. Wiegand, G. J. Sullivan, G. Bjøntegaard, A. Luthra, “Overview of the H.264/AVC video coding standard,” IEEE Trans. Circuits Syst. Video Technol., vol.13, no. 7, pp.560-576, Jul. 2003.

- Xin Zhao, Li Zhang, Siwei Ma, Wen Gao, “Rate-Distortion Optimized Transform”, ISO/IEC JTC1/SC29/WG11 MPEG, m16926, Xi’an, China, October 2009.

- H. Yang, J. Fu, S. Lin, etc., “Description of video coding technology proposal by Huawei Technologies & Hisilicon Technologies”, JCTVC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-A111, Dresden, DE, 15-23 April, 2010.

- TK Tan, G. Sullivan, and T. Wedi, “Recommended Simulation Common Conditions for Coding Efficiency Experiments Revision 4,” ITU-T Q.6/SG16 VCEG, VCEG-AJ10r1, San Diego, USA, July 2008.

- “Joint Call for Proposals on Video Compression Technology”, ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, W11113, January 2010.

- Y. Ye and M. Karczewicz, “Improved H.264 intra coding based on bi-directional intra prediction, directional transform, and adaptive coefficient scanning,” Proceedings of the 2008 IEEE International Conference on Image Processing (ICIP), pp. 2116-2119, Oct. 2008.

# Patent rights declaration(s)

Huawei Technologies may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).