| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B038 |
| --- | --- |

| Title: | Performance Improved Directional Transform Techniques for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Wang, Jin Song, Haitao Yang, Mingyuan Yang, Haoping Yu, Huawei Technologies Co., Ltd. Huawei Building, 3 Xinxi Road, Haidian District, Beijing, China. Xin Zhao, Siwei Ma Institute of Digital Media, Peking University Room 2641, No. 2 Science Building, No. 5 Yiheyuan Road, Haidian District, Beijing, China | Tel: Email: | +86 10 82882517 wangdong1210@huawei.com haopingyu@huawei.com zhaoforever@gmail.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the two updated techniques on directional transform since the last meeting. The first one is the directional multiple transform (DMT) for inter residual blocks. Based on the direction information of the residual signal, multiple transforms are used to match the texture feature of the residues. The syntax coding is optimized by using temporal predictive coding. The other technique is the rate distortion optimized transform (RDOT) for intra residual blocks, as described in detail in [2].

# Directional Multiple Transform

## Overview of the Technique

The DMT was included in the proposal for CfP in the Dresden meeting [3]. It is reviewed briefly in this section, and the following section 1.2 describes the updated technique since the last meeting.

It is observed that there exists structured texture information in the residual blocks. And the DCT transform, which is used in the current H.264/AVC standard, cannot deal perfectly with these blocks containing various textures. Based on this consideration, DMT introduces multiple transforms based on the directional texture of the residual block. The residual blocks are categorized according to the direction of the textures, and the K-L transforms are then trained for each category. These K-L transforms and the DCT transform are evaluated together through RDO process for the blocks with residue available, as shown in the Figure 1. Currently 8 transforms are used for 8 directions accordingly, and the technique is only applied on the 8x8 transform coefficients.

Figure 1. Encoding flow with DMT

## Temporal predictive coding

It is observed that the selected direction through RDO process is closely correlated with the texture information of the prediction block in the reference picture, calculated according to the motion vector.

The texture direction of the prediction block is derived based on the block gradient along all directions. The strongest gradient among them represents the vertical direction to the texture direction of the prediction block.

After the RDO process, the side information indicating the transform selection is transmitted in the bitstream, before writing residual coefficients. Two types of side information are needed as described below. Table 1 shows the related syntax in the bitstream, which is transmitted just before writing the 8x8 transform coefficients.

- One bit flag “directional_transform_flag” to indicate the use of directional transform or DCT transform.

- Transform index to indicate which directional transform is used, if directional transform is used. To represent this information, the straightforward coding is expensive. For example, in case of using 8 directional transforms, 3 bits are needed as the overhead. Based on the correlation described as above, the predictive coding is applied. The prediction of the direction is calculated according to the predictor block, and the actual syntax encoded is the difference between the current selected direction and the direction of the predictor block. Through the experiments, it is observed that most of the syntax values encoded are 0 or 1, resulting much lower overhead with CABC entropy coding.

Table 1. Syntax elements for transform index.

……

if( CodedBlockPatternOfCurrentBlock){

directional_transform_flag

if(directional_transform_flag)

transform_index

}

……

## Experimental Results

The proposed technique is implemented into the KTA software with version 2.6r1. Currently it is only applied on the 8x8 transform blocks, and implementation on other block sizes will be done soon later. The anchor of the test condition is based on the VCEG-AJ10r1. Since the new coding tools in the KTA software are subject to the evaluation in the TE or through other means, they are not switched on in the experiment.

In terms of the large MB, it is switched off because the non-AVC transforms such as 16x16 transform are used. Such AVC-beyond transform techniques as well as the usage of these transforms in various coding mode are the study subjects of the transform AHG. And at this moment, only AVC transform techniques exist in the anchor configuration, with UseExtMB=0.

Two sets of test conditions are used: CS1 and CS2, according to the test conditions defined by CfP, in the document N11113 in Jan 2010 meeting. The QPs used for I slices (QP_I) are 22, 27, 32, 37, including wide bitrate range. QPs of P slices are QP_I+1. QPs of the first B layer are QP_I+2. QPs of the second B layer are QP_I+3. QPs of the third B layer are QP_I+4.

Table 2 shows the performance of DMT. The evaluation criteria is based on BD-Rate and BD-PSNR, as described in [4]. From the table, it can be seen that the performance for CS2 and CS1 test condition is about -2.5% and -1.2%, respectively. Also the performance for 2560x1600 sequences is worse than other sequences. The analysis and possible further optimizations are described as below.

- The current implementation for B frame is the main reason to lower the performance under the CS1 test condition. Currently the DMT implementation is the same as that in P frame. However, since there are much less residuals in B frame compared with the P frame, it needs to be further optimized in order to reduce overhead cost.

- The results for 2560x1600 sequences are not that good, and the performance lowers the average performance of the DMT technique. The main reason is that the residuals are less that of the sequences with smaller sizes, and hence the better 8x8 transforms are not that useful compared with smaller sequences. In the future implementations, if larger transforms are used, it is expected to bring better performance with the DMT technique.

It should be noted that the current implementation is only applied on 8x8 transform blocks. The DMT for other transform sizes need to be implemented, especially when large block sizes are used.

In conclusion, the current implementation of the DMT technique brings the performance gain under the test condition discussed within the AhG. With further optimization described as above, it is expected to bring more performance gain, compared to the figures shown in this document.

Table 2. RD performance of DMT

|  |  | CS1 |  | CS2 |  |
| --- | --- | --- | --- | --- | --- |
| Sequence | Resolution | BD-RATE | BD-PSNR | BD-RATE | BD-PSNR |
| PeopleOnStreet | 2560x1600 | -0.789 | 0.031 | -2.193 | 0.078 |
| Traffic | 2560x1600 | -0.326 | 0.015 | -0.948 | 0.044 |
|  | Average: | -0.558 | 0.023 | -1.571 | 0.061 |
| Kimono | 1920x1080 | -0.011 | 0.000 | -0.732 | 0.028 |
| ParkScene | 1920x1080 | -0.388 | 0.014 | -1.129 | 0.040 |
| Cactus | 1920x1080 | -1.179 | 0.028 | -2.651 | 0.062 |
| BasketballDrive | 1920x1080 | -1.279 | 0.033 | -2.701 | 0.069 |
| BQTerrace | 1920x1080 | -2.443 | 0.055 | -4.285 | 0.114 |
|  | Average: | -1.060 | 0.026 | -2.300 | 0.062 |
| Basketball Drill | 832x480 | -2.783 | 0.116 | -5.515 | 0.220 |
| BQMall | 832x480 | -1.489 | 0.065 | -2.706 | 0.118 |
| Party Scene | 832x480 | -1.711 | 0.078 | -2.125 | 0.104 |
| Race Horses | 832x480 | -1.176 | 0.049 | -2.164 | 0.098 |
|  | Average: | -1.790 | 0.077 | -3.128 | 0.135 |
| BasketballPass | 416x240 | -0.946 | 0.047 | -2.375 | 0.119 |
| BQSquare | 416x240 | -0.537 | 0.023 | -0.908 | 0.039 |
| BlowingBubbles | 416x240 | -1.856 | 0.074 | -3.615 | 0.149 |
| Race Horses | 416x240 | -1.111 | 0.054 | -1.921 | 0.098 |
|  | Average: | -1.113 | 0.050 | -2.205 | 0.101 |
| Vidyo1 | 1280x720 |  |  | -2.081 | 0.057 |
| Vidyo3 | 1280x720 |  |  | -3.048 | 0.105 |
| Vidyo4 | 1280x720 |  |  | -1.690 | 0.052 |
|  | Average: |  |  | -2.273 | 0.071 |
|  | Average of all sequences: | -1.202 | 0.045 | -2.377 | 0.088 |

## Complexity Analysis

At the encoder side, the multiple transforms need to be evaluated and it introduces extra complexity. Through the experiment, the average complexity overhead in terms of encoding time is 50%. However, since there are one or more redundant RDO processes in the current codec, in order to ease the software implementation, this figure is not correct and could be easily optimized further.

As the decoder could know the transform selection, it does not introduce multiple transform computations at the decoder side. The only extra complexity is the calculation of the direction of the predictor block. The average complexity overhead in terms of decoding time is 14%. Again, it can be optimized as well, since fetching predictor block is done twice in the current decoder.

# Rate-Distortion Optimized Transform

## Algorithm description

The proposed rate-distortion optimized transform (RDOT) is an improved technique on the basis of the mode dependant directional transform (MDDT) technique [6]. Because it is observed that the residual usually present different statistical characteristics even for the same intra prediction (IP) mode, RDOT was invented to deal with these diversified residual blocks. At the encoder side, RDOT applies several different transforms to the residual obtained by intra prediction, and selects the best transform via rate-distortion optimization. To ease the computational burden of the encoder to choose the best transform, an early termination mechanism is introduced into the RDO selection process.

RDOT is combined with all IP modes in AVC standard. A flag is put in the MB header to indicate if RDOT is used. In case RDOT is used, additional bits are used to specify the index of selected transform matrix for both column and row transform. For more details, please refer to [2][3].

## Experimental results

Table 3. Coding performance in terms of BD_Rate and BD_PSNR, and computational complexity in terms of encoding and decoding time (the ratio compared to the anchor is presented)

| Sequence name | Coding performance |  |  |  | Computational complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All-Intra |  | CS1 |  | All-Intra |  | CS1 |  |
|  | BD_Rate | BD_PSNR | BD_Rate | BD_PSNR | Encoding | Decoding | Encoding | Decoding |
| Traffic | -8.44 | 0.481 | -3.75 | 0.140 | 4.28 | 1.28 | 1.11 | 1.07 |
| PeopleOnStreet | -9.08 | 0.557 | -2.89 | 0.134 | 4.23 | 1.25 | 1.15 | 1.08 |
| Class A avg | -8.76 | 0.519 | -3.32 | 0.137 | 4.25 | 1.26 | 1.13 | 1.08 |
| Kimono1 | -7.83 | 0.294 | -2.11 | 0.075 | 4.46 | 1.33 | 1.03 | 1.07 |
| ParkScene | -7.29 | 0.353 | -1.91 | 0.069 | 5.74 | 1.27 | 1.04 | 1.06 |
| Cactus | -7.80 | 0.329 | -4.09 | 0.097 | 5.23 | 1.29 | 1.09 | 1.09 |
| BasketballDrive | -7.03 | 0.226 | -4.26 | 0.110 | 4.31 | 1.31 | 1.14 | 1.08 |
| BQTerrace | -6.55 | 0.407 | -3.45 | 0.077 | 5.22 | 1.25 | 1.09 | 1.06 |
| Class B avg | -7.30 | 0.322 | -3.16 | 0.086 | 4.99 | 1.29 | 1.08 | 1.07 |
| BasketballDrill | -6.86 | 0.357 | -4.04 | 0.169 | 4.15 | 1.19 | 0.97 | 1.07 |
| BQMall | -8.19 | 0.516 | -3.55 | 0.154 | 4.16 | 1.24 | 1.01 | 1.06 |
| PartyScene | -6.45 | 0.538 | -2.75 | 0.126 | 3.48 | 1.20 | 1.59 | 1.07 |
| RaceHorses | -5.41 | 0.400 | -2.08 | 0.088 | 4.98 | 1.25 | 1.69 | 1.08 |
| Class C avg | -6.73 | 0.453 | -3.10 | 0.134 | 4.19 | 1.22 | 1.32 | 1.07 |
| BasketballPass | -6.99 | 0.432 | -3.24 | 0.163 | 4.67 | 1.26 | 1.01 | 1.08 |
| BQSquare | -6.71 | 0.637 | -2.61 | 0.108 | 3.71 | 1.20 | 1.03 | 1.08 |
| BlowingBubbles | -6.54 | 0.431 | -2.47 | 0.099 | 5.51 | 1.20 | 0.99 | 1.10 |
| RaceHorses | -7.04 | 0.498 | -2.23 | 0.111 | 5.32 | 1.26 | 1.05 | 1.11 |
| Class D avg | -6.82 | 0.500 | -2.64 | 0.120 | 4.80 | 1.23 | 1.02 | 1.09 |
| vidyo1 | -9.70 | 0.525 | -6.46 | 0.215 | 3.43 | 1.30 | 1.13 | 1.06 |
| vidyo3 | -10.44 | 0.643 | -4.10 | 0.145 | 3.71 | 1.29 | 1.22 | 1.04 |
| vidyo4 | -8.82 | 0.449 | -3.63 | 0.112 | 4.49 | 1.30 | 1.06 | 1.05 |
| Class E avg | -9.65 | 0.539 | -4.73 | 0.157 | 3.88 | 1.29 | 1.14 | 1.05 |
| All class avg | -7.62 | 0.449 | -3.03 | 0.115 | 4.50 | 1.26 | 1.13 | 1.07 |

As illustrated in Table 3, the proposed RDOT shows better performance compared with MDDT. Under All-Intra condition, on average -7.62%/0.65dB BD-Rate/BD-PSNR gains is obtained. The encoding time is 4.5 times of that of Anchor, and the decoding time 1.26 times. Under the CS1 condition, the gain is -3.03%/0.12dB. The encoding time is 1.13 times of that of Anchor, and decoding time 1.07 times.

We can see that the proposed RDOT can achieve good coding gains while not increasing the complexity too much under the CS1 test condition. Of course the comparison of encoding and decoding time itself is not accurate, but we can get a rough idea of the computational complexity.

# References

- T. Wiegand, G. J. Sullivan, G. Bjøntegaard, A. Luthra, “Overview of the H.264/AVC video coding standard,” IEEE Trans. Circuits Syst. Video Technol., vol.13, no. 7, pp.560-576, Jul. 2003.

- Xin Zhao, Li Zhang, Siwei Ma, Wen Gao, “Rate-Distortion Optimized Transform”, ISO/IEC JTC1/SC29/WG11 MPEG, m16926, Xi’an, China, October 2009.

- H. Yang, J. Fu, S. Lin, etc., “Description of video coding technology proposal by Huawei Technologies & Hisilicon Technologies”, JCTVC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-A111, Dresden, DE, 15-23 April, 2010.

- TK Tan, G. Sullivan, and T. Wedi, “Recommended Simulation Common Conditions for Coding Efficiency Experiments Revision 4,” ITU-T Q.6/SG16 VCEG, VCEG-AJ10r1, San Diego, USA, July 2008.

- “Joint Call for Proposals on Video Compression Technology”, ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, W11113, January 2010.

- Y. Ye and M. Karczewicz, “Improved H.264 intra coding based on bi-directional intra prediction, directional transform, and adaptive coefficient scanning,” Proceedings of the 2008 IEEE International Conference on Image Processing (ICIP), pp. 2116-2119, Oct. 2008.

# Patent rights declaration(s)

Huawei Technologies may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).